3GPP TSG-RAN WG3 Meeting #125	R3-244334

Maastricht, Netherlands, Aug. 19th – Aug. 23rd, 2024

Agenda Item:	12.2

Source: 	Canon Research Centre France

Title:	Discussion on enhancements for WAB

Document for:	Discussion

# Introduction

In this paper, we discuss some aspects related to Wireless Access Backhaul (WAB), based on the draft TR 38.799 on WAB [2], and on the following agreements/notes from the previous RAN3 meeting [1]:

- Whether a BH RAN node can be unaware of WAB needs to be further studied. - It is desirable if a WAB unaware BH RAN node could be supported so long as performance is within acceptable limits and needed functionalities can be supported. - Chairman suggests that the work on functionalities is carried on first before it is decided whether a BH RAN node may or may not be WAB-aware. - The NG connection(s) of a WAB-gNB whose authorization status changes from “authorized” to “not authorized” can be removed. It needs to be further studied whether they can be suspended. - Establishment of Xn Connections of the WAB-gNB with BH-RAN nodes, as well as with surrounding RAN nodes, is supported and can follow legacy procedures. - Addition of new information in the legacy procedures is not precluded

# Discussion

## WAB unaware BH RAN nodes

A WAB node may benefit from connecting to a WAB-aware BH RAN node, especially in terms of resource management.

However, in order to ease up the adoption of WAB deployment in 5G systems, it is essential for a WAB node to serve UEs while the co-located WAB-MT is connected to a legacy BH RAN node, i.e., a BH RAN node which is not WAB-aware.

Indeed, despite the services offered by WAB-aware BH RAN nodes, such enhanced RAN nodes may not always be visible by the WAB-MT, for instance this will greatly depend on the trajectory of a mobile WAB node, which may be unpredictable.

Thus, when there is no visible WAB-aware BH RAN node, the WAB-MT may then be served by a legacy BH RAN node (i.e., a RAN node which is not WAB-aware). In this case, the legacy BH RAN node would serve the WAB MT as a legacy UE, while the WAB-MT would keep its already-established PDU sessions, and the co-located WAB-gNB would keep on serving UEs, hence preventing, or at least limiting, service interruptions at the served UEs. If the legacy BH RAN node is not able to accommodate the whole traffic associated to the served UEs, some minimum level of service may still be possible in most cases.

Proposal 1: A WAB-MT may be allowed to connect to either a WAB-aware BH RAN node or to a legacy BH RAN node.

Proposal 2: A WAB-gNB may be allowed to serve UEs when the BH RAN node serving the co-located WAB-MT is not WAB-aware.

In order to benefit from the above advantages when served by a WAB-aware BH RAN node, a WAB node should be capable of prioritizing such RAN nodes when performing cell selection, upon network integration or when performing connection reestablishment. Moreover, when performing the handover of a WAB-MT, a WAB-aware BH RAN node should preferably select a target BH RAN node that is also WAB-aware.

It is therefore essential for a WAB-aware RAN node to advertise its ability to support WAB-specific features and services.

In this respect, a WAB support indication may be broadcasted by the WAB-aware BH RAN node, for instance using SIB1 signalling, so that a WAB-MT can consider a cell where such WAB support indication is broadcast as a preferred candidate cell for further cell selection.

Proposal 3: A WAB-aware RAN node advertises a WAB support indication using for instance SIB1 signalling.

## Xn Connections

The establishment of Xn connections between a WAB-gNB and BH RAN nodes or between a WAB-gNB and surrounding RAN nodes is supported. It can follow legacy procedures, but addition of new information in these legacy procedures is not precluded.

For instance, to support coordination for resource multiplexing between a WAB-gNB and a (WAB-aware) BH RAN node, the co-location between the WAB-gNB and the WAB-MT should be known by the BH RAN node serving the WAB-MT. Also, the BH RAN node serving the WAB-MT of a WAB node should be known by the WAB-gNB co-located with the WAB-MT.

Thus, some Xn procedures, for instance the Xn setup procedure, may be enhanced to support the indication of the co-location of WAB-MT/WAB-gNB in a WAB-node and/or the identification of a BH RAN node serving a WAB-MT.

Moreover, the Xn setup procedure may be initiated either by the WAB-gNB or by the BH RAN node serving the WAB-MT.

Proposal 4a: Some Xn procedures, e.g., Xn setup procedure, may be enhanced to support the sharing of some WAB-MT/WAB-gNB co-location information.

Proposal 4b: Some Xn procedures, e.g., Xn setup procedure, may be enhanced to allow a WAB-gNB to identify the BH RAN node serving a WAB-MT.

Proposal 4c: The Xn setup procedure may be initiated either by the WAB-gNB or by the BH RAN node serving the WAB-MT.

Besides, the establishment of Xn connection between two WAB-gNBs is not precluded.

In the context of two WAB-gNBs on top of two different vehicles having completely different trajectories, the interest of establishing Xn connection between these WAB-gNBs would be limited.

However, when considering vehicles embedding several WAB nodes – e.g., a cruise ship or a train – establishing Xn connections between the WAB-gNBs bound to a same vehicle would become relevant. For instance, for the served UEs moving inside the vehicle, the handover from one WAB-gNB to another could be facilitated by having Xn connection between the source and target WAB-gNBs.

Proposal 5: The establishment of Xn connections between WAB-gNBs is supported.

## NG Connection Removal and Suspension

The NG connection(s) of a WAB-gNB whose authorization status changes from “authorized” to “not authorized” can be removed. It needs to be further studied whether NG connection(s) can be suspended.

When considering a WAB node having a “loop” trajectory, i.e., where it is expected that the WAB node moves along a predefined path, while coming back to its starting point on a periodic basis, the existing NG connection(s) with AMF(s) may be suspended, and resumed later. For some WAB nodes, there may be no strong constraints on hardware resources (as it is the case for satellites), and there would be no issue to keep in memory some parameters related to suspended NG connections.

To prevent from maintaining some suspended NG connections forever, e.g., if the trajectory of a WAB node is finally not as planned, some timer may be used to trigger the removal of suspended NG connections.

Proposal 6: In addition to NG connection removal, NG connection suspension and resuming are supported.

# Conclusion

In this contribution, we have discussed some WAB-related subjects about WAB unaware BH RAN node, Xn connection, NG connection removal and suspension. Finally, we have made the following proposals:

Proposal 1: A WAB-MT may be allowed to connect to either a WAB-aware BH RAN node or to a legacy BH RAN node.

Proposal 2: A WAB-gNB may be allowed to serve UEs when the BH RAN node serving the co-located WAB-MT is not WAB-aware.

Proposal 3: A WAB-aware RAN node advertises a WAB support indication using for instance SIB1 signalling.

Proposal 4a: Some Xn procedures, e.g., Xn setup procedure, may be enhanced to support the sharing of some WAB-MT/WAB-gNB co-location information.

Proposal 4b: Some Xn procedures, e.g., Xn setup procedure, may be enhanced to allow a WAB-gNB to identify the BH RAN node serving a WAB-MT.

Proposal 4c: The Xn setup procedure may be initiated either by the WAB-gNB or by the BH RAN node serving the WAB-MT.

Proposal 5: The establishment of Xn connections between WAB-gNBs is supported.

Proposal 6: In addition to NG connection removal, NG connection suspension and resuming are supported.

# References

[1] Chair notes at RAN3#124 meeting

[2] RP-241262 - TR 38.799 v1.0.0