3GPP TSG-RAN WG3 Meeting #125	R3- 244335

Maastricht, Nedherlands, 18 – 23 August, 2024

Agenda item:	10.2

Source:	Nokia

Title:	MRO Enhancements for LTM and CHO with Candidate SCG(s)

Document for:	Discussion

# 1	Introduction

This is a contribution related to MRO for LTM and CHO with candidate SCG(s) in Rel.19.

# 2	Discussion

## 2.1 MRO for LTM

### 2.1.1 Agreements achieved in RAN3 and RAN2

The agreements achieved in RAN3 #124 [1]:

- RAN3 to prioritize MCG LTM over SCG LTM.

- RAN3 takes RAN2#125-bis agreement on MRO for LTM scenarios as baseline for further study.

- For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization.

- For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) source DU performs optimization.

- RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM.

For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) FFS if CU or DU performs root cause analysis.

For stage-2, FFS if existing problem definitions (Connection failure due to intra-system mobility) can be reused or if a new chapter needs to be created.

The agreements achieved in RAN3 #123bis [2]:

- Work on scenarios of near failure LTM

- Work on scenarios for the differentiation of too early LTM, too late LTM and LTM to wrong cell

The agreements achieved in RAN2 #126 [3]:

- We correct the definition of scenario 3a as follows: Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure in a cell other than the source cell and the target cell.

- If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

- Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures.

- Log the LTM cell ID upon performing recovery an LTM candidate cell.

- Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

The agreements achieved in RAN2 #125bis [4]:

- RAN2 will start work on MCG LTM

- RAN2 considers SHR, RA report and RLF for MCG LTM SON

- For LTM MRO, RAN2 considers the following three connection failure cases:

-	Too late LTM

-	Too early LTM

-	LTM to wrong cell

For too late LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

-	Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and performs reestablishment procedure.

-	Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell, detects HOF with the selected LTM cell.

-	Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM execution with the selected LTM candidate cell.

For too early LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

-	Case 2a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

-	Case 2b: the UE detects HOF/RLF in the LTM target cell, selects the source cell which is also an LTM candidate cell, detects HOF with the source cell, and performs reestablishment procedure.

-	Case 2c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected source cell which is also an LTM candidate cell.

LTM to wrong cell, the following sub-cases are considered but we may down prioritize later (not limiting):

-	Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

-	Case 3b: the UE detects HOF/RLF in the LTM target cell, selects an LTM candidate cell which is different from the source or target one, detects HOF with the selected LTM candidate cell, and performs reestablishment procedure.

-	Case 3c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected LTM candidate cell which is different from the source or target one.

## 2.1.2 Prioritization of different objectives within the LTM MRO

During the RAN3 # 124 meeting the following was agreed:

RAN3 to prioritize MCG LTM over SCG LTM.

In the remaining part of the contribution, we therefore focus on MCG MRO both for connection and near failure.

### 2.1.3 Near Failure Monitoring for MCG LTM

With 3GPP Rel-17 for the intra-NR mobility and consequent extension for inter RAT mobility in Rel. 18, the MRO concept based on RLFs has been extended towards the logging of risky or failed but recovered mobility cases which finally resulted in success handover by providing a successful handover report (SHR). LTM intends to allow intra NR mobility based on L1/L2 measurements instead of legacy L3 measurements. The end user shall not perceive any difference from the perspective of dropped services between those two mobility principles. The MRO concept including near failure monitoring is applicable also for LTM. As the outcome of the RAN3 # 123 bis meeting the following was agreed:

Work on scenarios of near failure LTM

As defined in the chapter 5.7.10.6 of the TS 38.331 the UE only logs SHR, if SHR configuration is received previous to the handover, the handover is successfully executed (i.e. the procedure triggered with RRC Connection reconfiguration with synchronization is successfully completed) and at least one of the following criteria is satisfied:

- T310 timer of the UE exceeds the configured threshold by the source node,

- T312 timer of the UE exceeds the configured threshold by the source node,

- T304 timer of the UE exceeds the configured threshold by the target node.

Regarding the SHR triggers in principle all of them may apply also for LTM. There was discussion in RAN2 whether also T304 shall be used to supervision the LTM cell switch procedure. Based on the RAN2 # 123 meeting outcome it was agreed to use T304 also for LTM. The open point was whether dedicated T304 timer shall be used. The fact that LTM Cell switch shall primarily be triggered as RACH-less procedure indicates dedicated T304 timer and SHR threshold value shall be configured for LTM.

Proposal 1: RAN3 agrees to send LS to RAN2 to double check the T304 value and possible dedicated threshold value for LTM.

Main benefits of the LTM shall be seen in avoiding the complete reset of L3, avoiding RACH procedure which shall result into reduction of the interruption time. The interruption time is significantly impacted with RACH procedure. In case the TA of the candidate target LTM cell is not included into Cell Switch Command (MAC CE) the UE will initiate the legacy RACH procedure to access the target candidate cell. Despite the procedure can be monitored by means of T304 timer, but the quality in terms of having it in a RACH-less manner is not considered. However, it is essential to get information whether the cell change occurred RACH-less which is key for LTM.

On the other hand, in case the RACH-less cell change is triggered, i.e. MAC-CE msg contains TA, the first UL transmission is done in RLC AM. So, it may require several RLC retransmissions to get the transmission through.

Observation 1: The legacy SHR triggers seem to be incomplete for LTM.

Proposal 2: RAN3 agrees to define new SHR triggers specific to LTM near failure cases related to RACH based LTM and new threshold related to number of RLC retransmissions of the first UL transmission in RLC AM for RACH less LTM.

The UE is configured for several mobility methods and the SHR triggered by the legacy threshold cannot be assigned to the mobility method that triggered the risky cell change documented with this SHR. The network thus may not be able to distinguish between the SHR related to legacy HO and LTM. However, due to their different triggers it is not possible to optimize them commonly.

Observation 2: The network for proper optimization may need to distinguish between SHR related to LTM and legacy HO.

Proposal 3: RAN3 agrees the UE triggers the SHR if at least one of the SHR trigger criteria is satisfied (including also new triggers as per proposal 2) with an indictor it is related to LTM cell switch.

### 2.1.4 Connection Failure Monitoring for MCG LTM

Similar failure cases as defined in TS38.300 [2] for HO (Too Late Handover, Too Early Handover, and Handover to Wrong Cell, ) shall be considered for MCG LTM. RAN2 # 125bis agreed the following::

For LTM MRO, RAN2 considers the following three connection failure cases:

-	Too late LTM

-	Too early LTM

-	LTM to wrong cell

For too late LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

-	Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and performs reestablishment procedure.

-	Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell, detects HOF with the selected LTM cell.

-	Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM execution with the selected LTM candidate cell.

For too early LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

-	Case 2a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

-	Case 2b: the UE detects HOF/RLF in the LTM target cell, selects the source cell which is also an LTM candidate cell, detects HOF with the source cell, and performs reestablishment procedure.

-	Case 2c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected source cell which is also an LTM candidate cell.

LTM to wrong cell, the following sub-cases are considered but we may down prioritize later (not limiting):

-	Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

-	Case 3b: the UE detects HOF/RLF in the LTM target cell, selects an LTM candidate cell which is different from the source or target one, detects HOF with the selected LTM candidate cell, and performs reestablishment procedure.

-	Case 3c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected LTM candidate cell which is different from the source or target one.

During the RAN3 # 124 meeting the following was consequently agreed:

RAN3 takes RAN2#125-bis agreement on MRO for LTM scenarios as baseline for further study.

The Case 3a is wrongly defined as it is identical with Case 2a. The Case 3a may read as follows:

Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the cell which is different from the source and target cell.

The above proposal is in accordance with the following agreement achieved during the RAN2 # 126 meeting:

- We correct the definition of scenario 3a as follows: Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure in a cell other than the source cell and the target cell.

For the case 1b and 1c it is not clear how UE attempts to access the selected LTM candidate cell. It shall clearly say that it is done via LTM recovery which must be activated for the UE. For the case 2 and 3 the information about sending the Cell Switch Command to UE is missing. Also, the statement “the UE detects HOF/RLF in the LTM target cell” requires some more clarification. In fact, either it is HO failure in source LTM cell or RLF shortly after successful HO in target LTM cell. Nor it is clear whether UE is allowed to initiate RRC Re-establishment procedure after LTM recovery failure.

Proposal 4: RAN3 agrees to send LS to RAN2 to clarify whether UE is allowed to initiate RRC Re-establishment procedure after LTM recovery failure.

The UE is configured for several mobility methods. The network thus may not be able to distinguish between the RLF report related to legacy HO and LTM. However, due to their different triggers it is not possible to optimize them commonly.

Observation 3: The network for proper optimization may need to distinguish between RLF report related to LTM and legacy HO.

The above observation is it is in accordance with the following agreement achieved during the RAN2 # 126 meeting:

- Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

Furthermore, main benefits of the LTM shall be seen in avoiding the complete reset of L3, avoiding RACH procedure which shall result into reduction of the interruption time. In case the TA of the candidate target LTM cell is not included into Cell Switch Command (MAC CE) the UE will initiate the legacy RACH procedure to access the target candidate cell. Therefore, it may be useful if UE reports the LTM execution was done for RACH based or RACH less case. During the RAN3 # 124 meeting the following was agreed:

RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM.

Some of the companies proposed during the RAN3 # 124 the following:

UE should omit RACH information in RLF Report during RACH-less LTM and further can add an explicit indicator that the LTM was RACH-less.

Some of the companies consider the presence of RACH information in RLF report may be used as an explicit indication form UE side the LTM execution was done for RACH based or RACH less case. However, as the RACH information in RLF report is related to T304 expiry the LTM RACH based cases with no T304 expiry the network will not be able to distinguish they are RACH less or RACH based. It may be therefore useful the UE provides information the LTM execution was RACH based or RACH less regardless of the presence of RACH information in the RLF report.

Proposal 5: RAN3 agrees on the extension of the RLF report for LTM to indicate the RACH based or RACH less LTM execution regardless of the presence of RACH information.

#### 2.1.4.1 CU and DU LTM Optimization

During the RAN3 # 124 meeting the following was agreed:

For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization.

For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) source DU performs optimization.

The following was left for discussion in the next meeting:

For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) FFS if CU or DU performs root cause analysis.

Based on the first agreement above considering the LTM is executed for intra CU CP scenario only and some further improvements of the RLF report in the chapter 2.1.4 the CU shall be able to distinguish between the too late, too early and wrong cell failure cases. Also based on the reported suitable cell belongs to candidate LTM cell list or not the CU shall be able to distinguish between the “failures due to wrong selection of candidate LTM cell” and “failures due to inappropriate cell switch triggering”.

Observation 4: The CU should have all the needed information to be able to distinguish between the too late, too early and wrong cell failure case and between the “failures due to wrong selection of candidate LTM cell” and “failures due to inappropriate cell switch triggering”.

The source DU after receiving the RLF report from CU shall initiate the own root cause analysis to identify the problem.

Proposal 6: CU identifies the failures due to inappropriate cell switch triggering and forwards the RLF report to the source DU which provides the root cause analysis.

It shall be noted that there may be plurality of problems causing the RLF. Besides the wrong cell selection at cell switch and wrong cell switch timing the problem may be related to wrong or missing TA in the Cell switch command.  In general, it may be related to an issue in Early TA Acquisition procedure which is in charge of source DU. Especially the scenarios when TA was reported in Cell Switch Command, but you did not succeed to attempt the target LTM cell and had to switch RACH based access maybe an info which you may report in the RLF report.

Proposal 7: RAN3 agrees the study on the extension of the parameters in RLF to identify issues related to Early TA Acquisition procedure.

#### 2.1.5.2 Stage 2 on Connection failure due to intra-system mobility for LTM

During the RAN3 # 124 meeting the following topic was left for discussion in the next meeting:

For stage-2, FFS if existing problem definitions (Connection failure due to intra-system mobility) can be reused or if a new chapter needs to be created.

In the 3GPP TS 38.300 the following definition is provided for legacy and CHO

##### 15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

---

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

-----

In case of CHO the same as for LTM, UE may attempt to re-establish radio link connection either via RRC re-establishment or recovery procedure. During the RAN3 # 124 meeting discussions some of the companies interpret the “re-establish” in the current definition as a term which covers both RRC re-establishment and CHO based recovery in case of CHO. Later in the same chapter of the 38.300 within the detection mechanism part it is clarified where it applies that the re-establish may be executed as “first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery”.

Observation 5: From the detection mechanism part related to “Connection failure due to intra-system mobility” chapter in the 38.300 it follows that the “re-establish” in the current definition covers both RRC re-establishment and CHO based recovery in case of CHO.

However, there is an issue with the current definition is that the "successful handover" refers to the UE state, namely the successful completion of the RA procedure. However, in case of LTM it shall be primarily executed as RACH-less.

Observation 6: The definition of the "successful handover" for connection failures in 38.300 does not fit for RACH-less LTM.

Proposal 8: RAN3 agrees to re-use existing connection failures definition from 38.300 also for LTM with the following additions in red:

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure. In this case of RACH-less LTM it refers to successful reception of the RRCConnectionReconfigurationComplete message from UE (TS 38.401).

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM, Too Early LTM and LTM to Wrong Cell.

## 2.2 MRO for CHO with Candidate SCG(s)

### 2.2.1 Agreements achieved in RAN3

The agreements achieved in RAN3 #124 [1]:

Do not distinguish between fast MCG recovery/no-fast MCG recovery for now (to simplify use cases).

Concurrent error cases (MCG+SCG) is FFS. The definition of these error cases needs to be further clarified.

RAN3 will start with the failure scenarios with UEs configured with CHO with candidate SCGs.

Whether to include failure and near failure scenarios related with configuration of CHO with candidate SCG(s) and CHO only is FFS.

The agreements achieved in RAN3 #123bis [2]:

- MRO for CHO with candidate SCG failure and near failure cases

- RAN3 focuses on NR-DC for MRO for CHO with candidate SCG in R19.

- R19 SON/MDT solution discussion is based on R18 work.

The agreements achieved in RAN2 #126 [3]:

- RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report.

The agreements achieved in RAN2 #125bis [4]:

- RAN2 to study failure and near failure scenarios for CHO with candidate SCGs.

## 2.2.2 3GPP Release 18 Mobility Enhancements - CHO + MR-DC

In Release 17, CHO in MR-DC is limited to the scenario where the target MN can prepare a single target PSCell (under the control of an SN). This limits the usefulness of the CHO MR-DC feature where UE may need to access different target PSCell when the CHO needs to be executed. In Rel-18 specified mechanisms for CHO in MR-DC where target MN can prepare multiple target PSCells for the same target PCell. The corresponding stage 2 in the TS 37.340 was improved in Rel. 18 to support the CHO with candidate SCG(s) and stage 3 of the impacted messages in the TS 38.423.

During the initial discussions on this topic in RAN3 #123bis meeting some of the companies considered that since when only CHO with candidate SCG(s) is configured, the UE does not execute CHO with candidate SCG(s) until the execution conditions for both the candidate PCell and the associated candidate PSCell are met in a complementary way, network may also configure at least one of legacy R16 CHO and legacy R17 CHO with SCG. In such a way,

- If at least one candidate PCell satisfies the corresponding execution condition and the associated candidate PSCell satisfies the corresponding execution condition, CHO with candidate SCGs is performed, e.g. the UE detaches from the source MN, applies the stored corresponding configuration for that selected candidate PCell and the associated candidate PSCell.

- Else, if at least one candidate PCell satisfies the corresponding execution condition, and a target PSCell associated to this candidate PCell is configured to the UE, legacy R17 CHO with SCG is performed, e.g. the UE detaches from the source MN, applies the stored corresponding configuration for that selected candidate PCell and the associated target PSCell.

- Else, if at least one candidate PCell satisfies the corresponding execution condition and there is no associated candidate PSCell or there is no associated target PSCell, legacy R16 CHO is performed, e.g. the UE detaches from the source MN, applies the stored corresponding configuration for that selected candidate PCell.

Then the companies’ proposal was that in R19, when we discuss MRO for CHO with candidate SCG(s), the following cases should be considered:

- Case 1: CHO with candidate SCGs is performed, when only CHO with candidate SCGs is configured, or CHO with candidate SCGs and at least one of legacy R16 CHO and legacy R17 CHO with SCG are configured,

- Case 2: legacy R17 CHO with SCG is performed, when CHO with candidate SCGs and legacy R17 CHO with SCG are configured, or, when CHO with candidate SCGs, legacy R16 CHO and legacy R17 CHO with SCG are configured,

- Case 3: legacy R16 CHO is performed, when CHO with candidate SCGs and legacy R16 CHO are configured, or, when CHO with candidate SCGs, legacy R16 CHO and legacy R17 CHO with SCG are configured.

RAN3 community during the RAN3 # 123bis meeting discussion was not able to conclude the above cases shall be covered in MRO in Rel-19 or not. During the RAN3 # 14 meeting discussions a progress was achieved on that meaning that the following was agreed:

RAN3 will start with the failure scenarios with UEs configured with CHO with candidate SCGs.

As indicated in the following the discussions about usefulness of the above case 2 and 3 were left for discussion to next meeting:

Whether to include failure and near failure scenarios related with configuration of CHO with candidate SCG(s) and CHO only is FFS.

Even we agree that configuring the UE with for example R16 or R17 CHO in parallel with R18 CHO is useful to avoid dropping or degradation the services from MRO perspective we recommend starting with MRO for UEs with configured R18 CHO with candidate SCG(s) only in Rel. 19. It simply because the MRO task for two procedures, CHO and CPAC running in parallel is not trivial task. Thus, combining the MRO for R18 CHO with the ones from lower releases may not guarantee the work is successfully completed in Rel. 19.

Proposal 9: RAN3 agrees on MRO for UEs with configured CHO with candidate SCG(s) only in Rel. 19.

### 2.2.3 Near Failure Monitoring for CHO with Candidate SCG(s)

With 3GPP Rel-17 for the intra-NR mobility and consequent extension for inter RAT mobility in Rel. 18, the MRO concept based on RLFs has been extended towards the logging of risky or failed but recovered mobility cases which finally resulted in success handover by providing a successful handover report (SHR). In similar way for PSCell addition or change the recovered cases which finally resulted in success scenario by providing a successful PSCell Addition or Change report (SPR).

Observation 7: The SHR and SPR monitoring is also applicable for CHO with candidate SCG(s).

When the UE is configured with R18 CHO with candidate SCG(s) configuration, there will be one candidate configuration that contains both MCG and SCG configuration. Although there is one candidate configuration, the UE will receive two orthogonal conditions to be monitored to execute that candidate configuration. Herein, there will be one (or more) CHO condition to reflect MCG quality measure, and one (or more) CPAC condition to reflect SCG quality measure in the condition evaluation. Herein, the CHO condition will be determined by the source MN whereas the CPAC condition will be determined by the candidate MN who prepares the candidate SN. Eventually, the candidate configuration will be executed only if both CHO and CPAC conditions are met.

Observation 8: CHO with candidate SCG(s) involves orthogonality meaning that candidate configuration will be executed only if both CHO and CPAC conditions are met.

It is nontrivial to set both CHO and CPAC conditions in a way that they will be in line with each other, i.e., one condition does not necessarily block other.

In legacy procedures, it is possible to optimize the execution conditions, by looking at the successful HO Report (SHR) for CHO, and Successful PSCell Report (SPR) for CPAC, as CHO and CPAC conditions were not tied to each other and execution of each was independent. When the CHO execution is delayed with an interruption at the UE side (T310 timer runs but does not expire) due to wrong CHO execution condition configuration, it is known by the network that what went wrong with the CHO execution. Similarly, when the CPAC execution is delayed and leads UE to experience interruption between UE and the S-SN, the wrong configuration can be known by the network if the UE indicates that CPAC execution was delayed (via SPR).

Observation 9: The legacy way of independent SHR and SPR monitoring and consequent optimization may be insufficient for CHO with candidate SCG(s).

Proposal 10: RAN3 agrees on analysis of the MRO for CHO with candidate SCG(s) from the perspective of the interdependency between CHO and CPAC configuration execution.

In case of CHO with candidate SCG(s), if one of the execution conditions is not configured properly, the entire execution will be delayed as CHO+CPAC configuration cannot be executed partially. Only when both conditions can be satisfied and the UE executes the configuration, i.e., access both target PCell and PScell.

Observation 10: For MRO for CHO with candidate SCG(s) it may be beneficial to identify the blocking/wrong configuration.

Proposal 11: RAN3 agrees on deeper analysis of the blocking CHO/CPAC configuration for CHO with SCG(s) mobility which may be for example reported as new information in SHR/SPR when CHO/CPAC execution is delayed by the time interval which is above the configured threshold after the CPAC/CHO execution.

### 2.2.4 Connection Failure Monitoring for CHO with Candidate SCG(s)

The UE may experience a failure (either RFL or SCG failure) while being configured with CHO with candidate SCG(s), more specifically one of the following scenarios may happen:

- CHO execution condition of a candidate PCell is fulfilled but CPAC execution condition of the associated candidate PSCell is not fulfilled or vice versa; or,

- neither CHO execution condition of a candidate PCell is fulfilled nor CPAC execution condition of the associated candidate PSCell is fulfilled.

- Both CHO execution condition of a candidate PCell and CPAC execution condition of the associated candidate PSCell are fulfilled but consequent execution completion will fail to either both or one of them

- Both CHO execution condition of a candidate PCell and CPAC execution condition of the associated candidate PSCell are fulfilled, consequent execution completion for both is successful but shortly after that there is SCG or MCG failure or both.

In this case it is important for the network to determine which condition or relationship between conditions lead to the failure.

Proposal 12: RLF report is enhanced for the case that an MCG failure occurs before CHO with candidate SCGs is executed.

Proposal 13: SCG Failure Information message is enhanced for the case that an SCG failure occurs before CHO with candidate SCGs is executed.

The above proposals 12 and 13 are in accordance with the following agreement achieved during the RAN2 # 126 meeting:

- RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report.

#### 2.2.4.1 Initial Discussion on CHO with Candidate SCG(s) Failure Scenarios

During the RAN3 # 124 meeting it was proposed to take as baseline the following nine failure cases, as shown in Fig. 1, from R3-243279 to start the discussions. During the same meeting, also the following was agreed:

Do not distinguish between fast MCG recovery/no-fast MCG recovery for now (to simplify use cases).

Considering this agreement the sub-cases a) and b) from the case 1,2, 3 and 9 shall be removed.  Cases 4, 5 and 6 shall be removed as current agreement is to start with UEs with CHO with candidate SCG(s) only configured. Concerning the sub-case d) in case 1, 2, 3, 7, 8 and 9 the following was marked as an FFS in the RAN3 # 124 meeting:

Concurrent error cases (MCG+SCG) is FFS. The definition of these error cases needs to be further clarified.

Fig. 1 The failure cases based on R3-243279

In case there is SCG and MCG failure declared at the same or MCG failure shortly after SCG failure which has not been yet reported in the form of SCG failure information via MCG to network then UE will generate RLF related to MCG failure which is then reported to network after re-establishment radio link connection in PCell. The SCG failure information is not reported at all.  The sub-case d) in the 1, 2, 3, 7, 8 and 9 then may indicate that only RLF report is communicated from UE to network. The case 9 requires some more clarification as it is not clear whether the fulfilment of the execution condition in time happens still for the same or different candidate PCell and PSCell. Nor it is clear how relevant is such case in live network.

Proposal 14: RAN3 agrees to remove the sub-cases a) and b) from the case 1,2, 3 and 9, to remove the sub-cases related to concurrent “MCG failure + SCG failure” from the case 1,2,3,7 and 8 and to completely remove the cases 4, 5, 6 and 9 and to consider the updated list of cases as a starter for discussion.

# 3	Conclusions

We have the following observations and proposals:

Proposal 1: RAN3 agrees to send LS to RAN2 to double check the T304 value and possible dedicated threshold value for LTM.

Observation 1: The legacy SHR triggers seem to be incomplete for LTM.

Proposal 2: RAN3 agrees to define new SHR triggers specific to LTM near failure cases related to RACH based LTM and new threshold related to number of RLC retransmissions of the first UL transmission in RLC AM for RACH less LTM.

Observation 2: The network for proper optimization may need to distinguish between SHR related to LTM and legacy HO.

Proposal 3: RAN3 agrees the UE triggers the SHR if at least one of the SHR trigger criteria is satisfied (including also new triggers as per proposal 2) with an indictor it is related to LTM cell switch.

Proposal 4: RAN3 agrees to send LS to RAN2 to clarify whether UE is allowed to initiate RRC Re-establishment procedure after LTM recovery failure.

Observation 3: The network for proper optimization may need to distinguish between RLF report related to LTM and legacy HO.

Proposal 5: RAN3 agrees on the extension of the RLF report for LTM to indicate the RACH based or RACH less LTM execution regardless of the presence of the RACH information.

Observation 4: The CU should have all the needed information to be able to distinguish between the too late, too early and wrong cell failure case and between the “failures due to wrong selection of candidate LTM cell” and “failures due to inappropriate cell switch triggering”.

Proposal 6: CU identifies the failures due to inappropriate cell switch triggering and forwards the RLF report to the source DU which provides the root cause analysis.

Proposal 7: RAN3 agrees the study on the extension of the parameters in RLF to identify issues related to Early TA Acquisition procedure.

Observation 5: From the detection mechanism part related to “Connection failure due to intra-system mobility” chapter in the 38.300 it follows that the “re-establish” in the current definition covers both RRC re-establishment and CHO based recovery in case of CHO.

Observation 6: The definition of the "successful handover" for connection failures in 38.300 does not fit for RACH-less LTM.

Proposal 8: RAN3 agrees to re-use existing connection failures definition from 38.300 also for LTM with the following additions in red:

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure. In this case of RACH-less LTM it refers to successful reception of the RRCConnectionReconfigurationComplete message from UE (TS 38.401).

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM, Too Early LTM and LTM to Wrong Cell.

Proposal 9: RAN3 agrees on MRO for UE with configured CHO with candidate SCG(s) only in Rel. 19.

Observation 7: The SHR and SPR monitoring is also applicable for CHO with candidate SCG(s).

Observation 8: CHO with candidate SCG(s) involves orthogonality meaning that candidate configuration will be executed only if both CHO and CPAC conditions are met.

Observation 9: The legacy way of independent SHR and SPR monitoring and consequent optimization may be insufficient for CHO with candidate SCG(s).

Proposal 10: RAN3 agrees on analysis of the MRO for CHO with candidate SCG(s) from the perspective of the interdependency between CHO and CPAC configuration execution.

Observation 10: For MRO for CHO with candidate SCG(s) it may be beneficial to identify the blocking/wrong configuration.

Proposal 11: RAN3 agrees on deeper analysis of the blocking CHO/CPAC configuration for CHO with SCG(s) mobility which may be for example reported as new information in SHR/SPR when CHO/CPAC execution is delayed by the time interval which is above the configured threshold after the CPAC/CHO execution.

Proposal 12: RLF report is enhanced for the case that an MCG failure occurs before CHO with candidate SCGs is executed.

Proposal 13: SCG Failure Information message is enhanced for the case that an SCG failure occurs before CHO with candidate SCGs is executed.

Proposal 14: RAN3 agrees to remove the sub-cases a) and b) from the case 1,2, 3 and 9, to remove the sub-cases related to concurrent “MCG failure + SCG failure” from the case 1,2,3,7 and 8 and to completely remove the cases 4, 5, 6 and 9 and to consider the updated list of cases as a starter for discussion.

# References

[1] Report of 3GPP TSG RAN3 meeting #124

[2] Report of 3GPP TSG RAN3 meeting #123-bis

[3] Report of 3GPP TSG RAN2 meeting #126

[4] Report of 3GPP TSG RAN2 meeting #125-bis