3GPP TSG-RAN WG3 Meeting #125	R3-244337

# Maastricht, Netherland, 19th - 23rd August 2024

Agenda item:	12.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to TR 38.799) Discussion on Xn support for WAB-gNB and Traffic Mapping

Document for:	Discussion and Decision

# 1	Introduction

RAN#102 agreed study item on topological enhancements for NR [1]. It covers both 5G femto and wireless access backhaul (WAB). The objectives for WAB SI are following, [1]:

. Study the support of WAB including [RAN3, RAN2]: - Study the architecture and protocol stack of supporting a gNB with MT function providing PDU session backhaul. - Study impact of WAB mobility within an existing RAN (e.g., inter-gNB neighbour relations). - Identify necessary inter-gNB- and gNB-to-CN signalling to address the support of WAB. - Study signalling enhancements on resource multiplexing for WAB. NOTE 1: No impact on the UE. NOTE 2: Coordination with other WGs (e.g. SA2) when needed. The WAB study does not preclude any backhaul scenario (e.g. NTN or TN).

This contribution analyses the architecture, protocol stack and high level aspects.

# 2	Xn between WAB-gNB and neighbouring gNBs

Last meeting RAN3 agreed

Establishment of Xn Connections of the WAB-gNB with BH-RAN nodes, as well as with surrounding RAN nodes, is supported and can follow legacy procedures.

However, there is no TP captured in the TR 38.799 for Xn management (e.g. Xn establishment) for the WAB-gNB.

Xn Establishment

Xn establishment between WAB-gNB and the BH-gNB serving WAB-MT

When the WAB-MT connects to a BH-gNB serving WAB-MT, the WAB-MT knows the TAI/CGI of the serving cell. After the integration procedure (i.e. WAB-MT connected with the gNB serving WAB-MT, WAB-gNB setup NG with the AMF, BH PDU session establishment), the WAB-gNB can initiate the Xn-C TNL address discovery procedure towards the BH-gNB serving WAB-MT. The WAB-gNB may then initiate the TNL/Xn Setup with the BH-gNB serving WAB-MT.

Proposal 1-1: WAB-gNB can reuse existing Xn-C TNL address discovery procedure to know the Xn-C TNL address of BH-gNB serving WAB-MT, then setup Xn with BH-gNB serving WAB-MT.

Xn establishment between WAB-gNB and neighboring gNB

The surrounding RAN node can be a stationary gNB, or a WAB-gNB. There is different view on whether the Xn can be setup between the two WAB-gNBs.

In Rel-18, the mobile IAB acts as a gNB-DU/cell. The Xn between the IAB-donor and its neighboring gNB may be already established before mobile IAB connects with IAB-donor. The arrival/departure of a mobile IAB does not affect the Xn between the IAB-donor and its neighboring gNB, other than possibly informing neighbors about newly deployed/released IAB cell(s). From the perspective of a neighboring gNB, the arrival/departure of a mobile IAB can be considered to add/remove cell(s) of the serving IAB-donor.

The WAB brings a new challenge to the inter-gNB neighbor relation. Unlike a new Rel-18 mobile IAB appearing as a new gNB-DU/cell to neighboring gNB, WAB appears as a new gNB to the neighboring gNB. The arrival/departure of mobile WAB causes dynamic setup/release of the Xn between the WAB-gNB and its neighboring gNB(s).

In legacy system, the gNB can initiate the ANR procedure whenever the UE reports an unknown PCI, i.e. a PCI that is not in the NCRT. This can be similar for WAB, e.g. WAB initiate the ANR procedure whenever the UE reports an unknown PCI. However, WAB faces another challenge, for example, when the UE reports a known PCI, the WAB cannot know whether the PCI is related to an existing cell/entry in its NCRT, or the PCI belongs to a new neighboring cell that is not in the NCRT. This is due to the mobility nature of WAB and PCI reuse for cells in different geographical areas.

Figure 1: PCI reuse by cells of gNB1 and gNB3

An example is shown in above figure.

- When the WAB is at location A, it detects the new cell with PCI#1 (i.e. gNB1’s cell), WAB initiate ANR procedure to add gNB1’s cell (PCI#1) in its NCRT.

- Later, when WAB move to location B which is covered by gNB3’s cell with same PCI#1, WAB cannot know whether the PCI#1 corresponds to an existing entry in NCRT that is related to gNB1’s cell, or corresponds to a new cell.

So, it is necessary for WAB-gNB to know the neighboring cell, and a neighboring gNB to know WAB cell as early as possible. In legacy system, the Xn setup is either triggered by OAM, or by ANR based on the UE’s measurement report. However, in the mobile WAB case, the WAB-gNB or neighboring gNB may not be able to know the right time that it needs to ask UE to perform a measurement in order to detect the cell using a specific PCI is changed.

Observation 1: Due to PCI reuse, it may be difficult for WAB-gNB (or gNB) to determine the right time to ask UE to perform a measurement in order to detect the new neighbouring cell.

One option is to exchange the neighboring cell information over Xn interface, in order for WAB-gNB to know the neighboring cell, and a neighboring gNB to know WAB cell as early as possible.

The WAB-gNB can reuse the existing mechanism to detect the neighboring cell of a gNB. Since the WAB has a gNB functionality, it can initiate the ANR procedure based on the measurement report from a connected UE, or from the measurement report of the co-located WAB-MT. In addition, the WAB-gNB can also use the information received from BH-gNB serving WAB-MT over Xn.  In most of the scenarios, the WAB’s neighboring gNB is also a neighboring gNB to the BH-gNB. Since the BH-gNB may already setup Xn with the neighboring gNB, BH-gNB can provide the Xn-C TNL address of neighboring gNB to WAB-gNB, thus WAB-gNB can directly initiate the Xn setup procedure with neighboring gNB. Alternatively, during the Xn Setup or NG-RAN node configuration update procedure, the BH-gNB serving WAB-MT can provide its neighboring cell information to the WAB-gNB. . The WAB-gNB can use the neighboring cell information received from the BH-gNB to update its NCRT, or initiate Xn-C TNL address discovery procedure for further TNL/Xn Setup with the neighboring gNB, without waiting for the measurement report from UE (or WAB-MT).

Proposal 1-2: BH-gNB can provide the Xn-C TNL address of neighboring gNB to WAB-gNB, so WAB-gNB can directly initiate Xn Setup with neighbour gNB.

Proposal 1-3: WAB-gNB can also use the neighboring cell information received from the BH-gNB to update its NCRT or initiate the Xn-C TNL address discovery procedure towards the neighboring gNB for further TNL/Xn Setup with the neighboring gNB, without waiting for the measurement report from UE (or WAB-MT).

Furthermore, in case Xn is to be avoided between WAB-gNBs (as proposed by some companies in the previous meeting due to the nature of WAB being mobile, which would lead frequent Xn setup and Xn release), this could even be avoided during TNL discovery procedure, i.e., as early as possible. For example, source WAB-gNB can inform the target gNB that it is a WAB during TNL discovery, and target can refrain sending TNL information. That way, Xn establishment can be avoided at the earliest time possible and there would be no unnecessary additional signalling.

Proposal 1-4: If Xn is to be avoided among WAB-gNBs, TNL discovery procedure can be enhanced to avoid Xn establishment as early as possible among WAB-gNBs.

Xn Removal

When the WAB move away, the WAB may need to remove the Xn with previous surrounding gNBs. This can be supported by the existing Xn Removal procedure.

Proposal 1-5: Existing Xn Removal procedure can be reused to remove the Xn between WAB-gNB and surround gNB.

The proposed TP for TR 38.799 can be found in Annex – TP for TR 38.799

# 3	Traffic Mapping for Backhaul PDU session carrying N2/N3 traffic

The WAB-gNB sets up NG with UE’s AMF. The N2/N3 (or NG-C/U) traffic is conveyed over the BH PDU session of the WAB-MT. At least one PDU session needs to be setup, and to be used for the UE’s N2/N3 backhaul. Up to the operator’s configuration, the WAB-MT may setup one or multiple PDU sessions, e.g. for OAM, for N2/N3, or for Xn, etc.

In uplink direction, the WAB-gNB knows the QoS for each UL packet received from the UE. The WAB-gNB can inform the co-located WAB-MT. Based on the QoS information for the UL packet, the WAB-MT can map the related N3 packet to a specific QoS flow of the WAB-MT’s BH PDU session. The WAB-MT can also determine whether an already established BH PDU session is enough, or modify the existing BH PDU session, or request the establishment of a new BH PDU session.

For the uplink N2 traffic, the co-located WAB-MT can know whether it is a UE-associated signaling or non-UE-associated signaling, the WAB-MT can map it to the related QoS flow of the BH PDU session.

Proposal 2-1: In the uplink direction, WAB can perform the traffic mapping based on the QoS of the UL packet, and the QoS of the QoS flow of its BH PDU session.

In downlink direction, UE’s UPF uses the packet filter provided by the UE for packet detection, e.g. identify the related QoS Flow and add the QFI in the NG-U GTP-U header. The NG-U GTP-U packet is routed to the UPF of the WAB-MT. The WAB-MT’s UPF performs a further traffic mapping to a QoS flow of the BH PDU session. The WAB-MT’s UPF can only see the IP header, but not the inside GTP-U packet from the UE’s UPF. So, the traffic mapping in the WAB-MT’s UPF can only be based on the IP Header.  This is also applicable to the downlink N2 traffic.

To perform a correct traffic mapping in the WAB-MT’s UPF, the WAB-MT’s UPF needs to be configured with the mapping rule.

Proposal 2-2: For downlink N2/N3 traffic, the WAB-MT’s UPF performs the traffic mapping based on the IP header of the N2/N3 traffic.

The mapping rule can be provided by the WAB-MT, e.g. based on the existing UE requested PDU session modification procedure.

If the N2 traffic is on the same PDU session as the N3 traffic, the traffic classification for N2, is based on identification of the SCTP protocol. Else, if the N2 traffic has its own PDU session, the traffic is identified in DL by the IP addresses used for N2 over this connection (i.e. the IP address of the connection) so normal IP routing is sufficient.

Note that the following conclusion is made by the SA2 on the matter:

- If QoS optimization of the N3 connection is required, then the MWAB-gNB is configured (e.g. by OAM) with the mapping of the 5QIs to the DSCP field for TNL used by the UPFs/gNBs in the PLMN/SNPN serving the UE via the MWAB-gNB. This is then used to establish SDFs at the BH-PDU session used for N3 connection(s) based on existing procedures.

The traffic mapping for N3 over the appropriate BH PDU session QoS flow, is therefore to be based on the approach documented in Solution 3 of TR 23.700-06 i.e based on the differentiated services code point (DSCP) value to be used by the UE’s UPF for sending the DL NG-U packet. This value can be known per 5QI and be configured by OAM at the WAB gNB which then can request the suitable update of the N3 PDU session by PDU session modification including this classification rule.

Proposal 2-3: For downlink traffic mapping to the appropriate N3 BH PDU session QoS flow, the WAB-gNB needs to know the DSCP information for the further DL NG-U packet during the UE’s PDU Session Resource Setup/Modification procedure.

The proposed TP for TR 38.799 can be found in Annex – TP for TR 38.799

# 4	OAM Connection in Roaming Scenarios

Regarding OAM connection over a PDU session, as explained in Section 3, there should be an extra consideration for the scenario where WAB-MT and WAB-gNB are connected to different PLMNs. More specifically, let us assume that  WAB-MT connects to a PLMN B, whereas WAB-gNB serves (and is served by and connected to the core network of) PLMN A. Considering this issue is in SA5 scope and SA5 is working on the similar issue for IAB, we would suggest to not discuss it in RAN3.

Proposal 3: OAM is in SA5 scope, and RAN3 should follows SA5 conclusion.

# 5	Detection of co-location of WAB-MT and WAB-gNB

For BH-gNB serving WAB-MT, it is beneficial in some scenarios to detect the co-location of the WAB-MT and WAB-gNB. For example, the BH-gNB may use this information, e.g. for resource multiplexing handling. In IAB case, the Donor detects the co-location based on the IAB-MT’s BAP address received in the F1 SETUP REQUEST message. In WAB, the co-location detection may be performed via the Xn interface. For example, when the WAB-gNB initiates the Xn Setup with the gNB serving WAB-MT, the WAB-gNB may include an indication in the XN SETUP REQUEST message. The indication may be a UE ID (e.g. C-RNTI) received from the BH-gNB  during the RRC establishment procedure. The detail of the indication can be further studied in the WI phase.

Proposal 4-1: The BH-gNB detects the co-location of the WAB-gNB and WAB-MT during the Xn Setup procedure initiated by the WAB-gNB, via the WAB-MT’s ID (e.g. C-RNTI) received from the WAB-gNB in XN SETUP REQUEST message.

The proposed TP for TR 38.799 can be found in Annex – TP for TR 38.799

# 6	Conclusion

In this contribution, we discussed the architecture and other high-level aspects of WAB. Our proposals are:

For Xn:

Proposal 1-1: WAB-gNB can reuse existing Xn-C TNL address discovery procedure to know the Xn-C TNL address of BH-gNB serving WAB-MT, then setup Xn with BH-gNB serving WAB-MT.

Observation 1: Due to PCI reuse, it may be difficult for WAB-gNB (or gNB) to determine the right time to ask UE to perform a measurement in order to determine the new cell.

Proposal 1-2: BH-gNB can provide the Xn-C TNL address of neighboring gNB to WAB-gNB, so WAB-gNB can directly initiate Xn Setup with neighbour gNB.

Proposal 1-3: WAB-gNB can also use the neighboring cell information received from the BH-gNB to update its NCRT or initiate the Xn-C TNL address discovery procedure towards the neighboring gNB for further TNL/Xn Setup with the neighboring gNB, without waiting for the measurement report from UE (or WAB-MT).

Proposal 1-4: If Xn is to be avoided among WAB-gNBs, TNL discovery procedure can be enhanced to avoid Xn establishment as early as possible among WAB-gNBs.

Proposal 1-5: Existing Xn Removal procedure can be reused to remove the Xn between WAB-gNB and surround gNB.

For Traffic Mapping:

Proposal 2-1: In the uplink direction, WAB can perform the traffic mapping based on the QoS of the UL packet, and the QoS of the QoS flow of its BH PDU session.

Proposal 2-2: For downlink N2/N3 traffic, the WAB-MT’s UPF performs the traffic mapping based on the IP header of the N2/N3 traffic.

Proposal 2-3: For downlink traffic mapping to the appropriate N3 BH PDU session QoS flow, the WAB-gNB need to know the DSCP information for the further DL NG-U packet during the UE’s PDU Session Resource Setup/Modification procedure.

Proposal 3: OAM is in SA5 scope, and RAN3 just follow SA5 conclusion.

Proposal 4-1: The BH-gNB detects the co-location of the WAB-gNB and WAB-MT during the Xn Setup procedure initiated by the WAB-gNB, via the WAB-MT’s ID (e.g. C-RNTI) received from the WAB-gNB in XN SETUP REQUEST message.

The proposed TP for TR 38.799 can be found in Annex – TP for TR 38.799

# References

- RP-234041 Study on additional topological enhancements for NR

- TR 23.700-06, Study on architecture enhancements for vehicle-mounted relays - Phase 2

- TR 38.799

# Annex – TP for TR 38.799

### 4.3.5 Resource multiplexing

In scenarios where WAB-node’s access link and backhaul link mutually interfere, resource coordination may be needed to facilitate the resource multiplexing of the WAB-node’s access links and backhaul link. For this purpose, the resource coordination mechanism introduced for IAB can be considered as the starting point. For resource coordination between the access link and backhaul link, the BH-gNB may need to discover co-location of the WAB-MT and the WAB-gNB. The BH-gNB serving WAB-MT detects the co-location of the WAB-gNB and WAB-MT during the Xn Setup procedure. For example, when the WAB-gNB initiate the Xn Setup procedure, the WAB-gNB includes the WAB-MT’s ID (e.g. C-RNTI) received from the WAB-gNB in XN SETUP REQUEST message, so the BH-gNB can detect the co-location of the WAB-gNB and WAB-MT.

### 4.3.x Xn management for WAB-gNB

The WAB-gNB can reuse existing Xn-C TNL address discovery procedure to know the Xn-C TNL address of BH-gNB serving WAB-MT, then setup Xn with BH-gNB serving WAB-MT. During the Xn setup procedure, the BH-gNB can provide the Xn-C TNL address of neighboring gNB to WAB-gNB, so WAB-gNB can directly initiate Xn Setup with neighbour gNB.

WAB-gNB can also use the neighboring cell information received from the BH-gNB to update its NCRT or initiate the Xn-C TNL address discovery procedure towards the neighboring gNB for further TNL/Xn Setup with the neighboring gNB, without waiting for the measurement report from UE (or WAB-MT).

If Xn is to be avoided among WAB-gNBs, TNL discovery procedure can be enhanced to avoid Xn establishment as early as possible among WAB-gNBs.

Existing Xn Removal procedure can be reused to remove the Xn between WAB-gNB and surrounding gNB.

### 4.3.y Traffic mapping for the traffic of WAB-gNB

For the downlink traffic to the WAB-gNB, the BH-UPF maps the downlink traffic to a QoS flow of the WAB-MT based on the IP header (e.g. DSCP).

For the uplink traffic from the WAB-gNB, the WAB-MT maps the uplink traffic to a QoS flow of the WAB-MT based on the IP header (e.g. DSCP).

## 4.4	Other