3GPP TSG-RAN WG3 Meeting #125	R3-244338

# Maastricht, Netherland, 19th - 23rd August 2024

Agenda item:	12.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to TR 38.799) Discussion on WAB mobility

Document for:	Discussion and Decision

# 1	Introduction

RAN#102 agreed study item on topological enhancements for NR [1]. It covers both 5G femto and wireless access backhaul (WAB). The objectives for WAB SI are following, [1]:

. Study the support of WAB including [RAN3, RAN2]: - Study the architecture and protocol stack of supporting a gNB with MT function providing PDU session backhaul. - Study impact of WAB mobility within an existing RAN (e.g., inter-gNB neighbour relations). - Identify necessary inter-gNB- and gNB-to-CN signalling to address the support of WAB. - Study signalling enhancements on resource multiplexing for WAB. NOTE 1: No impact on the UE. NOTE 2: Coordination with other WGs (e.g. SA2) when needed. The WAB study does not preclude any backhaul scenario (e.g. NTN or TN).

This contribution analyses WAB integration and mobility, and the support for RRC_INACTIVE.

# 2	NG interface during WAB mobility

Last RAN3 meeting, the following was agreed:

##### --- Start of TR38.799 text ---

#### 4.3.4.2	WAB-gNB mobility

During WAB-node movement, establishment, and removal of the WAB-gNB’s NG and/or Xn connections may be needed.

Editor’s NOTE: FFS whether NG connections can be suspended.

Establishment of Xn connections of the WAB-gNB with BH-RAN nodes, as well as with surrounding RAN nodes, is supported, and it can follow legacy procedures.

##### --- End of TR38.799 text ---

There is a similar EN in Section 4.3.2 (copied as below)

##### --- Start of TR38.799 text ---

### 4.3.2	WAB authorization

WAB authorization includes the authorization of the WAB-MT and the service authorization of the WAB-gNB. The authorization of the WAB-MT is different from the service authorization/configuration/activation of the WAB-gNB.

Authorization of the WAB-MT provides the WAB-MT with the right to support backhauling the co-located WAB-gNB’s traffic via BH PDU session(s).

Authorization of the WAB-gNB provides the service authorization, i.e., the right to serve UEs. The service authorization of the WAB-gNB is performed by e.g., OAM/SeGW using legacy procedures.

The WAB-gNB’s service authorization status may change. In case the WAB-gNB’s service authorization status changes from “authorized” to “not authorized”, the UEs served by the WAB-gNB can either be handed over to other RAN nodes or they can be released, after which the NG and Xn connection(s) of the WAB-gNB can be removed.

Editor’s NOTE: FFS whether NG connection(s) can be suspended.

##### --- End of TR38.799 text ---

When a WAB moves out the service area of the UE’s AMF and moves in the service area of UE’s new AMF, the WAB may need to disconnect from the old AMF and connect with the new AMF, up to the operator’s configuration. Current NGAP does not have a NG Removal procedure. This causes the WAB to unnecessarily keep the NG connection, IPSec tunnel and TNL association with the old AMF.

In addition, a WAB’s authorization status may be changed to not authorized. Orderly release is needed, in order not for the AMF to report the TNL connection lost after the WAB is changed to not authorized.

So it may be necessary to introduce a NG Removal procedure, which can be used to serve above scenarios. It is worthy to note that similar discussion is ongoing in NR NTN WI (including the discussions on NG suspension issue). The EN can be removed, and align with the discussion in NR NTN in the WI phase.

Proposal 1: Remove the EN in Section 4.3.2 and Section 4.3.4.2.

# 3	WAB-gNB mobility without change of UE’s AMF(s)

Last RAN3 meeting the following was agreed:

##### --- Start of TR38.799 text ---

##### 4.3.4.2.1 WAB-gNB mobility without change of UE’s AMF(s)

3CI and/or TAC) without the change of the UE’s AMF(s). This change of radio configuration parameters may require the UE handling by means of, e.g., intra-gNB handover and/or Mobility Registration Update as defined in TS 23.502.

Editor’s NOTE: Whether the TAC can be changed without the change of UE’s AMF needs to be confirmed.

##### --- End of TR38.799 text ---

As agreed by RAN3 in previous meeting that TAC of WAB-gNB’s cell need to reflect the WAB-node’s physical location.

The TAC/RANAC of WAB-gNB’s cell is configured by the OAM, and it can be reconfigured by the OAM during the mobility of WAB-node. The TAC/RANAC of the WAB-gNB’s cell may be the same as, or different than, the TAC/RANAC of the co-located WAB-MT’s serving cell. The TAC/RANAC broadcast by the WAB-gNB’s cell can be changed in order to reflect the WAB-node’s physical location.

The operator may configure a TAC to be used by WAB-gNB’s cell in a geographical area. When the WAB node moves within the coverage of UE’s AMF, its TAC may or may not be changed, up to operator’s plan and configuration.  In case the TAC is changed, the WAB-gNB provides the updated Supported TA List to UE’s AMF, and the UE follows normal mobility procedure (e.g. perform a Mobility Registration Update in case the new TAC does not belong to the assigned Registration Area).

Proposal 2: Remove the EN in Section 4.3.4.2.1

# 4	WAB-gNB mobility with change of UE’s AMF(s)

Last RAN3 meeting the following was agreed:

##### --- Start of TR38.799 text ---

##### 4.3.4.2.2 WAB-gNB mobility with change of UE’s AMF(s)

Due to WAB-node movement, the change of UE’s AMF(s) may be needed, based on, e.g., WAB-node’s current location and/or additional criteria. The NG connection handling and WAB-gNB configuration update may affect the served UEs.

- The WAB-gNB may obtain the configuration parameters needed to establish the connection to the UE’s new AMF(s).

- The WAB-gNB establishes NG connection(s) towards one or more new AMF(s).

Editor’s NOTE: FFS whether a new logical WAB-gNB needs to be established for the NG connections with the new AMF(s).

Editor’s NOTE: FFS whether the following steps are needed to support the AMF change for UE.

- The WAB-gNB may activate one or more new cells, with new cell configuration parameters related to the WAB-gNB’s current location. The new cells may broadcast the radio parameters configured for the new AMF(s), e.g., TAC, etc. The old cell(s) remain(s) active.

- The UEs are handled as follows:

- A UE in RRC_CONNECTED state is handed over between an old cell and a new cell served by the WAB-gNB via NG-based handover with AMF relocation, as defined in TS 23.502.

- When all UEs in RRC_CONNECTED state have been handed over, the old cell(s) are removed from service. A UE in RRC_IDLE or RRC_INACTIVE state camping on the old cell(s) reselects a new cell, and legacy procedure (e.g., Mobility Registration Update procedure as defined in TS 23.502) is performed.

- The NG connection(s) between the WAB-gNB and the old AMF(s) are removed.

##### --- End of TR38.799 text ---

RAN3 also agreed to further discuss following issues:

FFS if this procedure is needed for AMF relocation.

Discuss whether in case this procedure is used for AMF relocation, new cells have to belong to different logical gNBs or can belong to the same gNB.

Regarding the first FFS, the relocation of UE’s AMF is mainly caused by the WAB-gNB mobility. There is no need to have separate section/paragraph to describe the procedures to support the AMF change for UE.

SA2 had made the following conclusion:

The MWAB instantiates a new logical MWAB gNB when it is configured with a new TAC (and possibly Cell ID) and with a different gNB ID. This may be used to support migrating / handing over of UEs served by the MWAB to the new logical gNB. Once all UEs have been migrated, the MWAB removes the old logical MWAB-gNB instance. This can happen e.g. when AMF, serving the UEs of the MWAB, change is required based on the operator policy.

Observation 3-1: SA2 conclusion indicates that WAB-gNB initiates a new gNB when configured with a new TAC and a different gNB ID, e.g., in case of AMF change.

The main question left for RAN3 is whether it can be same logical gNB to setup NG with old AMF and new AMF.  This is possible. The main requirement is separate cells are required, i.e. separate “old cell” and “new cell”. Whether they belong to same logical gNB is just a matter of whether same gNB ID or different gNB ID is used in both old cell ID and new cell ID. A gNB may setup NG connection with multiple AMFs. In NG-Flex configuration, a gNB is connected to all AMFs of AMF Sets within an AMF Region supporting at least one slice also supported by the NG-RAN node. In NG-Flex case, the gNB uses same Supported TA List. In WAB case, the WAB-gNB connects with AMFs from different AMF Region, so the WAB-gNB shall use different Supported TA List for connecting with “Old AMF” and “New AMF”. It is not possible to use same Supported TA List for connecting with “Old AMF” and “New AMF”, since TAI is used for routing to different AMF. It may be possible to use same gNB ID, but should use different cell ID in order to be different cells to the UE.

Proposal 3-1: WAB-gNB shall use different Supported TA List for setting up NG connection with “New” AMF.

PCI (re-)configuration for WAB

When the mobile WAB moves, there may be a potential PCI collision between the cells from different WAB, or between WAB’s cell and a stationary gNB-DU’s cell. The WAB may need to change PCI for its cell. The connected UE will see the different cell before and after the PCI change. HO procedure can be used to move the UE from the “old” WAB cell using an old PCI and the “new” WAB cell using the new PCI. It can be considered as inter-cell intra-gNB HO. To support this HO, the WAB may need to have both “old” cell and “new” cell activate simultaneously.  This is similar to Rel-18 mobile IAB-DU migration that the UE is HO from the source logical IAB-DU’s cell to the co-located target logical IAB-DU’s cell.  After all connected UEs are HO to the new cell, the WAB-gNB may deactivate the old cell.

Proposal 3-2: Update TR38.799 to add the support for PCI change of a WAB’s cell, the WAB need to have two logical cells active simultaneously. The connected UE can be HO from the old cell to the new cell.

# 5	Change of backhaul resources during mobility

WAB-gNB resources/interfaces, e.g. NG, OAM, Xn, and on-board UE sessions may be mapped to different backhaul PDU sessions and network slices. During WAB-node mobility, the availability of different network slices and/or QoS resources from different BH-gNBs could change, which could impact the backhaul quality of the WAB-node and hence the resources of the WAB-gNB and the UEs connected to it (note that this problem could even happen when WAB is not moving, e.g. if the serving BH-gNB is experiencing congestion and needs to scale down BH resources).

Depending on which slices/resources are impacted the problem can be more severe. For example, if a slice or PDU session associated with the WAB-gNB’s NG interface can no longer be supported, there would be loss of connection to the UE’s 5GC, in which case both the WAB-gNB and the on-board UEs would be disconnected suddenly.

Observation 4-1: During WAB-node mobility, WAB-gNB resources can be degraded due different QoS resources and/or network slices that can be supported at BH-gNBs.

Therefore, it would be beneficial for the BH-gNB to be able to indicate to the WAB-gNB when backhaul service requirements can no longer be maintained so that the WAB-gNB can react accordingly, e.g. free up resources by releasing Xn with neighbouring nodes, handing over UEs to surrounding gNBs, remapping not supported slices/resources to supported slices/resources, etc.

Proposal 4-1: BH-gNB can indicate to the WAB-gNB that specific BH resources cannot be maintained.

In order to make this determination, it would be useful for the BH-gNB to know which slices/PDU sessions are associated with specific resources at the WAB-gNB (or to at least have some awareness of which slices/sessions should be prioritized). During handover preparation, such information could be used by the source BH-gNB to determine whether a candidate target BH-gNB can maintain BH service and inform the WAB-gNB if QoS is expected to be degraded.

Proposal 4-2: BH-gNB is aware which slices/PDU sessions are associated with specific resources at the WAB-gNB and may use this information to determine whether a target BH-gNB is able to maintain BH resources for the WAB-gNB.

# 6	Support for RRC INACTIVE

A UE connected to the WAB-gNB may transition to RRC INACTIVE, then move to the surrounding gNB’s coverage.  Similarly, a UE connected to the surrounding gNB may transition to RRC INACTIVE, then move to the WAB-gNB’s coverage. The RNA assigned to the UE may include the TAI/cell ID of WAB-gNB and surrounding gNB.

Observation 5-1: A UE’s RNA can consist of TAI(s)/cell ID(s) of WAB-gNB(s) and surrounding gNB(s).

Observation 5-2: A RRC_CONNECTED UE can transition to RRC_INACTIVE via WAB-gNB.

However, to support the Xn Retrieve UE Context procedure, the Xn Removal procedure should be carefuly designed to avoid the scenario where UE context cannot be retrieved due to released Xn interface between the target gNB and the WAB-gNB. This is especially an issue considering WAB’s are moving devices, and Xn may be established and removed dynamically as the WAB moves. In our view, there are at least a couple of ways to tackle the issue:

- “Too early Xn Removal”, i.e. Xn is removed before Xn Retrieve UE Context Procedure is avoided.

- In case Xn is to be released between the WAB-gNB and another gNB, whose cell(s) are in the RNA of at least one of the UEs that WAB-gNB sent to RRC_INACTIVE, UE context of the corresponding UE(s) are provided to the gNB by the WAB-gNB.

Considering Auguest meeting is the last meeting for the SI phase, the support for RRC_INACTIVE should be further discussed in the WI phase.

Proposal 5: RAN3 discuss the enhancement to avoid the UE context retrieval problems when Xn is removed between a WAB-gNB and a surrounding gNB.

# 5	Conclusion

In this contribution we have analyzed WAB mobility from RAN3 perspective.

Proposal 1: Remove the EN in Section 4.3.2 and Section 4.3.4.2.

Proposal 2: Remove the EN in Section 4.3.4.2.1

Proposal 3-1: WAB-gNB shall use different Supported TA List for setting up NG connection with “New” AMF.

Proposal 3-2: Update TR38.799 to add the support for PCI change of a WAB’s cell, the WAB need to have two logical cells active simultaneously. The connected UE can be HO from the old cell to the new cell.

Proposal 4-1: BH-gNB can indicate to the WAB-gNB that specific BH resources cannot be maintained.

Proposal 4-2: BH-gNB is aware which slices/PDU sessions are associated with specific resources at the WAB-gNB and may use this information to determine whether a target BH-gNB is able to maintain BH resources for the WAB-gNB.

Observation 5-1: A UE’s RNA can consist of TAI(s)/cell ID(s) of WAB-gNB(s) and surrounding gNB(s).

Observation 5-2: A RRC_CONNECTED UE can transition to RRC_INACTIVE via WAB-gNB.

Proposal 5: RAN3 discuss the enhancement to avoid the UE context retrieval problems when Xn is removed between a WAB-gNB and a surrounding gNB.

# References

- RP-234041 Study on additional topological enhancements for NR

- TR 23.700-06, Study on architecture enhancements for vehicle-mounted relays - Phase 2

- R3-24xxxx/S2-2407345, LS on questions regarding FS_VMR_Ph2

# Annex A – Text proposal for TR 38.799

### 4.3.2	WAB authorization

WAB authorization includes the authorization of the WAB-MT and the service authorization of the WAB-gNB. The authorization of the WAB-MT is different from the service authorization/configuration/activation of the WAB-gNB.

Authorization of the WAB-MT provides the WAB-MT with the right to support backhauling the co-located WAB-gNB’s traffic via BH PDU session(s).

Authorization of the WAB-gNB provides the service authorization, i.e., the right to serve UEs. The service authorization of the WAB-gNB is performed by e.g., OAM/SeGW using legacy procedures.

The WAB-gNB’s service authorization status may change. In case the WAB-gNB’s service authorization status changes from “authorized” to “not authorized”, the UEs served by the WAB-gNB can either be handed over to other RAN nodes or they can be released, after which the NG and Xn connection(s) of the WAB-gNB can be removed.

### 4.3.3 Configuration of WAB-node

Certain configurations of the WAB-node may need to be updated as the node moves, e.g.:

- The parameters that enable the WAB-gNB to select and connect to the AMF(s) to serve the UE(s).

- The parameters that enable the WAB-gNB to connect to, and communicate with, the OAM system.

- The configuration parameters that the WAB-gNB should broadcast, e.g., the TAC(s), the cell ID(s), the RANAC(s).

A WAB-node may be provisioned with the parameters pertinent to different potential locations of the WAB-node.

Alternatively, the OAM can provision configuration parameters to the WAB-node based on the location of the node. In that case the continuity of OAM connectivity needs to be ensured as the WAB-node moves.

#### 4.3.3.1 IP address configuration for WAB-gNB

A WAB-MT obtains IP address(es) for the PDU sessions in the same manner as a legacy UE.

The WAB-gNB can use the IP address(es) of the WAB-MT for the PDU sessions that backhaul the NG, Xn and OAM traffic. The WAB-gNB supports security protection of NG and Xn via IPsec, as defined by TS 33.501.

In case the WAB-gNB uses the IPsec tunnel mode to protect the OAM, NG and/or Xn traffic, the allocation of the inner tunnel IP address(es) is outside of 3GPP scope.

It is possible to transport OAM, NG or Xn traffic over other types of tunnel protocols on top of the WAB-MT’s PDU session(s), e.g., such as L2TP. In this case, the WAB-gNB uses different IP address(es) from WAB-MT. Since the support of these tunnel protocols is not defined for NG and/or Xn, such tunnel protocols are out-of-scope of this study.

#### 4.3.3.2	TAC/RANAC (re-)configuration for WAB-gNB’s cell

The TAC/RANAC of WAB-gNB’s cell is configured by the OAM, and it can be reconfigured by the OAM during the mobility of WAB-node. The TAC/RANAC of the WAB-gNB’s cell may be the same as, or different than, the TAC/RANAC of the co-located WAB-MT’s serving cell. The TAC/RANAC broadcast by the WAB-gNB’s cell can be changed in order to reflect the WAB-node’s physical location.

#### 4.3.3.x	PCI (re-)configuration for WAB-gNB’s cell

The PCI of WAB-gNB’s cell may need to be changed to avoid PCI collision during the mobility of WAB-node. Either OAM or WAB-gNB may decide to change the PCI.

- The WAB-gNB may activate one or more new cells using new PCI.

- The UEs are handled as follows:

- A UE in RRC_CONNECTED state is handed over between an old cell and a new cell served by the WAB-gNB via Xn-based handover, as defined in TS 38.300.

- When all UEs in RRC_CONNECTED state have been handed over, the old cell(s) are removed from service. A UE in RRC_IDLE or RRC_INACTIVE state camping on the old cell(s) reselects a new cell, and legacy procedure (e.g., Mobility Registration Update procedure as defined in TS 23.502) is performed.

### 4.3.4	Mobility handling

The following scenarios for WAB-node mobility are supported:

- The UE´s AMF/UPF remains unchanged as the WAB-gNB moves.

- The UE´s AMF/UPF changes as the WAB-gNB moves.

- The BH UPF/AMF remains unchanged as the WAB-MT moves inside a PLMN.

- The BH UPF/AMF changes as the WAB-MT moves inside a PLMN.

RAN3 assumes that the WAB-gNB does not change PLMN while the WAB-node moves across the network.

#### 4.3.4.1	WAB-MT mobility

The WAB-MT reuses legacy mobility handover procedures for the UE as it moves through the BH-RAN.

##### 4.3.4.1.1 The BH-UPF/BH-AMF remains unchanged as the WAB-MT moves inside a PLMN.

In case the WAB-MT’s PSA UPF does not change during these mobility procedures, the IP addresses allocated for the WAB-MT’s PDU sessions do not change. Therefore, the NG and Xn connections of the WAB-gNB carried over the BH PDU session(s) remain unaffected.

##### 4.3.4.1.2 BH-UPF/BH-AMF changes as the WAB-MT moves inside a PLMN.

When the WAB-MT’s PSA UPF need to be changed due to WAB-MT’s mobility, new BH PDU session(s) need to be established reusing existing mechanisms defined in TS 23.501/TS 23.502. In this case, the (outer) IP addresses used by the WAB-gNB for the transport of NG and Xn connections of the WAB-gNB will change.

##### 4.3.4.1.3 Roaming of the WAB-MT

During WAB-node movement, the WAB-MT may connect to a PLMN different than its HPLMN.

#### 4.3.4.2	WAB-gNB mobility

During WAB-node movement, establishment, and removal of the WAB-gNB’s NG and/or Xn connections may be needed.

Establishment of Xn connections of the WAB-gNB with BH-RAN nodes, as well as with surrounding RAN nodes, is supported, and it can follow legacy procedures.

##### 4.3.4.2.1 WAB-gNB mobility without change of UE’s AMF(s)

During WAB-node movement, radio configuration parameters of the WAB-gNB may be changed (e.g., cell ID, PCI and/or TAC) without the change of the UE’s AMF(s). This change of radio configuration parameters may require the UE handling by means of, e.g., intra-gNB handover and/or Mobility Registration Update as defined in TS 23.502.

##### 4.3.4.2.2 WAB-gNB mobility with change of UE’s AMF(s)

Due to WAB-node movement, the change of UE’s AMF(s) may be needed, based on, e.g., WAB-node’s current location and/or additional criteria. The NG connection handling and WAB-gNB configuration update may affect the served UEs.

- The WAB-gNB may obtain the configuration parameters needed to establish the connection to the UE’s new AMF(s).

- The WAB-gNB establishes NG connection(s) towards one or more new AMF(s). The WAB-gNB may use same or different gNB ID, but shall use different Supported TA List for establishing NG Connection with new AMF.

- The WAB-gNB may activate one or more new cells, with new cell configuration parameters related to the WAB-gNB’s current location. The new cells may broadcast the radio parameters configured for the new AMF(s), e.g., TAC, etc. The old cell(s) remain(s) active.

- The UEs are handled as follows:

- A UE in RRC_CONNECTED state is handed over between an old cell and a new cell served by the WAB-gNB via NG-based handover with AMF relocation, as defined in TS 23.502.

- When all UEs in RRC_CONNECTED state have been handed over, the old cell(s) are removed from service. A UE in RRC_IDLE or RRC_INACTIVE state camping on the old cell(s) reselects a new cell, and legacy procedure (e.g., Mobility Registration Update procedure as defined in TS 23.502) is performed.

- The NG connection(s) between the WAB-gNB and the old AMF(s) are removed.

### 4.3.5 Resource multiplexing

In scenarios where WAB-node’s access link and backhaul link mutually interfere, resource coordination may be needed to facilitate the resource multiplexing of the WAB-node’s access links and backhaul link. For this purpose, the resource coordination mechanism introduced for IAB can be considered as the starting point. For resource coordination between the access link and backhaul link, the BH-gNB may need to discover co-location of the WAB-MT and the WAB-gNB.

### 4.3.x1 support for RRC_INACTIVE

When a UE transition to RRC_INACTIVE via WAB-gNB or surrounding gNB, the UE’s RNA can consist of TAI(s)/cell ID(s) of WAB-gNB(s) and surrounding gNB(s). Enhancement may be considered to avoid the Xn removal before the Xn Retrieve UE Context Procedure is performed to/from the WAB-gNB, or proactively move the UE context to other gNB before the Xn with WAB-gNB is released.

## 4.4	Other