3GPP TSG-RAN WG3 Meeting #125	R3-244339

# Maastricht, Netherland, 19th - 23rd August 2024

Agenda item:	12.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on SA2 LS

Document for:	Discussion and Decision

# 1	Introduction

RAN3 received a SA2 LS with following questions:

Issue-1: SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases: a) MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and b) MWAB-UE accessing MWAB-gNB belonging to same MWAB. SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback. Issue-2: SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

This contribution analyses aspects related to questions in the SA2 LS.

# 2	Discussion on Issue-1

For scenario a), to prevent multi-hop, a WAB-MT cannot connect with any cell of a WAB-gNB. This is different to legacy IAB that a gNB-CU may have some cells belonging to IAB-DU, and other cells belonging to stationary gNB-DU. In WAB, it does not happen that some cell(s) of a gNB-CU belonging to WAB, and other cell(s) belonging to non-WAB.

There are multiple options

- Option 1: a WAB-gNB broadcast a new indication in SIB. By this indication, WAB-MT does not connect with the WAB-gNB

This option is similar to legacy IAB. However, this Option requires AS changes, and should be discussed in RAN2. It further requires mechanisms to disallow handover of WAB-MT to a WAB-gNB.

- Option 2: When the WAB-gNB initiates the NG Setup procedure with AMF, it can indicate it is a WAB-gNB. This enables the AMF to reject a further WAB-MT connect with the WAB-gNB (as in documented SA2 solution in the TR 23.700-06).

Note that SA2 conclusion indicates:

The MWAB-gNB indicates to the AMF(s) it is a MWAB when it establishes the N2 connection to the AMF(s).

NOTE 2: This requires RAN3 to define this indication.

Proposal 1-1: When the WAB-gNB initiates the NG Setup procedure with AMF, it indicates it is a WAB-gNB.

In order to avoid handover of the WAB-MT to a WAB-gNB, as SA2 had agreed, there will be specific slices for serving WAB-MT. For that reason, target gNB, i.e., WAB1-gNB can already see that those slices are there in the handover request for WAB2-MT, and reject the handover. This is similar to normal HO that target gNB rejects the PDU sessions associated to S-NSSAIs not supported by target NG-RAN. No enhancement is needed.

Another option would be to have such rejection from the AMF after path switch request is sent from WAB-gNB to AMF, but this means that the handover is already completed, so less preferred than the above slice-based option.

Proposal 1-2: Target WAB-gNB of a handover sees slices deployed for WAB-MTs in the WAB-MT’s handover request, and rejects the handover to avoid multi-hop in Xn/NG handovers.

For scenario b), WAB-UE and WAB-gNB are co-located, the WAB-UE can know whether a cell belongs to the co-located WAB-gNB by implementation.  This can be similar to legacy IAB/mobile IAB that the IAB-MT does not access the cell of the co-located IAB-DU.

Proposal 1-3: WAB-UE knows whether a cell belongs to the co-located WAB-gNB by implementation.

Proposal 1-4: For issue-1, RAN3 reply to SA2 with following:

a) SA2 agreed solution (as in documented SA2 solution in the TR 23.700-06) is feasible, i.e., when the WAB-gNB initiates the NG Setup procedure with AMF, it indicates it is a WAB-gNB. This enables the AMF to reject a further WAB-MT connect with the WAB-gNB.

To avoid multi-hops during handover of WAB-MT, target WAB-gNB of the handover sees slices deployed for WAB-MTs in the WAB-MT’s handover request, and rejects the handover which is similar to legacy HO that target gNB rejects the PDU session(s) associated to S-NSSAIs not supported by target gNB.

b) WAB-UE knows whether a cell belongs to the co-located WAB-gNB by implementation and does not connect to WAB-gNB belonging to same MWAB.

# 3	Discussion on Issue-2

SA2 conclusion indicates the MWAB-gNB provides to the AMF an Additional ULI. The Additional ULI is:

- TAC/Cell ID of the cell serving the MWAB-UE if the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are the same PLMN.

- If the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are not the same PLMN,

- option 1: mapped TAC/Cell ID based on geo-location of the MWAB based on input from OAM.

- option 2: geo-location of the MWAB.

- option 3: TAC/Cell ID of the cell serving the MWAB-UE in other PLMN.

The purpose of the additional ULI is to enable the UE’s CN to have a better knowledge of UE’s location and may perform location-based policy control.

For Option 1, it is unclear on what the Mapped TAI is, since TAI always corresponds to a geographic area. The Mapped Cell ID is used in NTN, but the WAB use case may be different to NTN. In NTN, a Uu cell ID corresponds to a very large geographic area and this geographical area can be dynamic when the satellite moves, thus requiring a Mapped Cell ID corresponds to a small fixed geographic area. In WAB, the TN cell (e.g. BH-gNB’s cell) corresponds to a small fixed geographic area, there is no need to introduce Mapped Cell.  In addition, Option 1 requires the WAB-gNB to know its geo-location, which is same as Option 2. Instead of providing the geo-location of WAB to CN (Option 2), Option 1 requires the WAB to map the geo-location to a Mapped Cell ID. So Option 2 is simple than Option 1.

For Option 3, it is similar to legacy IAB.

So it is preferred to adopt Option 2 or Option 3.

Proposal 2: For issue-2, RAN3 to reply to SA2 that Option 2 or Option 3 defined in clause 8.5 of TR 23.700-06 is preferred.

# 5	Conclusion

In this contribution we have analyzed issues raised by SA2. Our proposals are:

Proposal 1-1: When the WAB-gNB initiates the NG Setup procedure with AMF, it indicates it is a WAB-gNB.

Proposal 1-2: Target WAB-gNB of a handover sees slices deployed for WAB-MTs in the WAB-MT’s handover request, and rejects the handover to avoid multi-hop in Xn/NG handovers.

Proposal 1-3: WAB-UE knows whether a cell belongs to the co-located WAB-gNB by implementation.

Proposal  1-4: For issue-1, RAN3 reply to SA2 with following:

a) SA2 agreed solution (as in documented SA2 solution in the TR 23.700-06) is feasible, i.e., when the WAB-gNB initiates the NG Setup procedure with AMF, it indicates it is a WAB-gNB. This enables the AMF to reject a further WAB-MT connect with the WAB-gNB.

To avoid multi-hops during handover of WAB-MT, target WAB-gNB of the handover sees slices deployed for WAB-MTs in the WAB-MT’s handover request, and rejects the handover which is similar to legacy HO that target gNB rejects the PDU session(s) associated to S-NSSAIs not supported by target gNB.

b) WAB-UE knows whether a cell belongs to the co-located WAB-gNB by implementation and does not connect to WAB-gNB belonging to same MWAB.

Proposal 2: For issue-2, RAN3 to reply to SA2 that Option 2 or Option 3 defined in clause 8.5 of TR 23.700-06 is preferred.

Draft Reply LS can be found in Annex A – Text proposal for the reply-LS to SA2

# References

- RP-234041 Study on additional topological enhancements for NR

- TR 23.700-06, Study on architecture enhancements for vehicle-mounted relays - Phase 2

- R3-244019/S2-2407345, LS on questions regarding FS_VMR_Ph2

# Annex A – Text proposal for the reply-LS to SA2

3GPP TSG-RAN3 Meeting #125	R3-24xxxx

# Maastricht, Netherland, 19th - 23rd August 2024

Title:	Reply LS on LS on questions regarding FS_VMR_Ph2

Response to:	S2-2407345: LS on questions regarding FS_VMR_Ph2

Release:	Release 19

Work Item:	FS_VMR_Ph2

Source:	RAN3

To:	SA2

Cc:	RAN2

Contact person:	tba

Attachments:	None

# 1	RAN3 conclusions on requested information

RAN3 thanks SA2 for the LS in S2-2407345. RAN3 has analysed the issues and concluded following as the responses to SA2 questions.

SA2 Issue-1:

SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell)  for below two cases:

a)	MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and

b)	MWAB-UE accessing MWAB-gNB belonging to same MWAB.

SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

RAN3 Answer to Issue-1:

a) SA2 agreed solution (as in documented SA2 solution in the TR 23.700-06) is feasible, i.e., when the WAB-gNB initiates the NG Setup procedure with AMF, it indicates it is a WAB-gNB. This enables the AMF to reject a further WAB-MT connect with the WAB-gNB.

To avoid multi-hops during handover of WAB-MT, target WAB-gNB of the handover sees slices deployed for WAB-MTs in the WAB-MT’s handover request, and rejects the handover which is similar to legacy HO that target gNB rejects the PDU session(s) associated to S-NSSAIs not supported by target gNB.

b) WAB-UE knows whether a cell belongs to the co-located WAB-gNB by implementation and does not connect to WAB-gNB belonging to same MWAB.

SA2 Issue-2:

SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

RAN3 Answer to Issue-2: Option 2 or Option 3 defined in clause 8.5 of TR 23.700-06 is preferred.

# 2	Actions

To SA2:

ACTION: 	RAN3 kindly asks SA2 to take the above into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#125			2024-08-19 – 2024-08-23					Maastricht

RAN3#125bis		2024-10-14 – 2024-10-18					China