3GPP TSG-RAN WG3 Meeting #125	R3-244341

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	21.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to BL CR for TS 37.340) Discussion on the support of XR in NR-NR DC

Document for:	Discussion and Decision

# 1	Introduction

RAN3 received a LS ([1]) from SA2 with following questions to RAN3.

- Question1 [for SA4, RAN2 and RAN3]:

  - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach.

  - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension?

- Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

Last RAN3 meeting discussed the RAN3 impact to support XR DC, and there are some issues remaining open.

This contribution discusses the SA2 LS, and the remaining issues in RAN3.

# 2	Discussion on SA2 LS

For Question1, in case the UPF receives such information (i.e. periodicity) in RTP header, the UPF can convey such information via GTP-U to the gNB. It is up to SA4/RAN2 to decide the detail of such information. So for Question1, it is mainly in SA4/RAN2 scope.

Proposal 1-1: For Question1, reply SA2 that it is in SA4/RAN2 scope.

For Question3, it is actually related to RAN3 discussion in RAN3#124. There are different views on the available data rate for GBR, and the usage is unclear. With a compromise, last RAN3 meeting agreed the report of available data rate is feasible, but the usage of the available data rate is never clarified. For example, the QoS parameter for a GBR QoS Flow includes Guaranteed Flow Bit Rate (GFBR) and Maximum Flow Bit Rate (MFBR). TS23.501 defines

The GFBR denotes the bit rate that is guaranteed to be provided by the network to the QoS Flow over the Averaging Time Window. The MFBR limits the bit rate to the highest bit rate that is expected by the QoS Flow (e.g. excess traffic may get discarded or delayed by a rate shaping or policing function at the UE, RAN, UPF). Bit rates above the GFBR value and up to the MFBR value, may be provided with relative priority determined by the Priority Level of the QoS Flows (see clause 5.7.3.3).

It is unclear whether the available data rate is only below the GFBR, or between GFBR and MFBR, or even above MBFR. This needs to be clarified by SA2.

Observation 1: the usage of the available data rate is unclear. For example, whether the available data rate is only below the GFBR, or between GFBR and MFBR, or even above MBFR.

In case SA2 agreed to support the available data rate reporting, it requires enhancements to both CP and UP in RAN3.

- For Control Plane interface, the gNB need to be aware of which GBR QoS Flow is requested to report the available data rate.

In CU/DU split architecture, multiple QoS flows may be multiplexed in one DRB. The gNB-DU is only able to provide an estimation per DRB. This may add a new requirement to the implementation. For example, in case one QoS Flow requires estimation, gNB-CU should not multiplex this QoS Flows with other QoS Flows in the same DRB. It may be better that the DRB only support that QoS Flow, thus the estimation (per DRB) provided by the gNB-DU can be used to inform CN for the estimation of that QoS Flow.

It is possible that a handover is performed before the serving gNB completes the estimation, e.g. a Handover is performed during an Averaging Window. It may be better that target gNB re-start the estimation, without caring what source gNB has estimated. The request for available data rate reporting is transferred from source gNB to target gNB during the Xn handover preparation procedure, or from AMF to target gNB.

In case of DC, a QoS Flow may be supported by split bearer. Both MN and SN may estimate the available data rate, and it is up to the PDCP hosting gNB to combine them. This may be similar to ECN marking and congestion information exposure in DC.

- For User plane, both F1-U and NG-U need to be updated to convey the estimated data rate.

Overall, the required change is similar to enhancements to support ECN Marking and Congestion Information Exposure, i.e. via the control plane interface, a node requests the receiver node to perform available data rate reporting, and the receiver node report the available data rate via user plane. The request and report is per UL or DL direction.

Proposal 1-2: For Question3, reply to SA2 with following:

RAN3 would like SA2 to clarify the usage of the available data rate, e.g. whether the available data rate is only below the GFBR, or between GFBR and MFBR, or even above MBFR, etc. If it is required SA2, RAN3 can introduce enhancements to support the available data rate reporting.

# 3	End of Data Burst Indication and Indication of End PDU of the PDU Set

End of Data Burst Indication

Last meeting agreed the PDCP hosting gNB forward the End of Data Burst Indication to peer gNB, which can be used by the gNB to push the UE back to sleep when possible.

When the PDCP hosting gNB receives the End of Data Burst Indication of a QoS flow from the UPF, it always provides the End of Data Burst Indication of the QoS flow to the corresponding node when data transmission from the PDCP hosting node to the corresponding node is needed for the QoS flow.

Similar issue also exists in the handover case. For example,

- Source gNB received the PDU#3 and End of Data Burst Indication from UPF.

- Before handover, the End PDU (PDU#3) has been transmitted to UE successfully, but a PDU (PDU#2) before the End PDU have not been acknowledged from UE.

- In current data forwarding, the source gNB only forward PDU#2 to the target gNB.

Since the End of Data Burst Indication is not forwarded to the target gNB, the target gNB does not know the end of Data Burst and thus is not able to push the UE to sleep after handover.

Figure 1: Issue for End of Data Burst during handover

It is beneficial to forward the End of Data Burst indication to target gNB. The forwarding can be similar to the forwarding of the Data Burst Indication in DC. In case the source gNB forward one or more PDUs of the PDU set to target gNB, the source gNB should also provide the End of Data Burst indication to target gNB.

Proposal 2-1: During the data forwarding for handover, the source gNB provides the End of Data Burst Indication of the QoS flow to the target gNB in case one or more PDUs of the QoS flow is forwarded to target gNB.

TP to TS 38.300 can be found in Contribution ([2]).

Indication of End PDU of the PDU Set

The Indication of End PDU of the PDU Set is used by the gNB to support PDU Set based QoS handling. RAN3 agreed to provide the PDU Set QoS information to the SN, to enable the SN to perform PDU Set based QoS handling. To perform the PDU Set based QoS handling in the SN, SN need to know the End PDU of the PDU Set.

Observation 2-1: Since SN need to perform PDU Set based QoS handling, the SN need to know the End PDU of the PDU Set.

In case of split bearer is used, both MN and SN may still need to be aware of the Indication of End PDU of the PDU Set. For example, in case of MN-terminated split bearer, the MN receives PDU#3 and the Indication of End PDU of the PDU Set. MN send packet #3 to UE. SN does not know the Indication of End PDU of the PDU Set. The SN may not be able to perform the PDU set based QoS handling. So it is necessary that the PDCP Hosting gNB also forward the Indication of End PDU of the PDU Set to the peer gNB, which is similar to the forwarding of End of Data Burst Indication.

Figure 2: Example for MN-terminated split bearer with End of PDU

Proposal 2-2: Similar to the forwarding the End of Data Burst indication, the PDCP hosting gNB provides the Indication of End PDU of the PDU Set of the QoS flow to the corresponding node when data transmission from the PDCP hosting node to the corresponding node is needed for the QoS flow.

Update to the BL TS37.340 CR can be found in Annex B – TP to BL CR for TS 37.340.

# 4	Burst Arrival Time reporting

Last meeting agreed TS 37.340 TP to clarify how the SN use the received BAT, as below.

The UE may report uplink assistance information (jitter range, burst arrival time, UL data burst periodicity) per QoS flow via UE Assistance Information to the MN or the SN as configured. If the SN receives the burst arrival time, the SN may use it by considering the SFN offset of the MN.

The open question is whether SN can receive the BAT from MN. There is one comment that there is no use case for MN forward BAT to SN, with the following reason:

There is no UAIMCG in current inter-node message send from MN to SN i.e., only have UAIsourceSCG which is used for SN change. From other point view, UAI may help SN to configure (non-integer) DRX but whether SN would like to do such optimization is based on its own decision instead of MN. So we think if SN wants to receive UAI, it can trigger the UAI retrieval by itself, rather MN retrieval UAI and further send it to SN.

The comment may be a misunderstanding. It is true that the Inter-Node RRC message uses ueAssistanceInformationSourceSCG. However, the Inter-Node RRC messages defined in TS 38.331 Clause 11.2 is only used in following XnAP HO/DC messages:

- HANDOVER REQUEST

- HANDOVER REQUEST ACKNOWLEDGE

- S-NODE ADDITION REQUEST

- S-NODE ADDITION REQUEST ACKNOWLEDGE

- S-NODE RECONFIGURATION COMPLETE

- S-NODE MODIFICATION REQUEST

- S-NODE MODIFICATION REQUEST ACKNOWLEDGE

- S-NODE MODIFICATION REQUIRED

- S-NODE MODIFICATION REFUSE

- S-NODE CHANGE REQUIRED

- S-NODE CHANGE REQUIRED

- S-NODE RELEASE REQUEST

- RETRIEVE UE CONTEXT RESPONSE

Actually, the XnAP RRC TRANSFER message uses the RRC messages defined in TS 38.331 Clause 6.2.1

| UE Report |  | 0..1 |  |  | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >RRC Container | M |  | OCTET STRING | For NGEN-DC and NR-DC, includes the UL-DCCH-Message as defined in subclause 6.2.1 of TS 38.331 [10] containing the MeasurementReport message or the RRCReconfigurationComplete message or the FailureInformation message or the UEAssistanceInformation message. For NR-DC, includes the UL-DCCH-Message as defined in subclause 6.2.1 of TS 38.331 [10] containing the IABOtherInformation message. | – |  |

So the UEAssistanceInformation including BAT can be sent to the MN, and it is further forwarded to SN via Xn RRC Transfer procedure, e.g. via following procedure defined in TS 37.340:

Observation 3-1: SN can receive the UEAssistanceInformation including BAT from MN.

Current BL CR can cover this scenario. Small Stage-2 update may be needed to clarify that SN may receive the BAT from the MN or from the UE.

Proposal 3: Agree SN can receive the UEAssistanceInformation including BAT from MN.

# 5	DL PSI based discard

During the offline session of last meeting, companies agreed no enhancement is needed to support DL PSI based discard. However, this was missing in the summary ([4]). We propose to capture it as an agreement in the MCC notes.

Proposal 4: For DL PSI based discard, no enhancement is needed.

# 6	Conclusion

In this contribution, we discussed the RAN3 impact to support XR in DC. Our proposals are:

For SA2 LS:

Proposal 1-1: For Question1, reply SA2 that it is in SA4/RAN2 scope.

Proposal 1-2: For Question3, reply to SA2 with following:

RAN3 would like SA2 to clarify the usage of the available data rate, e.g. whether the available data rate is only below the GFBR, or between GFBR and MFBR, or even above MBFR, etc. If it is required SA2, RAN3 can introduce enhancements to support the available data rate reporting.

Proposal 2-1: During the data forwarding for handover, the source gNB provides the End of Data Burst Indication of the QoS flow to the target gNB in case one or more PDUs of the QoS flow is forwarded to target gNB.

TP to TS 38.300 can be found in Contribution ([2]).

Observation 2-1: Since SN need to perform PDU Set based QoS handling, the SN need to know the End PDU of the PDU Set.

Proposal 2-2: Similar to the forwarding the End of Data Burst indication, the PDCP hosting gNB provides the Indication of End PDU of the PDU Set of the QoS flow to the corresponding node when data transmission from the PDCP hosting node to the corresponding node is needed for the QoS flow.

Update to the BL TS37.340 CR can be found in Annex B – TP to BL CR for TS 37.340.

Observation 3-1: SN can receive the UEAssistanceInformation including BAT from MN.

Proposal 3: Agree SN can receive the UEAssistanceInformation including BAT from MN.

Proposal 4: For DL PSI based discard, no enhancement is needed.

# References

- R3-244020/S2-2407351, LS on FS_XRM Ph2

- R3-244109,	Support of XR enhancements	Nokia, Nokia Shanghai Bell, Lenovo, Qualcomm, Xiaomi, CATT, Ericsson, CMCC, Samsung

- R3-243173, R19 XR DC Signaling Enhancements  (Qualcomm Incorporated)

- R3-243820, Summary of Offline Discussion for CB: # XR2_NRDC

# Annex B – TP to BL CR for TS 37.340

### 13.x.2	Awareness

During the SN Addition Preparation procedure and the MN initiated SN modification procedure, the MN may send the PDU Set QoS Parameters as part of the QoS profile to the SN to enable PDU Set based QoS handling.

The UE may report uplink assistance information (jitter range, burst arrival time, UL data burst periodicity) per QoS flow via UE Assistance Information to the MN or the SN as configured. If the SN receives the burst arrival time, the SN may use it by considering the SFN offset of the MN.

When the PDCP hosting gNB receives the Indication of End PDU of the PDU Set or the End of Data Burst Indication of a QoS flow from the UPF, it always provides the Indication of End PDU of the PDU Set or the End of Data Burst Indication of the QoS flow to the corresponding node when data transmission from the PDCP hosting node to the corresponding node is needed for the QoS flow.