3GPP TSG-RAN WG3 Meeting #125	R3-244342

# Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	14.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to BL CR for TS 38.300) Discussion on the support of MBS Broadcast Service

Document for:	Discussion and Decision

# 1	Introduction

Last RAN3 agreed following:

WA: The mapped cell ID can be used to define MBS broadcast service in NTN.

Other options can be further checked.

This contribution analyses the issues and proposes a way forward.

# 2	Discussion

The main objective is to support the intended service area that is smaller than the coverage of a Uu cell. RAN3 need to specify the necessary signaling between CN and gNB to support this objective. Last meeting discussed the following options, and agreed the WA to use Option 1:

- Option 1: Mapped cell ID/list in MBS service area

- Option 2: New intended service area, e.g., smaller granularity for the MBS broadcast service area

- Option 3: Based on GNSS description

As discussed in [3],

- Option 1 has the benefit to reuse current MBS architecture and call flow, without requiring additional enhancements to CN nodes (e.g. AF, NEF/MBSF, MB-SMF, and AMF) and the various interfaces (e.g. Interface between AF and NEF/MBSF, Interface between NEF/MBSF and MB-SMF, Interface between MB-SMF and AMF, and Interface between AMF and NG-RAN).

- Option 2/3 require enhancements to above mentioned CN nodes and various interfaces.

It is beneficial to network vendor and operators to support NTN MBS by reusing existing network nodes and interfaces. So we propose turn the WA to an agreement.

Option 2 and Option 3 fulfil the same requirements as Option 1, but Option 2 and Option 3 requires enhancements to various CN nodes and interfaces. Since they fulfil the same requirement as Option 1, there is no need to develop multiple solution to fulfil the same requirements.

Proposal 1: turn the following WA to agreement

WA: The mapped cell ID can be used to define MBS broadcast service in NTN.

Proposal 2: No need to adopt Option 2 or Option 3.

Upon the reception of the MBS Service Area including Mapped Cell ID, since the gNB knows the mapping between geographical area and the Mapped Cell ID, the gNB can translate the Mapped Cell ID to Circle (like for TN coverage) or Geographical area information or any other small area information to be agreed by RAN2. Option 1 may only require a small Stage-2 update to clarify the Mapped Cell ID is used in MBS, and the gNB translate the Mapped Cell ID to small area information in SIB up to the gNB’s implementation.

TP for TS 38.300 can be found in Annex A – TP for TS 38.300

Proposal 3: adopt the TP for TS 38.300 BL CR to support NTN MBS

# 5	Conclusion

In this contribution, we discussed how to support MBS broadcast service in NR NTN. Our proposals are:

Proposal 1: turn the following WA to agreement

WA: The mapped cell ID can be used to define MBS broadcast service in NTN.

Proposal 2: No need to adopt Option 2 or Option 3.

TP for TS 38.300 can be found in Annex A – TP for TS 38.300

Proposal 3: adopt the TP for TS 38.300 BL CR to support NTN MBS

# References

- TS 38.300

- TS 38.413

- R3-243354, (TP for TS 38.300) Discussion on the support of MBS Broadcast Service (Nokia, Nokia Shanghai Bell)

# Annex A – TP for TS 38.300 BL CR

### 16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS.

-	The Cell Identity used for MBS Service Area.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.