3GPP TSG-RAN WG3 Meeting #125	R3-244350

Maastricht, NL, 19- 23 August 2024

Title: 	(TP for TR 38.743) TP on Continuous MDT feasibility

Source: 	Ericsson, InterDigital, Deutsche Telekom

Agenda item:	11.4

Document Type:	Discussion and decision

# TP to TR38.743

This TP mirrors the discussions and proposals in R3-244351.

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

## 5.4	Continuous MDT collection targeting the same UE across RRC states

Editor Note: Capture the description and its potential standard impacts.

### 5.4.1	Use Case Description

The problem of continuous data collection for management-based MDT can be described as follows: a UE in the NG-RAN can be configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) or when a UE is handed over between gNBs, the network does not have standardized means to select again the same UE for continuous MDT for subsequent MDT data collection.

The Data Collection continuity in this scenario can be split into two tasks as below:

-	Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

-	Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

Continuous MDT can be split in several different enhancement steps.

Step 1: Continuous MDT for Immediate MDT measurements

In order to correlate Immediate MDT reports and therefore achieve a continuous expression of Immediate MDT measurements, a UE identifier can be added to the trace file.

- In its simplest form, this can be achieved by adding the RAN UE ID to the Trace Record Header. This is already possible today as per section 5.2.2 of TS32.423, where the RAN UE ID is a parameter that can be added to the Trace Record Header. 
Re-using this feature could ensure correlation of Immediate MDT measurements collected by the UE while in RRC_Connected and while moving between cells of the same gNB, even if inter gNB-DU handovers are performed. Further, such approach enables correlation of Immediate MDT measurements also when a UE moves between RRC_Connected and RRC_Inactive, while remaining within the same gNB.
A small enhancement on top of this existing functionality could consist of enabling Immediate MDT measurement correlations by means of the RAN UE ID also possible when the UE transitions from RRC_Connected to RRC_Idle and back to RRC_Connected. This may be achieved by letting a gNB-CU-CP keep track of the association between RAN UE ID and other temporary UE identifiers. For example, the gNB-CU-CP could keep track of the mapping between RAN UE ID, S-TMSI and Masked IMEI-SV. The gNB-CU-CP would therefore assign the same RAN UE ID whenever a UE with a given S-TMSI and Masked IMEI SV moves to RRC_Connected.
This approach implies that Immediate MDT continuity is at least possible within a gNB.

- Alternatively, the UE identifier used to correlated Immediate MDT measurements could be the S-TMSI, as specified in TS28.558, where it is stated that the S-TMSI is associated to the M4, M5, M6, M7 measurements. With this approach, immediate MDT measurements could be correlated even if the UE moves across gNBs (assuming that the same UE can be selected for MDT configuration at the target NG-RAN node – see step 2 and 3) and so long as the S-TMSI remains unchanged (namely, so long as the UE remains within the same AMF Set).

Step 2: Enable correlation between Immediate MDT and Logged MDT reports

To additionally correlate immediate and logged MDT measurement reports, possible solutions could consist of adding a flag in the UE signalling when the UE moves from RRC_Idle to RRC_Conneected. 
This would enable a UE moving from RRC_Idle to RRC_Connected in a new NG-RAN node to be selected for continuous MDT. Methods in Step 1 would ensure correlation of the measurement reports at the new serving gNB with previous measurement reports.

Step 3: Enable correlation of MDT measurements during RRC_Connected mobility

To enable selection of a continuous MDT UE at the target NG-RAN node, after mobility has been carried out, possible solutions could consist of adding a flag in the HO signalling messages, e.g. in the HO Request, to indicate that the UE is a continuous MDT UE.
This would enable a UE moving from a source NG-RAN node to a target NG-RAN node to be selected for continuous MDT. Methods in Step 1 would ensure correlation of the measurement reports at the new serving gNB with previous measurement reports.

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

# 6	Conclusion

The AI/ML functionality and the following use cases are recommended to be specified in Rel-19 normative phase:

-	Continuous MDT including:

- Correlation of Immediate MDT measurements generated by a UE during a given time period

- Correlation of Immediate MDT and Logged MDT measurements generated by a UE during a given time period

- Identification of UEs selected for Continuous MDT at transitions to RRC_Connected state and after RRC_Connected mobility

Recommendations for each use case take the section of “Solutions and standard impacts” for each use case as basis.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>