3GPP TSG-RAN WG3 Meeting #R3-125	R3-244352

Maastricht, NL, 19 - 23 August 2024

Agenda Item:	11.4

Source:	Jio

Title: 	Network Energy Saving for EC in mobility

Document for:	Discussion

# Background

During Rel-19, new potential enhancements were proposed on document R3-243449 by Ericsson for the AI/ML-based Network Energy Saving use case, also stating that “have discussions on AI/ML based energy saving at the next meeting, where enhancements will be based on new solutions not discussed in Rel18….”

In this Discussion document, following the 3GPP TR 38.743 [4], the RAN3 #114bis-e R3-221329 [5] and 3GPP TR 37.817[6], we further elaborate the subject matter of Energy Cost Predictions in the scope of energy saving and propose some extra ideas. Moreover this document is also related to two Jio patents [7,8]

This Document is mirrored to the R3-244037 (TP for TR 38.743) Enhancements for AI/ML Energy Saving and the R3-244040 “Enhancements on AI-ML Network Energy Saving”.

# Energy Cost Prediction for Energy Efficient Mobility

## Energy Cost function considerations in Release 19

A key focus of Release 19 is the incorporation of advanced energy management techniques to handle increased traffic and the proliferation of connected devices. Energy efficiency in Next-Generation Radio Access Networks (NG-RAN) is crucial for sustainable network operations, especially as demand for mobile data continues to escalate. One important metric for managing energy consumption is the Energy Cost (EC) exchanged between NG-RAN nodes. This metric provides an inferred measure of energy consumption related to additional load, which can significantly influence mobility handovers. Understanding how EC is calculated and utilized can lead to more efficient network management and energy savings.

During Rel-18, RAN3 came to an understandingbut did not agree to itthat an Energy Cost (EC) prediction function or formula or content, which is to be exchanged between NG-RAN nodes, should be derived with respect to a tentative and potential offloading action between the nodes in order to be interpretable or actionable by the receiving node.

Energy Cost (EC) is a metric that quantifies the energy consumption associated with an additional load on a network node. It reflects how much more energy a node will consume when handling extra traffic, which is crucial for maintaining energy efficiency and ensuring optimal network performance.

EC provides an inferred measure of energy consumption by considering the following factors:

- Traffic Load: The amount of data traffic a node handles directly impacts its energy consumption. Higher traffic loads lead to increased power usage.

- Operational States: Different operational states (e.g., active, idle, sleep) have different energy profiles. The transition between these states, driven by traffic load, affects the overall energy consumption.

Observation 1:  An Energy Cost prediction should be combined with Traffic load and Operational status.

Energy cost function-based handover in 5G Release 19 represents a significant step towards more sustainable and cost-effective mobile networks. By incorporating energy efficiency into the handover decision process, this approach not only helps in reducing operational costs but also aligns with global environmental goals. However, successful implementation requires addressing the associated complexities and ensuring that QoS is not compromised.

Mobility handovers are transitions of user connections from one cell or node to another. EC can influence these handovers in several ways:

- Load Balancing: If a node's EC indicates high energy consumption due to increased traffic, the network can initiate handovers to redistribute the load. By offloading traffic to less burdened nodes, the network reduces overall energy consumption.

- Energy Optimization: Nodes with lower EC can handle additional traffic more efficiently. Handovers can be directed towards these nodes to optimize energy usage across the network.

- QoS Maintenance: Ensuring Quality of Service (QoS) is critical. High EC can lead to degraded performance, prompting handovers to maintain service quality without compromising energy efficiency.

Observation 2:  An Energy Cost prediction should be combined with Traffic load balancing and mobility actions.

An Energy Cost function in the context of mobility (handover) decisions quantifies the energy consumption associated with maintaining a user's connection during the handover process. This function aims to balance energy efficiency with network performance, ensuring that handovers do not lead to excessive energy consumption or degrade user experience.

There are several factors to consider, some of those are proposed here:

- Energy Efficiency: refers to minimizing the energy consumption of network elements while maintaining acceptable service quality.

  - Network Elements: Use energy-efficient hardware and software algorithms to reduce power consumption of base stations, core network components, and user devices.

  - Energy Consumption Metrics: Collect and analyze metrics such as the energy consumed per bit transmitted and the energy consumed per active user.

- Load Balancing: ensures that traffic is distributed evenly across network resources to avoid overloading specific nodes and to optimize resource utilization.

  - Load Metrics: Measure the load on each cell or base station in terms of user count, traffic volume, and resource utilization.

  - Balancing Algorithms: Apply algorithms to distribute users and traffic loads more evenly across available base stations. This can involve adjusting handover thresholds based on current load conditions.

- Traffic Load: Traffic load refers to the amount of data being transmitted or received by network nodes.

  - Real-Time Monitoring: Continuously monitor traffic load at different network nodes to make informed handover decisions.

  - Adaptive Thresholds: Set dynamic thresholds for handover decisions based on current traffic load to avoid unnecessary handovers and reduce energy consumption.

- Transmission Power: Transmission power is the amount of power used to send signals from a base station to user devices.

  - Power Control: Implement power control mechanisms to adjust the transmission power based on the distance between the base station and the user device and the current network conditions.

  - Energy-Efficient Protocols: Use protocols that optimize transmission power to reduce energy consumption while maintaining sufficient coverage and signal quality.

- User Throughput: User throughput is the rate at which data is successfully transmitted to or received by a user device.

  - Quality of Service (QoS): Ensure that handovers do not significantly impact user throughput by maintaining or improving QoS during the handover process.

  - Performance Metrics: Monitor and evaluate throughput before and after handover to ensure that performance is consistent with user expectations.

Proposal 1:  An Energy Cost prediction should include several factors in a weighted cost function optimization procedure. Factors in this cost function could be, among others, the energy efficiency, the load balancing considerations, the measured traffic load, the transmission power and the user throughput.

The Energy Cost function should integrate these factors to make optimal handover decisions. A possible approach is as follows:

E=λ1∙Eeff+λ2∙CLB+λ3∙CTL+λ4∙CTP+λ5∙CUThp

Where E  represent the energy cost, Eeffthe energy efficiency, CLBthe Load Balancing cost, CTL the traffic load cost, CTP the transmit power cost and CUThp the User Throughput cost. Moreover:

- Load Balancing Cost reflects the cost of redistributing traffic.

- Traffic Load Cost considers the impact of current traffic load.

- Transmission Power Cost is associated with the power needed for handover.

- User Throughput Cost measures the effect of handover on user throughput.

Implementing an Energy Cost function for handover decisions in 5G Release 19 requires a comprehensive approach that balances energy efficiency with performance factors such as load balancing, traffic load, transmission power, and user throughput. By carefully designing and optimizing this function, network operators can achieve significant improvements in both energy efficiency and overall network performance.

Proposal 2:  An Energy Cost prediction should optimize the energy cost function for proper mobility actions.

## EC Mobility Proposed Algorithm in Release 19

Implementing an Energy Cost function-assisted handover in 5G Release 19 involves a systematic approach to optimizing handover decisions based on multiple factors. Here’s a detailed step-by-step algorithm for implementing such a function, considering energy efficiency, load balancing, traffic load, transmission power, and user throughput.

- Data Collection

Step 1.1: Gather Network Metrics

- Energy Consumption Metrics: Collect real-time energy consumption data for each base station (e.g., energy per bit transmitted).

- Load Metrics: Monitor the current load on each base station, including the number of active users, traffic volume, and resource utilization (e.g., CPU, memory, bandwidth).

- Transmission Power: Measure the transmission power used by each base station and for each user connection.

- User Throughput: Record the throughput for each user connected to the base stations.

- Handover History: Store historical data on handover performance, including success rates, failure rates, and impact on user experience.

Step 1.2: Define Thresholds and Parameters

- Energy Efficiency Thresholds: Define thresholds for acceptable energy consumption per bit or per user.

- Load Balancing Parameters: Set parameters for balancing load, such as the maximum allowable load per base station.

- Transmission Power Limits: Define maximum and minimum transmission power levels.

- Throughput Requirements: Specify minimum throughput requirements for acceptable user experience.

- Energy Cost Function Definition

Step 2.1: Define Cost Components

- Energy Efficiency Cost Eeff​: Evaluate the energy consumption per bit or user for each base station.

Eeff​(i)=Energy Consumptionbit​(i)×Traffic Volume(i)

- Load Balancing Cost: Assess the cost associated with redistributing traffic from one base station to another.

|Load Balancing Cost(i,j)=∣Load(i)−Load(j)∣

- Traffic Load Cost: Consider the impact of the current traffic load on the base stations.

Traffic Load Cost(i)=Traffic Load(i)×Cost per Bit(i)

- Transmission Power Cost: Calculate the cost related to the transmission power required for handover.

Transmission Power Cost(i,j)=Powerused​(i)+Powerrequired​(j)

- User Throughput Cost: Evaluate the impact on user throughput due to the handover.

User Throughput Cost(i,j)=∣Throughputbefore​(i)−Throughputafter​(j)∣

Step 2.2: Define the Energy Cost Function

Combine the individual cost components into a single Energy Cost function E(i,j)

- Handover Decision Algorithm

Step 3.1: Identify Potential Handover Targets

- Candidate Selection: List all potential target base stations for handover based on user location and signal strength.

Step 3.2: Calculate Costs for Each Candidate

- For each potential target base station j, compute the total energy cost E(i,j) where i is the current base station.

Step 3.3: Evaluate and Select Optimal Target

- Optimal Target Selection: Choose the base station j that minimizes the energy cost function E(i,j).

j=argminjEi,j

Step 3.4: Perform Handover

- Initiate Handover: Carry out the handover to the selected base station j.

- Monitor Handover: Continuously monitor the handover process to ensure success and adjust if necessary.

- Post-Handover Evaluation

Step 4.1: Monitor Performance

- Evaluate Metrics: Assess the impact of the handover on energy efficiency, load balancing, traffic load, transmission power, and user throughput.

- Collect Feedback: Gather feedback on user experience and performance metrics.

Step 4.2: Update Parameters

- Adjust Weights: Based on performance data, adjust the weights λi  in the Energy Cost function.

- Refine Function: Modify the Energy Cost function as needed to improve decision-making accuracy.

Step 4.3: Continuous Improvement

- Data Analysis: Analyze historical data to refine the handover algorithm and improve overall efficiency.

- Algorithm Update: Periodically update the algorithm based on new insights and evolving network conditions.

Proposal 3:  By following these detailed steps, the Energy Cost function-assisted handover algorithm can effectively balance energy efficiency with other critical network factors, leading to optimized handover decisions in 5G networks.

# Conclusions

In this contribution, we discussed open issues and potential enhancements of the AI/ML based Network Energy Saving use case and made the following observations and proposals:

Observation 1:  An Energy Cost prediction should be combined with Traffic load and Operational status.

Observation 2:  An Energy Cost prediction should be combined with Traffic load balancing and mobility actions.

Proposal 1:  An Energy Cost prediction should include several factors in a weighted cost function optimization procedure. Factors in this cost function could be, among others, the energy efficiency, the load balancing considerations, the measured traffic load, the transmission power and the user throughput.

Proposal 2:  An Energy Cost prediction should optimize the energy cost function for proper mobility actions.

Proposal 3:  By following these detailed steps, the Energy Cost function-assisted handover algorithm can effectively balance energy efficiency with other critical network factors, leading to optimized handover decisions in 5G networks.

# Reference

[1]	RP-234054	New SID: Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

[2]	R3-242073	AI/ML Network Energy Saving, Ericsson

[3] Jio white paper on Network energy saving

[4] 3GPP TR 38.743

[5] R3-221329 Intel Corporation, TP to 37.817 on AI/ML based network energy saving

[6] 3GPP TR 37.817 Study on enhancement for Data Collection for NR and EN-DC (Release 17)

[7] Provisional patent 1

[8] Provisional patent 2