3GPP TSG-RAN WG3 #125																			   R3-244355

# Maastricht, The Netherlands, 19-23 August 2024

Agenda Item:	13.2

Source:	Ericsson

Title:	Further discussion on XnAP signaling for inter-CU LTM

Document for:	Discussion, Agreement

# Introduction

In relation to inter-CU LTM support for standalone scenario, the following agreements and FFS have been captured at previous RAN3 meetings:

Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first.

Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation.

Early data forwarding can be supported for inter-CU LTM.

Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution.

Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

Introduce a new procedure on Xn to transfer the TA information.

A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages.

A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS.

FFS on the LTM modification procedures.

FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

In this contribution, we further discuss aspects of Xn signaling design to support inter-CU LTM.

# Discussion

## General principles

Some general principles should be considered to design Xn signaling for inter-CU LTM:

- Independence from gNB internal architecture: from a logical perspective, a peer gNB is seen as a “black-box”. In general, the Xn interface has no notion of the logical nodes constituting a gNB (gNB-DU, gNB-CU), and as a consequence, transfer of information that relates to (or is associated with) such logical functions has no meaning over Xn. A direct implication of this is that an IE such as “LTM gNB-DU ID” defined in F1 should not be signaled over Xn.

- Minimize delay: since LTM aims to minimize mobility interruption times using some information (e.g., TA values) that can be short-lived, signaling this type of information over Xn should avoid unnecessary delays. For example, transferring a TA value calculated for a given UE should be done as quick as possible.

From the above, it is clear that Xn signaling design can consider choices made for F1 signaling design, but some differentiations are needed. More details are presented in the following sections.

- For inter-CU LTM, Xn has no notion of the logical nodes of a gNB (gNB-DU, gNB-CU).

- Keep the existing principle that Xn AP Identities and F1 AP Identities are independent.

- For inter-CU LTM, unnecessary delay should be avoided over Xn when designing the signalling (e.g., to transfer TA values).

## Initial request for LTM configuration

According to agreements taken at previous meetings, Handover Request and Handover Request Ack should be reused for Inter-CU LTM initial preparation, and the source gNB-CU initiates the handover preparation for inter-gNB LTM:

Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation.

Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

Some general aspects remain to be decided. In particular, we think the group should continue to discuss which of the two options below is to be used:

- a single Handover preparation that allows LTM preparation of multiple cells, or

- a single Handover preparation that allows LTM preparation of (only) one cell

If option A is selected, and the candidate gNB controls multiple LTM candidate cells, the source gNB can send a single HO Request message to a candidate gNB for LTM preparation of such cells.

If option B is selected, and the candidate gNB controls multiple LTM candidate cells, the source gNB sends multiple HO Requests to the candidate gNB, one for each candidate cell.

To select which one of the two options is more appropriate, we would like RAN3 to discuss the following aspects: 1) Handling of UE association(s), 2) Error Handling. A preliminary analysis in presented in the table below.

| Item | Option A (One HO Request for LTM preparation of multiple cells) | Option B (One HO Request for LTM preparation of (only) one cell) |
| --- | --- | --- |
| Handling of UE Association(s) | Only one UE association is established, regardless of the number of candidate cells. The target gNB, receiving the HO Request knows that all the candidate cells for which LTM is prepared are associated to a unique UE, the one referenced by the Source NG-RAN node UE XnAP ID reference IE. The handling of UE associations is simple. | In principle, a different UE association can be established for the same UE for each request to prepare a different candidate cell for that UE. Should this be allowed or not? If yes, what are the implications on the solution? For example, the target gNB receives for the same UE two HO Request, the first one for candidate cell 1 and containing a first Source NG-RAN node UE XnAP ID reference, and a second one for candidate cell 2 and containing a second Source NG-RAN node UE XnAP ID reference. How does the target gNB understand that the two Source NG-RAN node UE XnAP ID reference(s) point to the same UE? |
| Error handling | The error handling is simple, since it relates to a single procedure. For example, there can be only one value for non-LTM related IEs included in HO Request (e.g., GUAMI). | The error handling is complex, since it needs to consider that the same IE can be received, potentially with different values, across the multiple procedures. For examples, the non-LTM related IEs included in the multiple HO Requests may differ (e.g., different GUAMI). This requires more complex handling from implementation point of view. |

Based on the presented analysis, we think that one HO Request for LTM preparation of multiple candidate cells is preferrable as it provides simpler implementation.

- For inter-CU LTM configuration, enhance the Handover Preparation procedure to handle multiple LTM candidate cells.

## TA information transfer

During Early UL synchronization, the source gNB sends a PDCCH order to the UE, instructing the UE to send a RA Preamble towards a cell of a candidate gNB, so that the candidate gNB can calculate a TA to be later used by the UE at LTM execution. The source gNB knows which RACH resources are used by the UE when it sends the RA Preamble to the candidate, in particular the RA Preamble index and the RACH occasion (RO), since those were previously assigned by the candidate gNB and the source gNB received them during the LTM preparation, as per the agreement below:

Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages.

For intra-CU LTM, the DU-CU/CU-DU TA INFORMATION TRANSFER messages contain the following information, agreed at RAN3#121-bis:

- Source DU ID

- TA value

- Preamble index,

- Candidate cell ID.

- RO information (i.e. RA-RNTI)

A further clarification (applicable to F1 as well) is that the “RO information” cannot be captured as the RA-RNTI. This is because during Early UL synch, the UE only sends a RA preamble, i.e., the RAR is not part of this step. According to MAC specification, an RA-RNTI is only calculated in relation to the RAR, meaning that an RA-RNTI is not available. Also, an RA-RNTI would be calculated by the candidate cell, i.e., the source gNB is not able to understand which UE is associated with the RA-RNTI. Therefore, even if sending an “RO information” might be correct to uniquely identify the UE, we think that this part needs to be revisited. When the F1AP TA Information Transfer procedure is initiated, the RA-RNTI cannot be transferred together with the TA value. This applies to the already existing F1AP procedure, and needs to be considered for the newly introduced procedure to for TA information Transfer via Xn. RAN2 should be consulted for the appropriate correction.

- RA-RNTI should not be included in the TA Information Transfer procedures.

- Send an LS in Annex A to RAN2 and asks to reconsider the agreement on transferring RO information.

For proper design of the Xn message for transferring TA value (e.g., “TA information Transfer”) we think some clarifications are needed.

First, it should be discussed whether the procedure for TA value transfer should be UE-associated (to transfer only one TA value), or non-UE associated procedure, to transfer more than one TA values in one go.

A certain TA value is always associated to only one UE, therefore the UE associated identities assigned by the source gNB and the candidate target gNB should be included in the message, to uniquely identify the UE to which the TA value corresponds to.

- Include the UE XnAP IDs of the source and target gNB in the message for TA information transfer.

A TA value can be regarded as short-lived, i.e., it may become invalid in a short time. For this reason, it is crucial to avoid unnecessary delay to transfer this information via Xn. Some companies proposed to use a non-UE associated message to transfer more than one TA values in one go. However if this approach is used, before the message is sent, we will introduce an unnecessary delay due to the different times at which the individual TA values to be included in the message becomes available. For example, if a non-UE associated message is used to transfer 8 TA values, there is an additional delay introduced, from the time when the 1st TA value becomes available, until the time when the 8th TA value becomes available. There is a high chance that when the TA values need to be used they are not valid anymore, and the advantage of the Early UL synch is completely lost. If we use a UE-associated procedure instead, there is no extra delay, at the expense of an increased number of messages to be sent over Xn. We think that the cost of sending more messages is justified, otherwise the whole Early UL synch step becomes useless. In summary, even if existing F1 messages (DU-CU/DU-CU TA Information Transfer) used to transfer TA values are non-UE associated, there is no need to use the same approach for Xn, and we prefer to use a UE-associated procedure.

- The Xn procedure to transfer a TA information is UE-associated.

Regarding whether the procedure should be a class 1 or a class 2, in this case we think the same approach used in F1 can be kept, i.e. use a class 2 procedure.

- The Xn procedure to transfer a TA information is a class 2 procedure.

Regarding the presence or not of a “Source DU ID” in Xn, we have already mentioned that a gNB has no visibility of whether a peer gNB is deployed in non-split architecture or in split architecture, therefore a Source DU ID (or similar IE) cannot be included in the Xn message for TA value transfer.

- For inter-CU LTM, do not transfer via Xn information revealing gNB split architecture implementation (e.g., Source DU ID).

Another important topic to be addressed is the ongoing discussions regarding the allocation of the preamble indexes. In Release 18 intra-CU LTM, the allocation of preamble indexes takes place during the UE Context Setup and UE Context Modification procedures. We believe that the impact of assigning preamble indexes to multiple UEs has not been fully evaluated. The preamble indexes are provided in a list format on a per-cell basis within the Early UL Sync Configuration information over F1. In practice, for a certain candidate cell, a preamble index is associated with a certain UE, and this association remains until the UE switches to the candidate cell. Since the number of preamble indexes is limited (64) the source gNB-DU can receive from the candidate gNB-DU(s) the same preamble index for multiple UEs. When this happens, the source gNB-DU needs to make sure that those UEs for which the received preamble index is the same for a given candidate cell do not perform Early UL sync simultaneously. The source gNB-DU needs to ensure to reassign the same preamble index to another UE only once it confirms that the Early UL Sync procedure for the previous UE is completed. In other words, the source gNB-DU needs to track the use of the preamble index for each UE. While this issue may not be severe under the F1 interface, it could become critical in Rel-19 when the limited number of preamble indexes is managed over Xn. If the approach used for F1 would be used for Xn, the preamble indexes become depleted rapidly, and the burden at the source gNB-DU will become excessive. To prevent scalability issues, a more efficient procedure for requesting a preamble index is recommended.

When there is a trigger in the entity in the source DU or gNB serving the UE to perform an uplink synchronisation measurement one preamble, from a pool of NP preambles is allocated specifically for this UE. The random variable T describes the time a preamble is allocated. If there is no preamble available in the pool, the DU or gNB considers this a failure. Further details of the model and an example is provided in Appendix B where the following observation is made:

- The number of preambles needed for uplink synchronization scales poorly with the number of preamble pools.

Therefore, triggering preamble index allocation based on a request per UE is preferred.

- The existing design of preamble index allocation in Release 18 for intra-CU case would lead to scalability issues.

- Preamble index is allocated in the candidate node upon request from the source node as per UE basis in the new signalings.

## LTM modification

At last meeting, an FFS has been captured in relation to the need for modification of LTM candidate cells: “FFS on the LTM modification procedures.”

We think that a simple approach is preferrable, i.e., to achieve the modification of an existing LTM configuration, the existing one can be released and a new one setup. This simple approach is also used in other cases, e.g., for CPAC.

- To modify an existing LTM configuration, the LTM configuration is released, and a new one is setup.

## LTM cancel

At RAN3#124 the following agreement was taken: “A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS.”.

First, in our view it is reasonable that the source gNB can initiate the cancellation of configured LTM candidate cells. To reflect this, we propose to modify the previous agreement to reflect that.

- The source gNB and a candidate gNB can initiate cancellation of configured LTM candidate cell(s).

To resolve the FFS, we propose to RAN3 to discuss two options, presented below.

Option 1: reuse CHO approach: with this option, same procedures specified for CHO for handover cancel are reused. The source gNB will send a HANDOVER CANCEL to the candidate gNB to cancel one or more configured LTM candidate cells, and a candidate gNB will send a CONDITIONAL HANDOVER CANCEL to the source gNB to cancel one or more configured LTM candidate cells.

Option 2: introduce new LTM procedures: with this option, LTM candidate cells are cancelled following to the same logic used in F1 and NG to release a UE context. 
For UE context release, in F1/NG, either 1) the CU/AMF commands the DU/NG-RAN to release the UE Context, and the DU/NG-RAN executes it (class 1 procedure), or 2) the DU/NG-RAN requests the CU/AMF to initiate the release of the UE Context (class 2 procedure), and this triggers the CU/AMF-initiated command to release. 
To cancel configured LTM candidate cells, the same logic can be used, i.e., either 1) source gNB commands the candidate gNB to cancel a configured LTM candidate cell, and the candidate executes it, or 2) the candidate gNB requests the source gNB to cancel the configured LTM candidate cell, and this triggers the source gNB to initiate the cancellation.

In comparison, Option 2 appears to be a more straightforward approach. It avoids creating confusion with the existing HO and CHO procedures.

- RAN3 to discuss the following two options for cancelling configured LTM candidate cells:

        - Option 1: reuse the HANDOVER CANCEL message (from source gNB to candidate gNB) and the CONDITIONAL HANDOVER CANCEL message (from candidate gNB to source gNB)

        - Option 2: introduce new LTM procedures, to support the following:
- the source gNB sends a command to a candidate gNB to initiate the cancellation of LTM candidate cell, 
- a candidate gNB sends a request to the source gNB to cancel the configured LTM candidate cell, and then the source gNB initiates the cancellation of LTM candidate cell.

## Release of UE Context at source gNB

At last meeting the following point is left FFS: “FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.”.

We think that if no LTM candidate cell is left in the source gNB, it is reasonable to release the corresponding UE context. Therefore, we propose to remove the FFS and reuse the existing UE CONTEXT RELEASE message for this purpose.

- Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

- Agree on the TPs to TS 38.423 and TS 38.300 in [1][2].

# Conclusions and Proposals

In the previous sections we made the following observations:

Observation 1	For inter-CU LTM, Xn has no notion of the logical nodes of a gNB (gNB-DU, gNB-CU).

Observation 2	The number of preambles needed for uplink synchronization scales poorly with the number of preamble pools.

Observation 3	The existing design of preamble index allocation in Release 18 for intra-CU case would lead to scalability issues.

In this paper we propose:

Proposal 1	Keep the existing principle that Xn AP Identities and F1 AP Identities are independent.

Proposal 2	For inter-CU LTM, unnecessary delay should be avoided over Xn when designing the signalling (e.g., to transfer TA values).

Proposal 3	For inter-CU LTM configuration, enhance the Handover Preparation procedure to handle multiple LTM candidate cells.

Proposal 4	RA-RNTI should not be included in the TA Information Transfer procedures.

Proposal 5	Send an LS in Annex A to RAN2 and asks to reconsider the agreement on transferring RO information.

Proposal 6	Include the UE XnAP IDs of the source and target gNB in the message for TA information transfer.

Proposal 7	The Xn procedure to transfer a TA information is UE-associated.

Proposal 8	The Xn procedure to transfer a TA information is a class 2 procedure.

Proposal 9	For inter-CU LTM, do not transfer via Xn information revealing gNB split architecture implementation (e.g., Source DU ID).

Proposal 10	Preamble index is allocated in the candidate node upon request from the source node as per UE basis in the new signalings.

Proposal 11	To modify an existing LTM configuration, the LTM configuration is released, and a new one is setup.

Proposal 12	The source gNB and a candidate gNB can initiate cancellation of configured LTM candidate cell(s).

Proposal 13	RAN3 to discuss the following two options for cancelling configured LTM candidate cells:

	Option 1: reuse the HANDOVER CANCEL message (from source gNB to candidate gNB) and the CONDITIONAL HANDOVER CANCEL message (from candidate gNB to source gNB)

	Option 2: introduce new LTM procedures, to support the following: - the source gNB sends a command to a candidate gNB to initiate the cancellation of LTM candidate cell,  - a candidate gNB sends a request to the source gNB to cancel the configured LTM candidate cell, and then the source gNB initiates the cancellation of LTM candidate cell.

Proposal 14	Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

Proposal 15	Agree on the TPs to TS 38.423 and TS 38.300 in [1][2].

# References

[1] R3-244356, (TP for TS 38.423) Xn support for inter-CU LTM, Ericsson

[2] R3-244357, (TP for TS 38.300) Stage-2 signalings for inter-CU LTM, Ericsson

[3] Erlang Loss Formula: https://en.wikipedia.org/wiki/Erlang_(unit)#:~:text=The%20Erlang%20B%20formula%20(or,M%2FM%2Fc%2Fc

# Annex A

3GPP TSG-RAN WG3 #125		R3-24xxxx

Maastricht, NL, 19- 23 August 2024

Title:	[DRAFT] LS on the necessity of RA-RNTI within TA Information Transferring

Response to:

Release:	Rel-19

Work Item:	NR_Mob_Ph4-Core

Source:	Ericsson [to become RAN3]

To:	RAN2

Cc:

Contact Person:

#### Name:	Liwei Qiu

E-mail Address:	liwei.qiu@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

1. Overall Description:

RAN3 has introduced RA-RNTI in the TA values transfer procedure to F1AP in Rel-18 based on RAN2’s agreements. Nonetheless, RA-RNTI is not assigned without the RAR at the time when this procedure is initiated. Thus, RAN3 assumes that such information should not be transferred in the TA Information Transfer messages.

As a result, RAN3 favors removing RA-RNTI from the TA information transfer procedure and suggests that RAN2 reconsiders the agreement on transferring RA-RNTI as RO information via network signaling.

2. Actions:

To RAN2 group.

ACTION: 	RAN3 asks RAN2 to help provide the feedback on the above.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #125-bis	14 - 18 October 2024			Hefei, China

# Appendix B

The model is a M/G/k loss system where customers arrive to a system consisting of k identical servers according to a Poisson process with a rate λ. If at least one server is available the customer enters the system, otherwise the customer is lost to the system. The service time in a server follows some distribution G. This model is known as Erlang’s loss formula [3].

The purpose here is to make some estimates comparing the difference when using one large common pool of preambles in the candidate cell compared to allocating a set of preambles per DU. In the former case the preambles are retrieved one by one using UE associated signalling over Xn and in the latter case the preamble is retrieved internally the DU. The metric used for capacity is the number of UEs which can be served while the probability that all preambles are in use (corresponding to no available server) is smaller than some requirement.

Model:

- Service time T: Random variable with distribution G describing the time a preamble is allocated:

- Mean service time E[T] = 0.02s

- Five different requirements analysed for maximum allowed probability that all preambles are busy: 10-2 , 10-3, 10-4 , 10-5

Figure 3: The y-axis shows the maximum number of requests per second, for a pool of the size shown on the x-axis, given different requirements 10-2, 10-3, 10-4 and 10-5. The average time a preamble is allocated is 0.02s.

From Figure 3 we see that at least two preambles are required to meet the requirement that the probability for all preambles being busy at 10-2 and 10-3. For the requirements 10-4 and 10-5 at least three preambles are required. In this example, we assume that we need to design for a capacity of 100 uplink synchronizations per second with the requirement 10-4. Comparing the cases that we use one large pool, which would be located in the candidate DU, and three small pools, which each is located in a DU which can be a serving DU performing mobility to the candidate cell, we get the following results:

- One large pool: 10 preambles because max capacity is 113 requests/s) when the requirement is 10-4.

- Three small pools: 6 preambles per pool because 6 preambles correspond to max capacity 36 requests/s when the requirement is 10-4.

We note that one pool with 10 preambles have a similar capacity as 3 pools with 6 preambles each i.e. 10 preambles vs 18 preambles. However, keeping the pool in the candidate DU may be faster since no signalling over Xn is required. If we assume that pooling in the candidate DU gives twice the expected time (0.04s) compared to pooling in the serving DU the result becomes according to Figure 4.

Figure 4: Max capacity when the service time is increased from 0.02s to 0.04s.

To fulfil the capacity requirement 100 requests/s while the probability is less than 10-4, 14 preambles are required compared to 10 for the service time 0.02s.  This time penalty does however still result in better capacity compared to three smaller pools since 14 is smaller than 18.

Observation: The number of preambles needed for uplink synchronization scales poorly with the number of preamble pools.