3GPP TSG-RAN WG3 Meeting #125	R3-244358

Maastricht, Netherlands, 19th – 23th August, 2024

Title:	Discussion on UE Location Information for NB-IoT NTN

Agenda Item:	9.1

Source:	Huawei, Ericsson

Document for:	Discussion

# 1	Introduction

RAN3 received the Reply LS from SA2 on UE Location Information for NB-IoT NTN in RAN3 #123bis [1], with the following question.

To RAN2, RAN3: ACTION: SA2 asks RAN2 and RAN3 whether they think it may be useful for MME to further signal the coarse location information received from the UE in NAS back to eNB e.g. using one of the existing Location Reporting Procedure in order for the eNB to send updated ULI to MME

As observed from [1], SA2 agreed that the NB-IoT UE can report the coarse location information via NAS (SMC) to MME, and from the MME to the E-SMLC. SA2 asked RAN2 and RAN3 whether it may be useful for MME to further signal the coarse location information from the UE in NAS back to eNB.

In RAN3 #124, companies discussed this issue but there was no consensus. In this contribution, we further provide our views on SA2’s question from RAN3 perspective.

# 2	Discussion

As asked in the LS, SA2 asked RAN2 and RAN3 whether it may be useful for MME to further signal the coarse location information from UE in NAS back to eNB:

In addition SA2 also observes that the ULI information ("mapped” cell-id and TAC) that will be provided from eNB to MME and stored in CN may potentially not be very accurate given the cells may be quite large and the UE in case of NB-IoT will not be providing coarse location information to eNB. SA2 would therefore like to ask whether RAN2 and RAN3 think it may be useful for MME to further signal the coarse location information received from the UE in NAS back to eNB e.g. using one of the existing Location Reporting Procedure in order for the eNB to send updated ULI to CN. SA2 did not reflect this change in the attached SA2 approved TS 23.401 CR and would like to receive feedback from RAN2 and RAN3.

Basically, there is no requirement from SA2, neither from RAN to report back the UE coarse location to the eNB. We observe RAN2 replied to SA2 that it “may” be useful for the eNB if the MME further signals the coarse location information back to the eNB [2], but RAN2 did not give the specific evidence or the use case. Some companies in RAN3 proposed that it is useful to decide the target for release with redirection [3], and further during RAN3#124 it was discussed the case of load balancing. But in our view, we still do not think it is really beneficial, for the following reasons:

- Huge signalling overhead

The eNB has vague idea of the UE location until it really receives the UE coarse location. In NTN scenario, there may be a large number of UEs under the satellite coverage, but only the edge UEs’ location are needed for e.g. a load balancing. The vast majority of the signalling back the coarse location is useless. In particular, associated to the fact that the satellite or the UE might move, it is unclear if this information on coarse location will be accurate and need to be transmitted only once at UE attached or one later… This leads to a situation where a huge amount of signalling will be achieved for few UEs.

- Huge signalling overhead is needed when every served UE’s location would be reported to the eNB, but only few of the UEs’ location may be useful.

- Hard to identify the cell border

Even though the UE coarse location is transmitted back to the eNB, the eNB needs to know the relative location (e.g., distance) between the UE location and the its cell border, and the relative location (e.g., distance) between the UE location and the neighbour cell border, to identify whether the UE is suitable to be redirected to the neighbour cell. However, the cell coverage is actually not accurately regulated (especially for moving cell), and it is much tricky for the eNB to know the cell borders.

- It is much tricky for the eNB to know the cell borders for identifying whether the UE is suitable to be redirected to the neighbour cell.

- The eNB has no knowledge of the mapping of the location to the radio conditions

At last, even if the eNB can finally know the UE is near the cell border, the eNB has no knowledge of the mapping of the location to the radio conditions. It is not scientific for the eNB to determine how suitable the target cell/target frequency is just according to the UE coarse location. The coarse location under a neighbouring does not guarantee the good signalling to this neighbour. That’s also the main reason why RRM is never based on location, but on radio signal quality from different nodes. That’s also probably the reason why the discussion on SON for load balancing of idle UE has never been concluded neither lead to standard; this remains to statistical approach.

And considering there is neither handover nor measurement report for such NB-IoT UE, legacy behaviours seems reasonable, i.e., the blind redirection supported for load balancing (potentially with a dedicated offset for the frequency where the UE is redirected to), as specified in TS 36.300, or the eNB just releases the UE and then the UE makes a cell reselection autonomously. The eNB may just “learn” on possibility to balance the UE to other node via cell re-selection indication to the UE. An OAM process will be able e.g. to count and balance the UE able to be redirected. A slow learning process based on OAM e.g. release pseudo randomly a set of UE (e.g. 5%) and redirect them to the best target neighbour should be also efficient for the balancing (assuming that the UE not able to attached to the neighbour will be back). The legacy approach could not be avoided, and the coarse location might only be part of this pseudo random set of UE selection, which from discussion above remains random anyway. Also for such NB-IoT UEs, only applying nomadic mobility, the cell planning should roughly know the cell satellite location based on ephemeris, then without extra signalling, power, and cost, it helps a legacy release and new cell re-selection…

- eNB has no knowledge of the mapping of the location to the radio conditions. The radio signalling quality is always used in RRM process rather than the Location information. Based on OAM process, the eNB may be able to learn the UE who are able to re-select other neighbours.

Based on the observations above, there is no need for this information in the RAN.

- No need for the MME to signal back the UE coarse location to eNB.

In addition, if the MME has acquired the UE coarse location via the NAS from the UE, then the MME can perform the related actions based on the coarse location directly. Also, if the MME really needs to have the mapped cell ID information, the MME can easily derive it by implementation, e.g., asking the LMF. Therefore, the ULI report does not provide much help.

- No need for the eNB to report any updated ULI to the MME.

In summary, there is no need for the MME to signal back the UE coarse location to the eNB, and report any updated ULI again.

# 3	Conclusion

This document provides our observation and proposals on the UE location reporting issue in NB-IoT NTN:

- Huge signalling overhead is needed when every served UE’s location would be reported to the eNB, but only few of the UEs’ location may be useful.

- It is much tricky for the eNB to know the cell borders for identifying whether the UE is suitable to be redirected to the neighbour cell. The radio signalling quality is always used in RRM process rather than the Location information.

- eNB has no knowledge of the mapping of the location to the radio conditions. Based on OAM process, the eNB should be able to learn the UE who are able to re-select other neighbours.

- No need for the MME to signal back the UE coarse location to eNB.

- No need for the eNB to report any updated ULI to the MME.

# 4	References

- S2-2403851/R3-241523, Reply LS on UE Location Information for NB-IoT NTN, SA2

- R2-2403770/R3-243009, Reply LS on UE Location Information for NB-IoT NTN, RAN2

- R3-243554, (TP for TS36.300) Discussion on UE Location for NTN NB-IoT UE, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, RAN3 #124