3GPP TSG- RAN WG3 Meeting #125	R3-244359

# Maastricht, Netherlands, 19th – 23rd, August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0160 | rev | - | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Value UE Rx-Tx Time Difference ASN.1 presence correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, CATT, ZTE, Samsung, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The presence of the Value UE Rx-Tx Time Difference IE set as choice tag, within the E-CID Measurement Result, cannot be optional in ASN.1 and it must be corrected to mandatory. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the ASN.1 to mandate the presence of the Value UE Rx-Tx Time Difference IE within the E-CID Measurement Result. Impact Analysis: The CR has a protocol and function impact. The impact is limited to the positioning E-CID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect ASN.1 encoding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### //****** Beginning Informative no change******//

### 9.2.5	E-CID Measurement Result

The purpose of the E-CID Measurement Result information element is to provide the E-CID measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Serving Cell ID | M |  | NG-RAN CGI 9.2.6 | NG-RAN Cell Identifier of the serving cell | - |  |
| Serving Cell TAC | M |  | TAC 9.2.11 | Tracking Area Code of the serving cell | - |  |
| NG-RAN Access Point Position | O |  | 9.2.10 | The configured estimated geographical position of the antenna of the cell. If the Geographical Coordinates IE is used, the NG-RAN Access Point Position IE shall be ignored. | - |  |
| Measured Results |  | 0..1 |  | Measurement results of the serving RAT. | - |  |
| >CHOICE Measured Results Value |  | 1 ..<maxnoMeas> |  |  | - |  |
| >>Value Angle of Arrival EUTRA |  |  | INTEGER (0..719) | According to mapping in TS 36.133 [9] |  |  |
| ### //****** Omitted no change******// |  |  |  |  |  |  |
| >>Value Timing Advance NR |  |  | INTEGER (0.. 7690) | Report mapping as defined in TS 38.133 [16] | YES | ignore |
| >>Value UE Rx-Tx Time Difference |  |  | INTEGER (0.. 61565) | As defined in TS 38.215 [19] | YES | ignore |
| Geographical Coordinates | O |  | 9.2.46 |  | YES | ignore |
| Mobile Access Point Location Information | O |  | Mobile TRP Location Information 9.2.88 | The location information of the mobile access point of the cell that is associated to the mobile TRP. | YES | ignore |
| Measured Results Associated Information List |  | 0..1 |  | The Measured Results Associated Information Item IEs are in the same order as the Measured Results IEs. | YES | ignore |
| >Measured Results Associated Information Item |  | 1..<maxnoMeas> |  |  | - |  |
| >>Time Stamp | O |  | 9.2.42 | Time Stamp of the corresponding measured result. | - |  |
| >>Measurement Quality | O |  | 9.2.43 | Measurement Quality of the corresponding measured result. | - |  |

### //****** Beginning of Change ******//

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

### //****** Omitted no Change ******//

MeasuredResultsAssociatedInfoItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

MeasuredResultsValue ::= CHOICE {

valueAngleOfArrival-EUTRA				INTEGER (0..719),

valueTimingAdvanceType1-EUTRA			INTEGER (0..7690),

valueTimingAdvanceType2-EUTRA			INTEGER (0..7690),

resultRSRP-EUTRA						ResultRSRP-EUTRA,

resultRSRQ-EUTRA						ResultRSRQ-EUTRA,

choice-Extension			ProtocolIE-Single-Container {{ MeasuredResultsValue-ExtensionIE }}

}

MeasuredResultsValue-ExtensionIE NRPPA-PROTOCOL-IES ::= {

{ ID id-ResultSS-RSRP		CRITICALITY ignore	TYPE ResultSS-RSRP		PRESENCE mandatory	}|

{ ID id-ResultSS-RSRQ		CRITICALITY ignore	TYPE ResultSS-RSRQ		PRESENCE mandatory	}|

{ ID id-ResultCSI-RSRP		CRITICALITY ignore	TYPE ResultCSI-RSRP		PRESENCE mandatory	}|

{ ID id-ResultCSI-RSRQ		CRITICALITY ignore	TYPE ResultCSI-RSRQ		PRESENCE mandatory	}|

{ ID id-AngleOfArrivalNR	CRITICALITY ignore	TYPE UL-AoA				PRESENCE mandatory	}|

{ ID id-NR-TADV				CRITICALITY ignore	TYPE NR-TADV			PRESENCE mandatory	}|

{ ID id-UE-Rx-Tx-Time-Diff 	CRITICALITY ignore	TYPE UE-Rx-Tx-Time-Diff PRESENCE mandatory	},

...

}

Mobile-TRP-LocationInformation ::= SEQUENCE {

location-Information			OCTET STRING,

velocity-Information			OCTET STRING	OPTIONAL,

location-time-stamp				TimeStamp	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Mobile-TRP-LocationInformation-ExtIEs} } OPTIONAL,

...

}

Mobile-TRP-LocationInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

Mobile-IAB-MT-UE-ID ::= OCTET STRING

### //****** End of Change ******//