3GPP TSG-RAN WG3 Meeting #125	R3-244363

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	14.2

Source:	Huawei

Title:	(TP for TS 38.413 and TS 38.300) Support MBS broadcast service

Document for:	discussion

# 1	Introduction

In order to indicate the intended service area via NG interface for MBS broadcast service, the last RAN3#124 meeting agreed on the following work assumption, and would further check other options.

WA: The mapped cell ID can be used to define MBS broadcast service in NTN.

Other options can be further checked.

At the same time, RAN2-125 meeting made the following agreements, but did not decide yet the final geographical area format.

Agreements:

- For MBS broadcast service we don’t restrict the work to any satellite constellation type

- We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS

- We will cover at least the case where the indicated intended service area covers a portion of a NTN cell

- The intended service area can cover the area of more than one NTN cells (or portions thereof)

- Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended serive area)

- At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered):

- Circles (like for TN coverage)

- Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

Taking above into account, this contribution provides our considerations.

# 2	Discussion

From RAN2 agreements above, it can be observed that the Uu cell could be bigger than the intended service area, and the network would inform the UE to associate each MBS session with its intended service area. And RAN2 would continue to discuss that the UE outside the intended service area should not receive the MBS data.  Also, RAN2 agreed the following geographical area formats can be further considered.

- Circles (like for TN coverage)

- Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

Note: that information (circles, polygons) is enough to describe a mapped cell.

It is RAN3 work to pass the intended service area from AMF to the gNB so as to pass such area information to UEs. According to current NGAP specification, the MBS service area information is included in the MBS broadcast session related procedure, where the MBS service area information indicated from the CN to the gNB could be TA list or cell list.

#### 9.3.1.208 MBS Service Area This IE contains the MBS service area. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE Session Type; M. >location independent. >>MBS Service Area Information; M; 9.3.1.209. >location dependent. >>MBS Service Area Information List; 1. >>>MBS Service Area Information Item; 1..<maxnoofMBSServiceArea Information>. >>>>MBS Area Session ID; M; 9.3.1.207. >>>>MBS Service Area Information; M; 9.3.1.209. Range bound; Explanation. maxnoofMBSServiceAreaInformation; Maximum no. of MBS Service Area Information elements in the MBS Service Area Information Location Dependent List IE. Value is 256.. #### 9.3.1.209 MBS Service Area Information This IE contains MBS service area information. IE/Group Name; Presence; Range; IE type and reference; Semantics description. MBS Service Area Cell List; 0..<maxnoofCellsforMBS>. >NR CGI; M; 9.3.1.7. MBS Service Area TAI List; 0..<maxnoofTAIforMBS>. >TAI; M; 9.3.3.11. Range bound; Explanation. maxnoofCellsforMBS; Maximum no. of cells allowed within one MBS Service Area. Value is 8192.. maxnoofTAIforMBS; Maximum no. of TAs allowed within one MBS Service Area. Value is 1024..

## 2.1	Intended MBS broadcast service area

There are several options as follows.

- The working assumption of the mapped cell ID

The previous RAN3 meeting initially made the working assumption to reuse the mapped cell ID as the intended areas for NTN. It is our view that this WA can be turned into agreement. That is, the CGI between the gNB and core network for MBS service area information can correspond to the mapped cell ID. Then the gNB could map and translate the received mapped cell ID from the CN, into the intended area format broadcasted over the Uu interface e.g., via circles, polygons, to better approximate the intended shape of service area.

- Turn the WA to agreement, i.e., the mapped cell ID can be used to define MBS broadcast service area in NTN.

- Other options including the new defined geographic area, and TAI list

Under some circumstances, the mapped cell ID may not work well for MBS broadcast service. As indicated above, RAN2 confirms that intended area could be at least one portion of a Uu cell, or could be the area of more than one NTN Uu cells (or portions thereof), these areas may not exactly coincide with mapped cell ID area.  Hence, a new geographic area information can be defined over NG interface, and its formats can refer RAN2 definitions. In this case the gNB does not need to perform any mapping.

- A new geographic area can be defined for MBS broadcast service area, and its encoding refers to RAN2 specification.

In addition, the existing TAI list could also be reused to indicate the intended broadcast service, which is even bigger than Uu cells, e.g., covering multiple Uu cells. This is already supported by the specification.

- RAN3 to confirm that the TAI list can be used as intended MBS broadcast service area.

Note that multiple options can be supported, and it is up to the CN to indicate the intended service area to the gNB.

## 2.2 Stage 2 TP and Stage 3 TP

On stage 2 TP, it is suggested to remove and clean-up the Mapped Cell usage, to avoid to list all. The basic assumption is that the Mapped Cell are used for CN and all, unless a restriction is explicated. This is also beneficial to avoid making changes for each feature introduced in the future for NTN, as usual good practice in RAN3.

Based on the discussion in section 2.1, a new MBS Service Area Geographic Area List IE can be introduced in the 9.3.1.209	MBS Service Area Information, for which its format can refer to TS 38.331, with FFS to be further checked with other WGs.

- RAN3 to agree the NGAP TP and the stage 2 TP provided in the Annex.

## 2.3 MBS broadcast procedures

With the satellite movement, the MBS broadcast session resources need to be setup or removed in time. Typically, when the satellite covers the intended service area, the MBS broadcast session can be setup, otherwise, it can be released.

Currently, the related MBS Broadcast Session Management Messages over the NGAP can at least include:

- Broadcast Session Setup procedure: To request the NG-RAN node to setup MBS session resources for a broadcast MBS session

- Broadcast Session Release procedure: To release the MBS session resources related to a previously established broadcast MBS session

- Broadcast Session Release Required procedure: To trigger the AMF to release the MBS session resources related to a previously established broadcast MBS session.

It seems to us that the existing broadcast session procedures can be used for NTN to setup or release the broadcast session, without further changes.

- RAN3 to assume that no further update of MBS broadcast procedures is needed to enable the intended MBS service area for NTN.

# 3	Conclusion

In this paper, we provide the following observations and proposals:

- Turn the WA to agreement, i.e., the mapped cell ID can be used to define MBS broadcast service area in NTN.

- A new geographic area can be defined for MBS broadcast service area, and its encoding refers to RAN2 specification.

- RAN3 to confirm that the TAI list can be used as intended MBS broadcast service area.

- RAN3 to agree the NGAP TP and the stage 2 TP provided in the Annex.

- RAN3 to assume that no further update of MBS broadcast procedures is needed to enable the intended MBS service area for NTN.

# Annex1	Text Proposal for TS 38.413

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.209	MBS Service Area Information

This IE contains MBS service area information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Service Area Cell List |  | 0..<maxnoofCellsforMBS> |  |  | - |  |
| >NR CGI | M |  | 9.3.1.7 |  | - |  |
| MBS Service Area TAI List |  | 0..<maxnoofTAIforMBS> |  |  | - |  |
| >TAI | M |  | 9.3.3.11 |  | - |  |
| MBS Service Area Geographic Area List |  | 0..<maxnoofGeographicAreaforMBS> |  | Used only for NTN | YES | ignore |
| >Geographic Area | M |  | OCTET STRING | Encoded in the same format as the [FFS] IE as defined in TS 38.331 [18]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsforMBS | Maximum no. of cells allowed within one MBS Service Area. Value is 8192. |
| maxnoofTAIforMBS | Maximum no. of TAs allowed within one MBS Service Area. Value is 1024. |
| maxnoofGeographicAreaforMBS | Maximum no. of geographic area allowed within one MBS Service Area. Value is FFS. |

Editor’s Note: The details of the MBS Service Area Geographic Area List are to be further studied by RAN2.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex2	Text Proposal for TS 38.300

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported, except for:

-

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID.

-

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.

The MBS broadcast service is supported over the intended service area. The intended service area can be in a form of cell ID list, TAI list, or geographic area list or any of combinations as defined in TS 38.413 [26].

Editor’s Note: The above paragraphs may be reformulated, and the geographic area is also pending to RAN2 decision.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>