3GPP TSG-RAN WG3 Meeting #125	R3-244365

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	14.3

Source:	Huawei

Title:	(TP for BLCR for TS 38.300) Support of regenerative payload

Document for:	discussion

# 1	Introduction

In the last RAN3 meeting, the regenerative architecture and NG interface management messages were discussed, but no agreement was achieved. The divergence is mainly about whether to introduce the NG removal/suspend/resume procedures.

In addition, some companies discussed the INACTIVE enhancement in both previous RAN3 meeting and RAN2 meeting, and RAN2 agreement also indicates RAN3 to first discuss this issue and provide some inputs if any [1]:

Regarding potential issues with support of Inactive state, we wait for RAN3 input, if any.

In this paper, we continue to discuss the NG impact and the inactive support in the regenerative payload.

# 2	Discussion

## 2.1 SA2 SI conclusion

SA2 completed and concluded the support of regenerative payload in TR 23.700-29 as follows.

## 8.1 Conclusion for KI#1: Support of Regenerative-based satellite access The conclusion for supporting gNB/eNB on-board the satellite is based on the following principles: - When the eNB/gNB leaves the service area of an AMF/MME (e.g. when setting over the horizon) any new procedures or updates to existing procedures to handle the N2/S1 connection will be defined by RAN WG3. SA WG2 will align its specifications once this has been determined by RAN WG3. - gNB /eNB IP address change due to feeder link switch can be supported using existing N2 and S1 procedures. There are no normative impacts to SA WG2 specifications due to this. SA WG2 will align its specifications if new solution is developed by RAN WG3. - RAN transitioning between different NTN GWs, TAIs and potentially between MMEs/AMFs can be supported by using existing procedures, e.g. the eNB/gNB releasing UEs to (E)CM_IDLE before changing NTN gateway, or by using existing load balancing, trigger handover when it realizes it is going to leave a tracking area due to its own movement, etc. - AMF/MME can treat the Mapped Cell IDs as for Rel-17. - Knowledge of when regenerative payload type could be used to assist with CN PDB determination and policy decisions. Introducing new RAT types is not required and it is assumed deployments can determine the usage of regenerative payload type if needed.

First it can be observed that there is no new architecture on the regenerative payload is introduced. Hence the existing RAN/CN architecture can be used. And the figure on the Overall illustration of an NTN with regenerative payload hosting a gNB in the BLCR for TS 38.300 is fine.

Second, SA2 does not have any requirement on existing procedures when the gNB leaves the service area of an AMF, and will aligned on RAN3, if any from RAN3.

Observation 1: SA2 SI concluded to not introduce new RAN/CN architecture nor requirements on the regenerative payload. SA2 will update their specification, if any enhancement from RAN3.

## 2.2 NG impact

### 2.1.1 AMF change

Below we discusse whether to introduce the NG suspend/resume or dedicated procedure to remove NG.

- NG suspend/resume procedure

There is no strong need for introducing NG suspend/resume procedures. The only advantage for the NG suspend/resume procedure is a little signalling overhead reduction when establishing the NG connection with the same AMF again and not re-establishing the interface context (NG Configurations). However, the NG suspend operation requires the gNB stores those NG configurations related to multiple AMFs. Considering the earth rotation, it may take a long time for the gNB connects to the same AMF again, which causes much storage consumption on the gNB for retaining multiple AMFs’ NG configurations, particularly when the gNB/Satellite should use this time to power up the battery..

Observation 2: NG suspend/resume operation requires the gNB to store NG configurations relevant to multiple AMFs, which leads to much storage consumption on the regenerative payload/gNB.

- No need to introduce the NG suspend/resume towards the AMF.

- Dedicated procedure to remove NG interface

There are two options to tear down the NG interface.

- Graceful SCTP Shutdown chunk

- NG removal procedure

First the disconnection of the feeder link can lead to the natural NG release between the gNB and the CN, without any new procedure. And considering the deterministic satellite orbit and earth rotation, the NTN Control function can easily determine the point in time when the feeder link switch over between the gNB and the CN is performed. As this time is well predictable (via ephemeris), the gNB can tear down SCTP connection by sending a SHUTDOWN chunk to gracefully end the association with its peer and receives a SHUTDOWN-ACK so as to tear down the NG interface.

On the other hand, introducing the NG removal procedure also seems no harmful, e.g., used for OAM maintenance and statistics. It is proposed:

- The NG interface tear down can be accomplished via existing graceful SCTP SHUTDOWN chunk procedure. And introducing new NG removal procedure seems no harmful.

### 2.1.2 Feeder link switch

As described by the second bullet in SA2 LS [2], during the soft feeder link switch, the gNB IP/TNL address may be changed. We understand this can happen at least for no AMF change case, and agree with solution #35 in SA2 TR that such case can be supported by existing procedures.

Specifically, the solution #35 in SA2 TR 23.700-29 for this case is listed as follows.

---Excerpt from TR 23.700-29--- Scenario 1 (no AMF/MME change): In case of deployment scenario 1, the TNL between gNB/eNB and AMF/MME would change while the gNB/eNB is serving a specific set of cells and Tracking Areas. To update the TNL association between gNB/eNB and AMF/MME different options are possible based on the existing standard: - Support for multiple TNL associations is available in 5GS (i.e. N2), as described in TS 23.501 [2] and TS 38.413 [12]. In this case the gNB can initiate a new TNL association via the new feederlink and remove the old TNL association that was active over the old feederlink. - Support for replacing an old N2/S1 interface with a new N2/S1 interface while preserving the active UE contexts is supported for both N2 and S1, as described in TS 38.413 [12] and TS 36.413 [14]. In this case the gNB/eNB establishes a new SCTP association via the new feederlink and sends a N2/S1 SETUP Request including the UE Retention Information IE. The AMF/MME accepts to retain the existing UE related contexts and signalling connections. The AMF/MME sends a N2/S1 SETUP Response and includes the UE Retention Information IE. - To update the N3/S1-U GTP-U tunnels with the new gNB/eNB TNL address, the gNB/eNB can provide the new tunnel endpoint information (F-TEIDs) to the core network by sending PDU SESSION RESOURCE MODIFY INDICATION / E-RAB MODIFICATION INDICATION. ---End of excerpt---

And as observed from section 2.1, SA2 has concluded that gNB /eNB IP address change due to feeder link switch can be supported using existing N2 and S1 procedures.

For NG interface, as specified in TS 38.412, multiple TNL association can be bound to one NG interface and it is easily using a new TNL association to replace the old one. Another solution is establishing a new NG interface to the same AMF pool with the UE Retention Information IE, so that all UE context can be kept and reused by the new NG interface.

For the AMF change case, NG-Flex TS 38.401 is always possible and also applicable in same AMF pool. It is always allowing to setup a new NG interface. Also, it is possible to trigger the CN based handover and/or the release procedure towards UEs to trigger connect to another AMF.

With regards to SA2 LS, all these solutions are applicable to eNB/MME, when existing.

- RAN3 to confirm that existing procedures can be supported by the gNB in feeder link switch. And for AMF change case, the RAN can trigger the NG handover or UE context release procedure for its served UE as legacy.

## 2.2	INACTIVE supporting

The paging and UE context relocation for INACTIVE UEs are relying on the Xn interface. Due to the satellite mobility when there is no Xn interface between the UE’s last serving gNB and the UE’s current camped gNB, the RAN paging would fail.

To solve this problem, at last meeting one proposal in [3] is that the last serving gNB can proactively push the UE context to the next satellite (we call it candidate gNB in our following analysis). But in our view there exists the following problem for pushing the UE context:

When the INACTIVE UE resuming connection, the UE’s verification is executed by the candidate gNB via comparing the shortMAC-I reported by the UE’s RRCResumeRequest and included in the RETRIEVE UE CONTEXT REQUEST, and the shortMAC-I computed by itself. Specifically, the candidate gNB computes the shortMAC-I using the Krrcint as the key, and the source C-RNTI, source PCI, target cell ID as the input parameters. The key Krrcint should be the key of the gNB releasing the UE to INACTIVE, i.e., the last serving gNB, which is different from the candidate gNB. But normally the key of a gNB cannot be transferred to another gNB. Therefore, to make the context pushing solution feasible, the last serving gNB should infer all the possible target cells where the UE may resume the connection, and generate the possible shortMAC-I values using the Krrcint of itself and the possible target cell IDs, and send these together with the UE context to the candidate gNB, which is hard to be implemented and with heavy signalling overhead.

As an alternative, we think the INACTIVE UE’s last serving gNB can simply initialize the RAN paging for the UE, when the Xn interface is to be disconnected between the last serving gNB and a member in the gNBs relevant to the UE's RNA. In this way, the UE context can be guaranteed to be relocated to its current camped gNB, which is the UE’s current serving gNB after the paging, and the key is also refreshed. It is proposed:

- The UE's last serving gNB can send paging messages to candidate gNB(s) relevant to the UE's RNA, when the Xn interface is about to be disconnected between the last serving gNB and the candidate gNB relevant to the UE's RNA.

## 2.3	Update of Stage 2 BLCR

The BLCR was endorsed in [4], with good first phase introduction of regenerative payload.

First it can be observed that the detailed regenerative payload and non-regenerative payload are description are not necessary consistent, we suggest then to clarify when it applies to one only payload and nothing when it apply to both.

Second, there is an editor’s note to check the basic termination descriptions of the regenerative payload. It should be updated to describe that the regenerative payload terminates the Uu interface towards the UE (via the service link), and terminates the NG interface toward the 5GC (via the feeder link).

The Annex provides our suggested changes to the stage 2 BLCR.

- RAN3 to agree the TP on the stage 2 changes in the Annex.

# 3	Conclusion

This contribution discusses the regenerative payload for the NR NTN. It is proposed:

Observation 1: SA2 SI concluded to not introduce new RAN/CN architecture nor requirements on the regenerative payload. SA2 will update their specification, if any enhancement from RAN3.

Observation 2: NG suspend/resume operation requires the gNB to store NG configurations relevant to multiple AMFs, which leads to much storage consumption on the regenerative payload/gNB.

- No need to introduce the NG suspend/resume towards the AMF.

- The NG interface tear down can be accomplished via existing graceful SCTP SHUTDOWN chunk procedure. And introducing new NG removal procedure seems no harmful.

- RAN3 to confirm that existing procedures can be supported by the gNB in feeder link switch. And for AMF change case, the RAN can trigger the NG handover or UE context release procedure for its served UE as legacy.

- The UE's last serving gNB can send paging messages to candidate gNB(s) relevant to the UE's RNA, when the Xn interface is about to be disconnected between the last serving gNB and the candidate gNB relevant to the UE's RNA.

- RAN3 to agree the TP on the stage 2 changes in the Annex.

# 4	References

- R2-2405703, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2 126.

- R3-243017, LS on Support of Regenerative-based Satellite Access, SA2.

- R3-243355, (TP for TS 38.300 and TS 38.413) Discussion on the support of Regenerative payload, Nokia, Nokia Shanghai Bell, RAN3 124.

- R3-243949 Support for Regenerative Payload in NR NTN, Ericsson, Thales, Deutsche Telekom, Nokia, ESA, CATT, ZTE, Sateliot, Huawei, Dish Networks, Echostar, Eutelsat Group, Xiaomi, Samsung, CMCC, LG Electronics

# 5	TP for BLCR for TS 38.300

## 16.14	Non-Terrestrial Networks

### 16.14.1	Overview

Figures 16.14.1-1 and 16.14.1-X below illustrate examples of a Non-Terrestrial Network (NTN) providing non-terrestrial NR access to the UE by means of an NTN payload and an NTN Gateway, depicting a service link between the NTN payload and a UE, and a feeder link between the NTN Gateway and the NTN payload. In Figure 16.14.1-1 the payload is transparent; in Figure 16.14.1-X the payload is regenerative and hosts a gNB.

Figure 16.14.1-1: Overall illustration of an NTN with transparent payload

Figure 16.14.1-X: Overall illustration of an NTN with regenerative payload hosting a gNB

NOTE 1:	Figures 16.14.1-1 and 16.14.1-X illustrate an NTN; RAN4 aspects are out of scope.

The NTN transparent payload transparently forwards the radio protocol received from the UE (via the service link) to the NTN Gateway (via the feeder link) and vice-versa. The regenerative payload terminates the Uu interface towards the UE (via the service link), and terminates the NG interface toward the 5GC (via the feeder link).

The following connectivity is supported by the NTN payload:

-	An NTN gateway may serve multiple NTN payloads;

-	A NTN payload may be served by multiple NTN gateways;

NOTE 2:	In this release, the NTN-payload may change the carrier frequency, before re-transmitting it on the service link, and vice versa (respectively on the feeder link).

The following connectivity is supported by the NTN regenerative payload:

-	A regenerative NTN payload may terminate one or more inter-satellite links toward other regenerative payloads;

-	The feeder link is a transport link; it may transport the NG interface between the 5GC and the gNB hosted by the regenerative payload;

-	The inter-satellite link is a transport link; it may transport the Xn interface between the gNBs hosted by the two regenerative payloads.

NOTE 2:	In this release, the transparent NTN-payload may change the carrier frequency, before re-transmitting it on the service link, and vice versa (respectively on the feeder link).

For NTN, the following applies in addition to Network Identities as described in clause 8.2:

-	A Tracking Area corresponds to a fixed geographical area. Any respective mapping is configured in the RAN;

-	A Mapped Cell ID as specified in clause 16.14.5.

Three types of service links are supported:

-	Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites);

-	Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period and a different geographic area during another period (e.g., the case of NGSO satellites generating steerable beams);

-	Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

With NGSO satellites, the gNB can provide either quasi-Earth-fixed service link or Earth-moving service link, while gNB operating with GSO satellite can provide Earth fixed service link or quasi-Earth-fixed service link.

In this release, the UE supporting NTN is GNSS-capable.

In NTN, the distance refers to Euclidean distance.

NEXT CHANGE

### 16.14.4	Switchover

#### 16.14.4.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for an NTN payload. The feeder link switchover is a Transport Network Layer procedure. Service link switch refers to a change of the serving NTN payload.

Both hard and soft feeder link switchover are supported.

#### 16.14.4.2	Assumptions

A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs.

For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switch over, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption may occur during the transition between the feeder links.

#### 16.14.4.3	Procedures

The NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over between two gNBs is performed. The transfer of the affected UE(s)' context between the two gNBs at feeder link switch over is performed by means of either NG based or Xn based handover, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

NEXT CHANGE

INFORMATIVE: RAN2 MAY EXPECT RAN3 TO REVIEW THIS PART – NO CHANGE

### 16.14.6	AMF (Re-)Selection

The gNB implements the NAS Node Selection Function specified in TS 38.410 [16].

For an RRC_CONNECTED UE, when the gNB is configured to ensure that the UE connects to an AMF that serves the country in which the UE is located, if the gNB detects that the UE is in a different country to that served by the serving AMF, then it should perform an NG handover to change to an appropriate AMF, or initiate an UE Context Release Request procedure towards the serving AMF (in which case the AMF may decide to de-register the UE).

For the purpose of selecting an appropriate AMF, the 5GC may verify the UE location according to TS 23.501 [3] and TS 38.305 [42] after the UE has attached to the network.

NOTE:	UE location verification for AMF selection should not be necessary if NTN cell(s) do not extend across countries.

NEXT CHANGE

# B.4	Example implementation of Non-Terrestrial Networks

The following figure illustrates an example implementation of a Non-Terrestrial Network for transparent NTN payload:

Figure B.4-1: NTN based NG-RAN with transparent NTN payload

The gNB depicted in Figure B.4-1 may be subdivided into non-NTN infrastructure gNB functions and the NTN Service Link provisioning system. The NTN infrastructure may be thought of being subdivided into the NTN Service Link provisioning system and the NTN Control function. The NTN Service Link provisioning system may consist of one or more NTN payloads and NTN Gateways.

The NTN payload is embarked on a spaceborne (or airborne) vehicle, providing a structure, power, commanding, telemetry, attitude control for the satellite (resp. HAPS) and possibly an appropriate thermal environment, radiation shielding.

The NTN Service Link provisioning system maps the NR-Uu radio protocol over radio resources of the NTN infrastructure (e.g. beams, channels, Tx power).

The NTN control function controls the spaceborne (or airborne) vehicles as well as the radio resources of the NTN infrastructure (NTN payload(s) & NTN Gateway(s)). It provides control data, e.g. Ephemeris, to the non-NTN infrastructure gNB functions of the gNB.

Provision of NTN control data to the gNB is out of 3GPP scope.

NOTE:	The transport of NR-Uu protocol between the NTN Service Link provisioning system and the non-NTN infrastructure gNB functions is out of 3GPP scope.

At least the following NTN related parameters are expected to be provided by O&M to the gNB for its operation:

a) Earth-fixed beams: for each beam provided by a given NTN-payload:

-	The Cell identifier (NG and Uu) mapped to the beam;

-	The Cell's reference location (e.g. cell's center and range).

b) Quasi-Earth-fixed: for each beam provided by a given NTN payload:

-	The Cell identifier (NG and Uu) and time window mapped to a beam;

-	The Cell's/beam's reference location (e.g. cell's center and range);

-	The time window of the successive switch overs (feeder link, service link);

-	The identifier and time window of all serving satellites (resp. HAPS) and NTN Gateways.

c) Earth moving beams: for each beam provided by a given NTN payload:

-	The Uu Cell identifier mapped to a beam and mapping information to fixed geographical areas reported on NG, including information about the beams direction and motion of the beam's foot print on Earth;

-	Its elevation wrt NTN payload;

-	Schedule of successive serving NTN Gateways/gNBs;

-	Schedule of successive switch overs (feeder link, service link).

END OF CHANGES