3GPP TSG-RAN WG3 Meeting #125	R3-244378

Maastricht, Netherlands, 19 – 23 August, 2024

Agenda Item:	11.3

Source:	Nokia

Title:	Discussion on solution for AI/ML-based CCO

Document for:	Discussion

# 1	Introduction

With the aim to complete the study for Rel-19 AI/ML-based CCO, we here provide analysis of the following open points which need to be closed in order to define a solution for AI/ML-based CCO:

- Timing information

- Standards impact for signalling of information for CCO

A corresponding text proposal for TR 38.743 is submitted to this meeting in [3].

# 2	Discussion

## 2.1	Timing aspect for future CCO state

For AI/ML based CCO, RAN3 agreed that the generation of the predicted CCO issue is carried out by the gNB-CU which then informs the gNB-DU. Furthermore, at the last meeting RAN3 agreed that, once the predicted CCO issue is received by the gNB-DU, based on implementation (AI/ML based or legacy) gNB-DU determines the future coverage state corresponding to the predicted CCO issue. The future coverage state generated by gNB-DU can be shared with the neighbouring NG-RAN node(s) for the purpose of MRO, and also to enable the neighbouring nodes to adjust their coverage with a new matching coverage state. (TS 38.300: “The coverage state indicator may also be used at the receiving NG-RAN node to adopt coverage configurations matching with neighbouring cells coverage configurations.”)

Both for MRO and for the application of matching coverage configurations, a crucial information for neighbouring NG-RAN nodes is to know when to adjust their coverage to the new matching future coverage state. For this, the NG-RAN node sharing the future coverage state over Xn should also include the time when the said future coverage state is planned to be applied (here denoted as activation time). Currently the stage 3 allows to convey some timing related information by setting the Cell Deployment Status Indicator IE to ‘pre-change-notification’. This indicates that the Cell Coverage State is planned to be used at the next reconfiguration. Nevertheless, for indicating the exact time instant in future when the future coverage state will be applied, an additional IE should be considered in the normative phase.

Proposal 1: RAN3 to add signalling support for indicating activation time when the future coverage state is to be applied.

## 2.2	Signalling of information for CCO

An open issue related to standards impacts for network signalling was identified at the initial online discussion at RAN3#124, but could not be handled during the week due to lack of time:

Reusing the current defined CCO issue and CCO status information for predicted CCO issue and CCO status?

Over F1, the CCO issue information is currently sent in the GNB-CU CONFIGURATION UPDATE within the CCO Assistance Information IE which includes the CCO issue in the form of an ENUMERATED value and list of affected cells and beams.

Although the class 1 GNB-CU Configuration Update procedure is used to send the CCO issue from the gNB-CU to the gNB-DU, no new information was added to the corresponding response message or failure message to enable any feedback or error indication sent from the gNB-DU to the gNB-CU for CCO in this procedure. Specific error handling could also be seen as not in line with RAN3’s choice to define the CCO issue information over the F1 interface as ‘assistance information’.

The CCO state information is sent from the gNB-DU to the gNB-CU in the GNB-DU CONFIGURATION UPDATE message within the Coverage Modification Notification IE.

We believe that for sending predicted CCO issue information from the gNB-CU to the gNB-DU it will be possible to use the legacy CCO assistance information as baseline by additionally including timing information, i.e. indicate the prediction time. However, IE details will need agreement in normative phase, so within the present study it is sufficient to agree that signalling for predicted CCO issue needs to include prediction time information.

In our other discussion paper submitted to this meeting in [1] we focus on finding a suitable way forward on the issue of use of AI/ML inference to predict the future CCO state. There, we propose to support predicted (future) CCO state as a standardized output of an AI/ML model, deployed in the gNB-CU, serving as recommendation or assistance information in the gNB-DU. If this proposal is agreeable by RAN3, also the future recommended CCO state needs to be included in F1 signalling from the gNB-CU to the gNB-DU.

Proposal 2: F1 signalling from the gNB-CU to the gNB-DU for predicted CCO issue needs to include prediction time information, and the future recommended CCO state.

The open issue that was captured at last meeting additionally includes the potential reuse of existing signalling for the future CCO state sent from the gNB-DU to the gNB-CU. On this point we believe that the legacy signalling (Coverage Modification Notification IE) can serve as baseline, but that an activation time needs to be added as discussed in section 2.1.

Proposal 3: F1 signalling from the gNB-DU to the gNB-CU for future CCO state needs to include activation time information.

We further believe that RAN3 should analyze the two main options for conveying prediction information from the gNB-CU to the gNB-DU. The legacy signalling is based on the GNB-CU Configuration Update procedure, which is typically used when a node 1 has applied configuration changes and needs to inform neighbour node (node 2) about these changes, e.g. to enable node 2 to make its own matching changes. However, for most of the prediction information agreed in Rel-18, RAN3 made the choice to use a subscription-based mechanism based and introduced the XnAP Data Collection Reporting Initiation and Data Collection Reporting signalling procedures for this purpose. And in general terms, such subscription-based method is typically suitable when the node 1 (in this case the gNB-DU) is a consumer of some information that it would like the node 2 (the gNB-CU) to generate, which is the case for CCO issue prediction.

In the other direction, from the gNB-DU to the gNB-CU, the future CCO state corresponds to a configuration change in the gNB-DU, and use of the legacy GNB-DU Configuration Update procedure, which is a subscription-less (or ‘push’) signalling mechanism, seems a natural choice for signalling of this information over F1. However, similarly as for the information transferred from the gNB-CU to the gNB-DU, we believe that RAN3 should make the final choice in normative phase.

Proposal 4: Choice of signalling mechanisms for CCO related information over F1,  e.g. subscription-based vs. subscription-less, should be made in normative phase.

Although RAN3 has not yet captured the CCO solution description in the TR, it seems clear from earlier discussions that the AI/ML model in gNB1 will receive information over Xn. The following input information, captured in [2], appeared to be agreeable at last meeting:

From neighbouring gNBs:

-	Measured/Predicted radio resource status

However, also other information has been proposed by different companies for exchange over Xn, like UE radio measurements and UE performance measurements.  We believe UE performance measurement from own cell and neighbour cells can provide useful information for AI/ML based CCO, e.g. as feedback information.

Proposal 5: Include UE performance measurements from own cell and neighbour cells as feedback information for the AI/ML Model.

If inter-node exchange of such additional measurements is agreed by RAN3, we believe RAN3 also will need to design a signalling solution taking into account that signalling load over Xn should be kept as low as possible. However, the details of such solution will depend on the agreed measurements. Due to the limited time of the study item, we propose that the analysis and solution related to the exchange of input information to the AI/ML model over the Xn interface is left to normative phase.

Proposal 6: Solution for exchange of input information to the AI/ML model over the Xn interface to be left to normative phase.

Based on these proposals, the exchange of Rel-19 CCO information over F1 and Xn could look as illustrated in Figure 2.

Figure 2: Exchange of CCO information over F1 and Xn.

## 2.3	Solution for AI/ML Model Training and/or Inference in the gNB-CU

Figure 1 shows an overview of a solution where a gNB-CU performs CCO issue detection using an AI/ML model trained by OAM.

Figure 3: AI/ML Model Training in OAM, AI/ML Model Inference in the gNB-CU.

In steps 1-6 of Figure 1, the OAM collects input data for model training from the UE (via the gNB-CU), gNB-DU, gNB-CU and from the neighbouring NG-RAN node 2. In step 8, the OAM deploys or updates the model in the gNB-CU.

In steps 9-11, the gNB-CU collects input data for model inference (step 12), and optionally provides model performance feedback to the OAM in step 13.

In step 14, the gNB-CU sends model inference output (actual or predicted CCO issue and affected cells, and potentially also recommended CCO state as discussed in [1]) to the gNB-DU. The gNB-DU therefore takes the role of an actor according to the agreed NG-RAN framework for AI/ML included in TR 37.817, detail of the framework in Figure 2 below.

Figure 4: Functional Framework for RAN Intelligence (TR 37.817 Figure 4.2-1)

The role of an Actor is further described in TR 37.817:

Actor is a function that receives the output from the Model Inference function and triggers or performs corresponding actions. The Actor may trigger actions directed to other entities or to itself.

-	Feedback: Information that may be needed to derive training data, inference data or to monitor the performance of the AI/ML Model and its impact to the network through updating of KPIs and performance counters.

In step 15 the gNB-DU (Actor) determines the coverage state change (action), and provides feedback to the gNB-CU. The gNB-CU also receives feedback from UEs (step 17) and from the NG-RAN node 2 (step 18). Feedback is also sent to the OAM (steps 19, 20).

We believe the above description is well aligned with the AI/ML framework captured in TR 37.817, and propose to capture it in the TR.

Proposal 7: Capture AI/ML CCO solution details for the case when AI/ML Model Training is in OAM and AI/ML Model Inference is in the gNB-CU as described above in the TR 38.743.

In annex of this paper we also provide solution for AI/ML Model Training and AI/ML Model Inference in the gNB-CU along similar lines.

Proposal 8: Capture corresponding AI/ML CCO solution details for the case when both AI/ML Model Training and AI/ML Model Inference are in the gNB-CU in the TR 38.743.

Proposal 9: Agree the TP to TR 38.743 submitted to this meeting in [3].

# 3	Conclusion

We have made the following proposals:

Proposal 1: RAN3 to add signalling support for indicating activation time when the future coverage state is to be applied.

Proposal 2: F1 signalling from the gNB-CU to the gNB-DU for predicted CCO issue needs to include prediction time information, and the future recommended CCO state.

Proposal 3: F1 signalling from the gNB-DU to the gNB-CU for future CCO state needs to include activation time information.

Proposal 4: Choice of signalling mechanisms for CCO related information over F1,  e.g. subscription-based vs. subscription-less, should be made in normative phase.

Proposal 5: Include UE performance measurements from own cell and neighbour cells as feedback information for the AI/ML Model.

Proposal 6: Solution for exchange of input information to the AI/ML model over the Xn interface to be left to normative phase.

Proposal 7: Capture AI/ML CCO solution details for the case when AI/ML Model Training is in OAM and AI/ML Model Inference is in the gNB-CU as described above in the TR 38.743.

Proposal 8: Capture corresponding AI/ML CCO solution details for the case when both AI/ML Model Training and AI/ML Model Inference are in the gNB-CU in the TR 38.743.

Proposal 9: Agree the TP to TR 38.743 submitted to this meeting in [3].

# References

[1]		R3-244377, Discussion on how to determine the future CCO state, Nokia, Telecom Italia, Jio, Vodafone

[2]		R3-243939, (TP for TR 38.743) AI/ML enabled CCO, Qualcomm Incorporated, Ericsson, Huawei, ZTE, Nokia, CATT

[3]	R3-244379, (TP for TR 38.743) Solution for AI/ML-based CCO, Nokia