3GPP TSG-RAN WG3 Meeting #125	R3-244379

Maastricht, Netherlands, 19 – 23 August, 2024

Agenda Item:	11.3

Source:	Nokia

Title:	(TP for TR 38.743) Use case and solution for AI/ML-based CCO

Document for:	Text Proposal

# 1	Introduction

This paper provides a TP for TR 38.743 reflecting proposals in our paper R3-244378 submitted to this meeting.

# TP for TR 38.743

The TP is based on TR 38.743 v1.0.0.

<<< start of changes >>>

## 4.2	AI/ML based Coverage and Capacity Optimization

### 4.2.1	Use case description

The objective of NR Coverage and Capacity Optimization (CCO) function is to detect and resolve or mitigate CCO issues. An NG-RAN node may autonomously adjust within and switch among coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes with the list of cells and SSBs with modified coverage included.

In the legacy CCO solution, a reactive approach is used: when the gNB (gNB-CU in case of CU-DU split architecture) detects a CCO issue which negatively impacts network and UE performance after it has already occurred, the gNB (gNB-DU in case of CU-DU split architecture) attempts to resolve or mitigate it.

With an AI/ML based CCO, a more proactive approach is used to prevent (or limiting at an early stage) the rise of a CCO issue with the consequent degradation of network (and UE) performance.

### 4.2.2	Solutions and standard impacts

#### 4.2.2.1 Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based CCO:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.2.2.2	AI/ML Model Training at OAM and AI/ML Model Inference at NG-RAN

Figure x1 shows an overview of a solution where a gNB-CU performs CCO issue detection using an AI/ML model trained by OAM.

Figure x1: AI/ML Model Training in OAM, AI/ML Model Inference in the gNB-CU.

Steps 1-6: The OAM collects input data for model training from the UE (via the gNB-CU), gNB-DU, gNB-CU and from the neighbouring NG-RAN node 2.

Step 7, 8: AI/ML Model training in the OAM. The OAM deploys or updates the model in the gNB-CU.

Steps 9-11: The gNB-CU collects input data for model inference.

Step 12, 13: AI/ML Model inference in the gNB-CU. The gNB-CU optionally provides model performance feedback to the OAM.

Step 14: The gNB-CU sends model inference output (actual or predicted CCO issue and affected cells, timing information, [recommended future CCO state]) to the gNB-DU.

Steps 15, 16: The gNB-DU (Actor) determines the coverage state change (action), and provides feedback to the gNB-CU.

Steps 17-20: The gNB-CU receives radio measurements from UEs (step 17) and feedback from the NG-RAN node 2 (step 18). Feedback is also sent to the OAM (steps 19, 20).

#### 4.2.2.3		AI/ML Model Training and AI/ML Model Inference at NG-RAN

Figure x2 shows an overview of a solution where a gNB-CU trains an AI/ML Model and uses it for CCO issue detection.

Figure x2: AI/ML Model Training and AI/ML Model Inference in the gNB-CU.

Steps 1-5: The gNB-CU collects input data for model training from the UE, gNB-DU and from the neighbouring NG-RAN node 2.

Step 6: AI/ML Model training in the gNB-CU. Required measurements are leveraged to training AI/ML model for CCO.

Steps 7-9: The gNB-CU collects input data for model inference.

Step 10: AI/ML Model inference in the gNB-CU. Required measurements are leveraged into Model Inference to output the prediction.

Step 11: The gNB-CU sends model inference output (actual or predicted CCO issue and affected cells, timing of the predicted issue, [recommended future CCO state]) to the gNB-DU.

Steps 12, 13: the gNB-DU (Actor) determines the coverage state change (action), and provides feedback to the gNB-CU.

Steps 14-16: The gNB-CU receives radio measurements from UEs and feedback from the NG-RAN node 2.

#### 4.2.2.4		Input of AI/ML based CCO

For a proactive prediction and resolution of a CCO issue, a gNB may need the following information as input data for AI/ML-based CCO:

From local node:

-	Measured/Predicted radio resource status

-	Current CCO State

From neighbouring gNBs:

-	Measured/Predicted radio resource status

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

-	SON Reports (e.g., RLF, CEF, RA)

#### 4.2.2.5	Output of AI/ML based CCO:

AI/ML-based CCO model in a gNB can generate following information as output:

-	Predicted CCO issue, including timing information

-	Recommended future CCO state, serving as recommendation or assistance information in the gNB-DU

#### 4.2.2.6	Feedback data for AI/ML based CCO:

To optimize the performance of AI/ML-based CCO model, following feedback can be considered to be collected from gNBs:

- Measured radio resource status

- UE performance measurements

- SON Reports (e.g., RLF, CEF, RA)

#### 4.2.2.7	Potential standard impacts:

Following standard impacts are listed for subsequent Rel-19 normative work.

Xn interface:

- Between NG-RAN nodes: Future CCO State, activation time

NOTE: Solution for Xn exchange of input information and feedback to the AI/ML model to be left to normative phase.

F1 interface:

- From gNB-CU to gNB-DU: Predicted CCO Issue (including predicted cells/beams), prediction time

- From gNB-DU to gNB-DU: Future CCO State, activation time

NOTE: The signalling mechanisms to be used over F1, e.g. subscription-based or subscription-less, should be determined in normative phase.

Figure x3: Signalling of CCO information over F1 and Xn.

<<< skip unchanged text >>>

# 6	Conclusion

The following use cases are recommended by RAN3 to be specified in Rel-19 normative phase:

-	AI/ML-based Coverage and Capacity Optimization

Solution descriptions are provided in the clause “Solutions and standard impacts” for each use case and are recommended to be taken as basis for the normative work.

<<< end of changes >>>