3GPP TSG-RAN WG3 Meeting #125	R3-244380

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	17.2

Source:	Nokia

Title:	On-Demand SSB SCell operation for UEs in connected mode

Document for:	Discussion

# 1	Introduction

In this paper we provide information for monitoring of potential RAN3 impacts of on-demand SSB SCell operation for UEs in connected mode based on the current discussion state in RAN1 [1].

# 2	Discussion

RAN1 has made following agreements regarding on-demand SSB:

Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable Agreement For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE FFS: Whether the RRC is newly introduced or existing RRC is reused

Our understanding is that network triggering of SSB transmission will be decided by the gNB-DU in the discussed scenarios. RAN3 impact on e.g. F1 signalling, if any, can only be determined after further agreements taken by RAN1 and associated UE-related signalling decisions in RAN2.

RAN3 may be impacted in order to provide signalling support for L3 measurements of cells served by neighbour nodes, e.g. in terms of exchanged SMTC (SSB Measurement Timing Configuration) information. However, this aspect first requires handling in RAN2 as discussed in [1].

RAN3 should therefore continue monitoring agreements taken by RAN1 and RAN2 for this topic.

Proposal: RAN3 to continue monitoring agreements taken by RAN1 and RAN2 for on-demand SSB SCell operation for UEs in connected mode.

# 3	Conclusions

We have made the following proposal:

Proposal: RAN3 to continue monitoring agreements taken by RAN1 and RAN2 for on-demand SSB SCell operation for UEs in connected mode.

# References

[1]		R2-2406954, On-demand SSB SCell Operation, Nokia, Nokia Shanghai Bell