3GPP TSG-RAN WG3 Meeting #125	R3-244387

Maastricht, The Netherlands, 19th – 23rd August 2024

Agenda Item:	16.2

Source:	Ericsson

Title:	On AIoT RAN functional and logical architecture

Document for:	Discussions & Approval

# 1	Introduction

This document discusses furthe aspects on AIoT RAN functional and logical architecture.

# 2	Discussion

## 2.1	Principles and Requirements

### 2.1.1	New 3GPP IoT technology – as per expectations in the Study Item Description

Probably it is about time to revisit the AIoT RAN SID [1], which contains quite exquisite explications about expectations on the new technology envisaged to be studied in Rel-19 and probably specified by 3GPP (the “Justification” section didn’t change much from what was provided in the “Introduction” section of TR 38.848 [2] and TR 38.769 [4]):

Since existing technologies cannot meet all the requirements of target use cases, a new IoT technology is recommended to open new markets within 3GPP systems, whose number of connections and/or device density can be orders of magnitude higher than existing 3GPP IoT technologies. The new IoT technology shall provide complexity and power consumption orders of magnitude lower than the existing 3GPP LPWA technologies (e.g. NB-IoT and eMTC), and shall address use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technologies.

Let us summarize the content of the above paragraph from [1] in the following observations:

Observation 2.1-1:	3GPP TSG RAN has identified AIoT use cases to fill a gap for which no IoT technology yet exists and expects studies to be performed accordingly.

In terms of the “rcommended IoT technology to open new markets within 3GPP systems”, maybe the following figure catches the readers’ eyes better than actually reading through the paragraphs in the “Introduction” and “Justification” sections:

Figure 2.1.1-1: Technology gap intended to be addressed by 3GPP AIoT features.

Observation 2.1-2:	Respective studies are expected to envisage the introducing of new 3GPP IoT technologies, targeting new markets and being integrated into 3GPP systems – more specifically (along the SID tile): into 3GPP systems using NR access.

Some other statements from the justification section in [1]:

An example type of application in TR 22.840 is asset identification, which presently has to resort mainly to barcode and RFID in most industries. The main advantage of these two technologies is the ultra-low complexity and small form factor of the tags. However, the limited reading range of a few meters usually requires handheld scanning which leads to labor intensive and time-consuming operations, or RFID portals/gates which leads to costly deployments. Moreover, the lack of interference management scheme results in severe interference between RFID readers and capacity problems, especially in case of dense deployment. It is hard to support large-scale network with seamless coverage for RFID.

Some more observations, basically paraphrasing text from [1] and [2] as quoted above:

Observation 2.1-3:	While handheld UE readers are not ruled out, 3GPP mainly aims at studying planned, organised and fixed (mounted) deployment AIoT infrastructure, regardless the topology.

Deployment scenarios as of the SID [1] expect 3GPP AIoT to cover use of licensed NR resources (in-band to NR, in guard-band to LTE/NR or in stand-alone bands), which provides sufficient reason for assuming well planned deployment and respective control/management functions placed in the Radio Access Network, including interference management as mentioned in [1]. These functions are said to be missing in existing (non 3GPP) deployments, as quoted in the paragraph above.

Observation 2.1-4:	Planned, organised and mounted deployment of AIoT infrastructure is expected to include functions controlling the usage of AIoT radio resources within licensed 3GPP spectrum, including interference management.

And then on the statement “It is hard to support large-scale network with seamless coverage for RFID.”: While it is made clear in the SID [1], that the AIoT device is not expected to have RRC states, that there is no mobility support (in terms of cell selection/re-selection), etc., it can be stated that achieving seamless reachability of AIoT devices (at least in parts of an MNO network, where AIoT service availability is intended) will have to resort to existing (NR) infrastructure given the impracticality of rolling out a parallel infrastructure just for the sake of AIoT.

Observation 2.1-5:	3GPP specified AIoT building blocks, aiming at deployment of “large scale” AIoT network with “seamless coverage” will have to resort to existing (NR) infrastructure given the impracticality of rolling out completely new/parallel infrastructure.

Bearing in mind that the SID’s [1] expectation to study an extension of the reading range of currently available RFID technology of few meters to a few tens of meters can only be regarded as a first step towards true seamless coverage, where rather a range of a few hundreds of meters matches existing NR outdoor deployment scenarios.

Imagining such expected final stage of AIoT service provisions (seamless, large scale, larger ranges), reveals the fact that studying network side architecture for deployment scenarios outlined in the Rel-19 SID [1] will be rather limiting- This will leadon the one hand side to solutions that cannot cover the expected new markets and on the other hand side to solutions that are not able to cover sufficient aspects for support of seamless coverage in large scale networks.

Observation 2.1-6: The currently reading range extension from a few meters (as provided by RFID) to a few tens of meters (as of the SID, which concentrates on indoor scenarios) can only be regarded as a first (intermediate) step towards reaching the projected ultimate of seamless coverage in large scale networks.

Observation 2.1-7:	Limiting studies to a first (intermediate) deployment roll-out, targeting local/indoor scenarios only, without spending a single thought on the – expected – final roll-out scenario, bears the danger of ending up in solutions only applicable to a nieche (and probably already existing) market segment.

Proposal 2.1-1:	Studies on AIoT RAN architecture and respective RAN/CN functional split shall focus on the expected final roll-out scenario providing AIoT services in seamless coverage in large scale networks, preferably integrated into existing (NR) infrastructure to inherently provide means of coordinating AIoT and non-AIoT radio resource usage.

### 2.1.2	Functional principles for work on AIoT RAN architecture

With the observations from the pervious “expectation” chapters in mind, we would like to establish the following guiding principles for further work on functional and logical architecture during the Rel-19 SID [1]:

1.	Control of AIoT radio resources is an integral part of AIoT RAN, including admission control, interference management etc.

As discussed above, use of licensed bands calls for control of these radio resources. 3GPP AIoT systems expected to ultimately provide seamless coverage in large scale networks and the prospect of using radio resources within bands used for regular (MBB) traffic require explicit coordination, especially in order to protect AIoT resources from unwanted interference, given the low power budget of AIoT devices. Admission control needs to be able to coordinate resource allocation for AIoT and non-AIoT usage.

2.	5GS system provides means to control the authorized and secured use of AIoT radio resources.

Even if RAN3 architectural and protocol work is not directly impacted by finally specified security solutions, AIoT RAN has to rely on the provision of security and privacy, even if this is realised by non-AIoT RAN means. 5GS has to be able to control access to its core- (i.e. radio-) resources and, conversely, it has to provide means to allow AIoT devices to deduce the authenticity of the network via which it consumes AIoT services.
 If this cannot be provided by higher layers, then RAN would need to “step in”.

3.	AIoT RAN enables operations comprising multiple Common Reader Function operation. Related functions concern, among others, interference management and transactions involving more than one Common Reader Function.

This principle can be seen a derivative of the first one: seamless coverage and in large scale networks entail the concurrent usage of multiple Common Reader Functions and the respective control of radio resources protocol entities.

4.	AIoT RAN contains a function to enable multi-AIoT device operations.

This principle stems from already agreed multi-AIoT device operations for AIoT paging and inventory, FFS for command (see minutes from previous meetings).

5.	AIoT RAN functional architecture does not prevent support of mixed deployment of topologies 1&2.

Diverse roll-out strategies and deployment scenarios may result in a mixture of different topologies with the same AIoT radio network. Any solution developed for each individual topology should not prevent concurrent deployment of both topologies.

Proposal 2.1-2:	Agree the following functional principles for work on AIoT RAN architecture:

2.1.	Control of AIoT radio resources is an integral part of AIoT RAN, including admission control, interference management etc.

2.2.	5GS system provides means to control the authorized and secured use of AIoT radio resources.

2.3.	AIoT RAN enables operations comprising multiple Common Reader Function operation. Related functions concern, among others, interference management and transactions involving more than one Common Reader Function.

2.4.	AIoT RAN contains a function to enable multi-AIoT device operations.

2.5.	AIoT RAN functional architecture does not prevent support of mixed deployment of topologies 1&2.

## 2.2	Deployment scenarios for support of AIoT radio

### 2.2.1	The indoor deployment with UE based AIoT reading device (topology 2) – for local use cases

This deployment scenario corresponds to “Deployment scenario 2 with Topology 2”, as mentioned in the SID [1], i.e. outdoor base station, indoor UE Reader and indoor AIoT device. As mentioned in section 2.1, a deployment scenario with a few tens of meters distance between reader and device:

Figure 2.2.1-1: Indoor deployment with UE based AIoT reading device

As discussed in section 2.1 along the SID [1], UE handheld use cases require human interaction, leading to labor intensive and time-consuming operations.

However, what UE reader deployments may provide is a nice migration strategy from local AIoT service provision to large-scale and seamless coverage deployment, by starting off providing AIoT services in indoor scenarios, by means of a sufficient number of fixed mounted AIoT reading devices, probably exploiting well known positions of these AIoT reading devices for proximity/location applications.

A topology 2 deployment with fixed mounted UE based AIoT reading devices may look like a very simple to be achieved approach. However, it has to be considered how to operate such system, taking into account warehouse scenarios consisting of tens/hundreds of such UE based AIoT reading devices. It has to be ensured that the radio resources in which the AIoT devices should interact with the reading devices are as free of interference as possible – from (regular) non-AIoT traffic and from neighbouring AIoT radio traffic. and each of these UE based AIoT reading devices use respective AIoT radio resources in a controlled and organised fashion. Such requirements call – also in this scenario – for a radio network node centric radio resource management.

It is not too far fetched to assume – as a typical deployment scenario – NG-RAN nodes, in particular gNBs to be upgraded for taking over this task.

Observation 2.2-1:	According to expectations in the SID, indoor deployments with topology 2 can be assumed to be achieved with fixed mounted UE based AIoT reading devices which typicall requires the control of tens/hundreds of such reading devices in realistic use case scenarios.

Observation 2.2-2:	In such indoor scenario, it has to be ensured that the AIoT radio resources are free of interference from regular NR traffic and from neighbouring reading devices’ AIoT radio traffic , and each of these UE based AIoT reading devices use respective AIoT radio resources in a controlled and organised fashion. Such requirements call – also in indoor scenarios with UE based AIoT reading devices – for a radio network node centric radio resource management.

Proposal 2.2-1:	Further studies on architecture and protocol aspects assume that deployment scenarios with topology 2 require radio network node centric radio resource management located in the AIoT RAN node function, which is defined in TR 38.769 [2].

One can also assume that it is possible – and probably also typical – that existing NG-RAN deployments can be upgraded for incorporating the AIoT RAN node function. A possible deployment could look like this:

Figure 2.2.1-2: Possible deployment scenario with AIoT RAN node function containing radio resource management for control of AIoT radio resources.

Proposal 2.2-2:	Agree that it is possible for topology 2 to deploy the AIoT RAN node function within a gNB.

### 2.2.2	The indoor deployment with topology 1 – for local use cases solution

This deployment scenario corresponds to “Deployment scenario 1 with Topology 1”, as mentioned in the SID [1], i.e. indoor base stations and indoor AIoT device, as mentioned in section 2.1, a deployment scenario with a few tens of meters distance between reader and device:

Figure 2.2.1-1: Indoor deployment with topology 1

The functional aspects studied here for topology 1 differ from topology 2 in a way, that topology 1 does not explicate/specify how the functional split between AIoT RAN node function and Common Reader function should be achieved within AIoT RAN (RAN3 agreed to not spend study/specification effort on disaggregated NG-RAN aspects) – the interaction between the AIoT RAN node function and the Common Reader function is left to implementation.

However, from a functional point of view, arguments and requirements regarding the coordination of AIoT radio resources – and non-AIoT radio resources are the same, givent the usage of licensed bands, which also in such probably very specific indoor scenarios have to be coordinated.

Proposal 2.2-3:	Further studies on architecture and protocol aspects assume that deployment scenarios with topology 1 require radio network node centric radio resource management located in the AIoT RAN node function, which is defined in TR 38.769 [2].

Figure 2.2.1-3: Possible deployment scenario with AIoT RAN node function containing radio resource management for control of AIoT radio resources.

Proposal 2.2-4:	Agree that it is possible for topology 1 to deploy the AIoT RAN node function within a gNB.

### 2.2.3	Deployment scenario for large scale network deployment providing large area AIoT service access

As discussed in section 1, it is acknowledged that in the SID [1], mainly in order to reduce work in other RAN WGs, foresees only a limited set of radio-deployment scenarios to be studied. However, the expectation, as of the introduction section in the TR [2] and the justification section in the SID [1] rather gives the direction of aiming at providing AIoT services in the whole service area of an MNO.

While it might be possible to deploy AIoT service provision in local use cases by means of special solutions with rather weak integration in existing network infrastructure, as discussed in section 2.1, it is felt rather impractical to envisage the roll out completely new/parallel infrastructure for AIoT in large scale.

Proposal 2.2-5:	Agree that large scale roll out of AIoT service provision should be based on existing infrastructure with the AIoT RAN node function deployed in NG-RAN nodes/gNBs.

## 2.3	Some thoughts on possible Protocol Stack options

### 2.3.0	General

Next topic to look at is to define the means how a functional architecture achieves the communication between the functional elements contained in it. Hence different (generalised) protocol stack options are looked at, leaning towards classes of options discussed in past meetings in RAN3 and SA2.

Note, that topology 2 is explicitly depicted, foreseeing communication towards the UE based AIoT reading device. Such (explicitly depicted) communication (and respective protocol stacks) have to be regarded as an implementation specific variant of topology 1.

### 2.3.1	Direct communication between the application function and the Common Reader Function.

The protocol stack figure, put on top of the logical architecture for this option may look like as follows:

Figure 2.3-1: Protocol stacks for direct communication between AF and Common Reader Function.

This protocol stack option foresees any exchange of application data (the figure depicts an assumed communication of DL data – shown with red arrows) to be directly communicated between the application function and the common reader function, an option considered specifically for topology 2.

For the sake of functional architecture considerations, the “transport options” are of secondary concern. There is the possibility to achieve the transport of application data transparently by means of control plane solutions (AIoT specific RRC/NGAP messages/IEs) or user plane solutions (establishment of specific PDU session resources), one set of papers also suggested in the past the introduction of an SBI, which we regard to fall into the same category of “XXAP transport” related discussions.

Common to this group of proposals is that they do not foresee any functional interaction within AIoT CN or AIoT RAN by means of explicit termination of AIoT specific protocol elements and triggering AIoT functions.

As shown Figure 2.3-1, there is especially no influence for AIoT functions foreseen in 5GS at all.

While such approach is not infeasible to achieve, it is assumed to be outside the scope of studies to be performed in RAN3, probably also outside of the scope of 3GPP at all (probably it would only require some co-existance studies in RAN4).

Observation 2.3-1:	Protocol stack options foreseeing a direct communication between an application function and the common reader function can be regarded to be outside the scope of studies to be performed in RAN3 (probably even outside the scope of 3GPP). As such stack options would not foresee any “intervention” of 5GS, e.g. regarding admission control, authentication, radio resource management, etc., it is rather discouraged to follow such approach.

### 2.3.2	Direct communication between the AIoT CN and the Common Reader Function.

The protocol stack figure, put on top of the logical architecture for this option may look like as follows:

Figure 2.3-2: protocol stacks for direct communication between AIoT-CN and Common Reader Function.

Also here, the traversing of application-level DL data through the system, with “stop-overs” at various places, is shown by means of red arrows. Details on whether such information is completely transparent or needs to be processed at these “stop-over” places need to be discussed somewhere else. In general, we can assume, that device identification information is likely to need some interpretation/processing within 5GS.

Figure 2.3-2 (as well as Figure 2.3-1) attempts to depict both topologies. While not so relevant in Figure 2.3-1, Figure 2.3-2 can be simply “converted” into a toplogy 1 by collapsing the 2 groups of AIoT RAN stacks (and functions) into a single network entity.

Figure 2.3-2 does not show the stack for the topology 2 for the UE based AIoT reading device, but only the stacks for AIoT related communication.

The main functional differences between topology 1 and topology 2 are indicated in “callouts” in Figure 2.3-2 and are in principle discussed in section 2.3.1 already:

For topology 2, direct communication between the Common Reader Function and the AIoT CN does not allow AIoT RAN to perform admission control/RRM functions, unless an additional control plane structure - in parallel to XXAP – is foreseen, that first allows network side control of AIoT radio resources and only after AIoT radio resources are prepared in a controlled manner, XXAP and respective AIoT radio functions are performed.

Another aspect is that, for topology 2, this stack option would require the AIoT CN to control multi-reader operations, requiring the AIoT to select the readers applicable for an AIoT transaction, i.e. probably to translate a certain area scope as provided from the application into a set of readers. Given the envisaged use case for topology 2, expecting a potentially large number of readers, may give an hint on the scalability of this stack option.

Observation 2.3-2:	Protocol stack options foreseeing a direct communication between AIoT CN and common reader function for topology 2 do not allow to perform AIoT RAN specific functions to control AIoT radio resources, unless a control protocol structure is added on top to trigger respective AIoT RAN functions.
Reader selection in topology 2 might represent a scalability issue.

### 2.3.3	Hop-by-hop communication between AIoT AF, AIoT CN, AIoT RAN node function and Common Reader Function

The protocol stack figure, put on top of the logical architecture for this option may look like as follows:

Figure 2.3-3: protocol stack for hop-by-hop communication for AIoT.

Also here, the traversing of application level DL data through the system, with “stop-overs” at various places, is shown by means of red arrows. Topology 1 can be deduced by collapsing both AIoT RAN stacks into a single one.

The main functional differences between topology 1 and topology 2 can be seen in Figure 2.3-3: while for topology 1, the interaction between the Common Reader Function and the AIoT RAN node function is a matter of implementation, for topology 2, RRC needs to provide means to transport application data and respective control information.

Terminating XXAP in AIoT RAN has the advantage of creating a natural trigger point for performing AIoT RAN node level functions as discussed in the previous chapters.

It also has the advantage that the potentially large amount of Common Reader Function (instances) can be handled and controlled by AIoT RAN: AIoT RAN would only need to translate an application level/AIoT CN level area indicaton into a set of Common Reader Functions.

Another aspect for topology 2 is the handling of diverse RRC states of the UE based AIoT reading device. It is expected, that the UE based AIoT reading device should at least be allowed to move to RRC_INACTIVE, but there is no reason to preclude also RRC_IDLE. Triggering an AIoT transaction towards INACTIVE or IDLE UE based AIoT reading devices, would require to first bring those devices back to RRC_CONNECTED.

For paging INACTIVE UE based AIoT reading the following can be envisaged:

-	The AIoT CN provides an area within which an AIoT transaction needs to be performed and the AIoT RAN, containing functions to serve the UE based AIoT reading devices, trigger RAN paging of these reading devices within the indicated areas (as far as the AIoT transaction area is contained in the RNA allocated to those reading devices)

For paging IDLE UEs there are two options conceivable:

1.	The AIoT CN triggers the AMF(s) serving reader devices within a certain RA (that matches the AIoT transaction area) to perform paging of this set of reading devices within the AIoT transaction area.

2.	The AIoT CN triggers the gNB serving the cells within which the UE based AIoT reading devices should perform AIoT transactions to page the respective reading devices. This appears to result in a kind of group paging, which could be performed either by an actual group ID (no need to contact the serving AMF) or by an explicit indication to wake up all reading devices in the concerned area.

Observation 2.3-3:	Protocol stack options foreseeing a hop-by-hop communication with “stop -overs” at the AIoT CN and AIoT RAN allow triggering AIoT RAN node level functions. This allows control of multi-reader operations, especially for topology, where in addition paging aspects for UE based AIoT reading devices would need to be considered.

### 2.3.4	Concluding discussions on protocol stack options

Observation 2.3-4:	The more direct “end-to-end” communication between the AIoT device and the AIoT application is envisaged to be designed, the less “natural” interaction possibilities can be expected from the 5G system / AIoT RAN. If a certain 5GS control of network resources are expected, AIoT specific communication needs to be terminated at and processed within the relevant network entities.

Observation 2.3-5:	Given the expected number of Common Reader Function instances in a large scale network, scalbility in terms of reader selection (including paging to support IDLE and INACTIVE UE based AIoT reading devices) needs to be considered.

Proposal 2.3-1: Preference shall be given to protocol stack options (and respective allocation of functions) which allows 5GS and AIoT RAN control of access of system/radio resources.

Proposal 2.3-2: Preference shall be given to protocol stack options are able to handle reader selection in a scalable manner.

Proposal 2.3-3: Further study paging of IDLE/INACTIVE UE based AIoT reading devices for topology 2.

## 2.4	Developing a common logical architecture for AIoT topologies 1&2

### 2.4.1	Decomposing the Common Reader Function for topology 2

Discusson have been conducted at RAN3#124 on how to achieve a common logical architecture for AIoT.

We try to borrow some approaches on how to depict an “architecture” including instances of Uu interfaces from IAB discussions and introduce an “AIoT Mobile Termination” subfunction within the Common Reader Function. In the absence of discussions on disaggregated RAN, we propose to introduce an “AIoT radio front end” in order to clearly separated both interfaces, the Uu towards the gNB serving the UE based AIoT reading device and the AIoT radio towards the AIoT device – with the following proposed definitions:

AIoT MT: function within the Common Reader Function deployed as a UE based AIoT reading device, communicating via Uu interface to its serving gNB.

Editor’s Note:	It is FFS whether and in which way communication to the serving gNB is performed, i.e. whether the gNB performs e.g. as a mere relay of information provided via Uu UP or e.g. AIoT information is provided via legacy NAS signalling or by other means.

AIoT radio frontend: communicates to the AioT device by means of AIoT radio.

Further, we had also discussions on the nature of a “UE Reader” in the context of AIoT RAN discussions. Especially there was uncertainty as to which extent a “UE” as such could be introduced in an architectural discussion led by RAN3 (and not RAN2). RAN#104 discussions “allowed” RAN3 to continue discussions in that context.

As you might have realised, we have used, up to now, the term UE based AIoT reading device in order to denote the entity in between the AIoT device and the gNB serving that entity a proper name, which reduces this entity to the functions necessary to merely perform its AIoT specific operations.

We propose to introduce the term UE based AIoT reading device with the following definition:

UE based AIoT reading device: Equivalent to a device containing the Common reader function in topology 2 deployments. This term is used to denote all functions necessary for AIoT specific operations.

Proposal 2.4-1: agree on the following definitions regarding the decomposition of the Common Reader Function for topology 2:

AIoT MT: function within the Common Reader Function deployed as a UE based AIoT reading device, communicating via Uu interface to its serving gNB. Editor’s Note: It is FFS whether and in which way communication to the serving gNB is performed, i.e. whether the gNB performs e.g. as a mere relay of information provided via Uu UP or e.g. AIoT information is provided via legacy NAS signalling or by other means. AIoT radio frontend: communicates to the AIoT device by means of AIoT radio. UE based AIoT reading device: Equivalent to a device containing the Common reader function in topology 2 deployments. This term is used to denote all functions necessary for AIoT specific operations.

### 2.4.2	Where to place the common reader function for topology 2

There were discussions at the last RAN3 meeting on whether the Common Reader Function in topology 2, i.e. the UE based AIoT reading device, is part of what we called AIoT RAS (or AIoT RAN) at the last meeting.

We would like to apply the following logic:

-	Any function residing in between the AIoT device and the AIoT CN can be regarded as the AIoT RAS/RAN.

-	If the Common reader function is outside AIoT RAS/RAN, it has to be either the AIoT device or the AIoT CN, which is not possible. So it has to be inside the AIoT RAS/RAN. – There is no other option.

Proposal 2.4-2: Agree that the common reader function for topology 2, i.e. the UE based AIoT reading device, is part of AIoT RAS/RAN.

And then, if possible, rename “AIoT RAS” back to “AIoT RAN”.

Proposal 2.4-3: Agree to rename the “AIoT RAS” back to “AIoT RAN”.

### 2.4.3	Agreeable depictions of the logical AIoT RAN architecture and how to apply it to deployment scenarios for topologies 1&2

Now it is about drawing time.

We have all the tools and information at our hand (assuming the previous proposals are acceptable): A place for each function within the architectural entity, regardless the topology and respective definitions.

For topology 1, one possible deployment is to place the AIoT specific functions within an AIoT-enabled  gNB, which becomes part of the AIoT RAN. Coordination between non-AIoT- and AIoT-resources would become a gNB-internal matter. The interface towards the AIoT CN needs probably some more input from SA2 as the nature of the CN node towards AIoT RAN should connect is still to be determined. The resulting deployment scenario is depicted in Figure 2.4.3-1.

Figure 2.4.3-1: AIoT RAN deployment scenario for topology 1.

The previous chapters have attempted to provide an aligned architecture for both, topology 1 and topology 2, so it comes to no surprise that respective deployment scenario look rather similar, with the difference that the common reader function is realised in a AIoT RAN network node outside the AIoT enabled gNB, the “UE based AIoT reading device”.

In addition, deployment needs 5GS functions to serve the UE based AIoT reading device, the deployment scenario shows this be depicting an AMF and an AIoT enabled gNB that also provides radio resources to the UE based AIoT reading device. Like for topology 1, the nature of the XX interface needs further discussions.

Figure 2.4.3-2: AIoT RAN deployment scenario for topology 2.

Proposal 2.4-4: Capture AIoT RAN deployment scenarios for topologies 1 and 2 as shown in section 2.4.3 and in the TP for TR 38.769 [3].

## 2.5	Resolving some of the Editorial Notes in TR 38.769

Following the discussions and proposals in the previous chapters, some of the Editorial Notes in TR 38.769 [2] chapter 6.3 are proposed to be solved as follows:

EN4:

AIoT RAN (AIoT Radio Access Network): hosts certain functions for AIoT as part of the functional split between RAN and CN.

Editor’s Note 4: Further details regarding AIoT functions hosted in the AIoT RAS and the respective functional split to be decided by RAN2, RAN3 and SA2.

EN7:

XX interface: interface between the AIoT RAN and the AIoT CN on which certain AIoT specific functions are performed.

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 1 it may only represent a single interface instance, e.g. a new interface between AIoT RAS and AIoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between AIoT CN and AIoT RAS, or one instance for NG alone.

EN9(1):

AIoT RAN node function: a function residing in AIoT RAN. It hosts the following functions: AIoT radio specific resource management, admission control of the AIoT radio resources used towards the AIoT device by the common reader function, coordination of AIoT transactions involving multiple common reader functions, coordination of AIoT transactions concerning multiple AIoT devices, termination of the XX interface.

Editor’s Note 9: further details are FFS.

EN9(2):

Figure 6.3-1: Logical System Architecture for AIoT.

Proposal 2.4-5: Capture resolving Ens within section 6.3 of TR 38.769 as showin in section 2.4.4 and in the TP for TR 38.769 [3].

# 3	Summary and Conclusions

We have discussed AIoT RAN architecture aspects along the following observation and proposals

Observation 2.1-1:	3GPP TSG RAN has identified AIoT use cases to fill a gap for which no IoT technology yet exists and expects studies to be performed accordingly.

Observation 2.1-2:	Respective studies are expected to envisage the introducing of new 3GPP IoT technologies, targeting new markets and being integrated into 3GPP systems – more specifically (along the SID tile): into 3GPP systems using NR access.

Observation 2.1-3:	While handheld UE readers are not ruled out, 3GPP mainly aims at studying planned, organised and fixed (mounted) deployment AIoT infrastructure, regardless the topology.

Observation 2.1-4:	Planned, organised and mounted deployment of AIoT infrastructure is expected to include functions controlling the usage of AIoT radio resources within licensed 3GPP spectrum, including interference management.

Observation 2.1-5:	3GPP specified AIoT building blocks, aiming at deployment of “large scale” AIoT network with “seamless coverage” will have to resort to existing (NR) infrastructure given the impracticality of rolling out completely new/parallel infrastructure.

Observation 2.1-6: The currently reading range extension from a few meters (as provided by RFID) to a few tens of meters (as of the SID, which concentrates on indoor scenarios) can only be regarded as a first (intermediate) step towards reaching the projected ultimate of seamless coverage in large scale networks.

Observation 2.1-7:	Limiting studies to a first (intermediate) deployment roll-out, targeting local/indoor scenarios only, without spending a single thought on the – expected – final roll-out scenario, bears the danger of ending up in solutions only applicable to a nieche (and probably already existing) market segment.

Proposal 2.1-1:	Studies on AIoT RAN architecture and respective RAN/CN functional split shall focus on the expected final roll-out scenario providing AIoT services in seamless coverage in large scale networks, preferably integrated into existing (NR) infrastructure to inherently provide means of coordinating AIoT and non-AIoT radio resource usage.

Proposal 2.1-2:	Agree the following functional principles for work on AIoT RAN architecture:

2.1.	Control of AIoT radio resources is an integral part of AIoT RAN, including admission control, interference management etc.

2.2.	5GS system provides means to control the authorized and secured use of AIoT radio resources.

2.3.	AIoT RAN enables operations comprising multiple Common Reader Function operation. Related functions concern, among others, interference management and transactions involving more than one Common Reader Function.

2.4.	AIoT RAN contains a function to enable multi-AIoT device operations.

2.5.	AIoT RAN functional architecture does not prevent support of mixed deployment of topologies 1&2.

Observation 2.2-1:	According to expectations in the SID, indoor deployments with topology 2 can be assumed to be achieved with fixed mounted UE based AIoT reading devices which typicall requires the control of tens/hundreds of such reading devices in realistic use case scenarios.

Observation 2.2-2:	In such indoor scenario, it has to be ensured that the AIoT radio resources are free of interference from regular NR traffic and from neighbouring reading devices’ AIoT radio traffic , and each of these UE based AIoT reading devices use respective AIoT radio resources in a controlled and organised fashion. Such requirements call – also in indoor scenarios with UE based AIoT reading devices – for a radio network node centric radio resource management.

Proposal 2.2-1:	Further studies on architecture and protocol aspects assume that deployment scenarios with topology 2 require radio network node centric radio resource management located in the AIoT RAN node function, which is defined in TR 38.769 [2].

Proposal 2.2-2:	Agree that it is possible for topology 2 to deploy the AIoT RAN node function within a gNB.

Proposal 2.2-3:	Further studies on architecture and protocol aspects assume that deployment scenarios with topology 1 require radio network node centric radio resource management located in the AIoT RAN node function, which is defined in TR 38.769 [2].

Proposal 2.2-4:	Agree that it is possible for topology 1 to deploy the AIoT RAN node function within a gNB.

Proposal 2.2-5:	Agree that large scale roll out of AIoT service provision should be based on existing infrastructure with the AIoT RAN node function deployed in NG-RAN nodes/gNBs.

Observation 2.3-1:	Protocol stack options foreseeing a direct communication between an application function and the common reader function can be regarded to be outside the scope of studies to be performed in RAN3 (probably even outside the scope of 3GPP). As such stack options would not foresee any “intervention” of 5GS, e.g. regarding admission control, authentication, radio resource management, etc., it is rather discouraged to follow such approach.

Observation 2.3-2:	Protocol stack options foreseeing a direct communication between AIoT CN and common reader function for topology 2 do not allow to perform AIoT RAN specific functions to control AIoT radio resources, unless a control protocol structure is added on top to trigger respective AIoT RAN functions.
Reader selection in topology 2 might represent a scalability issue.

Observation 2.3-3:	Protocol stack options foreseeing a hop-by-hop communication with “stop -overs” at the AIoT CN and AIoT RAN allow triggering AIoT RAN node level functions. This allows control of multi-reader operations, especially for topology, where in addition paging aspects for UE based AIoT reading devices would need to be considered.

Observation 2.3-4:	The more direct “end-to-end” communication between the AIoT device and the AIoT application is envisaged to be designed, the less “natural” interaction possibilities can be expected from the 5G system / AIoT RAN. If a certain 5GS control of network resources are expected, AIoT specific communication needs to be terminated at and processed within the relevant network entities.

Observation 2.3-5:	Given the expected number of Common Reader Function instances in a large scale network, scalbility in terms of reader selection (including paging to support IDLE and INACTIVE UE based AIoT reading devices) needs to be considered.

Proposal 2.3-1: Preference shall be given to protocol stack options (and respective allocation of functions) which allows 5GS and AIoT RAN control of access of system/radio resources.

Proposal 2.3-2: Preference shall be given to protocol stack options are able to handle reader selection in a scalable manner.

Proposal 2.3-3: Further study paging of IDLE/INACTIVE UE based AIoT reading devices for topology 2.

Proposal 2.4-1: agree on the following definitions regarding the decomposition of the Common Reader Function for topology 2:

AIoT MT: function within the Common Reader Function deployed as a UE based AIoT reading device, communicating via Uu interface to its serving gNB. Editor’s Note: It is FFS whether and in which way communication to the serving gNB is performed, i.e. whether the gNB performs e.g. as a mere relay of information provided via Uu UP or e.g. AIoT information is provided via legacy NAS signalling or by other means. AIoT radio frontend: communicates to the AIoT device by means of AIoT radio. UE based AIoT reading device: Equivalent to a device containing the Common reader function in topology 2 deployments. This term is used to denote all functions necessary for AIoT specific operations.

Proposal 2.4-2: Agree that the common reader function for topology 2, i.e. the UE based AIoT reading device, is part of AIoT RAS/RAN.

Proposal 2.4-3: Agree to rename the “AIoT RAS” back to “AIoT RAN”.

Proposal 2.4-4: Capture AIoT RAN deployment scenarios for topologies 1 and 2 as shown in section 2.4.3 and in the TP for TR 38.769 [3].

Proposal 2.4-5: Capture resolving ENs within section 6.3 of TR 38.769 as showin in section 2.4.4 and in the TP for TR 38.769 [3].

Final Proposal:	Agree on the TP for TR 38.769 as provided in R3-244388 [3]

# 4	References

[1]	RP-240826 "Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR".

[2]	TS 38.848 "Study on Ambient IoT (Internet of Things) in RAN".

[3]	R3-244388 "Text Proposal for TR 38.769 on RAN architecture aspects".

[4]	TS 38.769 " Study on solutions for ambient IoT (Internet of Things)".