3GPP TSG-RAN WG3 Meeting #125	R3-244388

Maastricht, The Netherlands, 19th – 23rd August 2024

Agenda Item:	16.2

Source:	Ericsson

Title:	TP for TR 38.769 on AIoT Architecture aspects

Document for:	Discussions & Approval

# 1	Introduction

This TP follows discussions in R3-244387.

# 2	Text Proposal for TR 38.769 (currently based on agreed TP in R3-243972)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 6.3	RAN architecture aspects

### 6.3.1	General

Editor’s note 1: Corresponds to the second RAN3 objective in the SID, to identify RAN architecture aspects, including whether support for split architecture is necessary.

This chapter attempts to identify and describe architectural elements necessary to define a RAN architecture for support of Ambient IoT embedded in the overall 5G system architecture in support of topology 1 and topology 2 (as defined in TR 38.848 [2]).

Editor’s Note 2: What functionalities are hosted by the 5GS for AIoT is TBD.

This chapter also attempts to identify a functional split between RAN and CN.

### 6.3.2	Logical System Architecture for AIoT

Figure 6.3-1 depicts the logical system architecture for AIoT.

It consists of the following architectural elements:

AIoT device: equipment with characteristics outlined e.g. in TS 22.369 [x] and TR 38.848 [2].

Editor’s Note 3: Further details FFS, if any.

AIoT RAN (AIoT Radio Access Network): hosts certain functions for AIoT as part of the functional split between RAN and CN.

Editor’s Note 4: Further details regarding AIoT functions hosted in the AIoT RAN and the respective functional split to be decided by RAN2, RAN3 and SA2.

AIoT radio: radio interface between AIoT device and Common reader function.

Editor’s Note 5: Further details on AIoT radio to be discussed by RAN1 and RAN2.

AIoT CN: hosts certain functions for AIoT as of the functional split between RAN and CN

Editor’s Note 6: Further details regarding AIoT functions hosted in the AIoT CN and the respective functional split to be decided by RAN2, RAN3 and SA2.

XX interface: interface between the AIoT RAN and the AIoT CN on which certain AIoT specific functions are performed.

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 1 it may only represent a single interface instance, e.g. a new interface between AIoT RAN and AIoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between AIoT CN and AIoT RAN, or one instance for NG alone.

Common reader function: a function that communicates with the AIoT device by means of AIoT radio.

Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2.

AIoT RAN node function: a function residing in AIoT RAN. It hosts the following functions: AIoT radio specific resource management, admission control of the AIoT radio resources used towards the AIoT device by the common reader function, coordination of AIoT transactions involving multiple common reader functions, coordination of AIoT transactions concerning multiple AIoT devices, termination of the XX interface.

Editor’s Note 9: further details are FFS, e.g. scalabilty aspects regarding the expected numbers of Common reader function instances in large scale networks.

Figure 6.3.2-1: Logical System Architecture for AIoT.

### 6.3.3	Deployment scenarios based on Logical System Architecture for AIoT

#### 6.3.3.1	General

This chapter introduces further terms necessary for discussions on deployment scenarios:

AIoT MT: function within the Common Reader Function deployed as a UE based AIoT reading device, communicating via Uu interface to its serving gNB.

Editor’s Note:	It is FFS whether and in which way communication to the serving gNB is performed, i.e. whether the gNB performs e.g. as a mere relay of information provided via Uu UP or e.g. AIoT information is provided via legacy NAS signalling or by other means.

AIoT radio frontend: communicates to the AIoT device by means of AIoT radio.

UE based AIoT reading device: Equivalent to a device containing the Common reader function in topology 2 deployments. This term is used to denote all functions necessary for AIoT specific operations.

AIoT enabled gNB: a gNB able to serve Common Reader Functions and hosting the AIoT RAN node function, while performing legacy gNB functions as well.

#### 6.3.3.2	Deployment scenarios of AIoT functions in NG-RAN

Topology 1:

For topology 1, one possible deployment is to place the AIoT specific functions within an AIoT-enabled gNB, which becomes part of the AIoT RAN. Coordination between non-AIoT- and AIoT-resources would become a gNB-internal matter. The interface towards the AIoT CN needs probably some more input from SA2 as the nature of the CN node towards AIoT RAN should connect is still to be determined. The resulting deployment scenario is depicted in Figure 2.4.3-1.

Editor’s Note:	FFS on further details regarding the XX interface and the AIoT CN with dependency on at least SA2.

Figure 6.3.3.2-1: AIoT RAN deployment scenario for topology 1.

Topology 2:

For topology 2, one possible deployment scenario is to place the AIoT RAN node function within an AIoT-enabled gNB. The Common reader function is placed in a UE based AIoT reading device (as defined in section 6.3.3.1) and further decomposed in an AIoT Mobile Termination (AIoT MT, as defined in section 6.3.3.1) and an AIoT radio front end (as defined in section 6.3.3.1).

The AIoT-enabled gNB serves the UE based AIoT reading device and the (NR) radio resources consumed by the UE based AIoT reading device. It is also connected to the AMF serving the UE based AIoT reading device.

Editor’s Note:	FFS on further details regarding the XX interface and the AIoT CN with dependency on at least SA2.

Figure 6.3.3.2-2: AIoT RAN deployment scenario for topology 2.

### 6.3.4	Protocol Stack Options based on Logical System Architecture for AIoT

#### 6.3.4.1	General

The following chapters depict different approaches for protocol stacks, to discuss the impact of terminating and processing AIoT specific communication in an end-to-end manner, i.e. from the application to the AIoT device. Only principles of exchange of information are shown, no details e.g. on transport layers are provided. The figures explicate transfer of DL application layer information by means of red arrows.

Further, topology 2 is explicity depicted by separating the Common Reader Function from the AIoT RAN node function in AIoT RAN. Aspects of topology 1 could be regarded as a subset from toplogy 2 aspects and derived by collapsing the 2 respective stacks into a single one with communication between the Common Reader Function and the AIoT RAN node function being a matter of implementation.

Protocol stacks for topology 2 for the UE based AIoT reading device are not depicted, only the stacks for AIoT related communication.

Functions performed at termination points of protocol are hinted in “call-outs”.

#### 6.3.4.2	Direct communication between the application function and the Common Reader Function.

This protocol stack option foresees any exchange of application data to be directly communicated between the application function and the common reader function, an option considered specifically for topology 2.

Figure 6.3.4.2-1: Protocol stacks for direct communication between AF and Common Reader Function.

For the sake of functional architecture considerations, the “transport options” are of secondary concern. There is the possibility to achieve the transport of application data transparently by means of control plane solutions (AIoT specific RRC/NGAP messages/IEs) or user plane solutions (estalblishment of specific PDU session), one set of papers also suggested introduction SBI, which may be regarded to fall into the same category of “XXAP transport” related discussions.

Common to this group of proposals is that they do not foresee any functional interaction within AIoT CN or AIoT RAN by means of explicit termination of AIoT specific protocol elements and triggering AIoT functions.

As shown Figure 6.3.4.2-1, there is especially no influence for AIoT functions foreseen in 5GS at all.

While such is possible to achieve, it is assumed to be outside the scope of studies to be performed in RAN WGs.

#### 6.3.4.3	Direct communication between the AIoT CN and the Common Reader Function.

The protocol stack figure, put on top of the logical architecture for this option may look like as follows:

Figure 6.3.4.3-1: protocol stacks for direct communication between AIoT-CN and Common Reader Function.

From an AIoT RAN point of view, if topology 2 is applied, there is no difference to the previous stack option in section 6.3.4.2, as no functional interaction can take place at the AIoT RAN node function. In case of topology 1, that is possible by implementation:

For topology 2, direct communication between the Common Reader Function and the AIoT CN does not allow AIoT RAN to perform admission control/RRM functions, unless an additional control plane structure - in parallel to XXAP – is foreseen, that first allows network side control of AIoT radio resources and only after AIoT radio resources are prepared in a controlled manner, XXAP and respective AIoT radio functions are performed.

Another aspect is that, for topology 2, this stack option would require the AIoT CN to control multi-reader operations, requiring the AIoT to select the readers applicable for an AIoT transaction, i.e. probably to translate a certain area scope as provided from the application into a set of readers. Given the envisage use case for topology 2, expecting a potentially large number of readers, may give a hint on the scalability of this stack option.

#### 6.3.4.4	Hop-by-hop communication between AIoT AF, AIoT CN, AIoT RAN node function and Common Reader Function

This protocol stack option foresees protocol terminations at the AIoT CN, at the AIoT RAN node function and the Common reader function. This allows triggering respective functions in each of these entities.

Figure 6.3.4.4-1: protocol stack for hop-by-hop communication for AIoT.

The main functional differences between topology 1 and topology 2 can be seen in Figure 6.3.4.4-1: while for topology 1, the interaction between the Common Reader Function and the AIoT RAN node function is a matter of implementation, for topology 2, RRC needs to provide means to transport application data and respective control information.

Terminating XXAP in AIoT RAN provides the possibility of creating a natural trigger point for performing AIoT RAN node level functions as discussed in the previous chapters.

It also provides the possibility of handling the potentially large amount of Common Reader Function (instances) and controlling them by AIoT RAN: AIoT RAN would only need to translate an application level/AIoT CN level area indicaton into a set of Common Reader Functions.

### 6.3.5	Interactions between new AIoT functions and other 5GS functions

Editor’s Note 1:	This chapter studies the interaction between AIoT functions and 5GS functions that are either impacted by AIoT or support AIoT operation. Such interactions are assumed to exist for both topologies.

Editor’s Note 2:	Further studies are needed on whether and how support of UE based AIoT reading devices in RRC_IDLE and RRC_INACTIVE is possible. For support of RRC_IDLE options are conceivable that require interaction between AIoT CN functions and the AMF serving the UE based AIoT reading device.

Editor’s Note 3:	Further studies are needed on whether and how the use and designation of AIoT radio resources within licensed spectrum requires interactions between AIoT RAN functions and legacy NG-RAN functions on existing interfaces.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>