3GPP TSG-RAN WG3 Meeting #125	R3-244389

Maastricht, The Netherlands, 19th – 23rd August 2024

Agenda Item:	16.3

Source:	Ericsson

Title:	[TP for TR 38.769] Elements for AIoT RAN – AIoT CN communication and overall signalling

Document for:	Discussions & Approval

# 1	Introduction

This document discusses further details on elements for AIoT RAN to AIoT CN signalling, not losing sight of the overall system surrounding, based on discussions and principles worked on in [3].

# 2	Discussion

## 2.1	General

The signalling scenarios attempt a kind of “uniform” signalling on XXAP, using a single class1 procedure to trigger the transaction towards AIoT RAN, regardless whether the operation is inventory, command, registration or anything else. In addition a class 2 procedure provides the opportunity of multiple responses. This seems to possible by indicating the nature of the transaction. Protocol-design wise, this approach should be “completed” by RAN and CN triggered termination of the transaction.

Further, it should be attempted to avoid multiple rounds of communication between the AIoT device and the AIoT CN in order to avoid the AIoT device to run out of energy, i.e. in case of e.g. command, all the information necessary to page the device (device ID) and perform the command transaction (application layer DL data) should be provided within the triggering class 1 procedure.

Further details to be looked at the provision of an transaction ID, which would be allocated by the AIoT CN, and should be passed on to the device in order to allow the device to distinguish the transactions.

As discussed in [3], expecting a multitude of Common reader function instances, it is probably better to abstract on XX the area within which an AIoT transaction should take place from the identification of the reader and let the AIoT RAN translate the indicated transaction area into “reader IDs”.

Last not least we also consider discussing the information that should be passed on by the Common reader function on AIoT radio in order to allow devices to get information on (1) the nature of the AIoT transaction, (2) the identification of the network, (3) the identification of the reader, (4) the identification of the transaction, (5) identification of the AIoT device(s) concerned by the AIoT transaction and additional application layer data.

## 2.2	Scenarios

### 2.2.1	Inventory, assuming “hop-by-hop” communication within 5GS

Figure 2.2.1-1: Inventory, assuming “hop-by-hop” communication within 5GS, example message flow

Figure 2.2.1-1 shows the assumed logical architecture (including the application function), assumed protocol stacks and information flow under the assumption that the end-to-end communication allows specific functions to be triggered in the AIoT RAN node function, as discussed in [3].

This example message flow consists of the following steps:

0.	From an application point of view, the 5GS acts as a server. AIoT CN would need to translate the request into 5GS internal identification of the targeted AIoT devices and the area, i.e the set of Common reader functions that can perform the AIoT transaction.

1.	The AIoT CN triggers the AIoT RAN to perform the AIoT transaction by means of XXAP signalling. The AIoT CN allocates a transaction ID, provides transaction area information and the targeted AIoT devices and indicates that the transaction is an inventory. The AIoT RAN performs admission control and trigger the transaction towards the common reader function.

2.	In case of topology 2, Figure 2.2.1-1 shows the UE based AIoT reading device to be allocated AIoT resources by means of RRC signalling, in case AIoT information is provided by means of a control-plane-based approach. 
Further, basic all relevant information as received via XXAP signalling is provided to the Common Reader function as well.

3,-6.	AIoT radio signalling takes place, showing the AIoT device responding with its allocated temporary 5G AIoT device ID (details subject to SA3 discussions).

Further signalling relays the device’s responds back to the originator of the AIoT transaction.

### 2.2.2	Command, assuming direct XXAP communication between the Common Reader Function and the AIoT CN for topology 2

Figure 2.2.2-1: Command, assuming direct XXAP communication between the Common Reader Function and the AIoT CN for topology 2

Figure 2.2.2-1 the option for topology 2 where direction communication of AIoT specific information towards the Common reader function/UE based AIoT reading device form the AIoT CN is foreseen.

As such option would not allow a “natural” trigger point for allocating AIoT radio resources by the AIoT RAN node function, Figure 2.2.2-1 shows a “solutions” that overcomes such missing trigger point, by adding additional steps (1a-1c) to perform an NGAP AIoT Resource setup similar to the NGAP PDU Session Resource Setup, which allows admission control and selection and control of the UE based AIoT reading devices concerned by the AIoT transaction. The AIoT enabled gNB serving the UE based AIoT reading device maps the transaction area information into the set of readers necessary to perform the transaction.

Note, that the device correlates the RRC allocation of AIoT resources and the trigger of the AIoT transaction by means of a AIoT CN allocated AIoT transaction ID, which is also communicated to the AIoT device.

Remaining steps are in analogy to the steps described in section 2.2.1.

### 2.2.3	Authentication and registration, assuming “hop-by-hop” communication within 5GS

Figure 2.2.3-1: Authentication and registration, assuming “hop-by-hop” communication within 5GS

Figure 2.2.3-1 shows a possible message flow for authentication and registration, under the assumption that 5G security and privacy principles and respective functions are applied for AIoT devices as well, which is pending respective SA3 studies.

What is important to look at in Figure 2.2.3-1 steps 11-14, that for authentication and registration (providing a security challenge and receiving the respective response, allocating a CN allocated AIoT temporary AIoT devie ID), requires additional signalling means on XX and Uu, similar to DL/NAS transport on RRC and NGAP.

Figure 2.2.3-1 also shows that such approach, based on 5G security and privacy principles requires the handling of concurrent AIoT device associated signalling, which may be performed by introducing a AIoT device context identification as indicated in that figure.

# 3	Conclusion and proposal

It is proposed to discuss message flows and protocol principles as suggested in section 2.

It is further proposed to agree on the text proposal for TR 38.769 [2] summarizing the principles of the suggested protocol aspects for XXAP and other protocols, as shown in section 5.

# 4	References

[1]	RP-240826 "Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR".

[2]	TS 38.848 "Study on Ambient IoT (Internet of Things) in RAN".

[3]	R3-244387 "On AIoT RAN functional and logical architecture".

# 5	Text Proposal for TR 38.769(currently based on agreed TP in R3-243972)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 6.4	Impacts on CN-RAN interface

Editor’s note: Corresponds to the first RAN3 objective in the SID, to identify necessary impacts on signaling and procedures for CN-RAN interface.

### 6.4.1	Information exchanged between AIoT CN and AIoT RAN

#### 6.4.1.1	Inventory

Inventory can be sent by the AIoT CN for a single device, or a group of devices, or all devices.

The Inventory Request from the AIoT CN to the AIoT RAN, may include the following:

(1)	AIoT Device Identification (to find a single device, a group of devices, or all devices)

Note 1:	The definition of this identification is out of RAN3 scope.

Editor’s Note 1: It is FFS whether AIoT RAS needs to interpret/store/process it.

(2)	Scope of inventory request (e.g., a certain area in which the inventory is to be triggered)

Multiple individual AIoT Device IDs (one ID per device) can be provided to the AIoT CN via a single Inventory Report.

Editor’s Note 2: It is up to SA2 whether device ID is sent transparent or not.

#### 6.4.1.2	Command

Command can be sent by the AIoT CN for a single device.

Editor’s Note 1: it is FFS for command on a group of devices, or all devices.

Editor’s Note 2: it is FFS whether AIoT RAN can remain agnostic of the type of request from the AIoT CN (need to differentiate command and inventory)

### 6.4.2	Example signalling flows

#### 6.4.2.1	Inventory, assuming a “hop-by-hop” communication within 5GS, option x

Figure 6.4.2.1-1: Inventory, assuming “hop-by-hop” communication within 5GS, example message flow

Figure 6.4.2.1-1 shows the assumed logical architecture (including the application function), assumed protocol stacks and information flow under the assumption that the end-to-end communication allows specific functions to be triggered in the AIoT RAN node function, as discussed in [3].

This example message flow consists of the following steps:

0.	From an application point of view, the 5GS acts as a server. AIoT CN would need to translate the request into 5GS internal identification of the targeted AIoT devices and the area, i.e the set of Common reader functions that can perform the AIoT transaction.

1.	The AIoT CN triggers the AIoT RAN to perform the AIoT transaction by means of XXAP signalling. The AIoT CN allocates a transaction ID, provides transaction area information and the targeted AIoT devices and indicates that the transaction is an inventory. The AIoT RAN performs admission control and trigger the transaction towards the common reader function.

2.	In case of topology 2, Figure 6.4.2.1-1 shows the UE based AIoT reading device to be allocated AIoT resources by means of RRC signalling, in case AIoT information is provided by means of a control-plane-based approach. 
Further, basic all relevant information as received via XXAP signalling is provided to the Common Reader function as well.

3,-6.	AIoT radio signalling takes place, showing the AIoT device responding with its allocated temporary 5G AIoT device ID (details subject to SA3 discussions).

Further signalling relays the device’s responds back to the originator of the AIoT transaction.

#### 6.4.2.2	Command, assuming direct XXAP communication between the Common Reader Function and the AIoT CN for topology 2, option y

Figure 6.4.2.2-1: Command, assuming direct XXAP communication between the Common Reader Function and the AIoT CN for topology 2

As direct communication between the Common reader function and the AIoT CN does not allow a “natural” trigger point for allocating AIoT radio resources by the AIoT RAN node function, Figure 6.4.2.2-1 shows that it is possible to overcome this by adding additional steps (1a-1c) to perform an NGAP AIoT Resource setup similar to the NGAP PDU Session Resource Setup, which allows admission control and selection and control of the UE based AIoT reading devices concerned by the AIoT transaction. The AIoT enabled gNB serving the UE based AIoT reading device maps the transaction area information into the set of readers necessary to perform the transaction.

Note, that the device correlates the RRC allocation of AIoT resources and the trigger of the AIoT transaction by means of a AIoT CN allocated AIoT transaction ID, which is also communicated to the AIoT device.

Remaining steps are in analogy to the steps described in section 6.4.2.1.

#### 6.4.2.3	Authentication and registration, assuming “hop-by-hop” communication within 5GS, option z

Figure 6.4.3-1: Authentication and registration, assuming “hop-by-hop” communication within 5GS

Figure 6.4.2.3-1 shows a possible message flow for authentication and registration, under the assumption that 5G security and privacy principles and respective functions are applied for AIoT devices as well, which is pending respective SA3 studies.

What is important to look at in Figure 6.4.2.3-1 steps 11-14, that for authentication and registration (providing a security challenge and receiving the respective response, allocating a CN allocated AIoT temporary AIoT devie ID), requires additional signalling means on XX and Uu, similar to DL/NAS transport on RRC and NGAP.

Figure 6.4.2.3-1 also shows that such approach, based on 5G security and privacy principles requires the handling of concurrent AIoT device associated signalling, which may be performed by introducing a AIoT device context identification as indicated in that figure.

### 6.4.3	Resulting protocol functions

#### 6.4.3.1	Definitions

AIoT transaction:	Any kind of interaction between the 5GS and the AIoT device (e.g. inventory, command etc.).

Editor’s Note:	It is assumed that an AIoT device should become aware whether it has already reacted to an AIoT transaction, which could be achieved by providing an AIoT transaction ID (e.g. allocated by the AIoT CN) to distinguish different AIoT transactions from each other.

#### 6.4.3.2	XXAP protocol structure and other related protocols

XXAP

The AIoT CN triggers an AIoT transaction by issuing the class 1 XXAP: AIoT Transaction Request procedure which causes the AIoT RAN to page the AIoT device(s). This procedure interacts with a class 2 procedure, triggered by the AIoT RAN, to provide the result of the AIoT Transaction Request procedure (AIoT Transaction Report). Different kinds of AIoT transactions (inventory, command, etc) are distinguished by an explicit indication and a respective parameter set.

The XXAP AIoT Transaction Request procedure is accompanied by AIoT RAN and AIoT CN triggered AIoT Transaction Release procedures to terminate the AIoT transaction (FFS).

For command, the message triggereing the AIoT Transaction Request procedure preferably carries information to page the AIoT device and command related application-level data.

It is expected that AIoT CN does not necessarily need to know the Common reader functions to be involved in the AIoT transaction, but rather to provide an area indication within which the AIoT transaction shall be performed. The AIoT RAN is expected to identify the Common reader functions associated to the indicated transaction area.

AIoT radio

Overall 5GS functions, requiring the identification of the network and other AIoT transaction related information necessary to be provided to the AIoT device encompass the following: (1) the nature of the AIoT transaction, (2) the identification of the network, (3) the identification of the reader, (4) the identification of the transaction, (5) identification of the AIoT device(s) concerned by the AIoT transaction.

NGAP

NGAP might be impacted in case direct communication between the AIoT CN and Common reader function is performed for topology 2. Could be achieved by introducing a class 1 NGAP AIoT Resource Setup Request procedure to require allocation of AIoT resources.

RRC

RRC is impacted to provide respective RRC protocol functions for allocating of AIoT resources resources for topology 2 and triggering the AIoT transaction on AIoT radio, probably involving carrying application-level AIoT information.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>