3GPP TSG-RAN WG3 Meeting #125	R3-244390

Maastricht, The Netherlands, 19th – 23rd August 2024

Agenda Item:	16.4

Source:	Ericsson

Title:	[TP for TR 38.769] On locating AIoT devices

Document for:	Discussions & Approval

# 1	Introduction

This document provides a text proposal for TR 38.769 which contains reasoning for the proposed potential approaches and the approaches themselves.

It is evident, that applying proximity methods for locating an AIoT device is not sufficient to avoid additional human interaction, as the communication range for indoor scenarios, as envisaged in the Rel-19 SID, is about a couple of tens of meters. Therefore, the approaches introduced hereafter foresee multiple readers’ involvement in the location process.

# 2	Text Proposal for TR 38.769 (currently based on agreed TP in R3-243972)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 6.8	Locating ambient IoT devices

Editor’s note 1: Proximity determination may be in a 6.8.x sub-clause, or another arrangement, depending on how the study proceeds.

Editor’s note 2: Corresponds to the third RAN3 objective in the SID, to identify potential solutions for locating an Ambient IoT device with no specification impact, e.g., reusing existing user location report, or minimal specification impact to convey location information to core network.

### 6.8.1	General

AIoT device location information may be used for the following purposes:

(a) improving the AIoT operation itself, e.g. by AIoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the AIoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

Assuming that the Common reader function’s location is available, providing an AIoT device location “Reader ID granularity” may only give a rough approximation of the AIoT device’s location, still requiring (human) labour intensive final search of goods equipped with an AIoT device, probably in a 3D-environment.

For use case (a), if the UE is able to respond to more than one Common reader function instance, the AIoT RAN may report more than one reader ID to the AIoT CN, which may use.

For use case (b), further reduction of the enhancement of the locating accuracy may be achieved by involving multiple Common reader instances, measuring e.g. the power of the backscattered signal or by varying the outputpower of the carrier wave.

For use case (b), topology 2, it is assumed that 5GS is able to locate the UE based AIoT reading device by means of existing positioning functions; in case of topology 1, locating the reader can be seen as a matter of deployment.

Editor’s Note 2: Signalling details on how to provide the location information of the AIoT device to the AIoT CN needs further study.

### 6.8.2	Improving AIoT Operation using AIoT Device Location Information

AIoT device location information is useful in AIoT transactions which are based on the 5GS’s knowledge about the AIoT device’s reachability. This requires the 5GS to provide means to register the AIoT device.

For AIoT transactions subsequent to the registration, which foresee to page the AIoT device (i.e. for passive AIoT devices), reducing the effort to reach the AIoT device is of general advantage, as for UEs. CM-IDLE paging optimisation for UEs rely basically on (periodical or mobility induced) Registration Area Update and memorising e.g. the last serving cell by the AMF.

Equivalent approaches as for UEs can be foreseen for AIoT devices:

1.	Memorising the last serving reader by the AIoT CN, potentially a set of readers involved in the AIoT transaction, e.g. if the R2D communication is performed by a different reader than the D2R communication.

2.	Providing the network identity to the AIoT device and require it to re-register in case the network identity differs from the one indicated to the AIoT device in the previous AIoT transaction.

3.	Regularly/periodically causing AIoT devices re-register in order to keep the 5GS updated about the devices’ current location. It might be sufficient to operate AIoT transactions on the identity of the last serving reader, without the necessity to introduce a new AIoT area concept. This would require all AIoT devices to memorise the last serving reader’s ID and to react accordingly if a re-registration request is triggered by the network.

### 6.8.3	Providing AIoT Device Location Information to Consumer of AIoT Operation

#### 6.8.3.1	General

Reporting the proximity of an AIoT device to a reader may be sufficient in certain cases. However, given the indoor deployment scenarios envisaged during the Rel-19 studies assuming a range of tens of meters between the device and the reader, would still require additional manual labour to be spent, e.g. to exactly locate the goods to which the device is attached. It is assumed that proximity indication is not sufficient for typical AIoT uses cases.

Several approaches are conceivable, all characterised by the involvement of several readers and the reader’s ability to measure either the backscattered power (from a passive device) or the signal strength received from devices of higher complexity.

AIoT device location information may be used for the following purposes:

#### 6.8.3.x	Potential approach x – multiple inventories for the same location transaction

This approach foresees the AIoT RAN to trigger AIoT device inventory by means of different readers, all able to communicate to the AIoT device. All inventory instances are part of the same AIoT transaction.

This approach would foresee the AIoT RAN node function to compute the AIoT device location, based on the known readers’ locations and the measurements received from the readers.

This approach requires

a.	The AIoT device to react on several inventory instances belonging to the same AIoT transaction.

b.	In case of topology 2, the UE based AIoT reading device, to report e.g. the measured reflection, which represents a new function on RRC. In case of topology 1, reporting is a matter of implementation.

c.	AIoT RAN to schedule the individual inventories in a way that they do not interfere each other. It is assumed that passive/low complexity devices are not able to respond simultaneously to individual inventories, it is also assumed, that passive/low complexity devices would need each reader in its “reach” to mute at any given AIoT operation on AIoT radio.

Figure 6.8.3.x-1 shows a schematical example flow, involving 3 common reader function instances:

Figure 6.8.3.x-1: AIoT device locating by means of multiple inventory instances

#### 6.8.3.y	Potential approach y – multiple readers as “listeners” to a single inventory

This approach foresees the AIoT RAN to trigger a single AIoT device inventory by means of one reader, but to schedule AIoT radio resource configuration at suitable readers to measure the strength of the backscattered signal (in case of passive/low complexity AIoT devices) or the strength of the signal transmitted by higher complexity AIoT devices.

This approach foresees the calculation of the AIoT device location to take place at the AIoT RAN node function.

This approach requires:

a.	no specific functions implemented at the AIoT device.

b.	In case of topology 2, the UE based AIoT reading device, to report e.g. the measured reflection, which represents a new function on RRC. In case of topology 1, reporting is a matter of implementation.

c.	AIoT RAN to schedule the inventory in a way that they do not interfere with other transactions, however, it is likely that passive devices would require “silence/muting” in their vicinity, as they would not be able to distinguish CW excitations and respective information provision at AIoT paging when submitted by several readers in their vicinity (FFS in RAN1 whether FDMA in R2D signal is possible.)

Figure 6.8.3.y-1 shows a schematical example flow, involving 3 common reader function instances:

Figure 6.8.3.y-1: AIoT device locating by means of multiple “listening” readers

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>