3GPP TSG-RAN WG3 Meeting #125	R3-244391

# Maastricht, Netherlands, August 19 – 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR | draft | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing procedure description for pre-configured SRS activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Ericsson, Nokia, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As specified in TS 38.331 and TS 38.305, a UE may be pre-configured with one or more SRS for positioning configurations with validity area. A UE may send a RRC Resume Request message to the receiving gNB to request activation of the pre-configured SRS for positioning. However, a procedure description for this functionality is currently missing in Stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Area-specific SRS Configuration Activation Procedure description is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing Stage 2 support for area-specific SRS configuration activation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.14.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 38.423 CR 1246 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR 38.455 CR 0140 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 7.14.x	Area-specific SRS Configuration Activation Procedure

Figure 7.14.x-1 shows the Area-specific SRS Configuration Activation procedure.

Figure 7.14.x-1: Area-specific SRS Configuration Activation Procedure

0.	The UE in RRC_INACTIVE state is pre-configured with one or more area-specific SRS for positioning configurations as specified in clause 7.14.2 and receives a request from upper layers to activate a pre-configured SRS for positioning when the UE is camped on one of the cells indicated in the validity area(s) (e.g., when a configured periodic or triggered event is detected).

1.	The UE sends a RRC Resume Request message with the resume cause "srs-PosConfigOrActivationReq" to request activation of the pre-configured SRS.

2.	The receiving gNB which receives the request from the UE triggers the Retrieve UE Context procedure towards the last serving gNB and includes the pre-configured SRS activation request in the Retrieve UE Context Request message.

3.	The last serving gNB sends a NRPPa Positioning Information Update message to the LMF including the Cell ID of the receiving gNB where the UE resumes from together with a pre-configured SRS activation indication to notify the LMF that the pre-configured SRS is activated in the UE.

4.	The last serving gNB relocates the full UE context to the receiving gNB and includes the pre-configured SRS for positioning configuration(s), the Routing ID and the NRPPa Transaction ID in the Retrieve UE Context Response message.

NOTE:	If the last serving gNB decides not to relocate the full UE context, the last serving gNB provides a Retrieve UE Context Failure message to the receiving gNB, including a RRC Release message with the pre-configured SRS for positioning configuration and steps 5, 6, and 8 below are not performed.

5.	The receiving gNB triggers the Path Switch Request procedure towards the AMF.

6.	The AMF responds with the Path Switch Request Acknowledge.

7.	The receiving gNB activates the pre-configured SRS for positioning (e.g., via Positioning SRS Activation/Deactivation MAC CE) and releases the UE to RRC_INACTIVE state.

8.	The receiving gNB indicates the last serving gNB to release the UE context.