3GPP TSG-RAN WG3 Meeting #125	R3-244396

Maastricht, Netherlands, 19th – 23th August 2024

Agenda Item:	12.3

Source: 				LG Electronics Inc.

Title:			(TP for TR 38.799) Remaining open issues in 5G Femto

Document for:		Discussion and Decision

1.	Introduction

In order to complete this study item on NR Femto, the architecture options for NG and Xn should be down-selected for normative phase. Also, there is remaining open issue on access control for NR Femto. In this contribution, we examine these issues and then provide our view on it.

2.	Discussion

2.1 Architecture options for NG interface

RAN3 captured four architecture options for NG interface and initially evaluated the pros and cons of these options in TR 38.799. According to the initial evaluation, the NR Femto node in Option 1 can be easily deployed in the network because it does not require any architecture change. Also, it has less latency because there is no GW between NR Femto node and 5GC. However, it is not suitable for the case where the NR Femto nodes are frequently switched on/off. We think that the cons of Option 1 can be covered by the deployment of Option 2. That is, Option 1 and 2 are complementary to each other. Basically, the NR Femto nodes can be deployed as in Option 1. That is, it directly connects to the 5GC as a gNB via the existing NG interface. However, for the deployment scenarios with large number of NR Femto nodes or frequent switch on/off of NR Femto nodes, the operator may deploy the NR Femto GW to concentrate the NG-C interface. As described in Clause 5.2.3, Option 2 has some specification impact. However, this impact can be easily identified because 4G Femtos can used as a baseline. As a result, both options can be considered as a basis for normative work.

Proposal 1: Options 1 and 2 are used as a basis for normative work.

Option 3 does not have a NGAP impact because the SCTP concentrator is located at the transport layer. However, some new functionalities (e.g., mapping NGAP signaling to each SCTP stream association) on SCTP layer needs to be supported at the transport layer. Option 4 is intended to use the F1 interface to support the NR Femto. However, the RAN3 did not consider the use of F1 interface to support the NR Femto. For example, the F1 interface is not suitable for the deployments with frequent switch on/off of NR Femtos. Hence, it is required to check whether/how to enhance the F1 interface for the support of the NR Femto. However, there is no enough time to identify the F1 specification impact on how to support Option 4. Note also that both options cannot provide the evolution path for operators as well.

Proposal 2: Options 3 and 4 are ruled out.

2.2 Architecture options for Xn interface

In TR 38.799, there are two architectural options for the Xn interface. In Option A, the NR Femto Node directly connects to the other NR Femto Node(s)/gNB(s) as a gNB via the existing Xn interface. According to RAN3 agreement, Option A does not have any architecture impact in 3GPP. Also, there is no critical requirement on the concentration of the XnAP signalling because there are the limited number of surrounding gNBs. Hence, Option A can be considered as baseline for a normative work.

Option B introduces the Xn GW equivalent to the X2 GW in 4G. This option can support the deployment scenarios with large number of the surrounding gNBs. However, as mentioned above, this deployment scenario does not seem to be realistic from Xn point of view. This option may also cause additional processing delay and latency because of the concentration. However, this option can provide the evolution path for operators deploying the X2 GW in 4G. Therefore, we think that Option B can be used as an optional method.

Proposal 3: Options A and B are used as a basis for normative work.

2.3 Access control

In last RAN3 meeting, the following agreements were made:

- Initial access control to a CAG supported by a NR Femto node is performed by the AMF, reusing current PNI-NPN functionality.

- Reusing CAG configurations and mobility functionalities specified for PNI-NPN also for NR Femto nodes deployments. Whether enhancements to legacy CAG functionalities are needed is for further discussion.

However, the corresponding TPs on the access control are still missing in the TR 38.799. Therefore, it is needed to capture the corresponding TPs on RAN3 agreements into the TR 38.799

Observation 1: It is needed to capture the corresponding TPs on the access control into the TR 38.799.

For the access control, the remaining open issue is whether/how to consider applicability of use cases equivalent to those in 4G, i.e. open mode, closed mode, hybrid mode. In LTE, HeNB can be configured/operated as three access modes as follows:

- Open access mode: HeNB operates as normal PLMN cell. All UEs can access to this cell.

- Hybrid access mode: HeNB operates as hybrid cell. When the CSG ID of the hybrid cell belongs to the Permitted CSG list of the CSG-capable UE, this cell is considered by the CSG-capable UE as a CSG cell. For other UEs, this cell is the non-CSG cells. Therefore, the legacy UE is allowed to access to this cell.

  - Hybrid cell [1]: A cell broadcasting a CSG indication that is set to FALSE and a specific CSG ID.

- Closed access mode: HeNB operates as CSG cell. When the CSG ID of the cell belongs to the Permitted CSG list of the CSG-capable UE, the CSG-capable UE can access to this CSG cell. However, legacy UE is not allowed to access this cell.

  - CSG cell [1]: A cell broadcasting a CSG indication that is set to TRUE and a specific CSG ID.

RAN3 has already agreed that the PNI-NPN functionality can be used as a basis for the NR Femto. In PNI-NPN, the CAG concept is defined to prevent the UE, which are not allowed to access the network via the associated cell. According to TS 38.331 [2], the gNB can also broadcast the open cell, NPN cell, or NPN-only Cell as follows:

- NPN-only Cell (similar to CSG cell) [2]: A cell that is only available for normal service for NPNs' subscriber. An NPN-capable UE determines that a cell is NPN-only Cell by detecting that the cellReservedForOtherUse IE is set to true while the npn-IdentityInfoList IE is present in CellAccessRelatedInfo.

- NPN cell (similar to hybrid cell): An NPN-capable UE determines that a cell is NPN Cell by detecting that the cellReservedForOtherUse IE is set to false while the npn-IdentityInfoList IE is present in CellAccessRelatedInfo. For other UEs, this is a non-CAG cell.

- Open cell (similar to open cell in LTE): It is a normal PLMN cell.

Therefore, we think that three access modes (i.e., open, hybrid, closed) in HeNB can be supported in NR Femto by using the existing CAG functionality.

Proposal 4: Three access modes (i.e., open, hybrid, closed) in HeNB should be supported in NR Femto by using the existing CAG functionality.

In order to capture the above observation and proposals, the following proposal is also suggested to RAN3:

Proposal 5: It is proposed to agree the corresponding TPs in Appendix.

3.	Conclusion

In this contribution, we focused on the down-selection of architecture options for NG/Xn and remaining open issue on access control. Then we provided our view on it. The following proposals are kindly suggested to RAN3:

Proposal 1: Options 1 and 2 are used as a basis for normative work.

Proposal 2: Options 3 and 4 are ruled out.

Proposal 3: Options A and B are used as a basis for normative work.

Proposal 4: Three access modes (i.e., open, hybrid, closed) in HeNB should be supported in NR Femto by using the existing CAG functionality.

Proposal 5: It is proposed to agree the corresponding TPs in Appendix.

4.	Reference

- TS 36.304, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode,” v18.2.0.

- TS 38.331, “NR; Radio Resource Control (RRC) protocol specification,” v18.2.0.

5.	Appendix: TP for TR 38.799

This appendix provides the Text proposal for TR 38.799 based on the proposals of this contribution.

----------------Start of the First Change---------------

## 5.3	Access control

Editor Note: Study how to define the 5G access control mechanism by (re-)using the existing CAG functionality and identify needed enhancements (if any)

Reusing CAG configurations and mobility functionalities specified for PNI-NPN also for NR Femto nodes deployments. Whether enhancements to legacy CAG functionalities are needed can be decided in normative phase.

Initial access control to a CAG supported by a NR Femto node is performed by the AMF, reusing current PNI-NPN functionality. Also, three access modes (i.e., open, hybrid, closed) in HeNB can be supported in NR Femto by using the existing CAG functionality.

## 5.4	Local services access

Editor Note: Clarify the access to local services from the 5G Femto via collocated local UPF and identify issues, if any

## 5.5	Conclusions

During the study phase, four architecture options for NG interface are captured into Clause 5.2.1. However, as evaluated in Clause 5.2.3, Options 3 and 4 are out-of-scope of this study. Therefore, it is recommended that Options 1 and 2 as described in Clauses 5.2.1.1 and 5.2.1.2 are used as the basis for normative work. Also, it is concluded that regarding the architecture for the Xn interface, Options A and B as described in Clause 5.2.2 are used as the basis for normative work.

Initial access control to a CAG supported by a NR Femto node is performed by the AMF, reusing current PNI-NPN functionality. Also, three access modes (i.e., open, hybrid, closed) in HeNB can be supported in NR Femto by using the existing CAG functionality.

Reusing CAG configurations and mobility functionalities specified for PNI-NPN also for NR Femto nodes deployments. Whether enhancements to legacy CAG functionalities are needed can be decided in normative phase.

-----------------End of the First Change---------------