3GPP TSG-RAN WG3 Meeting #125	R3-244397

# Maastricht, Netherlands, August 19 – 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0162 | rev | - | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to SRS for positioning aggregation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The IE SRS Configuration (9.2.28) provides a list of SRS carrier defined by SRS Carrier List, where each item provides the SRS Config (incl. Positioning SRS Resource List and Positioning SRS Resource Set List) per carrier and active UL BWP. The aggregation information is implemented via IE Aggregated Positioning SRS Resource Set List in IE Positioning SRS Resource Set (9.2.32). However, the SRS aggregation is across carrier, and not across sets of a single carrier. Therefore, the Aggregated Positioning SRS Resource Set List must be at the same level as the SRS Carrier List. - The IE Aggregated Positioning SRS Resource Set List (9.2.94) provides the aggregated Positioning SRS Resource Set IDs together with Point A (possibly together with a PCI). However, the Point A (and PCI) alone does not define an SRS carrier. - The IE Aggregated Positioning SRS Resource Set List (9.2.94) can provide up to 48 aggregated/linked SRS Resource Set IDs. However, RRC can configure up to 2 aggregation groups, each with up to 3 x 16 = 48 SRS Resource Set IDs. Therefore, there can be up to 2 (groups) x [16 (Set IDs per carrier) x 3 (carrier) ] aggregated SRS for Positioning Resource Sets (see SRS-PosResourceSetAggBWCombinationList-r18 and SRS-PosResourceSetLinkedForAggBWList-r18 in RRC). - Aggregated semi-persistent SRS for positioning can be activated/deactivated via the "Aggregated SP Positioning SRS Activation/Deactivation MAC CE". This MAC CE allows the activation/deactivation of SRS resource set(s) in one or two or three of three aggregated carriers, where (in case of activation) a single spatial relation is indicated by the MAC CE for each of two or three aggregated SRS resources. - The activation is implemented in the POSITIONING ACTIVATION REQUEST message via IE Aggregated Positioning SRS Resource Set List. However, the CHOICE SRS type includes a single SRS Resource Set ID and it is unclear to which carrier the SRS Resource Set ID refers to (the SRS Resource Set ID is unambiguous within a single UL BWP only). - The deactivation is implemented in the POSITIONING DEACTIVATION message where a single SRS Resource Set ID can be deactivated. However, it is unclear to which carrier the SRS Resource Set ID refers to. In addition, only a single SRS Resource Set ID can be deactivated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The Aggregated Positioning SRS Resource Set List IE is moved into the SRS Configuration IE, indicating the linked SRS Resource Sets across carrier in the SRS Carrier List. - The SCS Specific Carrier IE is added to the Aggregated Positioning SRS Resource Set List to define the carrier to which the Positioning SRS Resource Set ID belongs to. - The value for maxnoaggregatedPosSRS-ResourceSets is changed to 96. - It is clarified that the SRS Resource Set ID for the Semi-persistent CHOICE in the POSITIONING ACTIVATION REQUEST is ignored when Aggregated Positioning SRS Resource Set List is present. - An additional CHOICE is added to the Abort Transmission in the POSITIONING DEACTIVATION message to deactivate aggregated positioning SRS Resource Set(s). Impact analysis: Impact assessment towards the previous version of the specification (same release): The CR has a protocol and function impact. The change is isolated to SRS with bandwidth aggregation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SRS for Positioning Aggregation is not aligned with RAN1/2 specifications and cannot be proper supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.9.2, 8.2.10.2, 9.1.1.17, 9.1.1.20, 9.2.28, 9.2.32, 9.2.94, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 38.473 CR xxxx |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.2.9	Positioning Activation

#### 8.2.9.1	General

The Positioning Activation procedure is initiated by the LMF to request the NG-RAN node to activate semi-persistent or trigger aperiodic UL SRS transmission by the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.9.2	Successful Operation

Figure 8.2.9.2-1: Positioning Activation procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING ACTIVATION REQUEST message to the NG-RAN node.

For semi-persistent UL SRS, the POSITIONING ACTIVATION REQUEST message includes an indication of the UL SRS resource set to be activated and may include the spatial relation for the semi-persistent UL SRS resource to be activated. For aperiodic UL SRS, if the SRS Resource Trigger IE is included in the POSITIONING ACTIVATION REQUEST message, the NG-RAN node shall take the value of this IE into account when triggering aperiodic SRS transmission by the UE.

If the Activation Time IE is included in the POSITIONING ACTIVATION REQUEST message, the NG-RAN node shall take the indicated value as the LMF’s requested time for activation of the UE’s SRS transmission.

Following successful activation of UL SRS transmission in the UE, the NG-RAN node shall respond with a POSITIONING ACTIVATION RESPONSE message. If the POSITIONING ACTIVATION RESPONSE message includes the System Frame Number and/or the Slot Number IEs, the LMF shall consider that the respective information indicates the activation time of SRS transmission by the UE.

If the Aggregated Positioning SRS Resource Set List IE is included in POSITIONING ACTIVATION REQUEST message and SRS bandwidth aggregation is configured in the UE, the NG-RAN node shall, if supported, activate the indicated SRS resource set(s) in the UE.

[…]

### 8.2.10	Positioning Deactivation

#### 8.2.10.1	General

The Positioning Deactivation procedure is initiated by the LMF to indicate to the NG-RAN node that UL SRS transmission should be deactivated in the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.10.2	Successful Operation

Figure 8.2.10.2-1: Positioning Deactivation procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING DEACTIVATION message to the NG-RAN node. This message shall include an indication of the UL SRS resource set(s) to be deactivated or release all the related resources.

#### 9.1.1.17	POSITIONING ACTIVATION REQUEST

This message is sent by the LMF to cause the NG RAN node to activate/trigger UL SRS transmission by the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| CHOICE SRS type | M |  |  |  | YES | reject |
| >Semi-persistent |  |  |  |  |  |  |
| >>SRS Resource Set ID | M |  | 9.2.33 | This IE is ignored if the Aggregated Positioning SRS Resource Set List IE is present. | - |  |
| >>SRS Spatial Relation | O |  | Spatial Relation Information 9.2.34 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. | YES | ignore |
| >>Spatial Relation Information per SRS Resource | O |  | 9.2.60 |  | YES | ignore |
| >Aperiodic |  |  |  |  |  |  |
| >>Aperiodic | M |  | ENUMERATED(true,…) |  | - |  |
| >>SRS Resource Trigger | O |  | 9.2.35 |  | - |  |
| Activation Time | O |  | Relative Time 1900 9.2.36 | Indicates the start time when the SRS activation is requested | YES | ignore |
| Aggregated Positioning SRS Resource Set List | O |  | 9.2.94 |  | YES | ignore |

[…]

#### 9.1.1.20	POSITIONING DEACTIVATION

This message is sent by the LMF to cause the NG RAN node to deactivate UL SRS transmission or release all the transmission by the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| CHOICE Abort Transmission | M |  |  |  | YES | ignore |
| >Deactivate SRS Resource Set |  |  |  |  |  |  |
| >>SRS Resource Set ID | M |  | 9.2.33 |  | - |  |
| >Release ALL |  |  | NULL | the NG-RAN node configures UE to stop transmitting SRS for the positioning purpose |  |  |
| >Deactivate aggregated SRS Resource Set |  |  |  |  |  |  |
| >>Aggregated Positioning SRS Resource Set List | M |  | 9.2.94 |  | YES | ignore |

### 9.2.28	SRS Configuration

This information element contains the SRS configuration configured by the NG-RAN node for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SRS Carrier List |  | 1 |  |  | - |  |
| >SRS Carrier List Item |  | 1..<maxnoSRS-Carriers> |  |  | - |  |
| >>Point A | M |  | INTEGER (0..3279165) | NR ARFCN | - |  |
| >>Uplink Channel BW-PerSCS-List |  | 1 |  | Corresponds to sCS-SpecificCarrierList in TS 38.331 [13] | - |  |
| >>>SCS Specific Carrier |  | 1. .<maxnoSCSs> |  |  | - |  |
| >>>>Offset To Carrier | M |  | INTEGER(0..2199,…) | First usable RB to Point A in the number of PRBs | - |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120,…, kHz480, kHz960) |  | - |  |
| >>>>Carrier Bandwidth | M |  | INTEGER(1..275,…) |  | - |  |
| >>Active UL BWP |  | 1 |  | Only the configuration in the active UL BWP is needed. | - |  |
| >>>Location And Bandwidth | M |  | INTEGER(0..37949,…) | Corresponds to information provided in locationAndBandwidth contained in BWP IE as defined in TS 38.331 [13] | - |  |
| >>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120,…, kHz480, kHz960) |  | - |  |
| >>>Cyclic Prefix | M |  | ENUMERATED(Normal, Extended) |  | - |  |
| >>>Tx Direct Current Location | M |  | INTEGER(0..3301,…) |  | - |  |
| >>>Shift7dot5kHz | O |  | ENUMERATED(true,…) |  | - |  |
| >>>SRS Config |  | 1 |  | Corresponds to information provided in SRS-Config IE as defined in TS 38.331 [13] | - |  |
| >>>>SRS Resource List |  | 0..<maxnoSRS-Resources> |  |  | - |  |
| >>>>>SRS Resource | M |  | 9.2.29 | Corresponds to information provided in SRS-Resource contained in SRS-Config IE as defined in TS 38.331 [13] | - |  |
| >>>>Positioning SRS Resource List |  | 0..< maxnoSRS-PosResources> |  |  | - |  |
| >>>>>Positioning SRS Resource | M |  | 9.2.30 | Corresponds to information provided in SRS-PosResource contained in SRS-Config IE as defined in TS 38.331 [13] | - |  |
| >>>>SRS Resource Set List |  | 0..<maxnoSRS-ResourceSets> |  |  | - |  |
| >>>>>SRS Resource Set | M |  | 9.2.31 | Corresponds to information provided in SRS-ResourceSet contained in SRS-Config IE as defined in TS 38.331 [13] | - |  |
| >>>>Positioning SRS Resource Set List |  | 0..<maxnoSRS-PosResourceSets> |  |  | - |  |
| >>>>>Positioning SRS Resource Set | M |  | 9.2.32 | Corresponds to information provided in SRS-PosResourceSet contained in SRS-Config IE as defined in TS 38.331 [13] | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID of the cell that contains the SRS carrier | - |  |
| Aggregated Positioning SRS Resource Set List | O |  | 9.2.94 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-Carriers | Maximum no of carriers for SRS. Value is 32. |
| maxnoSCSs | Maximum no of SCS spacings for a carrier. Value is 5. |
| maxnoSRS-Resources | Maximum no of SRS resources per UL BWP. Value is 64. |
| maxnoSRS-PosResources | Maximum no of positioning SRS resources per UL BWP. Value is 64. |
| maxnoSRS-ResourceSets | Maximum no of SRS resource sets per UL BWP. Value is 16. |
| maxnoSRS-PosResourceSets | Maximum no of positioning SRS resource sets per UL BWP. Value is 16. |

### 9.2.32	Positioning SRS Resource Set

This information element indicates a positioning SRS resource set in the UE for UL SRS transmission.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Positioning SRS Resource Set ID | M |  | INTEGER(0..15) |  | - |  |
| Positioning SRS Resource ID List |  | 1..<maxnoSRS-PosResourcePerSet> |  |  | - |  |
| >Positioning SRS Resource ID | M |  | INTEGER(0..63) |  | - |  |
| CHOICE Resource Type | M |  |  |  | - |  |
| >periodic |  |  |  |  |  |  |
| >>PosperiodicSet | M |  | ENUMERATED(true,…) |  | - |  |
| >semi-persistent |  |  |  |  |  |  |
| >>Possemi-persistentSet | M |  | ENUMERATED(true,…) |  | - |  |
| >aperiodic |  |  |  |  |  |  |
| >>SRS Resource Trigger | M |  | INTEGER(1..3) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-PosResourcePerSet | Maximum no of positioning SRS resources per positioning SRS resource set. Value is 16. |

### 9.2.94	Aggregated Positioning SRS Resource Set List

This information element is used to indicate the aggregated Positioning SRS Resource Set List.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Aggregated Positioning SRS Resource Set Item |  | 1.. < maxnoaggregatedPosSRS-ResourceSets> |  |  |
| >Point A | M |  | INTEGER (0..3279165) | NR ARFCN |
| >NR PCI | O |  | INTEGER(0..1007) |  |
| >Positioning SRS Resource Set ID | M |  | INTEGER(0..15) |  |
| >SCS Specific Carrier |  | 1 |  |  |
| >>Offset To Carrier | M |  | INTEGER (0..2199) |  |
| >>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, … , kHz480, kHz960) |  |
| >>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoaggregatedPosSRS-ResourceSets | Maximum no of aggregated SRS Positioning Resource Sets. Value is 96. |

### 9.3.5	Information Element definitions

[...]

-- A

AbortTransmission ::= CHOICE {

deactivateSRSResourceSetID		SRSResourceSetID,

releaseALL						NULL,

choice-extension							ProtocolIE-Single-Container { { AbortTransmission-ExtIEs } }

}

AbortTransmission-ExtIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-PosSRSResourceSet-Aggregation-List	CRITICALITY ignore TYPE PosSRSResourceSet-Aggregation-List		PRESENCE mandatory },

...

}

[...]

-- P

[...]

PosSRSResourceID-List ::= SEQUENCE (SIZE (1..maxnoSRS-PosResources)) OF SRSPosResourceID

PosSRSResourceSet-List ::= SEQUENCE (SIZE (1..maxnoSRS-PosResourceSets)) OF PosSRSResourceSet-Item

PosSRSResourceIDPerSet-List ::= SEQUENCE (SIZE (1..maxnoSRS-PosResourcePerSet)) OF SRSPosResourceID

PosSRSResourceSet-Item ::= SEQUENCE {

possrsResourceSetID				INTEGER(0..15),

possRSResourceIDPerSet-List		PosSRSResourceIDPerSet-List,

posresourceSetType				PosResourceSetType,

iE-Extensions		ProtocolExtensionContainer { { PosSRSResourceSet-Item-ExtIEs} }	OPTIONAL,

...

}

PosSRSResourceSet-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PosSRSResourceSet-Aggregation-List ::= SEQUENCE (SIZE (1.. maxnoaggregatedPosSRS-ResourceSets)) OF PosSRSResourceSet-Aggregation-Item

PosSRSResourceSet-Aggregation-Item ::= SEQUENCE {

pointA				INTEGER (0..3279165),

pCI-NR				INTEGER(0..1007)		OPTIONAL,

possrsResourceSetID	INTEGER(0..15),

scs-SpecificCarrier	SCS-SpecificCarrier,

iE-Extensions		ProtocolExtensionContainer { { PosSRSResourceSet-Aggregation-Item-ExtIEs} }	OPTIONAL,

...

}

PosSRSResourceSet-Aggregation-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PosResourceSetType  ::= CHOICE {

periodic			PosResourceSetTypePeriodic,

semi-persistent		PosResourceSetTypeSemi-persistent,

aperiodic			PosResourceSetTypeAperiodic,

choice-extension				ProtocolIE-Single-Container {{ PosResourceSetType-ExtIEs }}

}

PosResourceSetType-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

PosResourceSetTypePeriodic ::= SEQUENCE {

posperiodicSet			ENUMERATED{true, ...},

iE-Extensions		ProtocolExtensionContainer { { PosResourceSetTypePeriodic-ExtIEs} }	OPTIONAL,

...

}

PosResourceSetTypePeriodic-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PosResourceSetTypeSemi-persistent ::= SEQUENCE {

possemi-persistentSet	ENUMERATED{true, ...},

iE-Extensions		ProtocolExtensionContainer { { PosResourceSetTypeSemi-persistent-ExtIEs} }	OPTIONAL,

...

}

PosResourceSetTypeSemi-persistent-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PosResourceSetTypeAperiodic ::= SEQUENCE {

sRSResourceTrigger		 	INTEGER(1..3),

iE-Extensions				ProtocolExtensionContainer { { PosResourceSetTypeAperiodic-ExtIEs} }	OPTIONAL,

...

}

PosResourceSetTypeAperiodic-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

[...]

-- S

[...]

SRSConfig  ::= SEQUENCE {

sRSResource-List			SRSResource-List OPTIONAL,

posSRSResource-List			PosSRSResource-List OPTIONAL,

sRSResourceSet-List			SRSResourceSet-List OPTIONAL,

posSRSResourceSet-List		PosSRSResourceSet-List OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SRSConfig-ExtIEs } } OPTIONAL,

...

}

SRSConfig-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SRSCarrier-List ::= SEQUENCE (SIZE(1.. maxnoSRS-Carriers)) OF SRSCarrier-List-Item

SRSCarrier-List-Item ::= SEQUENCE {

pointA							INTEGER (0..3279165),

uplinkChannelBW-PerSCS-List		UplinkChannelBW-PerSCS-List,

activeULBWP						ActiveULBWP,

pCI-NR							INTEGER (0..1007)		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SRSCarrier-List-Item-ExtIEs } } OPTIONAL,

...

}

SRSCarrier-List-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SRSConfiguration ::= SEQUENCE {

sRSCarrier-List			SRSCarrier-List,

iE-Extensions			ProtocolExtensionContainer { { SRSConfiguration-ExtIEs } } OPTIONAL,

...

}

SRSConfiguration-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-PosSRSResourceSet-Aggregation-List	CRITICALITY ignore EXTENSION PosSRSResourceSet-Aggregation-List		PRESENCE optional },

...

}

[...]

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NRPPA-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NRPPA-CommonDataTypes;

[...]

-- **************************************************************

--

-- Lists

--

-- **************************************************************

maxNrOfErrors								INTEGER ::= 256

maxCellinRANnode							INTEGER ::= 3840

maxIndexesReport							INTEGER ::= 64

maxNoMeas									INTEGER ::= 64

maxCellReport								INTEGER ::= 9

maxCellReportNR								INTEGER ::= 9

maxnoOTDOAtypes								INTEGER ::= 63

maxServCell									INTEGER ::= 5

maxEUTRAMeas								INTEGER ::= 8

maxGERANMeas								INTEGER ::= 8

maxNRMeas									INTEGER ::= 8

maxUTRANMeas								INTEGER ::= 8

maxWLANchannels								INTEGER ::= 16

maxnoFreqHoppingBandsMinusOne				INTEGER ::= 7

maxNoPath									INTEGER ::= 2

maxNrOfPosSImessage							INTEGER ::= 32

maxnoAssistInfoFailureListItems				INTEGER ::= 32

maxNrOfSegments								INTEGER ::= 64

maxNrOfPosSIBs								INTEGER ::= 32

maxNoOfMeasTRPs								INTEGER ::= 64

maxnoTRPs									INTEGER ::= 65535

maxnoTRPInfoTypes							INTEGER ::= 64

maxnoofAngleInfo							INTEGER ::= 65535

maxnolcs-gcs-translation					INTEGER ::= 3

maxnoBcastCell								INTEGER ::= 16384

maxnoSRSTriggerStates						INTEGER ::= 3

maxnoSpatialRelations						INTEGER ::= 64

maxnoPosMeas								INTEGER ::= 16384

maxnoSRS-Carriers							INTEGER ::= 32

maxnoSCSs									INTEGER ::= 5

maxnoSRS-Resources							INTEGER ::= 64

maxnoSRS-PosResources						INTEGER ::= 64

maxnoSRS-ResourceSets						INTEGER ::= 16

maxnoSRS-ResourcePerSet						INTEGER ::= 16

maxnoSRS-PosResourceSets					INTEGER ::= 16

maxnoSRS-PosResourcePerSet					INTEGER ::= 16

maxPRS-ResourceSets							INTEGER ::= 2

maxPRS-ResourcesPerSet						INTEGER ::= 64

maxNoSSBs									INTEGER ::= 255

maxnoofPRSresourceSet						INTEGER ::= 8

maxnoofPRSresource							INTEGER ::= 64

maxnoofULAoAs								INTEGER ::= 8

maxNoPathExtended							INTEGER ::= 8

maxnoARPs									INTEGER ::=	16

maxnoUETEGs									INTEGER ::= 256

maxnoTRPTEGs								INTEGER ::= 8

maxFreqLayers								INTEGER ::= 4

maxNumResourcesPerAngle						INTEGER ::= 24

maxnoAzimuthAngles							INTEGER ::= 3600

maxnoElevationAngles						INTEGER ::= 1801

maxnoPRSTRPs								INTEGER ::= 256

maxnoVACell									INTEGER ::= 32

maxnoaggregatedPosSRS-Resources				INTEGER ::= 3

maxnoaggregatedPosSRS-ResourceSets			INTEGER ::= 96

maxnoAggPosPRSResourceSets					INTEGER ::= 3

maxnoofTimeWindowSRS						INTEGER ::= 16

maxnoofTimeWindowMeas						INTEGER ::= 16

maxnoPreconfiguredSRS						INTEGER ::= 16

maxnoofHopsMinusOne							INTEGER ::= 5

maxnoAggCombinations						INTEGER ::= 2

[...]