3GPP TSG-RAN WG3 Meeting #125	R3-244398

# Maastricht, The Netherlands, 19-23 August 2024

Agenda Item:	8.3

Source:	Ericsson

Title:	Considerations on RAN2 LS on emergency call support for UE in barred cells

Document for:	Discussion

# 1	Introduction

RAN3 received during RAN3#124 meeting a LS from RAN2 on emergency call support for (e)RedCap in barred cells [1], which states the following:

1. Overall Description: In RAN2#125bis RAN2 have endorsed the attached CRs for (e)RedCap UEs with the description of the CRs as below: If a (e)RedCap UE is barred in a cell where (e)RedCap is enabled since (e)RedCap UEs with 1Rx branch or 2Rx branches or both are barred in the cell, network may allow those (e)RedCap UEs to consider the cell as acceptable cell for emergency calls if cell selection criteria is fulfilled and, if the (e)RedCap UE supports only half duplex FDD operation, HD-FDD operation is allowed in the cell. The agreed RAN2 CRs also include the means for the cell to allow or not, the above operation via a SIB1 field. The CRs (R2-2402902, R2-2402903) when agreed, are considered to be implementable by Rel-17 UEs without any interoperability issue. 2. Actions: To RAN3 ACTION: RAN2 respectfully asks RAN3 to take the above information into account and update RAN3 specifications if needed.

The discussion was postponed based on RAN2 as was still continuing the revision of their CRs, which may impact RAN3’s encoding for the solution. No document was opened and cross-WG checking was encouraged to be done offline after the meeting.

In this document, we present our views about the RAN3 impacts based on the latest CRs agreed by RAN2, and which are copied below:

- R2-2405956 -  CR 380 Rev 5 38.304 CR

- R2-2405957 -  CR 1470 Rev 4 38.331 CR  linked with CR 380

- R2-2405958 -  CR 381 Rev 6 38.304 CR for eRedCap and 2Rx XR

# 2	Discussion

RAN2 have discussed as part of Rel-18 TEI, whether it is possible for a (e)RedCap UE to select/reselect in a cell that has status “barred”, or to be treated as if the cell status is “barred” for the purpose of emergency call. RAN2 has further generalized the case for any UE, for which it is advertised that bypassing the barring is possible based on an indication in the SIB1 message.

The new parameter barringExemptEmergencyCall was introduced in SIB1 as documented in R2-2405957 coped below:

SIB1 message -- ASN1START -- TAG-SIB1-START […] OPTIONAL, -- Need R eRedCap-ConfigCommon-r18 ERedCap-ConfigCommonSIB-r18 OPTIONAL, -- Need R cellBarredFixedVSAT-r18 ENUMERATED {barred, notBarred} OPTIONAL, -- Cond NTN cellBarredMobileVSAT-r18 ENUMERATED {barred, notBarred} OPTIONAL, -- Cond NTN reselectionMeasurementsNR-r18 ENUMERATED{true} OPTIONAL, -- Need R cellBarred2RxXR-r18 ENUMERATED {true} OPTIONAL, -- Need R intraFreqReselection2RxXR-r18 ENUMERATED {allowed, notAllowed} OPTIONAL, -- Need R barringExemptEmergencyCall-r18 ENUMERATED {true} OPTIONAL, -- Cond EM-Barring nonCriticalExtension SEQUENCE {} OPTIONAL }

[…]

SIB1 field descriptions

barringExemptEmergencyCall Indicates whether the cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [20].

Based on RAN2 CR, it can be understood that gNB indicates as part of the system broadcast information in SIB1 that it allows a UE (whether it is RedCap, eRedCap or 2Rx XR type UE) to bypass barring, if enabled in the serving cell, to make emergency calls.

Observation 1: Based on RAN2 agreements, a gNB indicates as part of the system broadcast information, in SIB1, that it allows a UE (whether it is RedCap, eRedCap or 2Rx XR type UE) to bypass barring, if enabled in the serving cell, to make emergency calls.

Regarding RAN3 signalling, for mobility purposes and in the case of a split gNB, exchange of information on whether barring is in place is indicated in the Served Cell Information NR IE, e.g., as part of the eRedCap Broadcast Information, eRedCap Broadcast Information and XR Broadcast Information IEs exchanged between gNBs and from gNB-DU to gNB-CU. To align with RAN2 agreements, F1AP and XnAP Served Cell Information NR information elements need then to be enhanced to indicate whether the UE’s exemption from the barring is set to perform emergency calls.

Proposal 1: Enhance the Served Cell Information NR IEs in XnAP and F1AP to indicate whether the UE can be exempted from the barring to perform IMS emergency call.

With respect to the encoding, it is preferable to encode with an ENUMERATED (true, …), that corresponds to information provided in the barringExemptEmergencyCall as defined in TS 38.331. CRs to Release 18 F1AP and XnAP are provided in [2-3].

Another aspect to consider, which was not discussed by RAN2, is the CN impact. In fact, for this UE it is actually registered as normal UE in the 5GC. But when it bypasses the barring of the cell to perform emergency PDU session call, the 5GC has no visibility that this UE is not allowed to establish a non-emergency service PDU Sessions(s). RAN3 should therefore discuss whether the CN needs to know whether the serving cell is allowing the UE, to bypass the barring to perform emergency calls only, in other words, that the UE is performing an emergency call while being served by a barred cell, so that non-emergency PDU sessions are not established.

Observation 2: The UE can be registered as normal UE in the 5GC. However when such UE bypasses the barring of the cell, the 5GC has no visibility that this UE is not allowed to establish a non-emergency service PDU Sessions(s).

Similarly, when such UE initiates a non-emergency call from a non-barred cell and is handed over to a barred cell, previously established non-emergency service PDU Sessions will be maintained, or new non-emergency service PDU Sessions can be established. This generates resource usage in 5GC which should be prohibited for such barred UE.

Observation 3: the barred UE may continue having non-emergency service PDU Session(s) established from previous node when it is being handed over from a non-barred cell to a barred cell.

Based on the above observations, it seems necessary to discussion about CN impacts, specifically:

- If AMF should have visibility that the accessing UE is not allowed to establish a non-emergency service PDU Sessions(s), and reject establishment of non-emergency service PDU Session(s)

- Let AMF know about the exemption, so that it removes any pre-established non-emergency PDU Session(s), i.e., established for this UE while it was being served by a non-barred cell.

Proposal 2: RAN3 to discuss the need to enhance NGAP to indicate to AMF whether the serving cell allows a UE to bypass the barring to perform emergency calls, so that AMF can decide to reject establishment of non-emergency service PDU Session(s) and also can decide to remove any pre-established non-emergency PDU Session(s), i.e., established for this UE while it was being served by a non-barred cell.

An example can be to impact the Initial UE Message, Path Switch Request, Handover Required and Handover Request Acknowledge messages to let AMF know of the exemption when the UE is accessing during initial establishment or after NG/Xn handover as described in CR [4]. A LS to RAN2 and SA2 is provided in [5].

# 3	Conclusion

Observation 1: Based on RAN2 agreements, a gNB indicates as part of the system broadcast information, in SIB1, that it allows a UE (whether it is RedCap, eRedCap or 2Rx XR type UE) to bypass barring, if enabled in the serving cell, to make emergency calls.

Proposal 1: Enhance the Served Cell Information NR IEs in XnAP and F1AP to indicate whether the UE can be exempted from the barring to perform IMS emergency call.

Observation 2: The UE can be registered as normal UE in the 5GC. However when such UE bypasses the barring of the cell, the 5GC has no visibility that this UE is not allowed to establish a non-emergency service PDU Sessions(s).

Observation 3: the barred UE may continue having non-emergency service PDU Session(s) established from previous node when it is being handed over from a non-barred cell to a barred cell.

Proposal 2: RAN3 to discuss the need to enhance NGAP to indicate to AMF whether the serving cell allows a UE to bypass the barring to perform emergency calls, so that AMF can decide to reject establishment of non-emergency service PDU Session(s) and also can decide to remove any pre-established non-emergency PDU Session(s), i.e., established for this UE while it was being served by a non-barred cell.

# 4	References

[1] R3-243012, LS on emergency call support for (e)RedCap in barred cells, RAN2

[2] R3-244399, Addition of UE barring exemption, XnAP CR, Ericsson

[4] R3-244400, Addition of UE barring exemption, F1AP CR, Ericsson

[6] R3-244401, Addition of UE barring exemption, NGAP CR, Ericsson

[8] R3-244402, LS to RAN2 and SA2 on UE barring exemption