3GPP TSG-RAN WG3 Meeting #125	R3-244409

Maastricht, The Netherlands, 19th – 23rd August 2024

Agenda Item:	18.2

Source:	Ericsson

Title:	Discussion of LPWUS impacts on NG-RAN architecture

Document for:	Discussions

# 1	Introduction

In Rel-19, a work item has been agreed to specify the wake-up signal for both RRC Idle/Inactive and RRC Connected states: [1], New WID: Low-power wake-up signal and receiver for NR (LP WUS/WUR). The objectives are the following:

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - This objective is to be further refined in RAN#103

In this contribution we discuss the potential impact to support the new LP-WUS feature in NG-RAN architecture, taking previous Rel-17 Paging Early Indication (PEI) as example.

# 2	Discussion on RAN3 impacts

## 2.1 PEI baseline

It was agreed by RAN2 that the PEI method is taken as baseline for LP-WUS:

RAN2 agreements The LP-WUS related configuration for IDLE/INACTIVE state is provided via system information. FFS if dedicated configuration is needed. Working assumption: the LP-WUS configuration in SIB at least includes the following information: - - LP-SS configuration - - LP-WUS configuration - - FFS on Entry/exit condition for LP-WUS monitoring The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups. RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE.

Observation 1 : RAN2 agreed that LP-WUS can be supported using same approach as PEI, using CN subgrouping (with NAS negotiation and CN signalling) or UE-ID subgrouping (configured within RAN).

Based on this agreement, the impacts for LP-WUS paging monitoring in NG-RAN architecture will follow similar impacts as those agreed in Rel-17 PEI work. The question is whether we can re-use the PEI-related IEs (e.g., PEIPS Assistance information IE present in RAN3 specifications) for the purpose of LP-WUS. Although there can be many common signaling aspects, LP-WUS is a Rel-19 new functionality that should not impact legacy. Furthermore it is still not determined by RAN2 the number of bits/codepoints in LP-WUS signal for subgrouping. During the discussion about the number of subgroups for PEI, the general understanding was that 8 subgroups is sufficient and is configured via OAM. It is assumed by RAN2 that 8 subgroups for CN-based LP-WUS subgrouping is enough. We assume that the knowledge will also be configured by OAM.

RAN2 agreements RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8.

Observation 2: the number of CN-subgroups for LP-WUS is FFS. We assume OAM configuration applies (same s PEI)

Below, we summarize the possible IEs that can be introduced to RAN3 specs for LP-WUS support:

- A CN-subgrouping LP-WUS assistance information IE is included in NGAP PAGING message for RRC_IDLE paging. FFS on number of subgroups;

- CN-subgrouping LP-WUS assistance information is included in NGAP Core Network Assistance Information for RRC_INACTIVE IE used for RAN Paging;

- LP-WUS assistance information IE is included in XnAP RAN PAGING message for paging an RRC_INACTIVE UE in the RNA;

- LP-WUS assistance information IE is included in F1AP PAGING message from gNB-CU to gNB-DU for paging in both IDLE and INACTIVE states.

Proposal 1: RAN3 to consider the following specification impacts for LP-WUS support:

- CN-subgrouping LP-WUS assistance information is included in NGAP PAGING message for RRC_IDLE paging. FFS on number of subgroups;

- CN-subgrouping LP-WUS assistance information is included in NGAP Core Network Assistance Information for RRC_INACTIVE IE used for RAN Paging;

- LP-WUS assistance information IE is included in XnAP RAN PAGING message for paging an RRC_INACTIVE UE in the RNA;

- LP-WUS assistance information IE is included in F1AP PAGING message from gNB-CU to gNB-DU for paging in both IDLE and INACTIVE states.

- FFS on number of CN subgroups.

A baseline CR to XnAP is provided in [2], which Ericsson candidates for being the rapporteur of.

The LP-WUS configuration parameters need to be configured in SIB, e.g.  time offset between LP-WUS and PO in SIB, the bits used for CN-assigned and UE-ID based subgroups, LP-WUS duration, etc. But these details depend on progress in RAN1. In RRC spec, the SIB1 contains the PEI-config. If the LP-WUS configuration info is limited it can be included in SIB1 otherwise new SIB to be defined. This needs further RAN2 monitoring how the LP-WUS/SS will be encoded.

There are differences between OFDM-based and OOK-based WUR in terms of coverage, accuracy, latency and need for LP-SS. Therefore, it should be possible for the gNB-DU to decide whether only OFDM-based or only OOK-based WUR or both types of WUR is used in the cell. If a new SIB is introduced for LP-WUS/SS, then further F1 impacts are needed. For now, RAN3 can assume that the gNB-DU decides the LP-WUS/SS configuration described in the RAN2 agreements:

RAN2 agreements RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions. The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - - LP-SS configuration - - LP-WUS configuration - - Entry/exit condition for LP-WUS monitoring (FFS if it is always configured)

Proposal 2: gNB-DU decides the LP-WUS/SS configuration. FFS if new SIB is introduced by RAN2. If yes, some F1 impacts are expected.

Another aspect is the paging limitation to one cell. Similar as with PEI the use of LP-WUS should be configurable in SIB and the gNBs supporting the LP-WUS monitoring to the last used cell function provide the UE's last used cell information to the AMF in the NG-AP UE Context Release Complete message for LP-WUS capable UEs. This will have stage 2 impacts.

Proposal 3: If it is indicated in SIB whether LP-WUS is only used in last used cell, RAN3 will follow same principle as PEI for paging monitoring to the last used cell function (i.e., the last serving gNB provides the last used cell information to AMF), this has stage2 impacts only.

## 2.2 Co-existence with long eDRX

RAN2 is discussing the co-existence impacts of eDRX and LP-WUS [3]. It is considered that when the UE requests to use LP-WUS with a long eDRX cycle, then LP-WUS should not be allowed with such long eDRX to avoid unnecessary LP-WUS transmissions during the UE long eDRX cycles. During the NAS negotiation signaling, the core network can omit the CN-assigned subgroup ID in the REGISTRATION ACCEPT message towards the UE when it includes the negotiated long eDRX cycle. However when UE is using UE-ID based subgrouping this option is missing.

Observation 3: Co-existence of LP-WUS with long eDRX may impact UE power saving gains due to frequent UE wake-ups during the long eDRX cycle. The CN can prevent the UE from using LP-WUS by not including the CN-assigned LP-WUS subgroups in the NAS REGISTRATION ACCEPT message. However for UE-ID based LP-WUS, this option is missing.

A solution to be discussed is that the UE indicates to CN during the NAS negotiation phase that it expects UE-ID LP-WUS. Afterwards, depending on the value of the negotiated eDRX, if e.g. longer than 20 seconds, the CN indicates to RAN during the INITIAL CONTEXT SETUP REQUEST message (for RRC_INACTIVE RAN Paging) and during the PAGING message (for RRC_IDLE paging) that UE-ID based subgrouping is not allowed. An illustration of signalling is presented below:

Example of signalling UE-ID allowed status to NG-RAN during long eDRX configuration for RRC-IDLE and RRC_INACTIVE

Some impacts pertain to CT1 and RAN2 feedback is needed. For now it is proposed to discuss this scenario in RAN3.

Proposal 4: RAN3 to discuss the scenario of CN indicating to NG-RAN whether UE-ID LPWUS should be allowed or not.

## 2.3 Emergency PDU session

When the UE has an emergency PDU session then eDRX is not used because eDRX may delay an (emergency) callback. Furthermore CN-assigned PEI is also not allowed when the UE has an emergency PDU session (see 24.501 section 5.3.25) for latency reasons. However UE-ID based PEI is allowed during an emergency PDU session, because the required signalling was overlooked initially, and the additional latency introduced by PEI was deemed tolerable.

For LP-WUS however, the average paging delay is increased due to the time offset between LP-WUS and PO. When a paging message is ready to be transmitted during the time offset, then the paging message is delayed with one DRX cycle. The time offset for LP-WUS is considerably larger (400-800 ms) compared to the time offset for PEI.

In our view CN-assigned and UE-ID based LP-WUS should be treated the same during an emergency session, because the additional latency is the same:

Proposal 5: CN-assigned and UE-based LP-WUS are treated the same when the UE has an emergency PDU session, i.e. either both are allowed or neither are allowed during an emergency PDU session.

Proposal 6: RAN3 to discuss whether the CN can indicate to RAN during Paging and ICSR that UE-ID based LP-WUS is not allowed (e.g. during an emergency PDU session).

# 3	Conclusion

Observation 1 : RAN2 agreed that LP-WUS can be supported using same approach as PEI, using CN subgrouping (with NAS negotiation and CN signalling) or UE-ID subgrouping (configured within RAN).

Observation 2: the number of CN-subgroups for LP-WUS is FFS. We assume OAM configuration applies (same s PEI)

Proposal 1: RAN3 to consider the following specification impacts for LP-WUS support:

- CN-subgrouping LP-WUS assistance information is included in NGAP PAGING message for RRC_IDLE paging. FFS on number of subgroups;

- CN-subgrouping LP-WUS assistance information is included in NGAP Core Network Assistance Information for RRC_INACTIVE IE used for RAN Paging;

- LP-WUS assistance information IE is included in XnAP RAN PAGING message for paging an RRC_INACTIVE UE in the RNA;

- LP-WUS assistance information IE is included in F1AP PAGING message from gNB-CU to gNB-DU for paging in both IDLE and INACTIVE states.

- FFS on number of CN subgroups.

Proposal 2: gNB-DU decides the LP-WUS/SS configuration. FFS if new SIB is introduced by RAN2. If yes, some F1 impacts are expected.

Proposal 3: If it is indicated in SIB whether LP-WUS is only used in last used cell, RAN3 will follow same principle as PEI for paging monitoring to the last used cell function (i.e., the last serving gNB provides the last used cell information to AMF), this has stage2 impacts only.

Observation 3: Co-existence of LP-WUS with long eDRX may impact UE power saving gains due to frequent UE wake-ups during the long eDRX cycle. The CN can prevent the UE from using LP-WUS by not including the CN-assigned LP-WUS subgroups in the NAS REGISTRATION ACCEPT message. However for UE-ID based LP-WUS, this option is missing.

Proposal 4: RAN3 to discuss the scenario of CN indicating to NG-RAN whether UE-ID LPWUS should be allowed or not.

Proposal 5: CN-assigned and UE-based LP-WUS are treated the same when the UE has an emergency PDU session, i.e. either both are allowed or neither are allowed during an emergency PDU session.

Proposal 6: RAN3 to discuss whether the CN can indicate to RAN during Paging and ICSR that UE-ID based LP-WUS is not allowed (e.g. during an emergency PDU session).

# 4	References

[1]: RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP WUS/WUR)

[2]; R3-244410: BL CR to XnAP: introduction of LP-WUS function, Ericsson

[3]: R1-2403277, LP-WUS operation in IDLE and INACTIVE modes, Ericsson, DISC, RAN1#116-bis