3GPP TSG-RAN WG3 Meeting #125	R3-244411

Maastricht, The Netherlands, 19th – 23rd August 2024

Agenda Item:	21.2

Source:	Ericsson

Title :	Discussion on SA2 LS questions to RAN3

Document for:	Discussions & Approval

# Introduction

In this contribution, we discuss the incoming LS from SA2 [1], where SA2 poses two questions to RAN3 asking for feedback on the CN providing dynamic in-band signaling of periodicity information over the GTP-U tunnel and the feedback on supporting “available data rate” exposure for GBR QoS Flow over user plane. The questions are repeated below highlighting the RAN3 queries.

-          Question1 [for SA4, RAN2 and RAN3]:

  - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach.

  - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension?

- Question2 [for SA4 and RAN2]: There is some discussion about time to next burst.

  - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6?

  - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server?

- Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

-          Question1 [for SA4, RAN2 and RAN3]:

  - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach.

  - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension?

- Question2 [for SA4 and RAN2]: There is some discussion about time to next burst.

  - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6?

  - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server?

- Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

In this contribution we discuss the RAN3 response to these questions.

# Discussion on Question 1

Dynamic updates of periodicity information in case of frequent traffic pattern change due to e.g. (frame)rate-adaptation may be useful to optimize resource scheduling in the NG-RAN, e.g. aligning grants with traffic arrival. It may also be useful to configure specific features such as optimal DRX pattern, alignment of SPS/CG allocations and PDCCH skipping. Such dynamic update of the periodicity value is beneficial to be signalled from the CN to the RAN via UP, so long as such updates are accurate and reliable and reflect the true behaviour of traffic received at the RAN. 
Otherwise, if the periodicity information is signalled via CP, it will require to run the PDU Session Management procedure many times, affecting many network interfaces (NG, F1, E1, Xn, Uu), which is too onerous in terms of signalling resources, especially for frequent changes of the periodicity value.  Whether and how dynamic changes of the traffic patterns can be supported in terms of radio resource configuration is an interesting (and important) question, but outside of RAN3 scope to discuss.

Observation1: Indicating dynamic changes of periodicity is beneficial so long as the information is accurate and reflects the true traffic trend received at the RAN. It seems to be better performed by means of in-band signaling as compared to control-plane signalling. How such dynamic changes can be supported in terms of optimization of resource scheduling in NG-RAN is a matter of implementation while feasibility in terms of respective configuration of radio resources is outside RAN3 scope.

To accommodate the awareness of periodicity changes in RAN by means of in-band signalling, there is need for adaptations to TS 38.415: add the periodicity information as new IE in the PDU Set Information user plane protocol in the GTP-extension header, as shown below:

### 6.5.2 Frame format for the PDU Set Information user plane protocol #### 6.5.2.1 DL PDU SET INFORMATION (PDU Type 0) This frame format is defined to allow the NG-RAN node or gNB-DU to receive PDU Set Information and indication of End of Data Burst of a QoS flow. The following shows the respective DL PDU SET INFORMATION frame. Bits; Number of Octets. 7; 6; 5; 4; 3; 2; 1; 0. PDU Type (=0); EDB; EPDU; PSSI; Spare; 1. QoS Flow Identifier (QFI); PSSN; 2. PSSN. Spare; PSI; 1. PSN; 1. Periodicity; 0 or 3. PSSize; 0 or 3. Padding; 0-3. Figure 6.5.2.1-1: DL PDU SET INFORMATION (PDU Type 0) Format ### 6.5.3 Coding of information elements in frames #### 6.5.3.1 PDU Type Description: The PDU Type indicates the structure of the PDU Set UP frame. The field takes the value of the PDU Type it identifies; i.e. "0" for PDU Type 0. The PDU type is in bit 4 to bit 7 in the first octet of the frame. Value range: {0= DL PDU SET INFORMATION, 1-15=reserved for future PDU type extensions}. Field length: 4 bits. Skipped text unchanged #### 6.5.3.2 Periodicity Description: This field indicates the Periodicity of the traffic flow as defined in TS 23.501. Periodicity expressed in units of 1 us. Value range: {0..232-1}. Field length: 4 octets.

Observation 2: Adding the Periodicity information as new IE in the PDU Set Information user plane protocol is straightforward.

Proposal 1 : RAN3 replies that dynamic in-band signaling indicating periodicity is useful for optimized resource scheduling, so long as the information is accurate and it reflects the true traffic trends received at the RAN. It is feasible for the information to be reported in the GTP-extension header as new IE in the PDU Set Information user plane protocol.

# Discussion on Question 3

On this question it should be noted that this topic has already been discussed in RAN3#124 meeting, based on the following LS [2] from SA2. In this LS SA2 queried the following:

Question3 [for RAN2 and RAN3]:SA2 would like to ask for to feedback on whether it is feasible for the NG-RAN to provide available data rate for the (non-)GBR QoS Flows.

Question3 [for RAN2 and RAN3]:SA2 would like to ask for to feedback on whether it is feasible for the NG-RAN to provide available data rate for the (non-)GBR QoS Flows.

And RAN3 provided the following reply in [3]:

RAN3 thinks that it is feasible to estimate available data rate for GBR QoS flows, which is up to RAN implementation. Regarding the report of available data rate, companies have different views on how to provide it to core network (e.g., via CP or UP) .There is no consensus on whether it is feasible for NG-RAN to estimate available data rate for non-GBR QoS flows.

Some companies in RAN3 would like to clarify the usage of available data rate for GBR QoS flows.

RAN3 thinks that it is feasible to estimate available data rate for GBR QoS flows, which is up to RAN implementation. Regarding the report of available data rate, companies have different views on how to provide it to core network (e.g., via CP or UP) .There is no consensus on whether it is feasible for NG-RAN to estimate available data rate for non-GBR QoS flows.

Some companies in RAN3 would like to clarify the usage of available data rate for GBR QoS flows.

Considering the past discussion, it was mentioned that RAN can only measure, by means of implementation, the ‘current data rate’ of a UE. It is not possible to derive future data rates, as this would imply to forecast aspects like future data channel quality, future user behaviour and more, for which the RAN is not prepared nor has the capability. Moreover, a future predicted ‘available data rate’ would likely be outdated by the time the information reaches the CN.

Observation 3: The concept of “available data rate” is inexistent in current 3GPP specifications

It was mentioned in last meeting that NG-RAN could know the available data rate based on estimation and learning algorithms, however this seems to be a shortcut to thinking, since several important questions need to be answered first in order to understand what this feature is about:

- What use case is covered by the notion of "available data rate"? SA2's question does not describe it.

- What is the timeline to which the – yet undefined - “available data rate” corresponds to? Does it correspond to a time frame or a specific epoch during the PDU session? Is such time indication up to the RAN to decide, or indicated initially by CN during the request/PDU Session Resource setup?

- If the estimation is for a fixed duration, then the data rate should rather be signalled via CP rather than via UP. Otherwise, we have Alternative QoS Profile function already.

- Another important question is for which UEs RAN should report the “available data rate”? those moving, or not moving?

- With which accuracy should this estimated data rate be provided?

- For how long should this estimation be (indicated) valid?

- In some RAN implementations, a fine balance is adopted for allocating the proper amount of radio resources for all users. “Availability” of additional resources as such - for a single user - is not possible without degrading performance of other users. Hence a scalability problem can easily arise.

- Etc.

All the above questions are for the most case prediction-related questions, and it will be hard for RAN3 to assess the feasibility and the benefit of such feature without understanding the full picture, which is lacking from the SA2 question. Besides, as far as AI/ML aspects are concerned, there is consensus in RAN3 that AI/ML predictions from the RAN are kept within  RAN side and not revealed to CN. Hence, RAN3 is still not in a position to discuss this topic as there is no clear definition of “available data rate” provided, not to speak of use case scenarios, applications etc.

Observation 4: Considering the little information provided, and without answering questions on the use cases and scenarios, RAN3 cannot assess the benefits nor usefulness of the feature of available data rate.

Furthermore, RAN3#125 is the last meeting in RAN3 for Rel-19 XR, and the questions above need to be answered first to understand the concept of “available data rate”, which is inexistent today. Since there is no time, it is proposed to reply to SA2 that the concept of available data rate is inexistent today in RAN specifications and that RAN3 cannot in the current state of the LS from SA2 assess the feasibility nor benefits of the proposed feature without having further discussion and questions answered regarding the use case and scenario. Specifically, since there are questions related to the use case and applicability of AI/ML algorithms, it should better be treated as new SI, if needed.

Proposal 2: RAN3 replies to SA2 that the concept of available data rate is inexistent in current 3GPP specifications and that RAN3 can neither assess the feasibility nor benefits of the proposed feature without having been provided with answers to fundamental questions and without subsequent discussions especially on the use cases and scenarios envisaged. Specifically, since there are questions related to the use case and applicability of AI/ML  algorithms, it should better be treated as new SI, if needed.

# Conclusion

In this contribution we discuss the questions to RAN3 from the SA2 LS [1].

Observations and proposals are listed below:

Observation1: Indicating dynamic changes of periodicity is beneficial so long as the information is accurate and reflects the true traffic trend received at the RAN. It seems to be better performed by means of in-band signaling as compared to control-plane signalling. How such dynamic changes can be supported in terms of optimization of resource scheduling in NG-RAN is a matter of implementation while feasibility in terms of respective configuration of radio resources is outside RAN3 scope.

Observation 2: Adding the Periodicity information as new IE in the PDU Set Information user plane protocol is straightforward.

Proposal 1 : RAN3 replies that dynamic in-band signaling indicating periodicity is useful for optimized resource scheduling, so long as the information is accurate and it reflects the true traffic trends received at the RAN. It is feasible for the information to be reported in the GTP-extension header as new IE in the PDU Set Information user plane protocol.

Observation 3: The concept of “available data rate” is inexistent in current 3GPP specifications

Observation 4: Considering the little information provided, and without answering questions on the use cases and scenarios, RAN3 cannot assess the benefits nor usefulness of the feature of available data rate.

Proposal 2: RAN3 replies to SA2 that the concept of available data rate is inexistent in current 3GPP specifications and that RAN3 can neither assess the feasibility nor benefits of the proposed feature without having been provided with answers to fundamental questions and without subsequent discussions especially on the use cases and scenarios envisaged. Specifically, since there are questions related to the use case and applicability of AI/ML  algorithms, it should better be treated as new SI, if needed.

# References

[1] S2-2407351, LS on FS_XRM Ph2, SA2

[2] S2-2405625 Response LS on LS on FS_XRM Ph2

[3] R3-243958 Response LS on FS_XRM Ph2

# Annex – LS Reply to SA2

3GPP TSG-RAN WG3 Meeting #125	R3-24xxxx

Maastricht, The Netherlands, 19th – 23rd August 2024

Title:	Response LS on LS on FS_XRM Ph2

Response to:	LS on FS_XRM Ph2

Release:	Rel-19

Work Item:	FS_XRM Ph2

Source:	RAN3

To:	RAN2, SA2

Cc:	SA4

Contact Person:

#### Name:	Yazid Lyazidi

Tel. Number:

E-mail Address:	yazid.lyazidi@ericsson.com

Attachments:

1. Overall Description:

RAN3 thanks SA2 for their questions and would like to provide the following answers:

- Question1 [for SA4, RAN2 and RAN3]: SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach.

- RAN3 answer:

  - RAN3 considers that indicating dynamic changes of periodicity seems to be better performed by means of in-band signaling as compared to control plane signalling, so long as the information is accurate and it reflects the true traffic trends received at the RAN. How such dynamic change be supported in terms of optimization or resource scheduling in NG-RAN is a matter of implementation while feasibility in terms of respective configuration of radio resources is outside RAN3 scope

- Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

- RAN3 answer:

  - The concept of “available data rate” is inexistent in current 3GPP specifications and RAN3 can neither assess the feasibility nor benefits of the proposed feature without having been provided with answers to fundamental questions and without subsequent discussions especially on the use cases and scenarios envisaged. Specifically, since there are questions related to the use case and applicability of AI/ML  algorithms, it should better be treated as new SI, if needed. Considering the little information provided, RAN3 is not able to provide any feedback.

2. Actions:

To SA2 group:

ACTION: 	RAN3 kindly asks SA2 to take the above RAN3 replies into account.

3. Date of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3#125bis	                    14 – 18 October			Hefei, PRC

TSG-RAN WG3#126	                    18 – 22 October			Orlando, U.S.