3GPP TSG-RAN WG3 Meeting #125	R3-244413

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	9.2

Source:	Nokia, Deutsche Telekom, ZTE

Title:	Issue with AoI reporting for Slice subject to Area Restriction

WID/SID:	TEI18

Document for:	Discussion and Decision

# 1	Introduction

In TS 23.501, SA2 has designed a new mechanism in release 18 related to enforcing Network Slice Area of Service not matching deployed tracking areas. In this mechanism, the SMF subscribes to “UE mobility event notification” to be informed by AMF when the UE is “IN” or “OUT” of the NS-AoS and the AMF configures for this AOI reporting towards the gNB where the AoI corresponds to the NS-AoS of the slice.  This is described in TS 23.501 section 5.15.18.3 as follows:

5.	When the AMF determines that the S-NSSAI of a PDU Session is restricted to an NS-AoS in the PDU session, the AMF indicates to the SMF that the PDU Session is subject to area restriction for the S-NSSAI. As a result, the SMF subscribes to "UE mobility event notification" event for reporting UE presence in Area of Interest by providing S-NSSAI to the AMF as described in clauses 5.6.11 and 5.3.4.4 if this event has not been subscribed before. The AMF determines the UE presence in Area of Interest as described in Annex D, clauses D.1 and D.2 of TS 23.502 [3] and notifies the result to the SMF. When SMF is notified that the UE location is outside of Area of Interest, SMF shall not send user data as payload of NAS message (see clause 5.31.4.1) in downlink directions and disable data notification.

Based on the information, when incoming packet arrives, the SMF refrains from triggering Data Notification, Network-triggered PDU session setup and Paging towards the UE for the slice.

However, this mechanism has an issue, especially in the typical case where the UE has several PDU sessions which can be of different S-NSSAIs. This paper describes this issue and investigates possible solutions.

# 2	Discussion

Suppose that a UE is operating on 3 slices with respective IDs 1, 2, 3. Each of those slices is associated with a respective area of interest AoI1, AoI2 and AoI3 which correspond to the slice Area of Service (AoS). For example, the AoI1 associated with slice 1 is defined as matching the slice AoS of slice 1. In this example, each of the AoIs is associated with the below cell list:

Slice 1: AoI 1 = {cell 1, cell 2, cell 3}

Slice 2: AoI 2 = {cell 2, cell 3, cell 4}

Slice 3: AoI 3 = {cell 3, cell 4, cell 5, cell 6}

The cells are deployed in an area as shown in the figure below, where each color represents the cells controlled by an NG-RAN node (gNB). For instance, green color (gNB1), orange color (gNB2) and pink color (gNB3). All the cells shown are within the registration area (RA) of the UE.

UE

UE

Figure 1: RA of the UE and cell distribution in TAs of the RA and UE positioning in a cell within the RA.

While the UE moves in the RA, UE is currently in cell 3 that is controlled by gNB1. gNB1 decides to bring the UE to RRC_INACTIVE state. In that case gNB1 has to build the RNA for the UE. Based on gNB1’s decision there are several ways for RNA construction as listed below:

Options: a) RNA = {cell 1, cell 2, cell 3}, b) RNA = {cell 1, cell 2, cell 3, cell 4, cell 5, cell 6}, c) RNA = {cell 3}

If NG-RAN AOI-based location reporting is enabled by SMF (via AMF) then based on the chosen RNA from the gNB we would have the following possible reporting options from NG-RAN reports towards SMF:

- Case 1: big-size RNA: If gNB1 builds RNA = {cell 1, cell 2, cell 3, cell 4, cell 5, cell 6}

  - AoI reporting to SMF looks as: AoI 1 -> UNKNOWN, AoI 2 -> UNKNOWN, AoI 3 -> UNKNOWN

  - SMF has no idea if UE is in the Area of Service of slice 1, slice 2 or slice 3. Therefore, SMF will enable data notification and reactivate PDU sessions all the time i.e. for any incoming packet of any slice.

  - This also leads to UE has to be paged in any of the cells of RNA with most of the time a wasted paging.

  - For example, paging is wasted for incoming packet of slice 1 if UE is in cells 4,5 or 6.

  - Paging is wasted for incoming packet of slice 2 if UE is in cells 1,5 or 6 and for an incoming packet of slice 3 if UE is in cells 1 or 2.

  - In a nutshell, the saving of avoiding data notification/network-triggered PDU session setup/Paging is broken most of the time.

- Case 2: middle-size RNA: If gNB1 builds RNA = {cell 1, cell 2, cell 3}

  - AoI reporting to SMF looks as: AoI 1 -> IN, AoI 2 -> UNKNOWN, AoI 3 -> UNKNOWN

  - SMF knows where the UE is regarding AoI1 related to NS-AoS of slice 1 (i.e. UE is “IN”), so it can enable data notification and reactivate PDU session for an incoming packet of slice 1 with successful paging and reconnection.

  - But if instead an incoming packet comes for slice 2 or slice 3 then the data notification/paging is wasted if e.g. UE is located in cell 1 for an incoming packet of slice 2, or e.g. if UE located in cells 1,2 for an incoming packet of slice 3.

  - Here the saving of avoiding data notification/network-triggered PDU session setup/Paging works partially but not fully.

- Case 3: small size RNA: If gNB1 builds RNA = {cell 3}

  - AoI reporting to SMF looks as:

  - AoI 1 -> IN, AoI 2 -> IN, AoI 3 -> IN

  - SMF enables trustworthy data notifications. Network-triggered PDU session setup and paging requests are not wasted but successful because the SMF knows where the UE is, and successful connection.

  - However, the drawback with such small RNA is high number of RNA updates as RNA contains only one cell.

The above examples show that the building of the RNA by a gNB will strongly influence how SA2 desired feature of avoiding data notification/paging when UE in outside slice AoS using AOI reproting effectively works.

The building of the RNA is currently fully under the responsability of the gNB but it can receive Core Network Assistance Information to help finding the best trade-off in size of the RNA.

In the above examples, if the gNB is not made aware by the AMF, when configured by AMF for AOI reporting, that this AOI reporting is for the purpose of slice AoS monitoring to prevent useless data notifications/paging then the gNB will not take this into account in the RNA building e.g. could typically decide big RNA to avoid frequent RNA update but then always report UE location as “unknown” which fails SA2 feature.

When AOI reporting is configured for slice AoS monitoring then gNB trade-off is:

1)  high number of RNA updates if RNA is restricted to a low number of cells.

2)  high number of useless data notifications/paging requests if RNA contains a large number of cells.

In order to have gNB make the right decision on RNA size, we think that gNB should be made aware when the AOI reporting is intended for the purpose of slice AoS monitoring.

RAN3 is invited to discuss the above issue. SA2 should be involved as well.

Proposal: RAN3 is invited to discuss the problem above.

# 3	Conclusion

The paper has shown why it is important when AMF configures the AOI reporting to gNB to tell the gNB when it is due to this slice AoS monitoring. This will help the gNB to make the best trade-off for the RNA size to avoid useless data notifications/network-triggered PDU session setup/Paging requests.

It is proposed to send an LS to SA2 to report this issue and seek feedback.

Proposal: liaise SA2 to report this issue and seek feedback (see annex A).

# 4	Annex A

3GPP TSG-RAN WG3#125			R3-24xxxx

Maastricht, 19th – 23rd August 2024

Title:	LS on Network Slice subject to area restriction

Response to:

Release:	Rel-18

Work Item:	TEI18

Source:	RAN3

To:	SA2

Cc:

Contact Person:

#### Name:	Philippe Godin

Tel. Number:	+33 1 6040 0304

E-mail Address:	philippe.godin@nokia.com

Attachments:	R3-24xxxx

1. Overall Description:

While working on the SA2 feature of “Network Slice subject to area restriction” as per section 5.15.18.3 of TS 23.501 RAN3 noticed the intended use of AOI (Area Of Interest) reporting in CM-connected state to have SMF notified (via AMF) whenever the UE is in or out of the slice AoS (Area of Service) and enable/disable data notification accordingly:

When SMF is notified that the UE location is outside of Area of Interest, SMF shall not send user data as payload of NAS message (see clause 5.31.4.1) in downlink directions and disable data notification.

However, RAN3 spotted a possible issue that gNB may decide at any point in time to send the UE to RRC_INACTIVE and the efficiency of the above AOI reporting would then be depend on how gNB configures the RNA (RAN Notification Area) for the UE. Especially if the gNB does not take into account when defining the RNA that the AOI reporting has been configured by AMF in the gNB for the above feature, the gNB will most of the time report “unknown” location in AOI which undermines totally the feature. The attached paper illustrates this in a few examples.

It seems beneficial to have AMF making gNB aware when AOI reporting is configured due to the above slice AoS monitoring purpose to assist gNB for RRC_INACTIVE RNA configuration and RAN3 would like to seek guidance from SA2 on this.

2. Actions:

To RAN2 group:

ACTION: 	RAN3 kindly ask SA2 to provide guidance on the AOI reporting issue presented above and possible assistance information from AMF to gNB that SA2 could propose.

3. Date of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3#125bis	                    14 – 18 October			Heifei, China

TSG-RAN WG3#126	                    18 – 22 November			Orlando, USA