3GPP TSG-RAN WG3 Meeting #125	R3-244414

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	12.3

Source:	Nokia, TMO US, AT&T, Verizon Wireless, BT, NTT Docomo, KDDI, Charter

Title:	[TP for TR 38.799] Conclusion on NR Femto Architecture Options

WID/SID:	FS_WAB_5GFemto_NR - Release 19

Document for:	Discussion and Decision

# 1	Introduction

At the last RANP#102 meeting, the study item of 5G femto was agreed in [1] including the following objective:

-	Study the overall RAN architecture and required functional and procedural impacts for supporting 5G Femto deployments [RAN3].

At RAN3#123bis meeting, a few architecture options were captured in the TR 38.799. At RAN3#124 an evaluation of the different architecture options was done.

RAN3#125 is the last meeting for the NR Femto Study Item. This paper proposes to draw conclusions based on the evaluation performed at last RAN3 meeting.

# 2	Discussion

The following evaluation has been captured at RAN3#124:

Option1: direct connection of NR Femto to 5GC

Pros:

- Already supported by current architecture.

- Less CP latency and no processing delay due to absence of a concentration stage.

- Suitable for certain deployments depending on number of NR Femtos to connect and/or virtualization support of the 5GC.

- Local breakout can be supported.

Cons:

- Not suitable for certain deployments with large number of NR Femtos and/or 5GC not virtualized.

- Not suitable for residential deployments with frequent switch on/off of NR Femtos.

Option 2: NR Femto GW

Pros:

- Only one SCTP association from 5GC to NR Femto GW,so it can support large number of femtos and/or no virtualization of 5GC.

- 5GC is shielded from frequent switch on/off of the NR Femtos.

- Enables operators who have already deployed 4G Femtos using HeNB GW to capitalize on operating model and integration process of 5G Femtos.

- Foreseen specification impacts are already well known from 4G.

- Allows to decouple concentration of CP and concentration of UP: concentration of UP is optional i.e. the NR Femto GW can concentrate CP only while the NR Femto connects directly to the UPF.

- Local breakout can be supported.

Cons:

- Some stage3 specification impact.

- Some processing delay for CP message.

Option 3: SCTP concentrator

Pros:

- Only one SCTP association from 5GC to SCTP concentrator due to using multi-streaming.

- Local breakout can be supported.

- Only stage2 specification impact.

Cons:

- The solution requires consistent configuration and handling of SCTP stream identifiers.

- The solution requires consistent SCTP implementation of AMF, SCTP concentrators and NR femtos.

- Some processing delay for CP message.

- Does not provide an evolution path for operators that have already deployed a HeNB GW for E-UTRAN.

Option 4: NR Femto as a gNB-DU

Pros:

- Reuse existing split gNB architecture.

Cons:

- F1-C was not designed to face frequent switch on/off. Usually F1-C is operated by the network operator and statically configured.

- Foreseen additional Interoperability issue of F1 compared to NG.

- F1-C carried over internet backhaul can lead to latency and reliability issue not meeting the stringent requirement for F1-C interface.

- This option forces the concentration of User Plane and not only control plane i.e. concentration of CP only while NR femto UP connects directly to UPF is not possible.

- Does not provide an evolution path for operators that have already deployed a HeNB GW for E-UTRAN.

- Specification impact for F1 needs to be further assessed.

- Local breakout requires collocation of NR Femto with CU UP.

In this evaluation we could group the options into two categories:

- Connection over NG interface without a concentration stage: option 1

- Connection to 5GC with concentration stage: option 2 (concentration done by HgNB GW, option 3 (concentration done by SCTP concentrator), option 4 (concentration done by the CU CP).

The concentration stage may be needed not only for scalability but also to hide frequent switch on/off in some deployment scenarios. In some other deployments it may not be needed. To be on the safe side, we propose that the 3GPP standards specifies one option without concentration and one option with concentration i.e. it is not needed to standardize multiple options with NG concentration. This will allow any operator can freely and flexibly select what it needs in the 3GPP toolbox.

Proposal 1: 3GPP to specify in standards one option without NG concentration and one option with optional NG concentration.

Assuming proposal 1 is agreed, we note that there is only one option without concentration which is option 1.

Then one option should be selected among the NG concentration options 2,3,4.

We note that option 4 has the most numerous drawbacks difficult to overcome.

Between remaining options 2 and 3, option 2 presents the big advantage of being a better fit with the operating model of operators who have already deployed 4G femtos using an HeNB GW. Also, the option 2 does not require the tedious configuration effort of SCTP stream identifiers and it does not require consistent SCTP implementations in AMF, NR Femto and the provider of the SCTP concentrator. Option 2 is therefore more trusted and has less impact on implementations and standardization effort.

Proposal 2: capture in the conclusion of the TR 38.799 that options 1 and 2 will proceed in the work item phase.

# 3	Conclusion and Proposals

This paper has evaluated the four options for the architecture which were captured in TR 38.799 and proposed text for both the evaluation part and conclusion part along following proposals:

Proposal 1: capture the above evaluation in the TR 38.799.

Proposal 2: capture in the conclusion of the TR 38.799 that options 1 and 2 will proceed in the work item phase.

The corresponding TP for TR 38.799 is available in Annex A.

# 4	References

- RP-234041, Study Item on Additional topological enhancements for NR, NTT Docomo, AT&T

# 5	Annex A –

## 5.2.Y	Conclusion for Architecture for the NG interface

To address different deployment scenarios, 3GPP should specify one option without NG concentration and one option with NG concentration. There is no need to specify multiple NG concentration options.

Among the options with NG concentration, option 4 would impose the most significant standardization effort, and presents numerous architectural challenges. Between options 2 and 3, option 2 is preferred as it has the significant advantage of being a better fit with the operating model of operators who have already deployed 4G femtos using an HeNB GW. Also, it does not require the tedious configuration effort of SCTP stream identifiers and it does not require consistent SCTP implementations across the AMF, the NR Femto and the provider of the SCTP concentrator. Option 2 is therefore more trusted and has less impact on implementations and standardization effort.

It is recommended to continue the work with options 1 and 2 in the work item phase.