3GPP TSG-RAN WG3 Meeting #125	R3-244415

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	12.3

Source:	Nokia

Title:	Answer to SA2 on UE mobility between CAG cell and CSG cell

WID/SID:	FS_WAB_5GFemto_NR - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The study item of 5G femto was agreed in [1] at RANP#102 and started at RAN3#123bis. In parallel, SA2 works on its own study item in [3] including different topics. One of the objectives of [3] is the mobility between CAG cell and CSG cell. In this context RAN3 has received the following LS from SA2:

As part of the FS_5G_Femto study (SP-231797), the following aspects are being studied:

- WT#1: Based on RAN3 outcome, enhance the overall architecture and enable the required functional and procedural changes for supporting 5G NR Femto deployment.

- WT#2: How to enable interworking between CAG and CSG cells.

In TR 23.700-45, the WT#2 (mapped to KI#1 in the TR) investigates the support of the UE moving between CAG cell of 5G Femto and CSG cell, with no impact on the RAN. Therefore, some companies proposed below solutions for this KI:

- The UE partitions CSG-CAG ID and constructs mapped CSG/CAG ID, and reports to the NG-RAN or E-UTRAN (depending on the considered mobility direction) as described in pCR (S2-2405814).

- RAN recognizes the target CSG cell (or the target CAG cell) as an open cell during the handover (e.g., via local configuration) and the core network performs access control as described in pCR (S2-2405789).

Question 1: SA2 would like to know whether the two solutions mentioned above have any impact on the RAN (e.g., for RAN procedures)?

Question 2:  SA2 has reserved the time units for the normative work of WT#1 based on the result of RAN3 work (RP-234041), which is expected to start in SA2 from SA2#164. Therefore, SA2 requests to confirm the conclusion of RAN3 on overall architecture, etc., which will be used as the basis for SA2's normative work.

2. Actions: SA2 asks RAN2 and RAN3 to answer the above questions.

This paper addresses this mobility question and proposes answers to SA2 LS in [4].

# 2	Discussion

SA2 ask feedback on two particular solutions for their WT#2. It is also necessary to feedback on both direction: 5g to CSG and 4g to CAG.

## 2.1	5g to 4g CSG Mobility

Solution 1:

The first SA2 solution is described in pCR S2-2405814:

There is a partition of CSG-CAG ID in the network i.e. some CAG ID are reserved to be mapped onto CSG IDs.

The UE reads the target CSG ID. It adds five leftmost bits set to “zero” to derive the corresponding mapped CAG ID. The UE sends measurement report including this CAG ID to source gNB. Currently only target 4g CGI can be reported, this requires the CAG ID to be added by RAN2 in the 4g measurement report to source gNB.

Then the source gNB has received the Allowed CAG List of the UE from AMF (ongoing in RAN3 for intra-5G mobility to NR Femto). The source gNB can do the access control check with the received CAG ID, similar to mobility to target NR Femto or PNI NPN (working assumption in RAN3). Handover will only be triggered if the received CAG ID matches the list.

Observation 1-1: in solution 1, the source gNB behaves as for intra-5G mobility to target NR Femto. The impact on source gNB is therefore minimum: only add the CAG ID in the measurement report of target CSG cell from the UE.

NOTE: alternatively, the UE could report the target CSG ID and the source gNB could map into the CAG ID.

On the target side, the CSG ID must be delivered to the target MME and target HeNB. The AMF can include the target CSG ID based on mapping from target HeNB ID (same in the solutions 1 and 2). No impact foreseen on target side, only on source AMF.

Observation 1-2: in solution 1, AMF can map the target HeNB ID into target CSG ID. No impact on target 4g RAN. Only impact on source AMF.

Solution 2:

The second SA2 solution is described in pCR S2-2405789:

The UE reports the target 4g CGI as an open cell as of today in a nominal 5g-4g handover preparation. The source gNB triggers the N2 Handover Required message including the target HeNB ID. The AMF is configured with the mapping of (target HeNB ID, target CSG ID, mapped CAG ID list).

NOTE 1: it is unclear here whether the mapped CAG ID List is dynamically configured (O&M burden) or result from a partition like in solution 1.

NOTE 2: it is also unclear whether single or multiple CSG IDs will be mapped to a single or multiple CAG IDs.

AMF performs the access control presumably comparing the UE Allowed CAG list with the mapped CAG ID List of AMF.

NOTE 3: This function is unusual for the AMF since the access control for PNI NPN is done in source RAN node (and same is assumed for target NR Femto). The AMF is impacted not only for heavy configuration, but also with this new additional function.

If the access control is positive, the AMF includes target CSG ID towards target MME. No impact to target side but impact on AMF to send this CSG ID to target MME. If the access control is unsuccessful, the AMF needs to reply with HANDOVER PREPARATION FAILURE message.

Assuming solution 2 pretends no impact on UE and source RAN node, the Handover preparation is therefore triggered whenever UE measures a closed CSG cell which it reports as an open cell. Assuming many CSG cells can overlap the source RAN (macro or femto) cell, for which only one may be allowed for the UE, the solution 2 will translate into many repeated handover preparation failures.

Observation 2: The solution 2 leads to frequent repeated handover preparation failures impacting the source gNB in terms of signaling processing and degraded handover KPI.

NOTE 4: For too many releases RAN WGs have been working on improving the HO performance, meaning reducing the handover failures. In contrast this solution increases the handover failure possibilities by blindly considering any closed CSG cell as open CSG cell.

The solution additionally impacts the source AMF more than solution 1 because in addition to sending the target CSG ID to target MME, the source AMF needs an additional new function of access control against CAG ID List which actually duplicates the function foreseen by RAN3 to take place in the source RAN for intra-5GS handover to target NR Femto (as per PNI NPN agreement).

## 2.2	4g to 5g CAG Mobility

Solution 1:

The first SA2 solution is described in pCR S2-2405814:

There is a partition of CSG-CAG ID in the network i.e. some CAG IDs are reserved to be mapped onto CSG IDs.

The UE measures the target CAG ID. It removes five leftmost bits set to “zero” to derive the corresponding mapped CSG ID. It is assumed that the UE also received the allowed CSG ID List from MME, as per legacy CSG mobility. Based on this the UE can evaluate the membership, as per legacy CSG mobility.

The UE reports the CSG ID and the membership to source eNB, same as legacy CSG mobility as shown below from TS 36.331:

AdditionalSI-Info-r9 ::= SEQUENCE {

csg-MemberStatus-r9 ENUMERATED {member} OPTIONAL,

csg-Identity-r9 CSG-Identity OPTIONAL

}

If the UE reports to be a member, the source eNB includes the CSG ID to source MME as per legacy CSG mobility. The source MME performs access control as per legacy CSG mobility and proceeds as per current 4g-5g mobility.

The impact to source eNB is only to add the CSG ID and Membership Status in the NR part of the 4g measurement report. After this the access control logic and algorithm is same as the one used towards a 4g target CSG cell.

At target side, PNI NPN Model is followed to avoid any impact i.e. the AMF sends the UE Allowed CAG List to target NR Femto and the target NR Femto checks access comparing with CAG ID of target cell. No impact to target RAN.

Observation 3: In solution 1 the source eNB has minimal impact compared to legacy CSG mobility because it just need to add the CSG ID and membership status to the NR part of 4g measurement report while eNB logic remains the same as mobility towards 4g CSG cell, source MME has no impact at all, and the target RAN has no impact compared to intra-5G CAG mobility (PNI NPN and RAN3 foreseen solution for target NR Femto).

Solution 2:

The second SA2 solution is described in pCR S2-2405789:

In solution 2, the UE should report the target CAG cell as an open cell.

It is unclear of UE reports the PCI or the target CGI. If target CGI is to be reported then UE is impacted because the 5g CGI is included in the NPN-List. If the solution is based on PCI reporting, then there can be PCI confusion.

NOTE: in PNI NPN the cell has a PLMN List (may not be valid PLMN ID) and an NPN List (containing the CGI).

The source eNB performs a 4g to 5g handover followed by MRU on target side. However, during the handover, on target side the target AMF will include the UE Allowed CAG List to target NR Femto.

Either:

- the target NR Femto will do the access control and will reject the handover sending back HO prep failure,

- Or the target NR Femto will not run access control and access control will be run in the MRU following the HO and will fail at this point in time which is even worse.

Similar as for 5g to 4g direction, assuming solution 2 pretends no impact on UE and source RAN node, the Handover preparation is therefore triggered whenever UE measures a closed CAG cell which it reports as an open cell. Assuming many CAG cells can overlap the source RAN (macro or femto) cell, for which only one may be allowed for the UE, the solution 2 will translate into many repeated handover preparation failures.  In the 4g to 5g direction, solution 2 is even more impacting because the complete handover signaling takes place down to target RAN node before being failed.

Observation 4: The solution 2 has no access control at all on source side: this leads to frequent repeated handover preparation failures impacting both the source eNB and target NR Femto node in terms of signaling processing and degraded handover KPI.

## 2.3	Evaluation Summary table

#### 5g to 4g direction

|  | Source gNB | Source AMF | Target MME | Target HeNB |
| --- | --- | --- | --- | --- |
| Solution 1 | CAG ID added into 4g part of 5g measurement report | Sends CSG ID to target MME | no | no |
| Solution 2 | Frequent Repeated handover failures | 1/ sends CSG ID to target MME 2/New access control in AMF duplicating access control in source gNB 3/ Frequent repeated handover failures 4/ mapping from target HeNB into list of CAG IDs | no | no |

#### 4g to 5g direction

|  | Source eNB | Source MME | Target AMF | Target NR Femto |
| --- | --- | --- | --- | --- |
| Solution 1 | CSG ID and membership status added into 5g part of 4g measurement report | no | no | no |
| Solution 2 | Frequent Repeated handover failures | Frequent Repeated handover failures | Frequent Repeated handover failures | Frequent Repeated handover failures |

## 2.4	New solution 2bis proposed at RAN3#124 to avoid handover failures using O&M in source RAN

Due to the frequent handover failures of solution 2 as explained above, which is a serious showstopper for solution 2, the proponent of solution 2 proposed at last RAN3#124 to modify solution 2 into solution 2bis. This is what can be seen from tentative LS back presented in tdoc R3-243947:

Solution 2 (S2-2405789) does not require to modify currently specified RAN or UE behavior. Any potential mobility issue (e.g. handover failure due to incorrect target selection) is expected to be addressed via local OAM configuration (e.g. cell ID-CSG ID-CAG ID mapping).

We can elaborate further on why solution 2bis does not work better than solution 2.

In solution 2bis, in order to avoid handover failures, each CAG (resp. CSG) cell is configured with so-called “friendly neighbor CSG (resp. CAG”). The assumption is that source gNB serving a UE in cell 1 of CAG x (resp. CSG x) will only select and trigger handovers towards neighbor cells of cell 1 which have a CSG y (resp. CAG y) which is a friend.

Friendship is assumed configured by O&M i.e. static. However, mobility decisions are not static but are per UE dependent. An example is given below representing different access rights per UE. This can correspond to staff within a campus or within an hospital where access is restricted in certain areas for certain types of personal staff.

Taking the example of a campus drawn below:

- UE1 is any student which has only access to certain open areas reserved for classes, some equipped with 4G femtos CSG 11 and some equipped with femtos CAG 10.

- UE2 is any teacher which has access to student classes + access to some teacher room areas equipped with 4G femtos of CSG 20.

- UE3 is any member of executive administrative staff leading the campus which has also access to student classes + some administrative areas equipped with 4g femtos of CSG 30.

- UE4 corresponds to some specific UE3 which has additionally access to the office of the director of the campus which is equipped with a 5g femto of CAG 40.

- UE5 is the director of campus and can access all areas.

It is obvious that solution 2bis cannot work in the above typical example. This is because:

- CSG 20 is a friend of CAG 10 only for UE2s and UE5 but UE1s, UE3s, UE4s should be restricted access.

- CSG 30 is a friend of CAG 10 only for UE3, UE4, UE5 but UE1s and UE2s should be restricted access.

- CAG 40 is a friend of CSG30 and CSG 11 only for UE4s and UE5 but UE1s, UE2s, UE3s should be restricted access.

Even in the above simple example with limited number of CSGs and CAGs, it is obvious that a mobility decision between a CAG cell and CSG cell cannot be statically configured by O&M but depends on each and every UE; and therefore, it requires either:

- Source RAN node to be made aware of the CSG/CAG allowed list of any UE

- Any UE to report some membership information.

Observation 5: solution 2bis cannot fulfil requirements of even basic CSG/CAG mixed campus/hospital deployments.

# 3	Conclusion and Way Forward

This paper has first shown that solution 2bis proposed at last minute of RAN3#124 does not work.

Proposal 1: eliminate solution 2bis proposed at RAN3#124 from the discussions.

Then this paper has evaluated the two solutions of the SA2 LS for inter-RAT mobility towards CAG/CSG cell. As seen from the summary table, the first conclusion is that any workable solution has source RAN impact.

NOTE: This is already what RAN3 replied to SA2 last year in their LS R3-226035.

The second conclusion is that solution 2 has no access control in source RAN node leading to repeated useless triggered and failed handovers every time the UE comes under a not allowed closed cell. Solution 2 is therefore not technically feasible.

Proposal 2: It is proposed to reply to SA2 by including the summary table and the following conclusion:

All solutions have source RAN node impact. In 5g to 4g direction, solution 2 has more AMF impact than solution 1 and leads to repeated handover failures. In 4g to 5g direction solution 2 has repeated handover failures impacting all source and target RAN and CN nodes and degrade handover KPI. Therefore, RAN3 think solution 2 is not technically feasible.  Solution 1 is technically feasible but has both UE and source RAN impacts.

Finally, in order to move forward, considering that the main drawback against solution 1 is the fact that UEs reports a misleading 4g ID (resp. 5G ID) concerning a 5G cell (resp. 4G cell), RAN3 could propose in their answer that solution 1 could be more acceptable if changed into solution 1bis whereby the UE reports instead to source RAN node the true CSG ID of a candidate target 4G femto (resp. the true CAG ID of a candidate target 5G femto) and the translation between CSG ID and CAG ID is done in the source RAN node which cannot anyway avoid impacts.

Proposal 3: suggest to SA2 to update solution 1 into solution 1bis whereby the UE reports instead to source RAN node the true CSG ID of a candidate target 4G femto (resp. the true CAG ID of a candidate target 5G femto).

The corresponding Reply LS is available in Annex A.

# 4	References

- RP-234041, Study Item on Additional topological enhancements for NR, NTT Docomo, AT&T

- R3-241832, [TP for TR 38.799] Solution for 5G Femto Architecture

- SP-231797, Study on System Aspects of 5G NR Femto

- S2-2405813, LS on Support of UE move between CAG cell of 5G Femto and CSG Cell

# 5	Annex A

3GPP TSG-RAN WG3#125			R3-24xxxx

Maastricht, Netherlands, 19 – 23 August 2024

Title:	Reply LS on Support of UE move between CAG cell of 5G Femto and CSG Cell

Response to:	LS on Support of UE move between CAG cell of 5G Femto and CSG Cell (R3-243020/S2-2405813)

Release:	Rel-19

Work Item:	FS_5G_Femto, FS_WAB_5GFemto_NR

Source:	RAN3

To:	SA2

Cc:	RAN2

Contact Person:

#### Name:	Philippe Godin

Tel. Number:	+33 1 6040 0304

E-mail Address:	philippe.godin@nokia.com

Attachments:

1. Overall Description:

RAN3 thanks SA2 for consulting on the inter-RAT mobility towards CAG/CSG cells. RAN3 would like to bring the following answers to SA2 questions:

Question 1: SA2 would like to know whether the two solutions mentioned above have any impact on the RAN (e.g., for RAN procedures)?

RAN3 answer: RAN3 has evaluated the two solutions and reached the following conclusions:

#### 5g to 4g direction

|  | Source gNB | Source AMF | Target MME | Target HeNB |
| --- | --- | --- | --- | --- |
| Solution 1 | CAG ID added into 4g part of 5g measurement report | Sends CSG ID to target MME | no | no |
| Solution 2 | Frequent Repeated handover failures | 1/ sends CSG ID to target MME 2/New access control in AMF duplicating access control in source gNB 3/ Frequent repeated handover failures 4/ mapping from target HeNB into list of CAG IDs | no | no |

#### 4g to 5g direction

|  | Source eNB | Source MME | Target AMF | Target NR Femto |
| --- | --- | --- | --- | --- |
| Solution 1 | CSG ID and membership status added into 5g part of 4g measurement report | no | no | no |
| Solution 2 | Frequent Repeated handover failures | Frequent Repeated handover failures | Frequent Repeated handover failures | Frequent Repeated handover failures |

In summary, all solutions have source RAN node impact. Please note that this was expected and was already communicated to SA2 two years ago in our previous LS in R3-226035.

In 5g to 4g direction, solution 2 leads to repeated handover failures and has more AMF impact than solution 1. In 4g to 5g direction solution 2 has repeated handover failures impacting all source and target RAN and CN nodes and degrade handover KPI. Therefore, RAN3 think solution 2 is not technically feasible.  Solution 1 is technically feasible but has both UE and source RAN impacts.

RAN3 would therefore like to suggest solution 1bis whereby the UE would report instead to source RAN node the true CSG ID of a candidate target 4G Femto (resp. the true CAG ID of a candidate target 5G Femto).

Question 2:  SA2 has reserved the time units for the normative work of WT#1 based on the result of RAN3 work (RP-234041), which is expected to start in SA2 from SA2#164. Therefore, SA2 requests to confirm the conclusion of RAN3 on overall architecture, etc., which will be used as the basis for SA2's normative work.

RAN3 answer: According to agreed workplan, RAN3 has concluded on overall architecture at this RAN3#125 (week 34 of 2024) and communicates the TR conclusion in attachment.

2. Actions:

To SA2 group:

ACTION: 	RAN3 kindly ask SA2 to take RAN3 answer into account.

3. Date of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3#125bis	                    14 – 18 October 2024				Heifei, China

TSG-RAN WG3#126	                    18 – 22 November 2024			Orlando, USA