3GPP TSG-RAN WG3 Meeting #125	R3-244417

Maastricht, NL, 19th Aug – 23rd Aug, 2024

Title: 	Further discussion on AI/ML for split architecture use cases

Source: 	Ericsson

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

In this contribution, we discuss the details of split architecture use-cases, primarily in the context of Load Balancing, Energy Saving, and Mobility Optimization use-cases, to be carried forward to normative discussions. During Release 18, it was agreed to first discuss non-split architecture for the above use-cases and that the principles and agreements made for the non-split architecture case will also be applied to the split architecture use-case in Release 19.

# 2. Discussion

The discussion and proposals in this paper are organized in terms of the different use-cases to which split architecture leftovers are applicable, followed by the required signalling enhancements, and finally a list of topics to be carried forward to normative phase.

## 2.1 UE performance

The first point for discussion is at what level of granularity should UE performance measurements be collected.

As part of discussions in Release 19 on AI/ML enabled slicing, it has been agreed that:

Agree to improve the granularity of UE performance feedback in Rel-19 for slicing.

But what should such improved granularity be?

It is worth noting that exchanging the measured UE performance between nodes included in the same gNB, e.g. between a gNB-CU-UP and a gNB-CU-CP, at a per DRB-level could be possible because all such nodes would be aware of the DRB to QoS flow mapping in use. However, the same cannot be said if the measured UE performance is exchanged per DRB across neighbour nodes, as the DRB to QoS flow mapping may change after a handover and the source would not know for what set of QoS Flows the performance corresponds. Evidently, in Release18, UE performance feedback for non-split architecture was specified at a UE-level granularity also for this reason.

Hence, one way to achieve an enhanced granularity for the measured UE performance when applied to AI/ML based network slicing is to enable collection of measured UE performance per slice. Namely, the reported UE performance in terms of packet delay, UL/DL throughput etc, will be provided per S-NSSAI in use by the UE. The performance metrics will consist of an average for all the DRBs in use at the UE for the same S-NSSAI.

On the basis of this, the following is proposed:

Proposal 1: RAN3 to agree to an enhancement of UE performance feedback for Release 18 (non-split architecture), where the measurements are exchanged at a per-slice-level granularity.

#### 2.1.1 Delay

As part of the meeting outcomes from RAN3 #124, it was agreed that UE performance metrics from the gNB-CU-UP shall be made available at the gNB-CU-CP.

Transferring Measured UE performance from CU-UP to CU-CP.

In the context of UE delay measurements, this corresponds to exchanging of the overall RAN UL and DL delay measurements, i.e., UL delay results and DL delay results at the UE, gNB-DU and gNB-CU-UP, from the gNB-CU-UP to the gNB-CU-CP.

It should be highlighted that the full set of RAN delay components could be available at the gNB-CU-UP, if the gNB-DU can provide its corresponding RAN delay components over F1-U and if the gNB-CU-CP could provide the UL PDCP delay to the gNB-CU-UP over E1. In current specifications the gNB-CU-UP receives such delay components from the gNB-DU and gNB-CU-CP only for QoS Flows for which QoS Monitoring has been activated. However, this could be changed and enhanced. Namely, in cases where packet delay is requested as UE performance feedback, the gNB-CU-CP could configure the gNB-DU to report packet delay components to the gNB-CU-UP for all the DRBs of the concerned UE. At the same time, the gNB-CU-CP could provide the UL PDCP delay to the gNB-CU-UP for each active DRB. The gNB-CU-UP would then be able to make use of its existing functionality to assemble together the full RAN delay components per DRB per slice and to return to the gNB-CU-CP a single value for the RAN delay component per S-NSSAI (as per Proposal 1). This per S-NSSAI delay measurement will then be provided to the requesting neighbour node over Xn, hence enabling slice-level and UE-level visibility on the delay performance.

Proposal 2: RAN3 to agree to the exchange of RAN delay measurements for UL and DL from the gNB-CU-UP to the gNB-CU-CP with a per-slice-level granularity.

#### 2.1.2 Packet loss

In Rel18 it was agreed to include Packet Loss as a parameter for UE performance feedback. It needs to be pointed out that UE performance feedback is meant to provide to the source RAN information about the service level performance with which a UE is served. The latter is useful to evaluate the performance of mobility actions derived from AI/ML-based inference in terms of how well services are served at the target cell. However, we observe that the Packet Loss parameter introduced in Rel18 may not represent the information originally desired and that it is challenging to collect it in a way that is meaningful and that cover all cases.

TS 28.552 defines measurement of DL Drop rate in gNB-CU-UP as the fraction of DL user plane packets that are dropped in relation to the total number of packets that entered the queue in the corresponding DRB over a period of time. The above measurement may be performed either at the PDCP level or at the RLC level to derive the packet loss rate at different levels in the protocol stack. In the context of UE performance reporting for packet losses, a value derived at a higher protocol layer, i.e., the measurements on the PDCP level packet loss rate, is considered more relevant as it better reflects the performance at service-level. Indeed, the packet loss at e.g. MAC level does not necessarily carry information about the quality of the service because a dropped MAC PDU may end up being successfully retransmitted with no negative effects on the service. Besides, a dropped MAC PDU may be due to aggressive modulation and coding scheme, which is an implementation choice and not a sign of good or bad performance.

It is also understood that, for a DRB delivered over RLC Unacknowledged mode, computing a PDCP packet loss metric as defined above is not possible. This is due to the fact that the PDCP entity does not keep track of packets that were acknowledged in RLC UM, but only keeps track of packets submitted to lower layers for transmission. Namely, PDCP does not know whether a RLC UM packet has been successfully delivered or not. For RLC unacknowledged mode packet loss rate may be computed at lower layers, i.e., at the RLC entity in the gNB-DU. However, as explained above, service level performance is not necessarily reflected by packet loss at RLC layer.

Moreover, packet loss rate at RLC layer is not available for UL traffic. To make this measurement available for UL traffic, a change at UE level would be needed, implying high impact on UEs and standard specifications and also implying that the measurement would not be available for legacy UEs.

The table below summarizes the drawbacks of Packet Loss rate collection at different protocol levels:

|  | Advantages | Disadvantages |
| --- | --- | --- |
| Packet Loss Rate Calculation at PDCP | It represents service level performance as it reflects IP packet drop rate | It cannot be calculated for RLC UM |
| Packet Loss Rate Calculation at RLC | It could be derived for RLC UM (from H-ARQ retransmissions result) | It is not available for UL unless new UL packet loss measurement reporting are introduced (this has UE impacts) |

As it can be seen from the above, measuring Packet Loss does not allow to cover all cases, e.g. see UL measurements or RLC UM bearers’ measurements.

Observation 1: Computing packet loss rate may not always be possible and it may not provide full information on service level performance.

Furthermore, there are some important distinctions to note when discussing the relationship between packet loss measurement at a lower layer and the service or application-level performance. 
Firstly, compared to other performance metrics, such as throughput and delay, the packet loss rate measured in the RAN is not necessarily reflected by the packet loss rate experienced at a higher layer. Namely, a loss of ‘n’ PDCP or RLC packets does not translate to the service or application experiencing ‘n’ higher layer packet drops. This is because, a number of packets at the lower layer may be retransmitted/recovered in time for delivery to the higher layer. Delay and throughput, on the other hand, are metrics directly linked to service performance at the higher layers, i.e., if the RAN has a minimum latency of 100ms or a maximum throughput of 100Mbit/s over a period of time, the service/application will not observe delivery of packets with a smaller delay or higher throughput over the same period. Based on the above, it may be argued that while delay and throughput are absolute measures of service level performance, packet loss is not.

Furthermore, it may be argued that compared to throughput and delays, packet loss is a much more volatile metric. 
In most situations, the choice of a modulation scheme is made in relation to a target BLER, and as a consequence a certain base packet loss rate is the consequence of a design choice of the BLER target. BLER targets are implementation specific and may change in relation to, e.g. the current SNR, the overall amount of resources available, the scheduling policy of a RAN node. Therefore, changes in the packet loss rate may not give information of whether the target cell is a good mobility target or not. The mobility target cell may be a perfectly good cell, but due to implementation choices the target RAN node may decide to e.g. apply an aggressive modulation and coding scheme to the UE and end up in higher packet loss rates, while the performance at the application layer is still good.

On the other hand, making packet loss rate available at the target gNB-CU is challenging. It requires changes over the split RAN interfaces. Hence the question is whether such impact to the standard is worthed, given that the Packet Loss metric does not cover all use cases and it may not represent performance at service level. On the basis of that, we propose the following:

Proposal 3: RAN3 to agree that packet loss is not a suitable candidate metric for AI/ML UE performance feedback in split architecture use cases.

Based on the above discussion, it is easy to deduce that packet loss rate may not be a good choice as a UE performance feedback metric even for a gNB in non-split architecture. Even if a new UL packet loss UL measurement was introduced in Rel19 (with associated UE impacts), such measurement would not be applicable to legacy UEs. On the other hand, lack of applicability of PDCP level packet loss to RLC UM transmission mode mean that such feedback may only be obtained for a fraction of the UEs (i.e. those not using RLC UM bearers).

Additionally, we reflect on the fact that release 18 discussions on packet loss and the inclusion of this IE in the UE performance feedback was largely derived from the QoS settings for packet loss rate. The specified range for the packet loss rate is an integer between 0 to 1000 with steps of 0.1%. In realistic scenarios, as specified for NR QoS parameters, the expected packet loss/error rate that the network is expected to offer to services is at most on the order of 10-2. Therefore, the Average Packet Loss DL IE, in its current standardised form, would not even be able to represent the measured packet loss rate in the most typical case (as the value range of the IE is too coarse). In other words, the Average Packet Loss DL IE introduced in TS38.423 is of no use and ignoring this IE would bring no impact to the overall functionality.

On the basis of this, we conclude that fixing the shortfalls to enable measurements of the packet loss rate in all use cases implies big impacts to the standard and in the end, it does not deliver information that are crucial to understand whether a mobility target is good or bad. We therefore propose the following:

Proposal 3: RAN 3 to agree on a correction to release 18 UE performance feedback, where the Average Packet Loss DL IE is not used and it is ignored.

#### 2.1.3 Throughput

As discussed in the previous section, any performance measure computed at a higher layer is generally considered closer to the application-level or service-level perceived performance. TS 28.552 defines different throughput measurements, however, the Average DL and UL UE throughput in gNB is defined as the ratio of successfully delivered RLC SDU data volume over time. Measuring throughput for UE performance at the RLC layer has the drawback that it may not consider other higher-layer interactions in the measure. Furthermore, this measure is computed at the gNB-DU (as the node hosting the RLC entity), and no mechanisms exists to exchange this with the gNB-CU-CP or the gNB-CU-UP.

On the other hand, TS 37.483 (E1 AP) defines the following IEs and corresponding functionality as part of the BEARER CONTEXT SETUP REQUEST and BEARER CONTEXT MODIFICATION request messages:

If the PDU Session Resource DL Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store and use the information for the down link traffic policing for the Non-GBR QoS flows for the concerned UE as specified in TS 23.501.

If the PDU Session Resource DL Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Setup List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall replace the information in the UE context and use it when enforcing downlink traffic policing for the non GBR QoS flows for the concerned UE, as specified in TS 23.501.

If the UE Slice Maximum Bit Rate List IE is included in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, store and use the information for the downlink traffic policing for each concerned slice as specified in TS 23.501.

If the UE Slice Maximum Bit Rate List IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, store and replace the previously provided UE Slice Maximum Bit Rate List by the received UE Slice Maximum Bit Rate List in the UE context, and use the received UE Slice Maximum Bit Rate List for the downlink traffic policing for each concerned slice as specified in TS 23.501.

It is evident from the above that a gNB-CU-UP has to measure the UE throughput at the PDCP level and for the DRBs corresponding to the slice for which the Maximum Bitrate was specified. However, it needs to be noted that no PDCP measurements have been defined to make such functionality work. Namely, it is not needed that PDCP throughput measurements are necessarily specified because in practice these measurements are collected already by the gNB-CU-UP.

Observation 2: There is already a requirement on the gNB-CU-UP to measure PDCP throughput without explicit definition of such measurements in the specifications.

In the context of throughput as UE performance, throughput at the PDCP layer is the most representative of the application-level or service-user performance. However, there exists no defined measurement for UE throughput at the PDCP layer as part of the NR user plane definitions. For the purpose of UE performance monitoring, throughput at PDCP may be computed as the average of measured data rates for UL and DL over all DRBs associated with a specific slice and with the UE.

Proposal 4: RAN3 to agree that the gNB-CU-UP can compute the UL and DL PDCP throughput for a UE.

Proposal 5: The measured average UL and DL PDCP throughput per slice is signalled from gNB-CU-UP to gNB-CU-CP as part of UE performance measurements.

## 2.2 Energy Saving

During RAN3 #121, the below agreement was reached, although further discussions in the context of split architecture were suspended to future releases, and the same is in scope in Release 19.

Work on the measured EC transmission from gNB-DU to gNB-CU over F1 in R18. Whether reusing the current F1AP procedures or defining new procedures needs to be further discussed.

The following in RAN3 #124 confirms the earlier agreement:

Measured Energy Cost can be transferred from DU to CU.

In addition to signalling the EC from the gNB-DU to the gNB-CU-CP, it needs to be discussed if the gNB-CU-UP should also signal its EC to the gNB-CU-CP or if it suffices to report EC from only the gNB-DU for use as node-level EC. Herein, we describe our views and argumentation in this matter.

It is common understanding that a significant portion of the energy consumed at a gNB in split architecture is expected to be at the different gNB-DUs, as it is the logical entity that hosts the baseband and radios. The energy consumed on a gNB-CU-UP, on the other hand, is expected to be negligible if compared to that of the gNB-DU. A complication worth considering in the generation of an EC associated to the gNB-CU-UP is that the gNB-CU-UP may be realized on a virtual/cloud environment hosting multiple instances of gNB-CU-UPs and either together with or independent of the gNB-CU-CP. In such cases, accurately determining the EC associated with a single instance of a gNB-CU-UP function may not be feasible. There is also precedence in NR specs when it comes to determining the energy consumption in split architecture, shown by the extract below from TS 28.310, where it is considered reasonable to compute the gNB’s energy consumption as just the energy consumption of the gNB-DU.

When gNBCU/gNBCU-CP/gNBCU-UP energy consumption is assumed to be very small compared to gNBDU and given that, in some cases, the gNBCU/gNBCU-CP/gNBCU-UP may be virtualized, the present document only considers the energy consumed in gNBDU(s) (in case of split scenarios) and in non-split gNBs (see clause 4.2.1 of 3GPP TS 28.541 [11] and clause 6.1.1 of 3GPP TS 38.401 [12]). There might be a need for some correction in KPI between the different deployment scenarios.

Based on the above, we have the following proposals:

Proposal 7: There is no need to signal EC from gNB-CU-UP to the gNB-CU-CP.

Proposal 8: RAN3 to only discuss signalling of EC from gNB-DU to gNB-CU-CP.

## 2.3 Signalling aspects

In light of what discussed in this paper it is straight forward that UE performance metrics in the split architecture case shall be signaled from the gNB-CU-UP to the gNB-CU-CP. Indeed, the only metric signalled from gNB-DU to gNB-CU-CP is the measured EC.

In this section, we tackle the signalling aspects of the aforementioned issues.

#### 2.3.1 UE performance

In the non-split architecture use-case, the request for UE performance metrics is implicitly carried over a handover request and it is indicated by the presence of a DataCollectionID in the handover request. The node collecting the UE performance transmits the collected information to the requesting node over the corresponding Data Collection Reporting procedure identified by the DataCollectionID. The configuration for the UE performance reporting is determined by the UE Performance Feedback Collection Configuration IE present in the Data Collection Request.

In the split architecture however, the UE performance metrics must be retrieved from the gNB-CU-UP of the target node, which does not receive the HANDOVER REQUEST from the source gNB. Furthermore, the DATA COLLECTION REPORTING procedure, which is used to signal back the collected UE performance, is not defined for the E1 interface.

Hence, to facilitate exchange of the UE Performance Feedback Collection Configuration and UE performance feedback reporting the following extensions are required:

Proposal 9: Extend the E1 AP specification with the DATA COLLECTION REPORTING procedure.

Proposal 10: At this stage, the proposed E1 AP DATA COLLECTION REPORTING procedure shall only consider aspects regarding UE performance feedback between gNB-CU-UP and the gNB-CU-CP.

Proposal 11: Extend the E1 AP Bearer Context Setup Request message to include the UE Performance Collection Configuration and the DataCollectionID.

The E1 AP Bearer Context Setup procedure is performed once per DRB that is setup at a target node. By taking advantage of a procedure that runs per DRB implies that a finer granularity of feedback can be obtained, however, additional information, with regards to the DRB structure, is required at a neighbor node that received per-DRB-level measurements. Therefore, the gNB-CU-UP, which collects the measurements at a per-DRB-level may signal the measurements as such to the gNB-CU-CP. However, when the gNB-CU-CP signals UE performance to a neighboring node, it may average the measurements over a single UE.

Proposal 12: RAN3 to discuss and agree to obtaining UE performance feedback at DRB-level granularity between a gNB-CU-UP and a gNB-CU-CP.

Proposal 13: RAN3 to agree that for UE performance reporting across nodes in non-split architecture to a per UE-level granularity.

Based on the above, UE performance feedback in split architecture will operate as the following description:

- A target gNB-CU-CP that receives a HANDOVER REQUEST with an implicit indication to collect UE performance for the corresponding handover,

- There exists a pre-configured Data Collection Reporting procedure between the gNB-CU-CP and the gNB-CU-UP, as part of which the UE Performance Collection Configuration was signalled.

- . The BEARER CONTEXT SETUP REQUEST message from the gNB-CU-CP to the gNB-CU-UP contains a DataCollectionID IE corresponding to an already configured E1 AP Data Collection Reporting Procedure between the gNB-CU-UP and the gNB-CU-CP.

- The gNB-CU-UP receiving the above request may collect the specified UE performance depending on the measurement and exit criteria and report the metrics back to the gNB-CU-UP in the next transmission instance of a DATA COLLECTION UPDATE message after the UE performance collection has terminated. The above measurement and reporting is performed per DRB configured for the UE

The figure below represents the full flow chart.

Step 1: Two neighbour gNBs have already been configured with a XN AP: Data Collection Reporting procedure between them, which also has a corresponding configuration for the UE performance collection configuration.

Step 2: gNB-CU-CP2 and gNB CU-UP2 also have a configured E1 AP: Data Collection Reporting procedure between them with a corresponding UE performance collection configuration.

Step 3a: gNB1 determines to handover one or more UEs to gNB2. This may be based on any AI/ML algorithm connected to use cases such as mobility optimization, load balancing, or energy savings.

Step 3b: gNB handovers UEs to the neighbour node. In the handover request message, the Data Collection ID corresponding to the Data Collection Reporting procedure (Step 1) is provided to the target gNB.

Step 4a: The UE connects to gNB2.

Step 4b: As part or continuation of the UE connecting to gNB2, DRBs are setup for the UE through the E1 AP: Bearer Context Setup Request message. This message also includes the ID of the E1 AP: Data Collection Reporting procedure over which the UE performance feedback shall be sent.

Step 5: Data delivered to the UE and legacy UP metrics (e.g. RAN level delay) are collected and reported

Step 6a: The exit criteria as specified in the UE performance collection configuration is fulfilled. This could be based on a timer, a handover, or other criteria.

Step 6b: The gNB-CU-UP reports the collected UE performance on the E1 AP: Data Collection reporting procedure.

Step 7: Upon receiving the collected UE Performance, the gNB-CU-CP2 generates per S-NSSAI, per UE performance metrics and transmits the information to the neighbour gNB-CU-CP1.

#### 2.3.2 Energy Saving

In this section, the actual stage 3 impact on RAN interfaces between the gNB-DU and the gNB-CU-CP, i.e., the procedure over which the EC signalling shall be performed over the F1 interface is discussed.

The exchange of the Energy Cost (EC) between gNBs (over Xn interface) is to be performed over a procedure dedicated for collection and exchange of AI/ML related information, called Data Collection Reporting procedure. This procedure is envisioned to exchange/fetch both measured and predicted information between NG RAN nodes, where either of the nodes may be providing inference, input, or predicted data to the other node simultaneously.

On the other hand, on the F1-C interface, it has been agreed that the inference functions will only be hosted at the gNB-CU-CP, and therefore the gNB-DU only reports measured values (either used for training or inference) to the AI/ML functions at the gNB CU-CP.

Observation 3:  In the context of exchanging EC, Xn and F1 interface have a different characteristic, namely no AI/ML inference results need to be signalled over the F1AP. For that, there is no need to add a new procedure over the F1 interface to accomplish exchange of EC from the gNB-DU to the gNB-CU.

Proposal 14: The required extensions to F1-C to facilitate the exchange of the EC from the gNB-DU to the gNB-CU-CP can be achieved by extending legacy procedures.

In the context of F1 AP, the Resource Status Reporting procedure is used by a gNB-CU to request the reporting of resource measurements from the gNB-DU. Energy Cost may also be considered a metric reflecting resources on the gNB-DU and therefore extending this procedure to include the measured EC metric would be a far simpler solution than defining an entirely new procedure to exchange AI/ML related information. As explained previously, the gNB-CU aggregates the information from the gNB-DUs to derive the node level EC and for use in AI/ML and potentially other use cases.

Proposal 15: Extend the F1: Resource Status Reporting procedure to include the Energy Cost metric.

# 3. Conclusion

Based on the discussion in this paper, we make the following observation and proposals.

Proposal 1: RAN3 to agree to an enhancement of UE performance feedback for Release 18 (non-split architecture), where the measurements are exchanged at a per-slice-level granularity.

Proposal 2: RAN3 to agree to the exchange of RAN delay measurements for UL and DL from the gNB-CU-UP to the gNB-CU-CP with a per-slice-level granularity.

Observation 1: Computing packet loss rate may not always be possible and it may not provide full information on service level performance.

Proposal 3: RAN3 to agree that packet loss is not a suitable candidate metric for AI/ML UE performance feedback in split architecture use cases.

Proposal 3: RAN 3 to agree on a correction to release 18 UE performance feedback, where the Average Packet Loss DL IE is not used and it is ignored.

Observation 2: There is already a requirement on the gNB-CU-UP to measure PDCP throughput without explicit definition of such measurements in the specifications.

Proposal 4: RAN3 to agree that the gNB-CU-UP can compute the UL and DL PDCP throughput for a UE.

Proposal 5: The measured average UL and DL PDCP throughput per slice is signalled from gNB-CU-UP to gNB-CU-CP as part of UE performance measurements.

Proposal 7: There is no need to signal EC from gNB-CU-UP to the gNB-CU-CP.

Proposal 8: RAN3 to only discuss signalling of EC from gNB-DU to gNB-CU-CP.

Proposal 9: Extend the E1 AP specification with the DATA COLLECTION REPORTING procedure.

Proposal 10: At this stage, the proposed E1 AP DATA COLLECTION REPORTING procedure shall only consider aspects regarding UE performance feedback between gNB-CU-UP and the gNB-CU-CP.

Proposal 11: Extend the E1 AP Bearer Context Setup Request message to include the UE Performance Collection Configuration and the DataCollectionID.

Proposal 12: RAN3 to discuss and agree to obtaining UE performance feedback at DRB-level granularity between a gNB-CU-UP and a gNB-CU-CP.

Proposal 13: RAN3 to agree that for UE performance reporting across nodes in non-split architecture to a per UE-level granularity.

Observation 3:  In the context of exchanging EC, Xn and F1 interface have a different characteristic, namely no AI/ML inference results need to be signalled over the F1AP. For that, there is no need to add a new procedure over the F1 interface to accomplish exchange of EC from the gNB-DU to the gNB-CU.

Proposal 14: The required extensions to F1-C to facilitate the exchange of the EC from the gNB-DU to the gNB-CU-CP can be achieved by extending legacy procedures.

Proposal 15: Extend the F1: Resource Status Reporting procedure to include the Energy Cost metric.

A detailed text proposal capturing the proposals in this paper is available in R3-244418.

Proposal 16: Agree on the TP provided in R3-244418.