3GPP TSG-RAN WG3 Meeting #125	R3-244418

Maastricht, NL, 19th Aug – 23rd Aug, 2024

Title: 	TP on Split architecture support for Rel-18 use cases

Source: 	Ericsson

Agenda item:	11.4

Document Type:	Discussion and decision

# TP to TR 38.743

This TP mirrors the discussions and proposals in R3-244417.

# 5	Rel-18 Leftovers and solutions

---text omitted---

## 5.2	Split architecture support for Rel-18 use cases

Editor Note: Capture the description and its potential standard impacts.

------------------------------------------- Start of changes -------------------------------------------

### 5.2.1 Use case description

The split architecture should be enhanced to support the Rel-18 use cases, e.g, Load Balancing, Energy Saving, and Mobility Optimization.

In case of CU-DU architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU(-CP);

- AI/ML Model Training and Model Inference are both located in the gNB-CU(-CP).

### 5.2.2 Potential Standard impacts

The following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

E1 interface:

- Measured UE performance from gNB-CU-UP to gNB-CU-CP.

F1 interface:

-	Measured Energy Cost from gNB-DU to gNB-CU.

The following are the new procedures introduced across the different nodes in the network:

- DATA COLLECITON REPORTING procedure over E1 interface (gNB-CU-CP – gNB-CU-UP).

The affected existing procedures are:

- E1 AP Bearer Context Setup procedure

- F1 AP Resource Status Reporting procedure

5.2.2.1 UE performance feedback in split architecture

Step 1: Two neighbour gNBs that have already been configured with a XN AP: Data Collection Reporting procedure between them, including a configuration for the UE performance collection configuration.

Step 2: gNB-CU-CP2 and gNB CU-UP2 also have a configured E1 AP: Data Collection Reporting procedure between them, including a corresponding configuration for UE performance collection configuration.

Step 3a: gNB1 triggers one or more handover to gNB2. This may be based on any AI/ML algorithm connected to use cases such as mobility optimization, load balancing, or energy savings.

Step 3b: gNB1 handovers UEs to gNB2. In the handover request message, the Data Collection ID corresponding to the Data Collection Reporting procedure (Step 1) is provided to gNB2.

Step 4a: The UE connects to gNB2.

Step 4b: DRBs are setup for the UE through the E1 AP: Bearer Context Setup Request message. This message also includes the ID of the E1 AP: Data Collection Reporting procedure over which the UE performance feedback shall be sent.

Step 5: Data delivered to the UE and legacy UP metrics (e.g. RAN level delay) are collected and reported

Step 6a: The exit criteria as specified in the UE performance collection configuration are fulfilled. This could be based on a timer, a handover, or other criteria.

Step 6b: gNB-CU-UP 2 reports the collected UE performance via the E1 AP: Data Collection reporting procedure.

Step 7: Upon receiving the collected UE Performance, the gNB-CU-CP2 singals the same information to the neighbour gNB-CU-CP1.

5.2.2.2		Energy Cost reporting in split architecture

Step 1: Two neighbour gNBs have already been configured with a F1 AP: Data Collection Reporting procedure between them, including a configuration for the reporting of EC from gNB2 to gNB1,

Step 2: At gNB2, the gNB-CU-CP 2 and the gNB-DU 2 are configured with a F1 AP: Resource Status Reporting procedure between them, including a configuration for the reporting of EC from gNB-DU2 to gNB-CU-CP2.

Step 3a: The reporting periodicity of the F1 AP: Resource Status Reporting procedure expires.

Step 3b: Signalling of F1 AP: Resource Status Update including the Energy Cost at the gNB-DU to gNB-CU-CP2.

Step 4a: The reporting periodicity of the XN AP: Data Collection Reporting procedure expires.

Step 4b: Signalling of XN AP: Data Collection Update with the measured Energy Cost of the gNB2 to gNB1.

------------------------------------------- End of changes -------------------------------------------