3GPP TSG-RAN WG3 Meeting #125	R3-244420

Maastricht, Netherlands, 19th – 23th August 2024

Agenda Item:	16.2

Source:	Nokia

Title:	[TP for TR 38.769] AIoT Architecture for Topology 1 and 2

Document for:	Discussions & Approval

# 1	Introduction

At the last RAN3 meeting, an architecture diagram was agreed for topology 1 in 3962 [3].

The tdoc R3-243807 [4] proposed for topology 2 but no conclusion could be reached for topology 2.

This paper resumes the discussion on ahcitecture diagram for topology 2, taking 243807[4] as baseline and explains why [4] coul not be agreed and which modifications are needed.

# 2	Discussion

At last RAN3#124 the following architecture diaram was proposed for topology 2 in [4]:

The first negative comment about the above proposal is the introduction of “RAS”.

Architectures have always been specified between UE, RAN and CN.

The 3GPP architecture has always been defined between UE, RAN and CN.

The introduction of an additional new RAS concept creates confusion. If AIoT RAS is not same as AIoT RAN does it include AIoT RAN? If it includes AIoT RAN what does it also include additionally if the AIoT device is not in it?

This confusion appears distinctly in the definition of AIoT RAS given below in

AIoT RAS (AIoT Radio Access System): hosts certain functions for AIoT as part of the functional split between RAN and CN.

Editor’s Note 4: Further details regarding AIoT functions hosted in the AIoT RAS and the respective functional split to be decided by RAN2, RAN3 and SA2.

As shown here-above in the highlighted part, the functional split intends to remain between AIoT RAN and CN, therefore the criteria to determine if an AIoT function is in RAS is based on whether it is either part of RAN or part of CN. Said differently, this means that AIoT RAS is either AIoT RAN or AIoT CN. Since it is obviously not AIoT CN, the above definition means that AIoT RAS is equal to AIoT RAN.

Proposal 1: the current definition of AIoT RAS is invalid. Remove the concept of AIoT RAS from the TR.

Going further, the interface between the AIoT device and the AIoT radio is defined as follows in [1]:

AIoT radio: radio interface between AIoT device and Common reader function.

Editor’s Note 5: Further details on AIoT radio to be discussed by RAN1 and RAN2.

Again, this definition does not make sense: an interface is defined between two logical nodes, and not between functions.

Proposal 2: the current definition of AIoT radio interface is invalid. Agree to redefine the AIoT radio interface.

Besides, the XX interface is defined as follows:

XX interface: interface between the AIoT RAS and the AIoT CN on which certain AIoT specific functions are performed.

Given that the AIoT RAS is undefined as explained above, the XX interface definition is also currently invalid. The editor’s note 7 represents this ambiguity:

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 1 it may only represent a single interface instance, e.g. a new interface between AIoT RAS and AIoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between AIoT CN and AIoT RAS, or one instance for NG alone.

How can this interface potentially be NG if the AIoT RAS is not RAN ?

Proposal 3: the current definition of XX interface is invalid. Agree to redefine the XX interface.

Even thougn tdoc [1] was only agreed for topology 1, the above contradictions prove that the current architecture is not a way forward and need complete redesign. RAN3 should come back to the two basic principles which prevail to define an architecture:

- An interface is defined between logical nodes,

- The logical nodes host the AIoT functions and these functions should be partitioned into UE, RAN and CN.

Proposal 4: agree to redesign the AIoT architecture according to the long lasting and sound principles of 3GPP RAN3.

Looking now at topology 2, it is clear from the WI that in toplogy 2 the logical node terminating the interface with the AIoT device is the Intermediate node as can be seen from the below diagram extracted from TR 38.848 [5]:

#### 4.2.1.2	Topology 2: BS ↔ intermediate node ↔ Ambient IoT device

Figure 4.2.1.2-1: Topology 2

With the intermediate node being the UE, this means that the UE will need to implement the AIoT interface in that case. Still, this is a logical node UE implementing the AIOT interface towards the AIoT device: the arrow showing the AIoT radio interface from AIoT device should stop at the UE.

A UE logical node involved in topology 2 will therefore implement common reader function in addition to usual non AioT functions which is already represented in the diagram.

Proposal 5: in topology 2, terminate the AIoT radio interface at the AIoT-enabled UE and not at the common reader function.

Similarly a gNB involved in topology 2, which can be named as an AIoT-enbaled gNB, will implement both AIoT RAN node function in addition to non-AIOT RAN node functions. Still, this is a logical node gNB and the arrow representing the Uu interface should terminate at the UE on one side and terminate in the logical node gNB on the other side.

In topology 2, the XX interface will link the AIOT CN with the logical node implementing the AIOT RAN function. In this case this is the logical AIOT enabled gNB: therefore, the XX arrow should terminate in the AIOT CN node on one side and terminate in the AIOT-enabled gNB on the other side.

Finally, From the above, it becomes clear that the AIOT RAN is comprised between the AIOT device and the AIOT CN: it therefore comprises the AIOT enabled UE and the AIOT enabled gNB: an additional brown rectangle can be drawn to represent the AIOT RAN.

Proposal 6: agree the below architecture diagram for the topology 2 re-introducing the AIoT RAN and removing the AIoT RAS.

It has been shown that there is no need to introduce a new concept of RAS to define the topology 2 which just bring more confusion because the XX interface is a RAN-Core interface.

Therefore, topology 1 should also be corrected accordingly, replacing RAS by RAN as follows:

Proposal 7: correct the architecture diagram for topology 1 accordingly replacing AIoT RAS by AIoT RAN.

# 3	Conclusion and proposals

It has been shown that the AIoT architecture can be defined without the need to introduce the new concept of AIoT RAS and the current AIoT RAS just brings confusion and contradictions. It is proposed to update the TR in line with the following proposals:

Proposal 1: the current definition of AIoT RAS is invalid. Remove the concept of AIoT RAS from the TR.

Proposal 2: the current definition of AIoT radio interface is invalid. Agree to redefine the AIoT radio interface.

Proposal 3: the current definition of XX interface is invalid. Agree to redefine the XX interface.

Proposal 4: agree to redesign the AIoT architecture according to the long lasting and sound principles of 3GPP RAN3.

Proposal 5: in Topology 2, terminate the AIoT radio interface at the AIoT-enabled UE and not at the common reader function.

Proposal 6: agree the below architecture diagram for the topology 2 re-introducing the AIoT RAN and removing the AIoT RAS.

Proposal 7: correct the architecture diagram for Topology 1 accordingly replacing AIoT RAS by AIoT RAN.

# 3	References

- RP-234058, New Study Item: Study on Solutions for Ambient IOT (Internet of Things), Huawei

- RP-240826, Revised Study Item: Study on Solutions for Ambient IOT (Internet of Things), CMCC

- R3-243962, [TP for TR 38.769] CB:#AIoT1_Architecture

- R3-243807, [TP for TR 38.769] CB:#AIoT1_Architecture

- TR 38.848, Study on Ambient AIoT in RAN

# 4	Text Proposal

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

AIoT RAN (AIoT Radio Access Network): hosts certain functions for AIoT as part of the functional split between RAN and CN.

Editor’s Note 4: Further details regarding AIoT functions hosted in the AIoT RAN and the respective functional split to be decided by RAN2, RAN3 and SA2.

AIoT radio: radio interface between AIoT device and AIoT RAN node hosting the Common reader function.

Editor’s Note 5: Further details on AIoT radio to be discussed by RAN1 and RAN2.

AIoT CN: hosts certain functions for AIoT as of the functional split between RAN and CN

Editor’s Note 6: Further details regarding AIoT functions hosted in the AIoT CN and the respective functional split to be decided by RAN2, RAN3 and SA2.

XX interface: interface between the AIoT RAN node hosting the AIoT RAN node function and the AIoT CN on which certain AIoT specific functions are performed.

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 1 it may only represent a single interface instance, e.g. a new interface between AIoT RAN and AIoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between AIoT CN and AIoT RAN, or one instance for NG alone.

Common reader function: a function that communicates with the AIoT device by means of AIoT radio.

Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2.

AIoT RAN node function: a function residing in AIoT RAN. It contains e.g. the control of the AIoT radio resources used towards the AIoT device.

Editor’s Note 9: further details are FFS. Note that “control of AIoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g. Inventory, Command) e.g. in case these functions have to be performed over a multitude of instances of the Common Reader Function are FFS.

Figure 6.4-1: Logical System Architecture for AIoT for topology 1.

Figure 6.4-2: Logical System Architecture for AIoT for topology 2.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>