3GPP TSG-RAN WG3 Meeting #125	R3-244424

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	18.2

Source:	Nokia

Title:	Overview of RAN3 Impacts of LP-WUS

WID/SID:	NR_LPWUS

Document for:	Discussion and Decision

# 1	Introduction

At the last RANP#102 meeting, the work item of LP-WUS was agreed in [1] including the following objective:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

This paper provides a brief summary of this feature and impact from the perspective of RAN3. It also proposes to endorse a draft stage 2 CR for TS 38.300.

# 2	Discussion

The LP-WUS feature has been designed to optimize further energy consumption of the device. It provides similar benefits as prior low power features based on wake-up signals such as PEI but with some differences. Even though the feature applies to both idle, inactive and connected state the main RAN3 impact is related to idle/inactive states.

For example, due to the presence of the new WUR (wake up receiver), some further optimization is reached due to the possibility for the UE to not turn on at all the main radio to monitor the PDCCH even for monitoring the wake-up signal. In this so-called “deep sleep” mode, the wake-up signal is monitored by the WUR instead, enabling further energy saving. Moreover, the main radio could be relaxed from doing some of the measurements taken over by WUR.

RRC Idle/Inactive state

As highlighted in the description of the objective, the feature reuses subgrouping functions in similarity with PEI subgrouping. RAN2 agreed the following at last RAN2#125bis:

  - The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups.

Foreseeable impacts to RAN3 could therefore be similar to PEI with impacts due to:

- the configuration of subgroups. NGAP impacts are foreseen for the Context setup procedure,

- paging subgroups of devices. NGAP and F1AP impacts for the paging procedures.

Proposal 1: LP-WUS has similar impacts for RAN3 as former PEI features of CN-based subgrouping and UE-ID based subgrouping with impact on Context Setup and Paging procedures on NGAP and F1AP protocols.

However, some notable differences with PEI/DCI include:

- Compared to PEI/DCI the payload is likely to be more reduced with only 16 bits. This means less energy consumed but this also means that identification of the subgroups may be limited to fewer subgroups. Also it should noticed that it is still undecided in RAN1 how many subgroups would be mapped per PO.

- A UE may a priori support PEI or LP-WUS features or both. For example, it is foreseen by RAN1/RAN2 that for idle/inactive state if a UE supports both features the UE could use either one or the other based on some trigger condition. Further, according to RAN1, even if a UE supports both features and network configures both, it is up to UE implementation whether to monitor PEI signal after the LP WUS signal, but this needs to be confirmed by RAN2.

The above points mean that the current signaling for PEI should preferably not be reused and separate signaling should be introduced allowing the LP-WUS parameters to co-exist with PEI parameters with new/different values.

Moreover, RAN2 took the following agreements at the last RAN2#126:

  - RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE.

  - RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8.

The above means that the number of subgroups is still undetermined, need to wait RAN2.

Proposal 2: PEI information elements over RAN3 protocols should preferably not be reused. New signaling should preferably be introduced in Context Setup and Paging procedures on NGAP and F1AP protocols. Exact values need to wait for RAN2.

As far as radio configuration aspects are concerned, RAN2#125bis took the following agreements:

  - The LP-WUS related configuration for IDLE/INACTIVE state is provided via system information. FFS if dedicated configuration is needed.

  - Working assumption: the LP-WUS configuration in SIB at least includes the following information: LP-SS configuration, LP-WUS configuration, FFS on Entry/exit condition for LP-WUS monitoring.

The entry/exit conditions are for example the RSRP/RSRQ quality thresholds upon which the UE will stop monitoring the legacy PO and start monitoring WUS and conversely. The above working assumption was further confirmed at RAN2#126.

We can assume that these configuration parameters will be configured directly in the gNB-DU to be delivered over SIB with no further impact.

Observation 1: it can be assumed that radio configuration parameters for SIB are configured in the gNB-DU with no signaling impact.

However, one aspect that differs in LP-WUS compared to PEI is that it takes time to turn on the main radio for the UE. Even though the mentioned order of magnitude is foreseen around 400 ms this delay will vary based on UE capabilities and can be more. It is important that this delay is taken into account for delivering the LP WUS signal early enough to make sure that all involved UEs have been able to turn on the main radio and not any of them misses the paging occasion. This may need to provide awareness of UE LP WUS capability into the CN.

Observation 2: depending on RAN1/RAN2 conclusion, additional delay for wake up may need to be taken into account by RAN3 for the paging.

RRC Connected state

LP-WUS in connected state is similar to DCP feature of release 16.

Besides, for RRC connected state, RAN2 took the following agreements at RAN2#126:

    - The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

    - E.g. for option 1-1 time offset before the start of drx-onDurationTimer for location of the LP-WUS monitoring occasion.

However, RAN1/RAN2 are still discussing which options to select. It could be option 1-1 or other option. so better to wait before making stage 3 CRs. However please note that according to WID in [1] the handling in idle/inactive state is considered higher priority than connected state:

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

In connected state, the UE would need to receive dedicated signaling from CU to help the monitoring of PDCCH. How this information is provided to CU might have RAN3 impact. This is pending further RAN2 work.

Observation 3: How CU receives the configuration to be sent to UE in RRC connected state is pending further RAN2 work.

Due to the various observations pending RAN1/RAN2 we make the following proposal:

Proposal 3: RAN3 to wait RAN1/2 conclusions before proceeding with stage 3 CRs.

Still, if above proposals 1 and 2 can be agreed, some stage 2 aspects can already be captured in stage 2 38.300 draft CR.

Proposal 4: It is proposed that RAN3 endorses the draft stage 2 CR in [2] to initiate the work.

# 3	Conclusion and Proposals

This paper has provided an overview of the new LP-WUS features and their potential impact on RAN3 protocols, emphasizing the main commonalities and differences with the PEI and Low Power features earlier introduced.

The following proposals and observations are made as way forward:

Proposal 1: LP-WUS has similar impacts for RAN3 as former PEI features of CN-based subgrouping and UE-ID based subgrouping with impact on Context Setup and Paging procedures on NGAP and F1AP protocols.

Proposal 2: PEI information elements over RAN3 protocols should preferably not be reused. New signaling should preferably be introduced in Context Setup and Paging procedures on NGAP and F1AP protocols. Exact values need to wait for RAN2.

Observation 1: it can be assumed that radio configuration parameters for SIB are configured in the gNB-DU with no signaling impact.

Observation 2: depending on RAN1/RAN2 conclusion, additional delay for wake up may need to be taken into account by RAN3 for the paging.

Observation 3: How CU receives the configuration to be sent to UE in RRC connected state is pending further RAN2 work.

Proposal 3: RAN3 to wait RAN1/2 conclusions before proceeding with stage 3 CRs.

Proposal 4: It is proposed that RAN3 endorses the draft stage 2 CR in [2] to initiate the work.

# 4	References

- RP-240801, Low Power Wake up signal and receiver for NR, Vivo, NTT Docomo, Ericsson, Mediatek, Samsung, Sony

- R3-244425, Support of LP-WUS