3GPP TSG-RAN3 Meeting #125	R3-244427

Maastricht, Netherlands, Aug 19th-23rd, 2024

Agenda Item:	16.2

Source: 	Qualcomm Incorporated

Title: 	Architecture, interfaces and transport for Ambient IoT

Document for:	Discussion/Decision

# Background

In this paper, we continue to provide our views on the architecture, interface and transport aspects for Ambient IoT (AIoT) based on the agreements and open issues identified in the previous RAN3 meetings.

# Discussion

## Logical System architecture for T1 and T2

In RAN3#123bis, RAN3 started the discussion identifying the logical nodes for T1 and T2 e.g., “AIoT RAN node”, “UE Reader”, “AIoT-enabled gNB”, “AIoT-aware CN node”

- [Topo1] AIoT RAN node:

  - Corresponds to the base station in Figure 4.2.1.1-1 in TR 38.848

  - A RAN node providing AIoT radio and connecting with an AIoT-aware CN node via the XX interface.

  - Details of the AIoT-aware CN node is subject to SA2.

- [Topo2] UE Reader:

  - A UE (corresponds to the intermediate node in Figure 4.2.1.2-1 in TR 38.848)

  - Providing AIoT radio and connecting with a gNB (may be an AIoT enhanced gNB, corresponds to the base station in Figure 4.2.1.2-1 in TR 38.848) via NR Uu interface.

- XX interface:

  - FFS whether it is the NG interface or a new interface, details to be discussed by RAN3 with SA2 progress of AIoT-aware CN node.

- XXAP (XX Application Protocol):

  - The radio network layer signaling protocol for the XX interface. It is FFS whether it is NGAP or a new XXAP.

  - It is FFS whether control plane transport or user plane transport is used for XXAP

However, in RAN3#124, RAN3 went away from the discussion of logical nodes and started discussing functionalities and drawing logical boxes such as “AIoT RAS”, “AIoT CN” to identify the functional split between AIoT RAN and AIoT CN.

In the end, the following was captured as the logical system architecture for AIoT (after several back-and-forth discussions) with the following Editor’s Note:

Editor’s Note 9: whether the logical system architecture above is applied to topology 2 to be discussed in next meeting.

AIoT RAS (AIoT Radio Access System): hosts certain functions for AIoT as part of the functional split between RAN and CN.

In our view, the way we have captured the above logical system architecture in the TR is not appropriate (and is not clear/complete) because:

- It doesn’t show the logical nodes (“AIoT RAN node” which was defined as the BS Reader for T1 and the “UE Reader” in T2 in R3#123bis) and just shows the logical boxes “AIoT RAS” and “AIoT CN”.

- T2 aspects are not shown or clear from the current figure e.g., the UE reader in case of T2 is not shown, whether the XX interface terminates at the UE Reader or at the “AIoT enabled gNB” is not clear.

- The figure is inconsistent in showing the logical interfaces e.g., “AIoT radio” interface terminates between a logical node (AIoT device) and a function (Common Reader function) whereas “XX” interface is drawn between two logical boxes (between AIoT RAS and AIoT CN) instead of between logical nodes.

- The AIoT RAN node function includes multiple functionalities e.g., resource allocation function, Inventory/Command coordination function which are not clearly defined yet. RAN3 should first discuss these functionalities in detail (e.g., whether resource allocation is pre-configured/static/dynamic/semi-static) or which entity (e.g., AIoT CN or RAN) can perform the routing of AIoT related messages and place them in logical nodes only when these aspects are clear.

We therefore propose the following:

- Proposal 1: For Topology 1, RAN3 should capture in TR a logical system architecture showing the AIoT device, AIoT RAN node and AIoT CN node and the logical interfaces involved, as shown in the figure below, where:

- AIoT radio is the interface between the AIoT device and the AIoT RAN node

- XX is the interface between the AIoT RAN node and the AIoT CN node

- Proposal 2: For Topology 2, RAN3 should capture in TR a logical system architecture showing the AIoT device, UE Reader, AIoT enabled gNB and AIoT CN node and the logical interfaces involved, as shown in the figure below:

- AIoT radio is the interface between the AIoT device and the UE reader hosting Common Reader Function

- N2 is the interface between the AIoT enabled gNB and the AIoT CN node

  - Note: Existing N2 interface might need enhancements for AIoT

- YY is the interface between the UE Reader and the AIoT CN node

  - Note: FFS the details of YY interface e.g., whether YY interface can use a control plane protocol (e.g., NAS protocol) or be carried over user plane (e.g., via a PDU session).

- Proposal 3: Remove “AIoT RAN node function” from the logical system architecture figure captured in the TR. RAN3 should discuss AIoT RAN node related functionalities (e.g., resource allocation aspects) and place them in logical nodes only when there is clear definition of those functionalities.

## Topology 1

The following figure depicts a simplified logical system architecture for Topology 1 (after removing the AIoT RAS and AIoT CN logical boxes and the common reader function):

For Topology 1, the interface between the AIoT RAN node and AIoT CN node (“XX interface”) is still not clear (e.g., whether it is a direct interface or are there any intermediate nodes (e.g., AMF) in between).

SA2 is still discussing whether existing 5GC entities (e.g., AMF) is involved in procedures related to AIoT and which of the legacy 5GC functionalities (e.g., registration, connection management) apply to AIoT.

In this section, we discuss two options for T1 for transporting AIoT related communication:

- T1-Option 1 (Indirect transport solution): AIoT RAN node indirectly (via the AMF) communicates with the AIoT CN node.

- T1-Option 2 (Direct transport solution): AIoT RAN node directly communicates with the AIoT CN node.

- Table 1 further compares the two options by showing its protocol stack, discussing the interfaces involved and discussing its pros/cons.

- Table 1: Evaluation of transport solutions for Topology 1

| - T1-Option 1 (Indirect transport solution) | - T1-Option 2 (Direct transport solution) |
| --- | --- |
| - NRPPa-like solution - SCTP-based stack between AIoT RAN node and AMF. - HTTP-based stack between AMF and AIoT CN node. - Different protocol stack at AIoT RAN node and AIoT CN node | - 5GC SBA like solution - HTTP-based stack at AIoT RAN node and AIoT CN node. - Common protocol stack at AIoT RAN node and AIoT CN node |
| - (Potentially) Two new protocols need to be defined. - Protocol b/w AIoT RAN node and AIoT CN node - (i.e., R-AP) - Protocol b/w AIoT RAN node and AMF - (i.e., XXAP) if N2 is not reused | - Only 1 new protocol needs to be defined. - Protocol between AIoT RAN node and AIoT CN node (i.e., R-AP). |
| - (Potentially) Two new interfaces need to be defined. - Interface between AIoT RAN node and AMF (XX) if N2 is not reused - Interface between AMF and AIoT CN node (Nac) | - Only 1 interface needs to be defined. - Interface between the AIoT RAN node and AIoT CN node (i.e., Nrc) |
| - Need upgrades to legacy AMF. | - No impact to legacy AMF. |
| - Less developer/cloud friendly - Uses point-to-point interface between AIoT RAN node and AMF (Nra interface) and service-based interface between AMF and AIoT CN node. | - More developer/cloud friendly - Uses Service Based Interface (SBI) between AIoT RAN node and AIoT CN node which is more developer/cloud friendly, RAT agnostic compared to a point-to-point interface. |
| - More backhaul latency due to involvement of AMF. - AMF is primarily used as a forwarding entity (“router”) for forwarding packets between AIoT RAN node and AIoT CN node. Potentially unnecessary involvement of AMF. | - Less backhaul latency (no involvement of AMF) |

- Proposal 4: RAN3 to capture in TP that the following 2 options are possible for the AIoT RAN node to communicate with AIoT CN node in T1:

- T1-Option 1 (Indirect transport solution): AIoT RAN node indirectly (via the AMF) communicates with the AIoT CN node.

- T1-Option 2 (Direct transport solution): AIoT RAN node directly communicates with the AIoT CN node.

- Proposal 5: RAN3 to capture in TP “Table 1” which evaluates the two transport options for AIoT communication in Topology 1 and lists their pros and cons.

Observation 1: T1-Option 1 (Indirect transport solution) has the following drawbacks compared to T1-Option 2 (direct transport solution):

- Need upgrades at legacy AMF to support two new interfaces (AIoT RAN node-AMF, AMF-AIoT CN node)

- More backhaul latency (due to involvement of AMF)

- Potentially unnecessary involvement of AMF (just as a “router”) for AIoT

- Potentially different protocol stack at AIoT RAN node (SCTP based stack) and AIoT CN node (HTTP based stack)

- Less developer/cloud friendly due to point-to-point interface between AIoT RAN node and AMF.

- Potentially additional protocol b/w AIoT RAN node and AMF (XXAP) needs to be defined to carry end-to-end packets between AIoT RAN node and AIoT CN node (R-AP)

- Proposal 6: RAN3 to capture in TP as a conclusion for T1 that “It is RAN3’s preference that the AIoT RAN node directly communicates with the AIoT CN node (without going via AMF) in the study of Topology 1 (referred to as T1-Option 2: Direct Transport Solution in this paper)”

Also, there is an open issue on whether to reuse NGAP for the interface between AIoT RAN node and AMF/AIoT CN node.

The following is from TS 38.410:

4.3        NG interface specification objectives

The NG interface specification facilitates the following:

-     inter-connection of NG-RAN nodes with AMFs supplied by different manufacturers;

-     separation of NG interface Radio Network functionality and Transport Network functionality to facilitate introduction of future technology.

4.4        NG interface capabilities

The NG interface supports:

-     procedures to establish, maintain and release NG-RAN part of PDU sessions;

-     procedures to perform intra-RAT handover and inter-RAT handover;

-     the separation of each UE on the protocol level for user specific signalling management;

-     the transfer of NAS signalling messages between UE and AMF;

-     mechanisms for resource reservation for packet data streams;

-     procedures to establish, maintain and release NG-RAN part of MBS sessions.

NG interface facilitates interconnection of NG-RAN nodes with AMF and support several procedures e.g., handling PDU sessions, mobility, NAS transfer for UE-specific signaling.

In our view, reusing NGAP is not appropriate for AIoT because of the following reasons:

- Unclear on the involvement of AMF for AIoT because most of the existing AMF functionalities (e.g., Registration/Connection/Mobility management) might not apply for AIoT and also SA2 has not yet concluded about the need of AMF for AIoT.

- Unclear whether there needs to be AIoT device-awareness at AIoT CN node. In NR, there is a NAS protocol between UE and AMF and UL/DL NAS messages are carried over UE-specific signaling in NGAP. However, in AIoT, it is not yet clear and perhaps no need to have awareness of each AIoT device in the AIoT CN node and thereby perhaps no need to carry AIoT-device specific signaling.

- Presence of several mandatory IEs in NGAP procedures

There are several mandatory IEs in the existing NGAP procedures that makes its reuse almost next to impossible and very inefficient.

For example, if we want to reuse NGAP and say reuse the NG Setup Request procedure for the Reader to setup a TNL association with the AMF/AIoT CN node, then there are several mandatory IEs e.g., the Global RAN Node ID, TAC, PLMN Identity which might not be relevant for the Reader.

So, we have to add clarifications in Semantics that these IEs don’t apply for Readers, or we have to define a completely new procedure/messages for Reader to setup a TNL association with the AIoT controller/Core network.

- Reusing NGAP is not clean from a spec or implementer perspective

If we have to reuse NGAP, we will have to end up creating entirely new messages e.g., “AIoT N2 Setup”. Also, someone who wishes to implement only the BS Reader functionality and not the legacy NG-RAN functionalities have to ignore all the legacy messages and read this big specification just for a few AIoT messages.

- Avoid creating a specification like TS 29.413 (Application of NGAP to non-3GPP access) where we had to add several sections mentioning “NGAP messages used for non-3GPP access”, “Exceptions for NGAP messages not specified to be applicable b/w the Non-3GPP access and AMF”.

Observation 2: In Topology 1, reuse of NGAP for AIoT related signalling between AIoT RAN node and AIoT CN node is not appropriate because of several reasons:

- Unclear on the involvement of AMF for AIoT

- Unclear whether there needs to be AIoT device-awareness at AIoT CN node

- Presence of several mandatory IEs in NGAP procedures

- Reusing NGAP is not clean from a spec or implementer perspective

- Avoid creating a specification like TS 29.413 (use of NGAP for non-3GPP access) with several exceptions.

We therefore think its cleaner to define a new application layer protocol (called Reader Application Protocol or R-AP in short) for the interaction between the Reader and the AIoT CN node. This is shown pictorially in the below figure. In other words, Alt 2 below should be the way forward in our view.

Proposal 7: RAN3 to capture in TP as a conclusion for T1 that “It is RAN3’s preference that NGAP is not reused for AIoT RAN node to communicate with the AIoT CN node in Topology 1”.

Proposal 8: RAN3 to capture in TP as a conclusion for T1 that “A new radio network layer protocol “R-AP (Reader-Application Protocol)” is defined for AIoT RAN node to directly communicate with the AIoT CN node in Topology 1”.

## Topology 2

It is still FFS whether we to use a control-plane based solution or user-plane based solution for T2. It has been raised before there can be 3 options for communication between the UE Reader and AIoT CN node.

T2-Option 1 (RRC-based solution): UE Reader uses its RRC connection with the gNB to send/receive AIoT messages explicitly in RRC. gNB then forwards the AIoT messages to the AIoT CN node explicitly over N2. No end-to-end protocol between the UE reader and the AIoT CN node.

T2-Option 2 (NAS-based solution): UE Reader uses its NAS association with the AMF to send/receive AIoT messages as NAS PDUs. AMF can then forward the AIoT messages to the AIoT CN node via SBI interfaces in the core. In this solution, the AIoT enabled gNB is unaware of the contents of the AIoT messages as it is received as a NAS container. This is similar to PDU Session Management messages carried in NAS (SMF is behind AMF and AMF does not process them).

T2-Option 3 (PDU session-based solution): UE Reader uses user plane signaling (establishes a PDU session till UPF) to communicate with AIoT CN node. UPF can then forward the AIoT messages to the AIoT CN node via SBI interfaces in the core. In this solution, the AIoT enabled gNB is unaware of the contents of the AIoT messages as it is sent over user plane.

In the Table 2 below, we compare the three options by discussing the protocol stack, interface impacts and their pros vs. cons.

| - RRC-based solution | - NAS-based solution | - PDU session-based solution |
| --- | --- | --- |
| - UE Reader uses its RRC connection with the gNB to send/receive AIoT messages explicitly in RRC. - gNB then decodes the AIoT RRC messages and forwards the AIoT messages to the AIoT CN node explicitly over N2. | - UE Reader uses its NAS association with the AMF to send/receive AIoT messages as NAS PDUs. - AMF can then forward the AIoT messages to the AIoT CN node via SBI interfaces in the core. | - UE Reader uses user plane signaling (establishes a PDU session till UPF) to communicate with AIoT CN node. - UPF can then forward the AIoT messages to the AIoT CN node via SBI interfaces in the core. |
| - AIoT enabled gNB is aware of the contents of the AIoT messages as it is received explicitly over RRC. However, it adds unnecessary RRC ASN.1 complexity. | - AIoT enabled gNB is unaware of the contents of the AIoT messages as it is received as a NAS container - AIoT enabled gNB can be made aware explicitly by the AIoT CN node providing assistance information in N2. | - AIoT enabled gNB is unaware of the contents of the AIoT messages as it is sent over user plane - AIoT enabled gNB can be made aware explicitly by the AIoT CN node providing assistance information in N2. |
| - No end-to-end protocol between the UE reader and the AIoT CN node | - There is an end-to-end protocol (R-AP) between the UE reader and the AIoT CN node. | - There is an end-to-end protocol (R-AP) between the UE reader and the AIoT CN node |
| - RRM like solution - This is like how gNB can explicitly request RRM measurements to UE explicitly via RRC | - LPP-like solution - This is like how LPP is defined between UE and LMF and carried over NAS/NGAP/RRC | - UPL like solution - This is like how UPL (User Plane Location Protocol) is used to carry location between a UE and location server |
| - UE Reader selection can be performed by gNB or AIoT CN node | - UE Reader selection performed by AIoT CN node | - UE Reader selection performed by AIoT CN node |
| - Overload on AMF if there are lot of AIoT devices | - Overload on AMF if there are lot of AIoT devices | - UPF can handle IP traffic better. |
| - RAN2 impact: High - RAN2 would need to define new RRC signaling to support the reception/transmission of AIoT related messages from gNB to UE Reader - RAN2 would need to define procedures when the UE Reader’s serving gNB changes (e.g., due to mobility) or RRC state changes | - RAN2 impact: No/minimal | - RAN2 impacts: No/minimal |
| - RAN3 impact: High - RAN3 would need to define explicit signaling over N2 for the AIoT messages - In case the serving gNB of the UE Reader changes (e.g., due to mobility) or the RRC state of the UE reader changes, - Xn coordination is needed to provide UE Reader’s context from source gNB to target gNB | - RAN3 impact: No/minimal - No new procedures needed when the UE Reader’s serving gNB changes or the UE Reader changes its RRC state | - RAN3 impact: No/minimal - No new procedures needed when the UE Reader’s serving gNB changes or the UE Reader changes its RRC state |
| - SA2 impact: Medium - SA2 needs to define the interface between AMF and AIoT CN node (Nac interface) | - SA2 impact: Medium - SA2 needs to define the interface between AMF and AIoT CN node - (Nac interface) and support the end-to-end R-AP protocol | - SA2 impact: Medium - SA2 needs to define the interface between UPF and AIoT CN node (Nuc interface) and support the end-to-end R-AP protocol |
| - Non-homogeneous control plane transport - RRC between UE Reader and gNB - N2 between gNB and AMF - API between AMF and AIoT CN node | - Non-homogeneous control plane transport - NAS between UE Reader and AMF - API between AMF and AIoT CN node | - Homogenous transport: End to-end IP - IP connectivity (PDU session) between UE Reader and UPF - IP connectivity between UPF and AIoT CN node |
| - Needs support of RRC at UE Reader (already supported by UEs today) | - Needs support of NAS at UE Reader (already supported by UEs today) | - Needs support of HTTP client at UE Reader (already supported by UEs today) |
| - Fragmented protocol stacks for Reader in T1 and T2 - AIoT RAN node in T1 will have R-AP over IP - UE Reader in T2 will receive signaling over RRC | - Fragmented protocol stacks for Reader in T1 and T2 - AIoT RAN node in T1 will have R-AP over IP - UE Reader in T2 will have R-AP over NAS | - Can support common SBI protocol stack for Reader in T1 and T2 - AIoT RAN node in T1 will have R-AP over IP - UE Reader in T2 will have R-AP over IP |

- Proposal 9: RAN3 to capture in the TP the following 3 options for the UE Reader to communicate with AIoT CN node:

- T2-Option 1 (RRC-based solution): UE Reader uses its RRC connection with the gNB to send/receive AIoT messages explicitly in RRC. gNB then forwards the AIoT messages to the AIoT CN node explicitly over N2.

- T2-Option 2 (NAS-based solution): UE Reader uses its NAS association with the AMF to send/receive AIoT messages as NAS PDUs. AMF can then forward the AIoT messages to the AIoT CN node via SBI interfaces in the core.

- T2-Option 3 (PDU session-based solution): UE Reader uses user plane signaling (establishes a PDU session till UPF) to communicate with AIoT CN node. UPF can then forward the AIoT messages to the AIoT CN node via SBI interfaces in the core

- Proposal 10: RAN3 to capture in TP “Table 1” which compares the three options for AIoT communication in Topology 2 and lists their pros and cons.

- Considering there are high impacts to RAN2/RAN3 with T2-Option 1 (RRC-based solution), it is our view that RAN3 should not prefer T2-Option 1. This should also be evaluated by RAN2, and their preference taken into consideration.

Observation 3: T2-Option 2 (NAS based solution) has the following drawbacks compared to T2-Option 3 (PDU session-based solution)

- Overload on AMF (e.g., if there are a lot of AIoT devices)

- Result in different protocol stacks for Reader in T1 and T2 (i.e., AIoT RAN node in T1 uses R-AP over IP whereas NAS based solution would use R-AP over NAS)

- Non-homogeneous control plane transport (NAS between UE Reader and AMF and API between AMF and AIoT CN node)

- Proposal 11: RAN3 should capture in TP as a conclusion for T2 that 
“RAN3 recommends a PDU session-based solution (T2-Option 3 in this paper) for UE Reader to communicate with the AIoT CN node in Topology 2 i.e., the UE Reader establishes a PDU session to send AIoT data to UPF which forwards it to the AIoT CN node”.

## Unified logical system architecture for T1 and T2

RAN3 made the following agreement last meeting:

RAN3 considers both Topologies at the same time looking whether commonalities are applicable.

Also, RAN2’s agreements below also show that they intend to use a common design for T1 and T2 and use the terminology “Reader” in several agreements. In other words, they don’t distinguish between T1 and T2 by using different terminologies e.g., AIoT RAN node and UE Reader

Unless explicitly stated all agreements apply to all device types and for both topologies.

From RAN2 perspective, the aim is that the design on the interface between reader and A-IoT device is common for topology 1 and topology 2.

Based on the service request, the reader sends the Initial Trigger Message indicating device(s) that need to respond; Details FFS

RAN2 assumes that RRC layer is not necessary between the reader and the device.

FFS about the level of visibility required by the reader and what information is necessary for AS layer operations.

Random Access is triggered by the reader

Reader provides the information that the device needs to respond to the random access trigger.  FFS what those parameters are

For the very first access message from the device to reader in random access an ID is included.  RAN2 to discuss whether a temporary identifier is included, or the permanent device ID is included (considering other WGs input as well).

Observation 3: RAN2 agreed that its aim is to design a common air interface between reader and AIoT device for T1 and T2 whereas RAN3 also agreed to look at both topologies while looking at the commonalities.

Observation 4: RAN2 uses the terminology “Reader” in many of their agreements and doesn’t distinguish between the topologies (unlike RAN3 which decided to use AIoT RAN node for T1 and UE Reader for T2).

We think there are a lot of commonalities in the architecture and protocol stack as well and RAN3 should look at those commonalities as well.

Let’s recap our proposal to use direct-transport solution for T1 and user plane solution for T2. Putting the two protocol stacks together in the Figure below, it can be seen that for both topologies, the Reader (AIoT RAN node in T1 and UE Reader in T2) supports an R-AP protocol over an SBI protocol stack (HTTP/TLS/TCP/IP/L2/L1) and uses an IP-based transport to carry R-AP packets (i.e., AIoT signaling/data between Reader and AIoT CN node). Only the lower layer (L1/L2) and the intermediate nodes might be different between the 2 topologies (e.g., Uu/N3 via gNB/UPF is used for T2 whereas L2/L1 of AIoT RAN node is used in T1)

Observation 5: Using a direct-transport solution for T1 and PDU session based solution for T2 results in a common protocol stack and interface design for the Reader in T1 and T2 i.e., the Reader uses an IP-based transport to carry R-AP packets (i.e., AIoT signaling/data between Reader and AIoT CN node)

- Alternatively, if we had used Indirect transport solution for T1 and NAS based solution for T2, we would not be having commonalities in the protocol stack and interface design for T1 and T2 e.g., T1 would use R-AP over XXAP whereas T2 would use R-AP over NAS. Reader in T1 (AIoT RAN node) would support a SCTP based stack whereas Reader in T2 just uses NAS/AS stack. This is shown via the figure below.

Observation 6: Using Indirect transport solution for T1 and RRC/NAS based solution for T2 would result in having a different protocol stack and interface design for Reader in T1 and T2 e.g.,

- T1 would use R-AP over XXAP whereas T2 would use R-AP over NAS.

- Reader in T1 (AIoT RAN node) would support a SCTP based stack whereas Reader in T2 just uses NAS/AS stack.

Considering the commonalities in functionalities and architecture with our solution (direct-transport solution for T1 and user plane solution for T2), we propose to merge the two entities “AIoT RAN node” and “UE Reader” into a unified terminology “Reader” and use that for subsequent discussion henceforth.

Defining separate logical entities based on topology (AIoT RAN node for T1 and UE Reader for T2) is inefficient because it would result in duplicate and complex specification efforts in defining procedures and protocol stack for T1 and T2 separately and possibly fragmented solutions.

This common “Reader” node can interact with the AIoT CN node via the R-AP protocol carried over SBI protocol stack (HTTP/TLS/TCP/IP/L2/L1) as shown below.

Observation 7: Defining separate logical entities based on topology (AIoT RAN node for T1 and UE Reader for T2) is inefficient because it would result in duplicate and complex specification efforts in defining procedures and protocol stack for T1 and T2 separately and possibly fragmented solutions.

- Proposal 12: Define a single logical node “Reader” that can depict the Reader functionality for both T1 and T2 (instead of AIoT RAN node for T1 and UE Reader for T2). The Reader can interact with the AIoT CN node via the R-AP protocol carried over SBI protocol stack (HTTP/TLS/TCP/IP/L2/L1)

# Summary

Logical System Architecture

- Proposal 1: For Topology 1, RAN3 should capture in TR a logical system architecture showing the AIoT device, AIoT RAN node and AIoT CN node and the logical interfaces involved, as shown in the figure below, where:

- AIoT radio is the interface between the AIoT device and the AIoT RAN node hosting the Common reader function

- XX is the interface between the AIoT RAN node and the AIoT CN node

- Proposal 2: For Topology 2, RAN3 should capture in TR a logical system architecture showing the AIoT device, UE Reader, AIoT enabled gNB and AIoT CN node and the logical interfaces involved, as shown in the figure below:

- AIoT radio is the interface between the AIoT device and the UE reader hosting Common Reader Function

- N2 is the interface between the AIoT enabled gNB and the AIoT CN node

  - Note: Existing N2 interface might need enhancements for AIoT

- YY is the interface between the UE Reader and the AIoT CN node

  - Note: FFS the details of YY interface e.g., whether YY interface can use a control plane protocol (e.g., NAS protocol) or be carried over user plane (e.g., via a PDU session).

- Proposal 3: Remove “AIoT RAN node function” from the logical system architecture figure captured in the TR. RAN3 should discuss AIoT RAN node related functionalities (e.g., resource allocation aspects) and place them in logical nodes only when there is clear definition of those functionalities.

Topology 1

- Proposal 4: RAN3 to capture in TP that the following 2 options are possible for the AIoT RAN node to communicate with AIoT CN node in T1:

- T1-Option 1 (Indirect transport solution): AIoT RAN node indirectly (via the AMF) communicates with the AIoT CN node.

- T1-Option 2 (Direct transport solution): AIoT RAN node directly communicates with the AIoT CN node.

- Proposal 5: RAN3 to capture in TP “Table 1” which evaluates the two transport options for AIoT communication in Topology 2, lists their pros and cons along with the WG impacts

Observation 1: T1-Option 1 (Indirect transport solution) has the following drawbacks compared to T1-Option 2 (direct transport solution):

- Need upgrades at legacy AMF to support two new interfaces (AIoT RAN node-AMF, AMF-AIoT CN node)

- More backhaul latency (due to involvement of AMF)

- Potentially unnecessary involvement of AMF (just as a “router”) for AIoT

- Potentially different protocol stack at AIoT RAN node (SCTP based stack) and AIoT CN node (HTTP based stack)

- Less developer/cloud friendly due to point-to-point interface between AIoT RAN node and AMF.

- Potentially additional protocol b/w AIoT RAN node and AMF (XXAP) needs to be defined to carry end-to-end packets between AIoT RAN node and AIoT CN node (R-AP)

- Proposal 6: RAN3 to capture in TP as a conclusion for T1 that “It is RAN3’s preference that the AIoT RAN node directly communicates with the AIoT CN node (without going via AMF) in the study of Topology 1 (referred to as T1-Option 2: Direct Transport Solution in this paper)”

Observation 2: In Topology 1, reuse of NGAP for AIoT related signalling between AIoT RAN node and AIoT CN node is not appropriate because of several reasons:

- Unclear on the involvement of AMF for AIoT

- Unclear whether there needs to be AIoT device-awareness at AIoT CN node

- Presence of several mandatory IEs in NGAP procedures

- Reusing NGAP is not clean from a spec or implementer perspective

- Avoid creating a specification like TS 29.413 (use of NGAP for non-3GPP access) with several exceptions.

Proposal 7: RAN3 to capture in TP as a conclusion for T1 that “It is RAN3’s preference that NGAP is not reused for AIoT RAN node to communicate with the AIoT CN node in Topology 1”.

Proposal 8: RAN3 to capture in TP as a conclusion for T1 that “A new radio network layer protocol “R-AP (Reader-Application Protocol)” is defined for AIoT RAN node to directly communicate with the AIoT CN node in Topology 1”.

Topology 2

- Proposal 9: RAN3 to capture in the TP the following 3 options for the UE Reader to communicate with AIoT CN node:

- T2-Option 1 (RRC-based solution): UE Reader uses its RRC connection with the gNB to send/receive AIoT messages explicitly in RRC. gNB then forwards the AIoT messages to the AIoT CN node explicitly over N2.

- T2-Option 2 (NAS-based solution): UE Reader uses its NAS association with the AMF to send/receive AIoT messages as NAS PDUs. AMF can then forward the AIoT messages to the AIoT CN node via SBI interfaces in the core.

- T2-Option 3 (PDU session-based solution): UE Reader uses user plane signaling (establishes a PDU session till UPF) to communicate with AIoT CN node. UPF can then forward the AIoT messages to the AIoT CN node via SBI interfaces in the core

- Proposal 10: RAN3 to capture in TP “Table 2” which compares the three options for AIoT communication in Topology 2, lists their pros and cons along with the WG impacts

Observation 3: T2-Option 2 (NAS based solution) has the following drawbacks compared to T2-Option 3 (PDU session-based solution)

- Overload on AMF (e.g., if there are a lot of AIoT devices)

- Result in different protocol stacks for Reader in T1 and T2 (i.e., AIoT RAN node in T1 uses R-AP over IP whereas NAS based solution would use R-AP over NAS)

- Non-homogeneous control plane transport (NAS between UE Reader and AMF and API between AMF and AIoT CN node)

- Proposal 11: RAN3 should capture in TP as a conclusion for T2 that 
“RAN3 recommends a PDU session-based solution (T2-Option 3 in this paper) for UE Reader to communicate with the AIoT CN node in Topology 2 i.e., the UE Reader establishes a PDU session to send AIoT data to UPF which forwards it to the AIoT CN node”.

Unified logical system architecture for T1 and T2

Observation 3: RAN2 agreed that its aim is to design a common air interface between reader and AIoT device for T1 and T2 whereas RAN3 also agreed to look at both topologies while looking at the commonalities.

Observation 4: RAN2 uses the terminology “Reader” in many of their agreements and doesn’t distinguish between the topologies (like RAN3 decided to use AIoT RAN node for T1 and UE Reader for T2).

Observation 5: Using a direct-transport solution for T1 and PDU session-based solution for T2 results in a common protocol stack and interface design for the Reader in T1 and T2 i.e., the Reader uses an IP-based transport to carry R-AP packets (i.e., AIoT signaling/data between Reader and AIoT CN node)

Observation 6: Using Indirect transport solution for T1 and RRC/NAS based solution for T2 would result in having a different protocol stack and interface design for Reader in T1 and T2 e.g.,

- T1 would use R-AP over XXAP whereas T2 would use R-AP over NAS.

- Reader in T1 (AIoT RAN node) would support a SCTP based stack whereas Reader in T2 just uses NAS/AS stack.

Observation 7: Defining separate logical entities based on topology (AIoT RAN node for T1 and UE Reader for T2) is inefficient because it would result in duplicate and complex specification efforts in defining procedures and protocol stack for T1 and T2 separately and possibly fragmented solutions.

- Proposal 12: Define a single logical node “Reader” that can depict the Reader functionality for both T1 and T2 (instead of AIoT RAN node for T1 and UE Reader for T2). The Reader can interact with the AIoT CN node via the R-AP protocol carried over SBI protocol stack (HTTP/TLS/TCP/IP/L2/L1)

TP

- Proposal 13: RAN3 to agree the TP to 38.769 in Annex 1.

# Annex 1 : TP to 38.769

## 6.3	RAN architecture aspects

### 6.3.1	Definitions

Editor’s note 1: Corresponds to the second RAN3 objective in the SID, to identify RAN architecture aspects, including whether support for split architecture is necessary.

This chapter attempts to identify and describe architectural elements necessary to define a RAN architecture for support of Ambient IoT embedded in the overall 5G system architecture in support of topology 1 and topology 2 (as defined in TR 38.848 [2]).

Editor’s Note 2: What functionalities are hosted by the 5GS for AIoT is TBD.

This chapter also attempts to identify a functional split between RAN and CN.

Figure 6.3-1 depicts the logical system architecture for AIoT.

It consists of the following architectural elements:

AIoT device: equipment with characteristics outlined e.g. in TS 22.369 [x] and TR 38.848 [2].

Editor’s Note 3: Further details FFS, if any.

AIoT RAS (AIoT Radio Access System): hosts certain functions for AIoT as part of the functional split between RAN and CN.

Editor’s Note 4: Further details regarding AIoT functions hosted in the AIoT RAS and the respective functional split to be decided by RAN2, RAN3 and SA2.

AIoT radio: radio interface between AIoT device and Common reader function.

Editor’s Note 5: Further details on AIoT radio to be discussed by RAN1 and RAN2.

AIoT CN: hosts certain functions for AIoT as of the functional split between RAN and CN

Editor’s Note 6: Further details regarding AIoT functions hosted in the AIoT CN and the respective functional split to be decided by RAN2, RAN3 and SA2.

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 1 it may only represent a single interface instance, e.g. a new interface between AIoT RAS and AIoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between AIoT CN and AIoT RAS, or one instance for NG alone.

Common reader function: a function that communicates with the AIoT device by means of AIoT radio.

Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2.

Editor’s Note 9: Further functionalities in the AIoT RAS are FFS e.g., the radio resource allocation functions and AIoT message routing functions.

AIoT RAN node: A RAN node providing AIoT radio and connecting with an AIoT-aware CN node via the XX interface.

AIoT CN node: A CN node with AIoT functionalities.

AIoT enabled gNB: A gNB with AIoT functionalities.

UE Reader: A UE (acting as an intermediate node) which provides AIoT radio to the AIoT device and connects with a gNB (may be an AIoT enhanced gNB) via NR Uu interface.

XX interface: interface between the AIoT RAN node and the AIoT CN node on which certain AIoT specific functions are performed.

YY interface: interface between the UE Reader and the AIoT CN node

Editor’s Note 10: FFS details of YY interface e.g., whether YY interface can use a control plane protocol (e.g., NAS protocol) or be carried over user plane (e.g., via a PDU session)

Figure 6.3-1: Logical System Architecture for AIoT for Topology 1

Figure 6.3-2: Logical System Architecture for AIoT for Topology 2

### 6.3.2	Interfaces used for Topology 1

There are 2 options for AIoT RAN node to communicate with AIoT CN node in T1:

- T1-Option 1 (Indirect transport solution): AIoT RAN node indirectly (via the AMF) communicates with the AIoT CN node.

- T1-Option 2 (Direct transport solution): AIoT RAN node directly communicates with the AIoT CN node.

#### 6.3.2.1	Evaluation

### Table 1 below compares the two options:

| - T1-Option 1 (Indirect transport solution) | - T1-Option 2 (Direct transport solution) |
| --- | --- |
| - NRPPa-like solution - SCTP-based stack between AIoT RAN node and AMF. - HTTP-based stack between AMF and AIoT CN node. - Different protocol stack at AIoT RAN node and AIoT CN node | - 5GC SBA like solution - HTTP-based stack at AIoT RAN node and AIoT CN node. - Common protocol stack at AIoT RAN node and AIoT CN node |
| - (Potentially) Two new protocols need to be defined. - Protocol b/w AIoT RAN node and AIoT CN node - (i.e., R-AP) - Protocol b/w AIoT RAN node and AMF - (i.e., XXAP) if NGAP is not reused | - Only 1 new protocol needs to be defined. - Protocol between AIoT RAN node and AIoT CN node (i.e., R-AP). |
| - (Potentially) Two new interfaces need to be defined. - Interface between AIoT RAN node and AMF (XX) if NGAP is not reused - Interface between AMF and AIoT CN node (Nac) | - Only 1 interface needs to be defined. - Interface between the AIoT RAN node and AIoT CN node (i.e., Nrc) |
| - Need upgrades to legacy AMF. | - No impact to legacy AMF. |
| - Less developer/cloud friendly - Uses point-to-point interface between AIoT RAN node and AMF (Nra interface) and service-based interface between AMF and AIoT CN node. | - More developer/cloud friendly - Uses Service Based Interface (SBI) between AIoT RAN node and AIoT CN node which is more developer/cloud friendly, RAT agnostic compared to a point-to-point interface. |
| - More backhaul latency due to involvement of AMF - AMF is primarily used as a forwarding entity (“router”) for forwarding packets between AIoT RAN node and AIoT CN node. Potentially unnecessary involvement of AMF. | - Less backhaul latency (no involvement of AMF) |

#### 6.3.2.2	Conclusion

- RAN3 recommends that NGAP is not reused for AIoT RAN node to communicate with the AIoT CN node in Topology 1

- RAN3 recommends that a new radio network layer protocol “R-AP (Reader-Application Protocol)” is defined for AIoT RAN node to directly communicate with the AIoT CN node in Topology 1

### 6.3.3	Interfaces used in Topology 2

In Topology 2, there can be 3 options for communication between the UE Reader and AIoT CN node.

T2-Option 1 (RRC-based solution): UE Reader uses its RRC connection with the gNB to send/receive AIoT messages explicitly in RRC. gNB then forwards the AIoT messages to the AIoT CN node explicitly over N2. No end-to-end protocol between the UE reader and the AIoT CN node

T2-Option 2 (NAS-based solution): UE Reader uses its NAS association with the AMF to send/receive AIoT messages as NAS PDUs. AMF can then forward the AIoT messages to the AIoT CN node via SBI interfaces in the core. In this solution, the AIoT enabled gNB is unaware of the contents of the AIoT messages as it is received as a NAS container.

T2-Option 3 (PDU session-based solution): UE Reader uses user plane signaling (establishes a PDU session till UPF) to communicate with AIoT CN node. UPF can then forward the AIoT messages to the AIoT CN node via SBI interfaces in the core. In this solution, the AIoT enabled gNB is unaware of the contents of the AIoT messages as it is sent over user plane.

#### 6.3.3.1	Evaluation

Table 2 below compares the three options:

| - RRC-based solution | - NAS-based solution | - PDU session-based solution |
| --- | --- | --- |
| - UE Reader uses its RRC connection with the gNB to send/receive AIoT messages explicitly in RRC. - gNB then decodes the AIoT RRC messages and forwards the AIoT messages to the AIoT CN node explicitly over N2. | - UE Reader uses its NAS association with the AMF to send/receive AIoT messages as NAS PDUs. - AMF can then forward the AIoT messages to the AIoT CN node via SBI interfaces in the core. | - UE Reader uses user plane signaling (establishes a PDU session till UPF) to communicate with AIoT CN node. - UPF can then forward the AIoT messages to the AIoT CN node via SBI interfaces in the core. |
| - AIoT enabled gNB is aware of the contents of the AIoT messages as it is received explicitly over RRC. However, it adds unnecessary RRC ASN.1 complexity. | - AIoT enabled gNB is unaware of the contents of the AIoT messages as it is received as a NAS container | - AIoT enabled gNB is unaware of the contents of the AIoT messages as it is sent over user plane |
| - No end-to-end protocol between the UE reader and the AIoT CN node | - There is an end-to-end protocol (R-AP) between the UE reader and the AIoT CN node. | - There is an end-to-end protocol (R-AP) between the UE reader and the AIoT CN node |
| - RRM like solution - This is like how gNB can explicitly request RRM measurements to UE explicitly via RRC | - LPP-like solution - This is like how LPP is defined between UE and LMF and carried over NAS/NGAP/RRC | - UPL like solution - This is like how UPL (User Plane Location Protocol) is used to carry location between a UE and location server |
| - UE Reader selection can be performed by gNB or AIoT CN node | - UE Reader selection performed by AIoT CN node | - UE Reader selection performed by AIoT CN node |
| - Overload on AMF if there are lot of AIoT devices | - Overload on AMF if there are lot of AIoT devices | - UPF can handle IP traffic better. |
| - RAN2 impact: High - RAN2 would need to define new RRC signaling to support the reception/transmission of AIoT related messages from gNB to UE Reader - RAN2 would need to define procedures when the UE Reader’s serving gNB changes (e.g., due to mobility) or RRC state changes | - RAN2 impact: No/minimal | - RAN2 impacts: No/minimal |
| - RAN3 impact: High - RAN3 would need to define explicit signaling over N2 for the AIoT messages - In case the serving gNB of the UE Reader changes (e.g., due to mobility) or the RRC state of the UE reader changes, - Xn coordination is needed to provide UE Reader’s context from source gNB to target gNB | - RAN3 impact: No/minimal - No new procedures needed when the UE Reader’s serving gNB changes or the UE Reader changes its RRC state | - RAN3 impact: No/minimal - No new procedures needed when the UE Reader’s serving gNB changes or the UE Reader changes its RRC state |
| - SA2 impact: Medium - SA2 needs to define the interface between AMF and AIoT CN node (Nac interface) | - SA2 impact: Medium - SA2 needs to define the interface between AMF and AIoT CN node - (Nac interface) and support the end-to-end R-AP protocol | - SA2 impact: Medium - SA2 needs to define the interface between UPF and AIoT CN node (Nuc interface) and support the end-to-end R-AP protocol |
| - Non-homogeneous control plane transport - RRC between UE Reader and gNB - N2 between gNB and AMF - API between AMF and AIoT CN node | - Non-homogeneous control plane transport - NAS between UE Reader and AMF - API between AMF and AIoT CN node | - Homogenous transport: End to-end IP - IP connectivity (PDU session) between UE Reader and UPF - IP connectivity between UPF and AIoT CN node |
| - Needs support of RRC at UE Reader (already supported by UEs today) | - Needs support of NAS at UE Reader (already supported by UEs today) | - Needs support of HTTP client at UE Reader (already supported by UEs today) |
| - Fragmented protocol stacks for Reader in T1 and T2 - AIoT RAN node in T1 will have R-AP over IP - UE Reader in T2 will receive signaling over RRC | - Fragmented protocol stacks for Reader in T1 and T2 - AIoT RAN node in T1 will have R-AP over IP - UE Reader in T2 will have R-AP over NAS | - Can support common SBI protocol stack for Reader in T1 and T2 - AIoT RAN node in T1 will have R-AP over IP - UE Reader in T2 will have R-AP over IP |

#### 6.3.2.2	Conclusion

- RAN3 recommends that a PDU session-based solution (T2-Option 3) is recommended for UE Reader to communicate with the AIoT CN node in Topology 2 i.e., the UE Reader establishes a PDU session to send AIoT data to UPF which forwards it to the AIoT CN node.

### 6.3.3	Unified logical system architecture for T1 and T2

### To unify the two topologies, a new

logical node called “Reader” is defined that supports an AIoT radio protocol stack (referred to as AIoT AS layer in the below Figure) to communicate with an AIoT device over the AIoT radio interface

Reader supports a radio network layer protocol “R-AP (Reader-Application Protocol)” to communicate with the AIoT CN node to receive AIoT related signaling (e.g., Inventory/Command) over the XX interface.

R-AP protocol is carried over SBI protocol stack (HTTP/2/TLS/TCP/IP/L2/L1).

This unified architecture can be used to realize both topologies. Reader may be collocated with the AIoT RAN node in Topology 1 and with a UE in Topology 2 as shown belpw. In Topology 2, L2/L1 of the Reader uses the collocated UE’s Uu connectivity with the gNB.