3GPP TSG-RAN3 Meeting #125	R3-244428

Maastricht, Netherlands, Aug 19th-23rd, 2024

Agenda Item:	16.3

Source: 	Qualcomm Incorporated

Title: 	Inventory, Command and device context management for AIoT

Document for:	Discussion/Decision

# Background

In this paper, we provide our views on AIoT related procedures e.g., Inventory, Command and device context management based on the progress and open issues identified in the previous WG meetings.

# Discussion

## 2.1 General

In the draft TP by RAN2, the following definitions for Inventory and Command are used:

Inventory: The service provided by the network to discover and acquire the identifier of A-IoT device(s).

Command: The service provided by the network to send the operation instruction to the A-IoT device (e.g. read, write, etc.).

Further the following figure and text is used in the RAN2 TP to describe the steps for different AIoT procedures:

The overall AS procedures can be formulated as:

- Step A: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message indicating device(s) that need to respond.

NOTE 1:	In the sub-clause 6.2, the term of “A-IoT paging message” is equal to the “(initial) trigger message”. For simplification, only the former is used.

- Step B: D2R data transmission. Triggered A-IoT device(s) perform the device ID transmission via the A-IoT random access procedure or without using the A-IoT random access procedure. See sub-clause 6.2.4.

- Step C1: Possible R2D data transmission (e.g. for sending the command).

- Step C2: Possible D2R data transmission (e.g. the corresponding response to command).

There is an ongoing discussion in RAN3 (and in SA2) on the architecture, Reader’s protocol stack, interfaces used between the Reader and AIoT CN node, data transport (control plane vs. user plane).

In our view, RAN3 should study the use cases identified by RAN2 and focus on the contents of the Inventory/Command messages exchanged between the Reader and the AIoT CN node agnostic of the interface/architecture/transport discussion. We therefore have the following proposal:

RAN3 should study the contents of Inventory/Command related messages between the Reader and AIoT CN node for both Topology 1 and 2, agnostic of the interface/architecture/transport discussion.

To be in alignment with RAN2, RAN3 should study the following use cases and identify potential impact to Reader-CN interfaces:

- Inventory-only

- Command-only (with Device ID)

- Command-only (without Device ID)

- Inventory and Command (i.e., Inventory followed by Command)

## 2.2 Inventory-only

In the RAN2 TP, the following is captured for the “inventory-only” use case:

For the detailed use case of “inventory-only”, it is supported by the procedure with step A and step B as baseline.

- Step A: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message indicating device(s) that need to respond.

- Step B: D2R data transmission. Triggered A-IoT device(s) perform the device ID transmission via the A-IoT random access procedure or without using the A-IoT random access procedure. See sub-clause 6.2.4.

SA2 is discussing aspects such as how the Application Function (AF) initiates the inventory process and what kind of information it provides to the AIoT CN node. For example, AF may  provide the following:

- Filter Criteria (e.g., product category, area) to limit the inventory to AIoT Devices matching those criteria (e.g., only Pepsi cans in a warehouse).

- a periodicity value to request the inventory to be performed periodically.

- a list of reader IDs to limit the inventory to specific Readers.

AIoT CN node might limit the inventory to specific Readers (if AF requests it) or can select Readers on its own. Irrespective of that, once the Reader(s) are selected, the AIoT CN node sends the Inventory Request to those Reader(s).

Each Reader that received the Inventory Request from the AIoT Controller performs the Inventory procedure towards the AIoT device. The Readers then collects the Device IDs and provides the Device IDs to the AIoT Controller. The Readers may further provide some enrichment data e.g., the Reader ID to the AIoT CN node.

Further, in our view the Readers should be able to either perform a one-time inventory or perform the inventory periodically (if a periodicity value is received from the AIoT CN node).

The inventory-only procedure call flow is captured in the Figure 1 below.

Figure 1: Inventory-only procedure

Based on the discussion above, we have the following proposal:

Capture Figure 1 in the TP describing the Inventory-only procedure

In the Inventory Request sent by the AIoT CN node to the Reader, AIoT CN node can provide a Periodicity value with which the Reader can trigger inventory towards the AIoT device(s)

In the Inventory Request sent by the AIoT CN node to the Reader, AIoT CN node can provide assistance information to the Reader e.g., expected response from AIoT device(s) to assist in resource scheduling

In the Inventory Report sent by the Reader to the AIoT CN node, the Reader can also include Enrichment data (e.g., the Reader ID) in addition to the device ID(s)

## 2.3 Command-only (with Device ID)

In the RAN2 TP, the following is captured for the “command-only” use case:

For the detailed use case of “command-only”:

-	It can be also supported by the baseline procedure with step A, step B, step C1 and step C2.

- Step A: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message indicating device(s) that need to respond.

  - NOTE 1:	In the sub-clause 6.2, the term of “A-IoT paging message” is equal to the “(initial) trigger message”. For simplification, only the former is used.

- Step B: D2R data transmission. Triggered A-IoT device(s) perform the device ID transmission via the A-IoT random access procedure or without using the A-IoT random access procedure.

- Step C1: Possible R2D data transmission (e.g. for sending the command).

- Step C2: Possible D2R data transmission (e.g. the corresponding response to command).

As can be seen from here in the RAN2 TP on the Command-only use case, the Command can also be sent towards a group of devices(s) e.g., an AF might want to read water usage from all water meters in a certain area. Therefore, we propose RAN3 to also study the use case when Command is sent towards a group of devices as well in addition to targeting a single device. Hence, the following is proposed:

RAN3 agrees to study the use case where Command is sent towards a group of devices meeting a certain filter criterion (and not just targeting a single device)

Upon receiving the Command from the AF, the AIoT CN node should determine the list of Readers to use for sending the Command taking the Reader IDs (if provided by the AF) and any stored information (e.g., about the last known Reader for specific Device IDs) into account.

The AIoT CN node then should provide the Command and the Device IDs or Filter Criteria (whichever has been provided by the AF) to the Readers.

Each Reader that receives the Command request from the AIoT CN node, sends the Command to the AIoT Device(s) e.g., in step 8 after AIoT paging and an AIoT random access procedure. The AIoT Device(s) that respond to the Reader provide a Command Response which the Reader forwards it to the AIoT CN node. The Readers may optionally include Enrichment Data. The above procedure is captured in Figure 2.

Figure 2: Command-only procedure (with Device ID)

Capture Figure 2 in the TP describing the Command-only procedure (with Device ID)

Upon receiving Command from AF, AIoT CN node sends at least the following to the Reader in a “Command Request” message:

- Command (e.g., Read/Write)

- Filter Criteria (e.g., device identifier(s) or a product code for which the Command is to be sent)

- Assistance info e.g., expected packet response

Upon receiving Command Response(s) from the AIoT device(s), the Reader sends at least the following to the AIoT CN node in a “Command Report” message:

  - Command Response(s)

  - Enrichment data (e.g., Reader ID, signal strength)

Multiple Command Response(s) can be provided by the Reader to the AIoT CN via a single Command Report.

## 2.4 Command-only (without Device ID)

In the RAN2 TP, the following is also captured as another candidate solution for the “command-only” use case:

- For the detailed use case of “command-only”:

-	In addition, another candidate to support this use case is following, whose feasibility still depends on the conclusion from [SA2 TR 23.700-13] and [SA3 TR 33.713]:

-	Step A’: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message including the command, indicating device(s) to process/respond the command.

-	Step C2: Possible D2R data transmission (e.g. the device ID or the corresponding response to command), via the A-IoT random access procedure or without using the A-IoT random access procedure.

- The above Command-only procedure (without Device ID) follows exactly the same as the Command-only procedure (with Device ID) in section 2.3 expect some differences in the R2D/D2R communication (steps 4-9 in section 2.3 vs. steps 4-7 in section 2.4). There is no difference in the two procedures from the Reader-CN interface perspective. The following figure captures the Command-only procedure (without Device ID):

Figure 3: Command-only procedure (without Device ID)

Capture Figure 3 in the TP describing the Command-only procedure (without Device ID)

## 2.5 Inventory and Command

In the RAN2 TP, the following is captured for the “Inventory and Command” use case:

For the detailed use case of “inventory and command”, it is supported by the procedure with step A, step B, step C1 and step C2, as baseline.

NOTE 2:	For the use case of “inventory and command”, it does not imply that the A-IoT paging message includes both the inventory and command and it does not imply the inventory and command are received by the reader at the same time from upper layer.

- Step A: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message indicating device(s) that need to respond.

NOTE 1:	In the sub-clause 6.2, the term of “A-IoT paging message” is equal to the “(initial) trigger message”. For simplification, only the former is used.

- Step B: D2R data transmission. Triggered A-IoT device(s) perform the device ID transmission via the A-IoT random access procedure or without using the A-IoT random access procedure. See sub-clause 6.2.4.

- Step C1: Possible R2D data transmission (e.g. for sending the command).

- Step C2: Possible D2R data transmission (e.g. the corresponding response to command).

The inventory and command use case is just a back-to-back inventory and command procedure. So the procedure simply follows the inventory-only procedure in section 2.2 followed by command procedure in section 2.3 or 2.4. The figure below shows the “inventory and command” procedure without drawing all the steps again and simply refers to the steps in the inventory-only and command-only procedure.

Figure 4: Inventory and Command

Capture Figure 4 in the TP describing the “Inventory and Command”  use case

## 2.6 Device context management

In current NR specifications, UE context management refers to procedures related to setup/release/modification of UE context at the NG-RAN and AMF during state transitions and during mobility.

Once there is a RRC connection established between a UE and an NG-RAN, the NG-RAN becomes aware of the UE AS context (e.g., L2 configurations), AS security context, bearer context (e.g., DRB/SRB) and maintains them as long as the UE is in RRC_CONNECTED (some of them in RRC_INACTIVE as well). NG-RAN further will allocate some UE identifiers (e.g., the C-RNTI, RAN UE NGAP ID) to send UE-specific signaling over different interfaces (over RRC, NGAP).

Once there is a NAS connection established between a UE and an AMF, the AMF becomes aware of the UE NAS-MM context, NAS security context and maintains it until the UE detaches or deregisters. AMF further will allocate some UE identifiers (e.g., the AMF UE NGAP ID) to send UE-specific signaling over NGAP to NG-RAN.

In NR, UE context included several things such as:

- UE identifiers over air interface (e.g., C-RNTI)

- UE identifiers over backhaul interface (e.g., AMF UE NGAP ID or RAN UE NGAP ID to uniquely the identify the UE over the NG interface)

- AS configurations

- AS/NAS security context

- Radio Bearer context

- PDU session context

In NR, UE context is established/released depending on the RRC/NAS CM state.

- UE context is established at NG-RAN and AMF upon UE establishing an RRC/NAS connection.

- UE context is released at NG-RAN upon UE going to RRC_IDLE.

- UE context is maintained at AMF (for both CM_IDLE/CM_CONNECTED states) even if UE goes to RRC_IDLE and is maintained unless UE deregisters or detaches.

For AIoT, it is not yet clear what the “device context” would look like and whether if it even needs to be maintained/managed.

We therefore think RAN3 should study what is meant by AIoT device context, whether there are benefits in maintaining the AIoT device context (or a subset of it), which entity (Reader or AIoT CN node) should maintain the AIoT device context, when should the device context be established/released/modified etc.

RAN3 should study the following:

- What is AIoT device context?

- Whether there are benefits in maintaining the AIoT device context (or a subset of it)?

- Which entity (Reader or AIoT CN node or both) should maintain the AIoT device context?

- When should the device context be established/released/modified etc.?

Further we think RAN3 should study what is Reader context and whether it needs to be maintained along with the device context.

Reader context can include any information related to the Reader e.g.., the Reader’s identifier, Reader’s configurations used for AIoT, association between Reader and devices.

As discussed in section 2.2, during inventory, the Reader provides the device ID(s) and possibly enrichment data (e.g., Reader ID) to the AIoT CN node.

It might be beneficial to store the Reader-Device association in the AIoT CN node so that the AIoT CN node use this knowledge in the future e.g., for Reader selection. For example, upon receiving a Command to a specific device, the AIoT CN node can use the last associated Reader to reach the device instead of blindly selecting multiple readers to send the Command.

Capture in TR that “it might be beneficial to store the Reader-Device association (e.g., AIoT devices inventoried by the Reader) in the AIoT CN node so that the AIoT CN node use this knowledge in the future e.g., for Reader selection while sending subsequent Commands”.

We summarize all the discussion above in Annex as a TP to 38.769 and we propose to agree it.

Agree the TP to 38.769 in Annex.

# Summary

General

- Proposal 1: RAN3 should study the contents of Inventory/Command related messages between the Reader and AIoT CN node for both Topology 1 and 2, agnostic of the interface/architecture/transport discussion.

- Proposal 2: To be in alignment with RAN2, RAN3 should also study the following use cases and identify potential impact to Reader-CN interfaces:

- Inventory-only

- Command-only (with Device ID)

- Command-only (without Device ID)

- Inventory and Command (i.e., Inventory followed by Command)

Inventory

- Proposal 3: Capture Figure 1 in the TP describing the Inventory-only procedure

- Proposal 4: In the Inventory Request sent by the AIoT CN node to the Reader, AIoT CN node can provide a Periodicity value with which the Reader can trigger inventory towards the AIoT device(s)

- Proposal 5: In the Inventory Request sent by the AIoT CN node to the Reader, AIoT CN node can provide assistance information to the Reader e.g., expected response from AIoT device(s) to assist in resource scheduling

- Proposal 6: In the Inventory Report sent by the Reader to the AIoT CN node, the Reader can include some Enrichment data (e.g., the Reader ID) in addition to the device ID(s)

Command-only (with Device ID)

- Proposal 7: RAN3 agrees to study the use case where Command is sent towards a group of devices meeting a certain filter criterion (and not just targeting a single device)

- Proposal 8: Capture Figure 2 in the TP describing the Command-only procedure (with Device ID)

- Proposal 9: Upon receiving Command from AF, AIoT CN node sends at least the following to the Reader in a “Command Request” message:

- Command (e.g., Read/Write)

- Filter Criteria (e.g., device identifier(s) or a product code for which the Command is to be sent)

- Assistance info (e.g., expected packet response)

- Proposal 10: Upon receiving Command Response(s) from the AIoT device(s), the Reader sends at least the following to the AIoT CN node in a “Command Report” message:

  - Command Response(s)

  - Enrichment data (e.g., Reader ID, signal strength)

- Proposal 11: Multiple Command Response(s) can be provided by the Reader to the AIoT CN via a single Command Report.

Command-only (without Device ID)

- Proposal 12: Capture Figure 3 in the TP describing the Command-only procedure (without Device ID)

Inventory and Command

- Proposal 13: Capture Figure 4 in the TP describing the “Inventory and Command” use case

Device context management

- Proposal 14: RAN3 should study the following:

- What is AIoT device context?

- Whether there are benefits in maintaining the AIoT device context (or a subset of it)?

- Which entity (Reader or AIoT CN node or both) should maintain the AIoT device context?

- When should the device context be established/released/modified etc.?

- Proposal 15: Capture in TR that “it might be beneficial to store the Reader-Device association in the AIoT CN node so that the AIoT CN node use this knowledge in the future e.g., for Reader selection”.

Proposal 16: Agree the TP to 38.769 in Annex.

# 4 Annex: TP to 38.769

## 6.4	Impacts on CN-Reader interface

Editor’s note: Corresponds to the first RAN3 objective in the SID, to identify necessary impacts on signaling and procedures for CN-RAN interface.

### 6.4.1	Information exchanged between AIoT CN node and Reader

Editor’s note 1: The information identified to be exchanged between the AIoT CN node and the Reader is common for both topologies, unless stated otherwise.

Editor’s note 2: This section does not preclude the involvement of any intermediate nodes (e.g., AIoT-enabled gNB in case of T2) when the information is exchanged between the AIoT CN node and Reader. The final decision on the involvement of intermediate nodes depends on the transport solution adopted.

#### 6.4.1.1	Inventory

Inventory procedure is used when the AIoT CN node wants to discover and acquire the identifier of A-IoT device(s) meeting certain criteria.

The Inventory Request from the AIoT CN to the Reader, may include the following:

(1)	AIoT Device Identification (to find a single device, a group of devices, or all devices)

Note 1:	The definition of this identification is out of RAN3 scope.

Editor’s Note 1: It is FFS whether the Reader needs to interpret/store/process it.

(2)	Scope of inventory request (e.g., a certain area in which the inventory is to be triggered)

(3) Filter Criteria (e.g., a certain type of product code e.g., EPC ID)

(4) Assistance information e.g., the periodicity with which the Reader can perform inventory towards the AIoT device(s), expected response from the AIoT device(s).

Multiple individual AIoT Device IDs (one ID per device) can be provided by the Reader to the AIoT CN via a single Inventory Report.

Editor’s Note 2: It is up to SA2 whether device ID is sent transparent or not.

In the Inventory Report, the Reader(s) may further provide some enrichment data (e.g., the Reader ID) to the AIoT CN node. The AIoT CN node can store the association between the Reader ID and device ID(s) for subsequent AIoT related signaling.

The figure below depicts a call flow for the Inventory-only use case.

Figure 1: Inventory-only use case

#### 6.4.1.2	Command

Command is a service provided by the AIoT CN node to send an operation instruction (e.g. read, write, etc.) to one or more A-IoT device(s) meeting a certain filter criterion.

In the Command Request sent from AIoT CN node to the Reader, the following may be included:

(1) Command (e.g., Read/Write/Disable)

(2) Filter Criteria (e.g., a certain area or certain product types for which Command is to be sent)

(3) Device identifier (e.g., if Command is sent to a specific A-IoT device)

(4) Assistance information e.g., expected response size

Each Reader that receives the Command request from the AIoT CN node sends the Command to the AIoT Device(s).

After receiving Command Response(s) from the AIoT device(s), the Reader sends at least the following to the AIoT CN node in a “Command Report” message:

(1) Command Response(s)

(2) Enrichment data (e.g., Reader ID, signal strength)

Multiple Command Response(s) can be provided by the Reader to the AIoT CN via a single Command Report.

Editor’s Note 2: it is FFS whether AIoT RAS can remain agnostic of the type of request from the AIoT CN (need to differentiate command and inventory)

Command-only use case can be performed with or without Device ID as shown in Figure 2 and 3 respectively.

##### 6.4.1.2.1 Command-only with Device ID

Figure 2: Command-only procedure (with Device ID)

##### 6.4.1.2.2 Command-only without Device ID

Figure 3: Command-only procedure (without Device ID)

#### 6.4.1.3	Inventory and Command

Figure 4: Inventory and Command

### 6.3.3	Device context management

RAN3 will study the following:

- What is AIoT device context e.g, whether it includes AIoT device identifiers, AIoT radio configurations, security context, bearer context.

- Whether there are benefits in maintaining the AIoT device context (or a subset of it)

- Which entity (Reader or AIoT CN node or both) should maintain the AIoT device context

- When should the device context be established/released/modified etc.?

It might be beneficial to store the Reader-Device association in the AIoT CN node so that the AIoT CN node use this knowledge in the future e.g., for Reader selection.