3GPP TSG-RAN3 Meeting #125	R3-244430

Maastricht, Netherlands, Aug 19th -23rd, 2024

Agenda Item:	10.2

Source: 	Qualcomm Incorporated

Title: 	MRO enhancements for R18 mobility mechanisms

Document for:	Discussion/Decision

# Background

In this paper, we discuss potential MRO enhancements for R18 mobility mechanisms for the following objective in Rel-19 SON/MDT WI based on the open issues and agreements last meeting.

MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

# Discussion

## MRO for LTM

### LTM connection failure definitions and detection mechanisms

RAN2 agreements:

For LTM MRO, RAN2 considers the following three connection failure cases: Too late LTM, Too early LTM, LTM to wrong cell

RAN2 will start work on MCG LTM.

RAN2 considers SHR, RA report and RLF for MCG LTM SON.

RAN3 agreements:

Work on scenarios for the differentiation of too early LTM, too late LTM and LTM to wrong cell.

Work on scenarios of near failure LTM.

For stage-2, FFS if existing problem definitions (Connection failure due to intra-system mobility) can be reused or if a new chapter needs to be created.

Previously, both RAN2 and RAN3 agreed to consider the 3 LTM connection failure cases: Too late LTM, Too early LTM and LTM to wrong cell.

Rel-16/Rel-17 already defined definitions for different MRO scenarios for L3 HO (including CHO) e.g., Too Late Handover, Too Early Handover, Handover to Wrong Cell.  For LTM, we think we can reuse the existing definitions and add clarifications wherever applicable instead of defining new failure types for LTM to avoid duplication in stage-2 as much as possible. An example TP is shown below.

15.5.2.2.2            Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-     Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-     Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-     Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM Execution, Too Early LTM Execution and LTM Execution to Wrong Cell.

We therefore have the following proposal:

Proposal 1: To simplify stage-2, reuse the existing definitions of Intra-system Too Late Handover, Intra-system Too Early Handover and Intra-system Handover to Wrong Cell for Too Late LTM, Too early LTM and LTM to Wrong Cell respectively with just the below clarification:

- In case of LTM, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM Execution, Too Early LTM Execution and LTM Execution to Wrong Cell.

Further, RAN2 agreed last meeting to extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

With this enhancement, a gNB can distinguish L3 HO failures from LTM failures. A gNB-CU receiving such enhanced RLF Report from UE (with lastHO-Type = LTM) can detect that the handover failure is an LTM failure

Further, RAN2 agreed to enhance RLF Report with cell identifiers (e.g., the LTM recovery cell ID) and timers (e.g., similar to timeConnFailure for LTM) as shown below:

Log the LTM recovery cell ID in RLF Report upon performing recovery on a LTM candidate cell. Details TBD e.g., which field.

Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures, details TBD.

However, the discussion on required cell identifiers and timers for LTM failures is not yet complete and that is the focus of our proposals below.

For L3 HO, UE reports a timer (e.g., time elapsed since last HO initialization until connection failure or time elapsed since CHO execution until connection failure) which is compared against an OAM configured threshold Tstore_UE_cntxt to check if there has been an RLF shortly after a successful HO/CHO execution.

We think a similar timer needs to be reported by the UE to detect if there was a RLF shortly after a successful LTM execution and therefore the following is proposed:

Proposal 2: To detect RLF shortly after a successful LTM execution (in case of Too early LTM and LTM to wrong cell), UE can report Time since LTM execution until RLF to gNB in RLF Report.

Also, for L3 HO, UE reports the identifiers of cells (e.g, source/target cell, failed cell, reestablished cell, reconnect cell, CHO recovery cell) in RLF Report which is used by the gNB to detect the HO failure type. We think similar cell identifiers should be reported for LTM as well and therefore the following is proposed:

Proposal 3: Similar to L3 HO, UE can report the following cell identifiers in RLF Report:

- Previous cell: Source cell of the last LTM execution

- Failed cell: Cell in which the RLF is detected or the target cell of the failed LTM execution.

- Reestablished cell: Cell in which the re-establishment attempt was made after LTM failure.

- Reconnect cell: Cell in which the UE comes back to connected after LTM failure and after failing to perform reestablishment.

- LTM Recovery cell: Cell which the UE selected for LTM based recovery.

Regarding detection mechanisms for LTM connection failures, we think we can just modify the detection mechanisms of Intra-system Too Late Handover, Intra-system Too Early Handover and Intra-system Handover to Wrong Cell. An example TP is shown below:

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-     Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO is configured but the CHO execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if LTM is configured but the LTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt)..

-     Intra-system Too Early Handover: there is a recent handover/LTM for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell is the cell that served the UE at the last handover/LTM initialisation or fall back to the source cell configuration in case of DAPS HO.

-     Intra-system Handover to Wrong Cell: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery/the cell UE attempts LTM recovery is neither the cell that served the UE at the last handover/LTM initialisation nor the cell that served the UE where the RLF happened or the cell that the handover/LTM was initialized toward.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO execution until connection failure or the time elapsed since the LTM execution until connection failure.

In line with the above discussion and the above TP, we propose the following:

Proposal 4: To simplify stage-2, reuse the existing text on detection mechanisms of Intra-system Too Late Handover, Intra-system Too Early Handover and Intra-system Handover to Wrong Cell with necessary modifications for LTM e.g., clarify that the UE reported timer refers to the time elapsed since the LTM execution until connection failure in case of LTM.

### Optimizing LTM candidate cells at gNB-CU

For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization.

During LTM preparation, gNB-CU determines the LTM candidate cells (e.g., based on L3 measurements from UE). It was also agreed in last RAN3 meeting that gNB-CU is responsible for optimizing LTM candidate cells. But it is not yet clear how the gNB-CU optimizes the LTM candidate cell list (e.g., based on UE assistance in RLF Report or based on a network-based solution).

In Rel-17, we similarly discussed how to optimize the CHO candidate cell list and there were 2 options discussed - a network-based solution and a UE based solution. After a contentious discussion, both solutions were agreed, albeit for different sub cases:

- UE based solution (for the case of CHO execution failure): UE reports the CHO candidate cell list in RLF Report, in case of CHO execution failure.

- Network based solution (for the case of RLF shortly after a successful CHO execution): Source gNB provides the CHO configuration (which includes CHO candidate cell list and CHO execution conditions) to target gNB in SN STATUS TRANSFER. Target gNB stores it and provides it back to source gNB in HANDOVER REPORT

In Rel-17, the UE based solution was agreed first, but then we realized that the UE releases the CHO configuration after a successful CHO execution and therefore UE based solution can’t work in the case that there is a RLF shortly after the successful CHO execution. Therefore, the network-based solution was also agreed.

Observation 1: To optimize CHO candidate cell list, Rel-17 specified two solutions - a UE based solution (UE reports the CHO candidate cell list in RLF Report) and a network-based solution (source gNB provides the CHO configuration to target gNB in SN STATUS TRANSFER which stores it and provides it back to source gNB in HANDOVER REPORT).

LTM is different from CHO in that we support subsequent LTM and therefore UE stores the LTM configuration even after a successful LTM execution. Also, Rel-18 only supports intra-CU LTM and therefore the gNB-CU can have the LTM candidate cells in its UE context even if there is inter-DU mobility within the gNB-CU.

In Rel-19 MRO for LTM, we should avoid having two solutions (i.e., both UE based and network-based solution) for optimizing LTM candidate cell list. Also existing information e.g., the timeConnFailure and C-RNTI reported in RLF Report can be used to retrieve the UE’s LTM candidate cells at the time of RLF.

We propose to use a network-based solution as follows:

Proposal 5: There is no need for UE to report the LTM candidate cell list (or an indication whether a cell was LTM candidate or not) in RLF Report. Existing information e.g., the timeConnFailure and C-RNTI reported in RLF Report can be used to retrieve the UE’s LTM candidate cells at the time of RLF.

### Optimizing LTM execution at gNB-DU

For failures due to inappropriate cell switch triggering (e.g., wrong cell selection at cell switch, wrong cell switch timing, …),

- Source DU performs optimization

- FFS whether CU or DU performs root cause analysis.

If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

In LTM, a gNB-DU might have selected the wrong target cell for LTM execution and therefore an RLF might have happened. Since gNB-DU determines the target cell to which LTM is executed, it should also be responsible to root cause why it selected a wrong target cell for LTM execution.

Observation 2: gNB-DU determines the target cell to which LTM is executed and the timing of LTM execution (e.g., based on L1 measurements).

As per 38.473, ACCESS and MOBILITY INDICATION sent from gNB-CU to gNB-DU already includes the RLF Report Container, which the gNB-DU can take into account for optimization of mobility parameters. With the enhanced RLF Report in Rel-19 (including LTM related information), gNB-DU can use that information to do root cause inappropriate cell switch triggering as well.

Observation 3: ACCESS and MOBILITY INDICATION sent from gNB-CU to gNB-DU already includes the RLF Report Container and will automatically include the RLF Report enhancements in Rel-19 for LTM. gNB-DU can use this enhanced RLF Report container for LTM related optimizations (e.g., for optimizing inappropriate cell switch triggering)

Further to assist the gNB-DU, gNB-CU can indicate the HO failure type (Too early LTM/Too late LTM/LTM to wrong cell) via F1AP along with the ACCESS and MOBILITY INDICATION.

Proposal 6: gNB-CU is always responsible to identify the HO failure type (Too early/Too late/wrong cell), for both L3 HO and LTM

Proposal 7: In case of LTM failures, gNB-CU can indicate the LTM failure type (Too early LTM, Too late LTM or LTM to wrong cell) via F1AP ACCESS and MOBILITY INDICATION.

From TS 38.300,

The gNB-DU decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state, or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available.

As mentioned above, the gNB-DU can indicate beam information (TCI state ID, UL TCI state ID) in LTM cell switch command MAC CE to the UE.

The UE can use this information during LTM execution e.g., considers the SSB associated to the TCI state indicated by TCI state ID field as the one used for configured uplink grant selection for the initial uplink transmission towards the candidate cell during RACH-less LTM cell switch.

It is possible that the gNB-DU might not have indicated the right TCI state ID and therefore the UE might have used the wrong one thereby resulting in LTM cell switch failure. We therefore propose the following:

Proposal 8: RAN3 should discuss solutions on how to optimize the target beam(s) (indicated via TCI state(s) in LTM cell switch command MAC CE to UE) for LTM execution.

### RACH information in RLF Report

Currently UE includes RACH information (ra-InformationCommon) in RLF Report if connectionFailureType is hof and if the failed handover is an intra-RAT handover. But LTM can be RACH-based or RACH-less. Therefore, the RACH information should be only included in case of RACH-based LTM and in case of RACH-less HO, UE should omit RACH information in RLF Report during RACH-less LTM and further can add an explicit indicator that the LTM was RACH-less.

Proposal 9: UE can include existing RACH information (ra-InformationCommon) in case RLF Report is collected during RACH-based LTM failure.

Proposal 10: UE should omit RACH information (ra-InformationCommon) in case RLF Report is collected during RACH-less LTM failure. UE can further add an explicit indicator that the LTM was RACH-less.

### LTM-CHO coexistence

Suppose CHO and LTM are both configured and there is a LTM failure (or a RLF shortly after a successful LTM). The following recovery mechanisms are possible, if configured by the network:

- UE will perform CHO recovery if the selected cell after RLF is a CHO candidate cell

- UE will perform LTM recovery if the selected cell after RLF is a LTM candidate cell

- It is up to UE on what recovery UE should perform if the selected cell after RLF is both a CHO candidate and LTM candidate cell, then

If no recovery is configured by the network, the UE simply performs cell selection and performs reestablishment on the selected cell

In such a scenario where a UE is configured with both CHO candidate cells and LTM candidate cells and UE encounters an RLF, RAN3 should discuss if any optimizations can be made for such coexistence scenarios.

Therefore, the following is proposed.

Proposal 11: RAN3 should discuss failure scenarios when a UE is configured with both LTM and CHO and if any optimizations can be made for such coexistence scenarios.

### UHI/MHR enhancements for LTM

A UE supporting LTM might undergo both LTM and normal L3 HO scenarios depending on the gNB implementation. A gNB might do a back-to-back L3 HO and LTM due to L3 execution conditions being met in gNB-CU and LTM execution condition met in gNB-DU.

Therefore, it useful for the gNB to know whether the UE’s visited cells were due to LTM or L3 HO. Without such knowledge, the gNB might unnecessarily be trying to optimize ping-pong handovers even though they happened due to different reasons (i.e., L3 HO and LTM)

We therefore propose RAN3 to discuss how to distinguish genuine ping-pong handovers vs. back-to-back L3 HO and LTM. One solution could be to optimize the UHI to include a HO-type (i.e., LTM or L3 HO) similar to how it has been agreed by RAN2 for RLF Report.

Proposal 12: RAN3 should discuss how gNB can distinguish genuine ping-pong handovers vs. back-to-back L3 HO and LTM (e.g., via MHR/UHI enhancements)

Proposal 13: Enhance UHI and/or MHR to include a “HO-type” (i.e., LTM or L3 HO).

## 2.2 MRO for CHO with candidate SCG

RAN2 agreements:

RAN2 to study failure and near failure scenarios for CHO with candidate SCGs.

RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report.

RAN3 agreements:

RAN3 focuses on NR-DC for MRO for CHO with candidate SCG in R19.

RAN3 will start with the failure and near failure scenarios with UEs only configured with CHO with candidate SCGs.

FFS whether to also consider failure and near failure scenarios related with configuration of CHO with candidate SCG(s) and CHO only.

Do not distinguish between fast MCG recovery/no-fast MCG recovery (after the CHO with candidate SCG execution failure)

FFS whether to support concurrent failure cases (RLF + SCG failure). The definition of these cases needs to be further clarified.

In Rel-18, CHO with candidate SCG(s) was specified for NR-DC and the following is captured in 37.340:

- A CHO with candidate SCG(s) is defined as a PCell change with PSCell addition/change that is executed by the UE when the execution conditions for both candidate PCell and the associated candidate PSCell are met.

- The UE starts evaluating the execution conditions for candidate PCell(s) and candidate PSCell(s) simultaneously upon receiving the CHO with candidate SCG(s) configuration

- UE stops evaluating the execution conditions once a PCell change or a PSCell change is triggered.

- The UE does not execute CHO with candidate SCG(s) until the execution conditions for both the candidate PCell and the associated candidate PSCell are met

Further the following is captured in 37.340:

If at least one candidate PCell satisfies the corresponding execution condition and the associated candidate PSCell satisfies the corresponding execution condition, the UE detaches from the source MN, applies the stored corresponding configuration for that selected candidate PCell and the associated candidate PSCell, synchronises to that candidate PCell, and completes the RRC handover procedure by sending RRC reconfiguration complete* message to the target MN.

Else if at least one candidate PCell satisfies the corresponding execution condition and there is no associated execution condition for a candidate PSCell, the UE detaches from the source MN, applies the stored corresponding configuration for that selected candidate PCell and, if included, the associated PSCell, synchronises to that candidate PCell and completes the RRC handover procedure by sending RRC reconfiguration complete* message to the target MN.

Below we consider different scenarios for optimizing CHO with candidate SCG.

Example

- Suppose UE is connected to (PCell A0, PSCell B0)

- UE is configured with the CHO with candidate SCG configurations i.e., (A1, B1), (A1, B2), (A2, B3)

- UE starts evaluating the execution conditions of candidate PCells (A1, A2) and candidate PSCells (B1, B2, B3) simultaneously

Scenario 1: UE encounters an RLF or SCG failure before the CHO with candidate SCG execution and execution conditions of no candidate PCell and no candidate PSCells were met before the RLF or SCG failure

Scenario 2: UE encounters an RLF or SCG failure before the CHO with candidate SCG execution and one execution condition was “blocking” the whole execution e.g.,

- A candidate PCell (A1 in the example below) satisfies the execution condition but none of its associated candidate PSCells (i.e., B1 or B2) satisfy the corresponding execution condition.

- A candidate PSCell (B3 in the example below) satisfies the execution condition but the associated candidate PCell (i.e., A2) doesn’t satisfy the corresponding execution condition.

In scenarios 1 and 2, it might be useful for the network to know why the CHO with candidate SCG execution e.g., it didn’t happen because of one “blocking” execution condition (even though the other execution condition was met) or because none of the execution conditions were met

Scenario 3: UE encounters an RLF or SCG failure before the CHO with candidate SCG execution and non-associated PCells and PSCells met the execution condition

In the example below, Execution condition of candidate PCell A1 and candidate PSCell B3 is met but the CHO with candidate SCG execution didn’t happen because there is no (A1, B3) associated configuration.

It would be beneficial if the network can know what execution conditions were met (i.e., A1 and B3) so that the network can fine-tune the CHO with candidate SCG configuration eg., by including a (A1, B3) configuration or (A2, B3) confguration to avoid RLF or SCG failures in the future.

Considering the above example scenarios, we propose the following:

Proposal 14: RAN3 considers how to optimize the following cases when UE receives a CHO with candidate SCG config but encounters an RLF or SCG failure before the CHO with candidate SCG execution

- Neither any candidate PCell met the CHO execution condition nor any candidate PSCell met the CPC execution condition

- Only one type of execution condition was met (i.e., either the CHO execution condition or CPC execution condition) thereby blocking the complete execution

- A candidate PCell met the CHO execution condition and a candidate PSCell met the CPC execution condition, but they were not associated PCell and PSCell pairs

Proposal 15: Enhance RLF Report with:

- The type of fulfilled execution condition(s) (i.e., CHO execution condition or CPC execution condition or none)

- Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

- Identifier of candidate PSCell(s) which met the configured CPC execution conditions before the RLF is encountered

- Time elapsed between the fulfilled execution and RLF

Similar enhancements to RLF Report can be done for SCGFailureInformation.

Proposal 16: Enhance SCGFailureInformation with:

- The type of fulfilled execution condition(s) (i.e., CHO execution condition or CPC execution condition or none)

- Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

- Identifier of candidate PSCell(s) which met the configured CPC execution conditions before the RLF is encountered

- Time elapsed between the fulfilled execution and RLF

CHO Recovery

When initial CHO execution attempt fails or HO fails, the UE performs cell selection, and if the selected cell is a CHO candidate and if network configured the UE to try CHO after handover/CHO failure, then the UE attempts CHO execution once, otherwise re-establishment is performed.

Suppose a UE receives a CHO with candidate SCG config, both the execution conditions are met but the CHO execution fails (e.g., RACH in target cell fails). UE performs cell selection, and the selected cell is also a CHO candidate cell.

As per the below text from 38.331, UE sets the choCellId (CHO recovery cell ID) in RLF Report only in case CHO recovery is attempted after a legacy CHO failure and not in case of CHO recovery is attempted after a CHO with candidate SCG execution failure.

1>  if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG, except for an LTM cell switch procedure following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or mobility from NR failure, and

1>  if attemptCondReconfig is configured; and

1>  if the selected cell is not configured with CondEventT1, or the selected cell is configured with CondEventT1 and leaving condition has not been fulfilled; and

1>  if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig and the condExecutionCondPSCell is not configured for the corresponding condReconfigId in the MCG VarConditionalReconfig:

2>  if the UE supports RLF-Report for conditional handover, set the choCellId in the VarRLF-Report to the global cell identity, if available, otherwise to the physical cell identity and carrier frequency of the selected cell;

2>  apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3;

NOTE 1:  It is left to network implementation to how to avoid keystream reuse in case of CHO based recovery after a failed handover without key change.

Observation 4: Currently, UE sets the choCellId (CHO recovery cell ID) in RLF Report only in case CHO recovery is attempted after a legacy CHO failure and not in case of CHO recovery is attempted after a CHO with candidate SCG execution failure.

We therefore have the following proposal:

Proposal 17: RAN3 should consider scenarios where UE attempts CHO recovery after a CHO with candidate SCG execution failure.

Proposal 18: UE should log the CHO recovery cell ID in RLF Report even when UE attempts CHO recovery after a CHO with candidate SCG execution failure.

Consider Scenario 2 again where the UE encounters an RLF before the CHO with candidate SCG execution because candidate PSCell (B3 in the example below) satisfies the execution condition but the associated candidate PCell (i.e., A2) doesn’t satisfy the corresponding execution condition.

Suppose the UE attempts CHO recovery and observed that both PCell A2 and PCell A3 are suitable cells, the UE should be able to prioritize selecting PCell A2 for CHO recovery because its associated PSCell’s (B3) execution condition was met before rather than selecting PCell A3 for CHO recovery because its associated PCell’s (B4) execution condition was not met before.

We therefore have the following proposal:

Proposal 19: In case the UE attempts CHO recovery after a CHO with candidate SCG execution failure and observes that there are 2 or more suitable cells, UE may prioritize selecting a cell for CHO recovery based on whether its associated PSCell’s execution condition was met before or not

## 2.3 MRO for Subsequent CPAC (S-CPAC)

RAN3 agreements:

Work on the scenarios of failure in S-CPAC.

The optimization of non-failure scenarios (e.g., near failure and ping-pong) is not excluded.

R19 SON/MDT solution discussion is based on R18 work.

A subsequent CPAC is defined as a conditional PSCell addition or change procedure that is executed after a PSCell addition, a PSCell change, a PCell change or an SCG release based on pre-configured subsequent CPAC configuration of candidate PSCell(s) without reconfiguration and re-initiation of CPC/CPA.

The UE keeps the configured subsequent CPAC configuration (unless the network indicates to release it) and evaluates the execution conditions of candidate PSCells after completion of a PSCell addition, a PSCell change, a PCell change or an SCG release.

Observation 5: UE keeps the configured S-CPAC configuration unless the network indicates to release it

The S-CPAC configuration contains

- candidate SCG configuration(s) of candidate PSCell(s)

- execution conditions

- MCG configuration (to be applied when subsequent CPAC execution is triggered)

- reference configuration

- security update configuration

In case a UE configured with S-CPAC configuration encounters an SCG failure, it sends a SCGFailureInformation to MN (including the measurement results) and waits for MN’s subsequent action e.g., whether it wants UE to add a new PSCell or keep it in standalone mode

In case the MN wants to add a new PSCell for the UE and the new PSCell is an already prepared candidate PSCell whose configuration is provided to the UE as part of S-CPAC configuration, MN should be able to simply indicate the PSCell to add without providing the whole configuration. UE can simply activate the indicated PSCell’s configuration and start evaluating the S-CPAC configuration.

For example, UE encounters an SCG failure in source PSCell A0 and reports in SCGFailureInformation that B4 (an already prepared candidate, but not the immediate candidate) was the strongest cell at the time of SCG failure

We therefore have the following proposals

Proposal 20: RAN3 should discuss solutions how to perform an optimized PSCell addition post an SCG failure for a UE configured with S-CPAC configuration.

Proposal 21: In case the MN wants to add a new PSCell for the UE and the new PSCell is an already prepared candidate PSCell whose configuration is provided to the UE as part of S-CPAC configuration, MN should be able to simply indicate the PSCell to add without providing the whole configuration.

Proposal 22: Upon receiving such an indication to activate a PSCell, UE can simply activate the indicated PSCell’s configuration and start evaluating the S-CPAC configuration.

# Summary

MRO for LTM

LTM connection failure definitions and detection mechanisms

Proposal 1: To simplify stage-2, reuse the existing definitions of Intra-system Too Late Handover, Intra-system Too Early Handover and Intra-system Handover to Wrong Cell for Too Late LTM, Too early LTM and LTM to Wrong Cell respectively with just the below clarification:

- In case of LTM, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM Execution, Too Early LTM Execution and LTM Execution to Wrong Cell.

Proposal 2: To detect RLF shortly after a successful LTM execution (in case of Too early LTM and LTM to wrong cell), UE can report Time since LTM execution until RLF to gNB in RLF Report.

Proposal 3: Similar to L3 HO, UE can report the following cell identifiers in RLF Report:

- Previous cell: Source cell of the last LTM execution

- Failed cell: Cell in which the RLF is detected or the target cell of the failed LTM execution.

- Reestablished cell: Cell in which the re-establishment attempt was made after LTM failure.

- Reconnect cell: Cell in which the UE comes back to connected after LTM failure and after failing to perform reestablishment.

- LTM Recovery cell: Cell which the UE selected for LTM based recovery.

Proposal 4: To simplify stage-2, reuse the existing text on detection mechanisms of Intra-system Too Late Handover, Intra-system Too Early Handover and Intra-system Handover to Wrong Cell with necessary modifications for LTM e.g., clarify that the UE reported timer refers to the time elapsed since the LTM execution until connection failure in case of LTM.

Optimizing LTM candidate cells at gNB-CU

Observation 1: To optimize CHO candidate cell list, Rel-17 specified two solutions - a UE based solution (UE reports the CHO candidate cell list in RLF Report) and a network-based solution (source gNB provides the CHO configuration to target gNB in SN STATUS TRANSFER which stores it and provides it back to source gNB in HANDOVER REPORT).

Proposal 5: There is no need for UE to report the LTM candidate cell list (or an indication whether a cell was LTM candidate or not) in RLF Report. Existing information e.g., the timeConnFailure and C-RNTI reported in RLF Report can be used to retrieve the UE’s LTM candidate cells at the time of RLF.

Optimizing LTM execution at gNB-DU

Observation 2: gNB-DU determines the target cell to which LTM is executed and the timing of LTM execution (e.g., based on L1 measurements).

Observation 3: ACCESS and MOBILITY INDICATION sent from gNB-CU to gNB-DU already includes the RLF Report Container and will automatically include the RLF Report enhancements in Rel-19 for LTM. gNB-DU can use this enhanced RLF Report container for LTM related optimizations (e.g., for optimizing inappropriate cell switch triggering)

Proposal 6: gNB-CU is always responsible to identify the HO failure type (Too early/Too late/wrong cell), for both L3 HO and LTM

Proposal 7: In case of LTM failures, gNB-CU can indicate the LTM failure type (Too early LTM, Too late LTM or LTM to wrong cell) via F1AP ACCESS and MOBILITY INDICATION.

Proposal 8: RAN3 should discuss solutions on how to optimize the target beam(s) (indicated via TCI state(s) in LTM cell switch command MAC CE to UE) for LTM execution.

RACH information in LTM

Proposal 9: UE can include existing RACH information (ra-InformationCommon) in case RLF Report is collected during RACH-based LTM failure.

Proposal 10: UE should omit RACH information (ra-InformationCommon) in case RLF Report is collected during RACH-less LTM failure. UE can further add an explicit indicator that the LTM was RACH-less.

LTM-CHO coexistence

Proposal 11: RAN3 should discuss failure scenarios when a UE is configured with both LTM and CHO and if any optimizations can be made for such coexistence scenarios.

MHI/UHR enhancements for LTM

Proposal 12: RAN3 should discuss how gNB can distinguish genuine ping-pong handovers vs. back-to-back L3 HO and LTM (e.g., via MHR/UHI enhancements)

Proposal 13: Enhance UHI and/or MHR to include a “HO-type” (i.e., LTM or L3 HO).

MRO for CHO with candidate SCG

Proposal 14: RAN3 considers how to optimize the following cases when UE receives a CHO with candidate SCG config but encounters an RLF or SCG failure before the CHO with candidate SCG execution (see Annex 2 which depicts example scenarios)

- Neither any candidate PCell met the CHO execution condition nor any candidate PSCell met the CPC execution condition

- Only one type of execution condition was met (i.e., either the CHO execution condition or CPC execution condition) thereby blocking the complete execution

- A candidate PCell met the CHO execution condition and a candidate PSCell met the CPC execution condition, but they were not associated PCell and PSCell pairs

Proposal 15: Enhance RLF Report with:

- The type of fulfilled execution condition(s) (i.e., CHO execution condition or CPC execution condition or none)

- Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

- Identifier of candidate PSCell(s) which met the configured CPC execution conditions before the RLF is encountered

- Time elapsed between the fulfilled execution and RLF

Proposal 16: Enhance SCGFailureInformation with:

- The type of fulfilled execution condition(s) (i.e., CHO execution condition or CPC execution condition or none)

- Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

- Identifier of candidate PSCell(s) which met the configured CPC execution conditions before the RLF is encountered

- Time elapsed between the fulfilled execution and RLF

Observation 4: Currently, UE sets the choCellId (CHO recovery cell ID) in RLF Report only in case CHO recovery is attempted after a legacy CHO failure and not in case of CHO recovery is attempted after a CHO with candidate SCG execution failure.

Proposal 17: RAN3 should consider scenarios where UE attempts CHO recovery after a CHO with candidate SCG execution failure.

Proposal 18: UE should log the CHO recovery cell ID in RLF Report even when UE attempts CHO recovery after a CHO with candidate SCG execution failure.

Proposal 19: In case the UE attempts CHO recovery after a CHO with candidate SCG execution failure and observes that there are 2 or more suitable cells, UE may prioritize selecting a cell for CHO recovery based on whether its associated PSCell’s execution condition was met before or not

MRO for S-CPAC

Observation 5: UE keeps the configured S-CPAC configuration unless the network indicates to release it

Proposal 20: RAN3 should discuss solutions how to perform an optimized PSCell addition post an SCG failure for a UE configured with S-CPAC configuration.

Proposal 21: In case the MN wants to add a new PSCell for the UE and the new PSCell is an already prepared candidate PSCell whose configuration is provided to the UE as part of S-CPAC configuration, MN should be able to simply indicate the PSCell to add without providing the whole configuration.

Proposal 22: Upon receiving such an indication to activate a PSCell, UE can simply activate the indicated PSCell’s configuration and start evaluating the S-CPAC configuration.

# 4 Annex 1: TP to 38.300

15.5.2.2.2            Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-     Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-     Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-     Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM Execution, Too Early LTM Execution and LTM Execution to Wrong Cell.

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-     Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO is configured but the CHO execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if LTM is configured but the LTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt)..

-     Intra-system Too Early Handover: there is a recent handover/LTM for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell is the cell that served the UE at the last handover/LTM initialisation or fall back to the source cell configuration in case of DAPS HO.

-     Intra-system Handover to Wrong Cell: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery/the cell UE attempts LTM recovery is neither the cell that served the UE at the last handover/LTM initialisation nor the cell that served the UE where the RLF happened or the cell that the handover/LTM was initialized toward.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO execution until connection failure or the time elapsed since the LTM execution until connection failure.

# 4 Annex 2: MRO for CHO with candidate SCG - Scenarios

Scenario 1: UE encounters an RLF or SCG failure before the CHO with candidate SCG execution and execution conditions of no candidate PCell and no candidate PSCells were met before the RLF or SCG failure

Scenario 2: UE encounters an RLF or SCG failure before the CHO with candidate SCG execution and one execution condition was “blocking” the whole execution e.g.,

- A candidate PCell (A1 in the example below) satisfies the execution condition but none of its associated candidate PSCells (i.e., B1 or B2) satisfy the corresponding execution condition.

- A candidate PSCell (B3 in the example below) satisfies the execution condition but the associated candidate PCell (i.e., A2) doesn’t satisfy the corresponding execution condition.

Scenario 3: UE encounters an RLF or SCG failure before the CHO with candidate SCG execution and non-associated PCells and PSCells met the execution condition

In the example below, Execution condition of candidate PCell A1 and candidate PSCell B3 is met but the CHO with candidate SCG execution didn’t happen because there is no (A1, B3) associated configuration.