3GPP TSG-RAN3 Meeting #125	R3-244431

Maastricht, Netherlands, Aug 19th -23rd, 2024

Agenda item:	10.3.1

Source:	Qualcomm Incorporated

Title:	SON MDT for NTN

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss potential SON/MDT enhancements for NTN based on agreements and open issues last meeting.

# Discussion

  - MRO enhancements for NTN

RAN3 agreements so far:

MRO mechanisms for intra-NTN should be addressed based on failure scenarios first.

MRO for intra-NTN mobility shall consider both conditional and non-conditional intra-NTN mobility

MRO for intra-NTN conditional mobility shall consider the following trigger conditions:

- Measurement-based trigger condition

- A time-based trigger condition

- A location-based trigger condition

- A time-based and Measurement-based combined trigger condition

- A location-based and Measurement-based combined trigger condition

Do not consider MRO for feeder link switch.

Work on near failure scenarios (e.g., SHR enhancements) later

### Time-based CHO

The following time-based CHO trigger condition is supported for NTN:

CondEvent T1: Time measured at UE becomes more than configured threshold t1-Threshold but is less than t1-Threshold + duration

In the last meeting, there was a discussion that the time-based CHO parameters in CondEvent T1 (e.g., t1-Threshold and/or duration) might not have been configured properly and thereby resulting in CHO failures. But there was also a point raised that the UE already reports some “relative” timers in RLF Report.

Below we discuss different cases and identify the optimization goals and whether any existing timers can be reused.

Case 1a: RLF before time-based CHO window has started

UE encounters RLF before window for time-based CHO has even started (i.e., UE has not even attempted time-based CHO execution yet)

If the network is able to determine that a lot of RLFs (e.g., 90%) are seen before the T1-Threshold, the network can move the T1-Threshold to the left (i.e., decrease its value) so that the UEs can start time-based CHO execution earlier and thereby reducing the possibility of these RLFs.

Case 1b: RLF after time-based CHO window has started (but before CHO execution)

UE encounters RLF after window for time-based CHO has started but CHO has not been executed

(i.e., UE has not yet executed CHO as it has till t1-Threshold + Duration to execute the CHO)

If the network is able to determine that a lot of RLFs (e.g., 90%) are seen after the T1-Threshold but before the Duration and the CHO is not yet executed, it is perhaps because the Duration is set too large. With this knowledge, the network can perhaps reduce the Duration so that UEs execute CHO more quickly.

There are also other cases where RLF occurs “shortly”: after CHO execution or the time-based CHO execution itself fails.

Case 2a: RLF “shortly” after time-based CHO execution but before T1-Threshold + Duration

Case 2b: RLF “shortly” after time-based CHO execution but after T1-Threshold + Duration

Case 3: Time-based CHO execution failure

Considering the two cases above, we make the following proposal.

Proposal 1: RAN3 should consider the following cases as part of MRO for time-based CHO:

- Case 1a: RLF before time-based CHO window has started (i.e., before T1-Threshold)

- Case 1b: RLF after time-based CHO window has started but before CHO execution

- Case 2a: RLF “shortly” after time-based CHO execution but before T1-Threshold + Duration

- Case 2b: RLF “shortly” after time-based CHO execution but after T1-Threshold + Duration

- Case 3: Time-based CHO execution failure

Proposal 2: In order to optimize T1-Threshold and/or Duration, RAN3 should discuss solutions on how to enable the network to distinguish between case 1a and case 1b

Currently, RLF Report already supports the following timer:

| timeSinceCHO-Reconfig | In case of handover failure, this field is used to indicate the time elapsed between the initiation of the last handover execution towards the target cell and the reception of the latest conditional reconfiguration. In case of radio link failure, this field is used to indicate the time elapsed between the radio link failure and the reception of the latest conditional reconfiguration while connected to the source PCell. |
| --- | --- |

Case 1a and Case 1b is shown below now with the timeSinceCHO-Reconfig also shown:

Also, it is to be noted that the gNB knows the time CHO config is sent to the UE, T1-Threshold and Duration

Therefore, the network can determine from the timeSinceCHO-Reconfig sent in RLF Report whether the RLF encountered by UE is due to case 1a (occurs before T1-Threshold) or case 1b (occurs after the T1-Threshold)

Observation 1: gNB knows the time CHO configuration is sent to the UE and the CHO configuration parameters (T1-Threshold and Duration).

Proposal 3: gNB can distinguish between case 1a and case 1b (i.e., whether an RLF occurred before T1-Threshold or after T1-Threshold) by adding the time it sent CHO reconfig with timeSinceCHO-Reconfig received in RLF Report and comparing it with T1-Threshold.

Also, there were few proposals last meeting to report the absolute timestamp (e.g., timestamp when RLF occurred or timestamp when CHO occurred). But we think reporting the absolute timestamp adds to a lot of overhead and not all UEs might have absolute time sync (e.g., with the UTC time). We therefore propose the following:

Observation 2: Reporting the absolute time stamp e.g., timestamp when RLF occurs adds to a lot of overhead

Proposal 4: There is no need for UE to report any absolute timestamp information in RLF Report

### Location-based CHO

The following location-based CHO trigger conditions are supported for NTN:

CondEvent D1:

Distance between UE and a reference location referenceLocation1 becomes larger than configured threshold distanceThreshFromReference1 and

between UE and a reference location referenceLocation2 of conditional reconfiguration candidate becomes shorter than configured threshold distanceThreshFromReference2

CondEvent D2:

Distance between UE and a moving reference location determined based on movingReferenceLocation and its corresponding satellite ephemeris and epoch time broadcast in SIB19 for the serving cell becomes larger than configured threshold distanceThreshFromReference1 and

distance between UE and a moving reference location determined based on referenceLocation2 of conditional reconfiguration candidate becomes shorter than configured threshold distanceThreshFromReference2

In the last meeting, there were proposals that Location-based CHO parameters in CondEvent D1/D2 (e.g., distanceThreshFromReference1, distanceThreshFromReference2) might not have been configured properly, therefore resulting in a Too early/Too late/wrong cell CHO.

And there were proposals to report the relative distance between UE’s exact location and the reference location/moving reference location of source/target/candidate cell when the connection failure happens (or when CHO execution happens).

We think that the existing location information reported in RLF Report is sufficient and there is no need to report any relative distance(s) to optimize location-based CHO.

Proposal 4: Existing location information reported in RLF Report is sufficient to optimize location-based CHO failures and there is no need for UE to report any relative distance information in RLF Report.

### Optimizing multiple CHO triggers

NTN supports multiple trigger conditions for CHO e.g., the RRM measurement-based event A4, time-based trigger condition and location-based trigger condition.

The following is from 38.300:

Time-based or location-based trigger conditions may be configured independently from the measurement condition for CHO in NTN in at least hard satellite switch case where the service discontinuity gap time length is assumed to be zero or negligible.

However, if the service discontinuity gap time length is not zero or negligible, then a time-based or a location-based trigger condition is always configured together with one of the measurement-based trigger conditions (CHO events A3/A4/A5) as defined in TS 38.331 [12].

It is up to UE implementation how the UE evaluates the time- or location-based trigger condition together with the RRM measurement-based event.

In case multiple trigger conditions are configured for CHO in NTN (e.g., time-based + measurement-based or location-based + measurement-based) for a candidate cell, it might be useful for the network to know if a subset of the trigger conditions were satisfied before an RLF occurred, so that it can fine tune the right CHO triggers. UE can include such information in the RLF Report and report it to the gNB. Therefore, the following is proposed:

Proposal 6: In case a UE is configured with multiple trigger conditions for CHO in NTN (e.g., time-based + measurement-based or location-based + measurement-based) and the UE encounters an RLF before CHO execution, UE can include the CHO trigger condition(s) that were satisfied before the RLF occurred.

  - Logged MDT enhancements for NTN

Logged MDT enhancement based on scenarios to be worked first.

FFS whether to consider any enhancement for MDT configuration e.g., area scope and trigger condition for NTN

Currently, logged MDT is used to collect radio measurements while UE is in RRC_IDLE and RRC_INACTIVE to develop coverage maps. Further, there are specific triggers defined to detect coverage holes e.g., when UE is “Out of Coverage” or to detect when the radio conditions are below a certain threshold.

An NTN might have a different coverage compared to a TN. Especially in initial NTN deployments, there might be discontinuous coverage which are well known in advance. Or an operator might be interested in only collecting NTN coverage or only TN coverage.

Further, there might be enhancements needed in the Area Scope of logged MDT to indicate which neighbour cells should the UE measure while being in Idle/Inactive mode e.g., only the NTN neighbor cells or only TN cells or both NTN+TN cells and how long or when it should measure. In all these cases, there are some enhancements needed to the logged MDT procedure to be able to collect NTN specific information.

We therefore propose the following:

Proposal 7: Area Scope of logged MDT should indicate which neighbour cells the UE should measure while being in Idle/Inactive mode (e.g., only the NTN neighbor cells or only TN cells or both NTN+TN cells)

Proposal 8: RAN3 should discuss how to distinguish true coverage holes vs. known coverage holes (e.g., avoid UE collecting logged MDT in known coverage holes)

# Conclusion

MRO enhancements for NTN

Time-based CHO

Proposal 1: RAN3 should consider the following cases as part of MRO for time-based CHO (see Annex for figures):

- Case 1a: RLF before time-based CHO window has started (i.e., before T1-Threshold)

- Case 1b: RLF after time-based CHO window has started (i.e., after T1-Threshold) but before CHO execution

- Case 2a: RLF “shortly” after time-based CHO execution but before T1-Threshold + Duration

- Case 2b: RLF “shortly” after time-based CHO execution but after T1-Threshold + Duration

- Case 3: Time-based CHO execution failure

Proposal 2: In order to optimize T1-Threshold and/or Duration, RAN3 should discuss solutions on how to enable the network to distinguish between case 1a and case 1b

Observation 1: gNB knows the time a CHO configuration is sent to the UE and the CHO configuration parameters (T1-Threshold and Duration).

Proposal 3: gNB can distinguish between case 1a and case 1b (i.e., whether an RLF occurred before T1-Threshold or after T1-Threshold) by adding the time it sent CHO configuration with timeSinceCHO-Reconfig received in RLF Report and comparing it with T1-Threshold.

Observation 2: Reporting the absolute time stamp e.g., timestamp when RLF occurs adds to a lot of overhead

Proposal 4: There is no need for UE to report any absolute timestamp information in RLF Report

Time-based CHO

Proposal 5: Existing location information reported in RLF Report is sufficient to optimize location-based CHO failures and there is no need for UE to report any relative distance information in RLF Report.

Optimizing multiple CHO triggers

Proposal 6: In case a UE is configured with multiple trigger conditions for CHO in NTN (e.g., time-based + measurement-based or location-based + measurement-based) and the UE encounters an RLF before CHO execution, UE can include the CHO trigger condition(s) that were satisfied before the RLF occurred.

Logged MDT enhancements for NTN

Proposal 7: Area Scope of logged MDT should indicate which neighbour cells the UE should measure while being in Idle/Inactive mode (e.g., only the NTN neighbor cells or only TN cells or both NTN+TN cells)

Proposal 8: RAN3 should discuss how to distinguish true coverage holes vs. known coverage holes (e.g., avoid UE collecting logged MDT in known coverage holes)

# Annex (Time-based CHO scenarios)

Case 1a: RLF before time-based CHO window has started

Case 1b: RLF after time-based CHO window has started but before CHO execution

Case 2a: RLF after time-based CHO execution but before T1-Threshold + Duration

Case 2b: RLF after time-based CHO execution but after T1-Threshold + Duration

Case 3: Time-based CHO execution failure