3GPP TSG-RAN3 Meeting #125	R3-244432

Maastricht, Netherlands, Aug 19th-23rd, 2024

Agenda item:	10.3.2

Source:	Qualcomm Incorporated

Title:	SON MDT for network slicing

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss several aspects related to SON/MDT for network slicing.

# Discussion

  - Slice-aware MRO

RAN3 agreements:

Work on the Slice aware MRO scenarios.

For successful handover, FFS if solutions are needed to optimize handovers in the case of slicing discontinuity, e.g., target cell not supporting slice.

The following is captured in TS 38.300

To make mobility slice-aware in case of Network Slicing, S-NSSAI is introduced as part of the PDU session information that is transferred during mobility signalling. This enables slice-aware admission and congestion control.

Both NG and Xn handovers are allowed regardless of the slice support of the target NG-RAN node i.e. even if the target NG-RAN node does not support the same slices as the source NG-RAN node.

Even though handovers are allowed regardless of the slice support of the target NG-RAN node, this leads to discontinuity in the slice service and interruption, if UE moves towards an area that does not support one of UE's ongoing slices (e.g., during inter-RA mobility).

To detect such scenarios and take correction action in the future, it was proposed by few companies last meeting to add slice-related information in SON reports (e.g., in UHI or SHR) e.g., include UE’s ongoing slice(s) in the source and target cell in UHI, collect SHR when a certain handover resulted in a slice service interruption.

Currently, a gNB already knows the slices supported by its served cells and the slices supported in the cells served by its neighbor gNBs via Slice Support List exchange in F1 and Xn. During handover preparation, the source gNB will know that a PDU session (which is associated to a slice) it is offloading to a certain target gNB might get interrupted due to non-support of that slice in the target gNB.

We therefore are not sure whether any enhancements to SON reports (e.g., collecting SHR in case of such handover to target cell supporting the ongoing slice) to do non-real optimizations are really needed when such things are known already at the gNB.

Also, a certain gNB via implementation can already take slice aware mobility decisions and mitigate slice service discontinuity. For example, a gNB can prioritize a target cell that supports UE’s ongoing slice in the source cell over another target cell that doesn’t support the UE’s ongoing slice (if radio conditions of the two target cells are comparable).

Observation 1: A gNB knows the slices supported by its served cells and the slices supported in the cells served by its neighbor gNBs via Slice Support List exchange in F1 and Xn.

Observation 2: Based on the knowledge of slice(s) supported at neighboring gNBs, a source gNB has knowledge whether a PDU session (which is associated to a slice) will get interrupted or not upon offloading to a target gNB.

Proposal 1: There are no enhancements needed from UE in SON reports to assist the gNB to optimize handovers with slice service discontinuity (e.g., due to target cell not supporting UE’s ongoing slice). gNB already has sufficient knowledge to identify such scenarios.

  - Slice-specific MDT Area Scope

RAN3 to support immediate MDT measurements collection on specific area per slice(s).

FFS on which immediate MDT measurements can be collected per slice.

The previous agreement, “..on specific area per slice(s)” is not very clear. It is not clear whether it meant:

- Understanding 1: Support immediate MDT measurements collection only when the UE is camped on cells supporting a certain slice

- Understanding 2: Support immediate MDT measurements collection only when the UE is using a certain slice

Proposal 2: RAN3 to clarify the previous agreement “RAN3 to support immediate MDT measurements collection on specific area per slice(s)” and which of the understanding below is to be supported:

- Understanding 1: Support immediate MDT measurements collection only when the UE is camped on cells supporting a certain slice

- Understanding 2: Support immediate MDT measurements collection only when the UE is using a certain slice

Understanding 1: Support immediate MDT measurements collection only when the UE is camped on cells supporting a certain slice

An operator might be interested in collecting immediate MDT measurements only when the UE is camped on cells supporting a certain slice. For example, an operator might be interested in observing the packet delay (M6) measurements only in areas where the URLLC slice is supported

Consider the following example deployment consisting of cells 1, 2 and 3:

- Cell 1 supports S-NSSAI 1, 2

- Cell 2 supports S-NSSAI 1

- Cell 3 supports S-NSSAI 1, 2

For the above example, let’s consider where the operator is only interested in collecting immediate MDT measurements where S-NSSAI 2 is support (i.e., cells 1 and 3):

To support this

- OAM needs to indicate to AMF a “Slice Scope for MDT” i.e., a list of S-NSSAI(s) (e.g. S-NSSAI 2) on which it is interested to collect immediate MDT measurements

- AMF then needs to forward this “Slice Scope for MDT” to gNB when it sends the s-based MDT Configuration over NGAP

- gNB need to select only those cells that support the slices within the Slice Scope (i.e. only cells 1 and 3) and configure/collect immediate MDT measurements only UE is camped on cell 1 and cell 3.

Summarizing the above, we have the following proposals:

Proposal 3: OAM can indicate to AMF a “Slice Scope for MDT” i.e., a list of S-NSSAI(s) on which it is interested to collect immediate MDT measurements. Send LS to SA5 to confirm if it is feasible.

Proposal 4: AMF should forward the “Slice Scope for MDT” (if received from OAM) to gNB when it sends the signaling based MDT Configuration over NGAP

Proposal 5: Upon receiving a “Slice Scope for MDT” from AMF, a gNB should identify its served cells that support the slices within the Slice Scope and configure/collect immediate MDT measurements only UE is camped on those cells.

Logged MDT

FFS on whether to support logged MDT on specific area per slice(s).

Now suppose the same operator is also interested in collecting logged MDT only in cells where a certain slice is supported.

- This can be achieved by gNB identifying the cells where the slice of interest is supported and thus create an area scope of logged MDT. In the above example, gNB needs to create an Area Scope of Logged MDT comprising of only cells 1 and 3 and indicate that to the UE.

- There is no need for gNB to indicate the “Slice Scope of MDT” to the UE in the Logged MDT configuration and this can be achieved by reusing existing “Area Scope of Logged MDT”.

The following is proposed:

Proposal 6: RAN3 should support logged MDT measurements collection only when the UE is camped on cells supporting certain slice(s) of interest

Proposal 7: Upon receiving a “Slice Scope for MDT” from AMF, a gNB should identify its served cells that supports the slices within the Slice Scope, create an Area scope of Logged MDT only comprising those cells and indicate that area scope to the UE in the logged MDT configuration.

Proposal 8: There is no need for gNB to indicate the “Slice Scope of MDT” to the UE in the Logged MDT configuration

Understanding 2: Support immediate MDT measurements collection only when the UE is using a certain slice

If this is the understanding, there can be following solutions:

- UE-based solution: gNB indicates the Slice Scope to the UE and UE collects immediate MDT measurements only when UE is camped on a cell supporting the S-NSSAI(s) within the Slice Scope. With this, a UE doesn’t need to perform MDT measurements (e.g., M6) when it is not using a slice belonging to the Slice Scope. But the drawback is that MDT configuration needs to be enhanced to include the Slice Scope and this also adds complexities at the UE to check the Slice Scope.

- gNB-based solution: In this solution, gNB doesn’t need to indicate the Slice Scope to the UE. gNB collect/configures immediate MDT measurements for the UE only when the UE is using a certain S-NSSAI belonging to the Slice Scope. gNB currently knows the DRB that a UE is using and can decipher the S-NSSAI/PDU session associated to that DRB and configure/release immediate MDT as seen necessary.

To avoid impacts to UE, we propose to not consider the UE based solution and consider the gNB-based solution. But first we want RAN3 to discuss whether Understanding 2 is to be supported.

Proposal 9: RAN3 to discuss whether to support a mechanism where a gNB if configured with a “Slice Scope of MDT” should collect/configure immediate MDT measurements for the UE only when the UE is using a certain S-NSSAI belonging to the Slice Scope.

  - Logged MDT enhancements for slice-based cell reselection

RAN3 to work on logged MDT enhancement for slice-based cell reselection.

In order to support slice-based cell reselection, a UE may be provided with slice-based cell reselection information via RRCRelease or SIB16 e.g., reselection priorities per NSAG per frequency and corresponding list(s) of cells where the slices of the NSAG are supported or not supported.

When a UE supports slice-based cell reselection, and when slice-based cell reselection information is provided to the UE, then the UE uses the slice-based cell reselection information.

When no slice-based cell reselection information is provided for any NSAG that was determined to be considered during cell reselection, then the UE uses the general cell reselection information, i.e., without considering the NSAG(s) and their priorities.

A UE provided with slice-based cell reselection information may reselect different cells compared to a UE not provided with slice-based cell reselection information parameters because of different reselection priorities and based on a bias introduced by whether the cell supports a certain slice or not. An operator trying to build coverage maps via logged MDT might not be able to build a true coverage map if it doesn’t know why UEs traversing a similar path see a different coverage (i.e., connect to different cells) even though the radio conditions are similar.

Proposal 10: RAN3 should discuss whether there are benefits in making the OAM aware that the logged MDT reports were collected by a UE using slice-based cell reselection information (and not general cell reselection information).

# Conclusion

Slice aware MRO

Observation 1: A gNB knows the slices supported by its served cells and the slices supported in the cells served by its neighbor gNBs via Slice Support List exchange in F1 and Xn.

Observation 2: Based on the knowledge of slice(s) supported at neighboring gNBs, a source gNB has knowledge whether a PDU session (which is associated to a slice) will get interrupted or not upon offloading to a target gNB.

Proposal 1: There are no enhancements needed from UE in SON reports to assist the gNB to optimize handovers with slice service discontinuity (e.g., due to target cell not supporting UE’s ongoing slice). gNB already has sufficient knowledge to identify such scenarios.

Slice-specific MDT Area Scope

Proposal 2: RAN3 to clarify the previous agreement “RAN3 to support immediate MDT measurements collection on specific area per slice(s)” and which of the understanding below is to be supported:

- Understanding 1: Support immediate MDT measurements collection only when the UE is camped on cells supporting a certain slice

- Understanding 2: Support immediate MDT measurements collection only when the UE is using a certain slice

Understanding 1: Support immediate MDT measurements collection only when the UE is camped on cells supporting a certain slice

Proposal 3: OAM can indicate to AMF a “Slice Scope for MDT” i.e., a list of S-NSSAI(s) on which it is interested to collect immediate MDT measurements. Send LS to SA5 to confirm if it is feasible.

Proposal 4: AMF should forward the “Slice Scope for MDT” (if received from OAM) to gNB when it sends the signaling based MDT Configuration over NGAP

Proposal 5: Upon receiving a “Slice Scope for MDT” from AMF, a gNB should identify its served cells that support the slices within the Slice Scope and configure/collect immediate MDT measurements only UE is camped on those cells.

Understanding 1 for logged MDT: Support logged MDT measurements collection only when the UE is camped on cells supporting a certain slice

Proposal 6: RAN3 should support logged MDT measurements collection only when the UE is camped on cells supporting certain slice(s) of interest

Proposal 7: Upon receiving a “Slice Scope for MDT” from AMF, a gNB should identify its served cells that supports the slices within the Slice Scope, create an Area scope of Logged MDT only comprising those cells and indicate that area scope to the UE in the logged MDT configuration.

Proposal 8: There is no need for gNB to indicate the “Slice Scope of MDT” to the UE in the Logged MDT configuration

Understanding 2: Support immediate MDT measurements collection only when the UE is using a certain slice

Proposal 9: RAN3 to discuss whether to support a mechanism where a gNB if configured with a “Slice Scope for MDT” should collect/configure immediate MDT measurements for the UE only when the UE is using a certain S-NSSAI belonging to the Slice Scope.

Logged MDT enhancements for slice-based cell reselection

Proposal 10: RAN3 should discuss whether there are benefits in making the OAM aware that the logged MDT reports were collected by a UE using slice-based cell reselection information (and not general cell reselection information).

# 4. References