3GPP TSG-RAN WG3 #125	R3-244433

Maastricht, Netherlands, Aug 19–23, 2024

Agenda Item:	10.2

Source:	Ericsson

Title:	MRO enhancements for LTM and CHO with candidate SCGs

Document for:	Discussion and Decision

# 1	Introduction

One of the main objectives of the SONMDT Rel-19 WID [RP-234038] is to enhance the MRO features to optimize the LTM cell switch procedure.

MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

This contribution discusses considerations regarding MRO feature enhancements for LTM and for and CHO with candidate SCGs.

# 2	MRO enhancements for LTM

During RAN3#123-bis and RAN3#124, the following agreements were taken on MRO for LTM:

- Work on scenarios of near failure LTM

- Work on scenarios for the differentiation of too early LTM, too late LTM and LTM to wrong cell

- RAN3 takes RAN2#125-bis agreement on MRO for LTM scenarios as baseline for further study.

Next sections will therefore discuss further the scenarios to be discussed in RAN3.

## 2.1	LTM scenarios

During RAN2#125bis, it was agreed in [1] that the following scenarios were to be studied (with Ericsson highlighting):

For too late LTM, the following sub-cases are considered but we may down prioritize later (not limiting): - - Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and performs reestablishment procedure. - - Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell, detects HOF with the selected LTM cell. - - Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM execution with the selected LTM candidate cell. For too early LTM, the following sub-cases are considered but we may down prioritize later (not limiting): - - Case 2a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell. - - Case 2b: the UE detects HOF/RLF in the LTM target cell, selects the source cell which is also an LTM candidate cell, detects HOF with the source cell, and performs reestablishment procedure. - - Case 2c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected source cell which is also an LTM candidate cell. LTM to wrong cell, the following sub-cases are considered but we may down prioritize later (not limiting): - - Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell. - - Case 3b: the UE detects HOF/RLF in the LTM target cell, selects an LTM candidate cell which is different from the source or target one, detects HOF with the selected LTM candidate cell, and performs reestablishment procedure. - - Case 3c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected LTM candidate cell which is different from the source or target one.

Despite these use cases being agreed by RAN2, we have found some minor issues with them. As these use cases will not be treated during the current RAN2 meeting, and as they are important for RAN3 as well, we would like them to be updated and agreed by RAN3.

The issues we have found are (see also highlighted text above):

- For case 1b, there were no suitable cell for the LTM cell switch, and no suitable cell for the recovery to the first failure either, because LTM Recovery or RRC re-establishment failed. As there were no suitable cells at all, we are hesitant to describe this use case as a "Too late LTM cell switch". We would like to denote this use case as FFS.

- For case 3a, which is a "LTM to wrong cell" case, it is written that the UE reestablishes at the source cell. However, that would be a "Too early LTM" case. Instead, the UE reestablishes to a "third cell", not being the source nor target cell. This correction has already been agreed during RAN2#126, in order to comply with classic MRO scenarios.

For these reasons, we would like to rewrite the agreements accordingly:

For too late LTM, the following sub-cases are considered but we may down prioritize later (not limiting): - - Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and performs reestablishment procedure. - - Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell, detects HOF with the selected LTM cell. [This use case is FFS.] - - Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM execution with the selected LTM candidate cell. For too early LTM, the following sub-cases are considered but we may down prioritize later (not limiting): - - Case 2a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell. - - Case 2b: the UE detects HOF/RLF in the LTM target cell, selects the source cell which is also an LTM candidate cell, detects HOF with the source cell, and performs reestablishment procedure. - - Case 2c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected source cell which is also an LTM candidate cell. LTM to wrong cell, the following sub-cases are considered but we may down prioritize later (not limiting): - - Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with a third cell, not being the source or target cell. - - Case 3b: the UE detects HOF/RLF in the LTM target cell, selects an LTM candidate cell which is different from the source or target one, detects HOF with the selected LTM candidate cell, and performs reestablishment procedure. - - Case 3c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected LTM candidate cell which is different from the source or target one.

This leads to the following proposal, with RAN3 agreeing the updated use-cases.

RAN3 to agree the MRO use cases for LTM:

- For too late LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

    - Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and performs reestablishment procedure.

    - Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell, detects HOF with the selected LTM cell. [This use case is FFS.]

    - Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM execution with the selected LTM candidate cell.

- For too early LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

    - Case 2a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

    - Case 2b: the UE detects HOF/RLF in the LTM target cell, selects the source cell which is also an LTM candidate cell, detects HOF with the source cell, and performs reestablishment procedure.

    - Case 2c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected source cell which is also an LTM candidate cell.

- LTM to wrong cell, the following sub-cases are considered but we may down prioritize later (not limiting):

    - Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with a third cell, not being the source or target cell.

    - Case 3b: the UE detects HOF/RLF in the LTM target cell, selects an LTM candidate cell which is different from the source or target one, detects HOF with the selected LTM candidate cell, and performs reestablishment procedure.

    - Case 3c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected LTM candidate cell which is different from the source or target one.

These failure cases should also be captured in an update to TS 38.300 [2]. Here we propose the following definitions to be added to clause 15.5.2.2.2, covering all the sub-cases defined above:

- Too Late LTM cell switch: an RLF occurs after the UE was configured with at least one LTM configuration; the UE attempts LTM recovery or to re-establish the radio link connection in a candidate cell different from the last serving cell. Editor's note: In the above definition it is FFS if the failed LTM recovery should be considered as a too late LTM switch. - Too Early LTM cell switch: an RLF occurs shortly after a successful LTM cell switch from a source cell to a target cell or a LTM cell switch failure occurs during the LTM cell switch procedure; the UE attempts LTM recovery or to re-establish the radio link connection in the source cell. - LTM cell switch to Wrong Cell: an RLF occurs shortly after a successful LTM cell switch from a source cell to a target cell or a LTM cell switch failure occurs during the LTM cell switch procedure; the UE attempts LTM recovery or to re-establish the radio link connection in a cell other than the source cell or the target cell.

Note that we’ve added an Editor's note corresponding to the FFS part in Proposal 1. A TP to TS 38.300 mirroring the definitions above is proposed to be agreed in Annex 1.

During RAN3#124, there was an opinion that these definitions of failures in the LTM cell switch procedure should be merged with the definitions for legacy handover. However, we are of the opinion that this would result in an unnecessarily complicated definition. Furthermore, the existing text concerns handovers, while the proposed text concerns LTM cell switching, which is a different procedure.

Agree LTM problem definitions in TP in Annex 1.

- Now, looking at scenarios described in Proposal 1, it needs to be noticed that there are 2 ways for the UE to perform RRC re-establishment instead of LTM Recovery after a first RLF/HOF. Either the selected cell is not an LTM candidate cell, or the network did not allow the UE to perform LTM Recovery (attemptLTM-Switch-r18 in RRC). Therefore, 3GPP needs to differentiate (e.g. by means of information contained in RLF Report) cases were the selected cell was not an LTM candidate, or if LTM Recovery was not allowed by the network,

For LTM, in case of RLF/HOF, and for root cause analysis, the network needs to know the reason if and why LTM Recovery was not attempted (i.e. the selected cell was not a candidate cell, or LTM Recovery was not allowed).

## 2.2 	Successive failure cases.

During the RAN3#124 meeting, successive failure cases were discussed. The following figure was contributed to discuss further scenarios:

We analyse the figure as follows:

- Case a is already covered, as it is a failure with reestablishment.

- Case b is already covered, as it is a failure with a failed recovery.

- Case c is already covered, as it is a failure with a successful recovery.

- Case d has not been discussed in the context of LTM. It consists of a failure with a successful recovery, quickly followed by another failure and a reestablishment. This case has been discussed for CHO, and there is a solution defined for CHO. In this solution, the second RLF Report does not log the source cell ID, the time to connection failure and the last handover type. The reason for not logging those is that the report could then mistakenly be interpreted as describing a too early cell switch failure.

As case d has very similar properties to the same case for CHO, we propose to copy the solution defined for CHO (i.e. keep logging the source cell ID, time to connection failure and last handover type from the first failure).

RAN3 to copy the CHO solution for the case of LTM failure with a subsequent successful LTM recovery, quickly followed by a new failure and a reestablishment.

## 2.3 	RACH-less and RACH-based optimization.

During RAN3#123-bis, RACH-based vs. RACH-less LTM optimizations were discussed, but with no conclusion. As RACH-less is a critical addition to Rel-18 and essential for LTM, and as RACH-based can also be performed, there is a need to optimize RACH-less access itself. Furthermore, it is also a need to be able to differentiate RACH-less access from RACH-based access when performing root cause analysis of LTM failure and near-failure cases.

RAN3 to discuss the differentiation of RACH-less access from RACH-based access when performing root cause analysis of failure and near failure cases.

In an LTM cell switch attempt, the UE may be configured to obtain uplink sync in a candidate cell in two different ways.

- The first way is that early UL sync is achieved prior to the LTM cell switch execution to support a RACH-less procedure. Configuration for this procedure is optional. Early UL sync can be achieved in two different ways:

  - The network determines the TA based on a PDCCH ordered RA preamble transmission in the candidate cell.

  - The UE autonomously determines the TA based on measurements of the difference in time of arrival for DL signals in the candidate cell and the source cell.

- The second way is to use a RACH-based procedure, much like legacy handover. Configuration for this procedure is mandatory.

The RACH-less procedure may fail for a number of reasons, for example the TA configuration becoming stale after some time. In this case the UE may fall back to the RACH-based procedure, where fall back means e.g. that the TA validity timer has expired or that UE failed to autonomously calculate TA. However, this fallback is not mandatory. If this fallback is not employed by the UE, the UE will declare an RLF.

When attempting a LTM cell switch, and a RACH-less access is configured in the UE, the UE will first attempt a RACH-less access. If that fails, the UE may fall back to a RACH-based access.

When attempting a LTM cell switch, and if no RACH-less access is configured in the UE (i.e. no TA is available for that candidate), the UE will attempt a RACH-based access.

If an LTM cell switch fails, this should be reported using a RLF Report in the same way that a legacy handover failure results in a RLF Report.

UE to generate a RLF Report after a failed LTM cell switch.

For LTM, we need to distinguish between a set of failure cases:

- The UE was configured for both RACH-less and RACH-based access. The RACH-less access cannot be executed, and the UE used a RACH-based access as fallback. The RACH-based access succeeded. This case should not result in a RLF Report, as the access finally succeeded.

- The UE was configured for both RACH-less and RACH-based access. The RACH-less access cannot be executed, and the UE used a RACH-based access as fallback. The RACH-based access failed as well. This case should result in a RLF Report.

- The UE was configured for only a RACH-based access. The RACH-based access failed. This case should result in a RLF Report.

Case 1 above should not result in an RLF. However, cases 2 and 3 should generate an RLF. As it is now, there is no possibility of separating these failures.

For current RLF Reports, there is no possibility to observe whether a LTM cell switch failure attempted both RACH-less and RACH-based access, or only RACH-based access.

However, for root cause analysis, it is important to distinguish these cases. Therefore, it is proposed that the RLF Report is enhanced with information indicating that RACH-less access was attempted but failed.

Enhance RLF Report with information indicating that RACH-less access was attempted but failed.

With the early UL sync, the NW can instruct the UE to perform LTM cell switch procedure by sending the LTM Cell Switch Command MAC CE. In the LTM cell switch command, if a TA value is included or if the UE has successfully measured the TA, then the RA procedure can be skipped and RACH-less LTM can be executed. If RACH-less LTM cell switch cannot be executed, the UE may fall back to a RACH-based LTM. However, even if the fall-backed RACH-based LTM is successful, the interruption time will increase compared to RACH-less LTM. Hence it is necessary to log the early UL sync related information to optimize this undesired fallback behavior.

If the UE falls back from RACH-less LTM falls back to RACH-based LTM, this will increase the LTM interruption time which is a sub-optimal scenario.

In order to log the early UL sync related information in the SHR, a new triggering condition can be considered, e.g., SHR can be triggered when the UE falls back to RACH-based LTM from RACH-less LTM.

Introduce fallback from RACH-less LTM to RACH-based LTM as a new trigger to log SHR.

Enhance SHR with information indicating that RACH-less access was attempted but failed.

Similar observations and enhancements can be made for the RA Report. As described above, in the LTM cell switch command, if a TA value is included or if the UE has successfully measured the TA, then the RA procedure can be skipped and RACH-less LTM can be performed.

Failure of the RACH-less LTM cell switch will lead to an RRC connection re-establishment procedure, with an increased interruption time for the UE. With enhancement to RLF Report proposed above, the RACH-less failure can be known to the network.

However, if RACH-less procedure cannot be executed, the UE may fall back to RACH-based procedure before expiry of T304. In order to detect if there is a previous RACH-less failure before the RA procedure, a fallback indication can be included in RA-Report when the fall-backed RACH procedure is successfully completed. In case that the fall-backed RACH-based procedure also fails, this fallback indication is then useful in the RLF-Report for the network to detect the root cause for the LTM cell switch failure. Furthermore, in a near-failure case, this fallback indication is also beneficial in SHR for the network to optimize the LTM cell switch procedure.

RA-InformationCommon is included in the RA-Report, the RLF-Report and the SHR, it is simple and straightforward to log the fallback indication from RACH-less LTM to RACH-based LTM in RA-InformationCommon.

Include an indication of fallback from RACH-less LTM to RACH-based LTM (e.g. TA validity timer has expired or that UE failed to autonomously calculate TA) inside RA-InformationCommon to enable logging it in the RA-report, the RLF-Report and the SHR.

## 2.4	Optimization triggered by RLF Report and its impact on F1

In case of LTM handover failure, there are at least two types of handover optimisation that the source gNB can perform:

Firstly, and similar to L3 HO, LTM cell switch triggers can be optimized (e.g. trigger LTM earlier to avoid too late LTM). For LTM, the trigger for execution of LTM is performed at MAC level, which is part of DU. Therefore, the DU should be in charge of optimisations related to LTM trigger.

DU is in charge of optimisations related to LTM trigger (i.e. sending the MAC CE command).

During RAN3#124, there was an agreement that:

For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization.

Therefore, both the CU and the DU need to receive the RLF report. When the UE notifies the gNB that an RLF Report is available, this notification is received by the CU, as the RRC protocol is terminated in the CU. For this reason, the CU needs to transfer the RLF Report to the DU, for the DU to be able to use it for its optimisations. Such a procedure (i.e. Access And Mobility Indication) already exists over F1, and can be reused for LTM.

For the purpose of LTM, the CU shall transfer the RLF Report to the DU(s), using existing mechanism (F1), and the ACCESS AND MOBILITY INDICATION message.

## 2.5	Use of SHR triggers for LTM

The RAN2#124 SOD on MRO for LTM [3], clause 2.5.2 contains questions about triggers for SHR for LTM. We will try to answer one of these questions below:

For LTM SHR, the T304, T310 and T312 related triggering conditions are taken as baseline? Send LS to RAN2 about T304?

As timers T304, T310 and T312 are used in the same way for LTM cell switches as for handovers, we propose to reuse these timers as SHR triggers. There might be a need to re-evaluate the trigger percentages (for T304 and T310, they are 40%, 60% or 80%, while for T312 they 20%, 40%, 60% or 80%.)

Another observation is that the time for LTM cell switch can be quite different if RACH-less LTM switch is used, or RACH-based LTM switch is used. We assume that a RACH-less LTM switch is faster than a RACH-based LTM switch. The network node can set the timers and thresholds differently depending on whether it will configure the UE to do a RACH-less or RACH-based access.

There are triggers defined for T304, T310 and T312 for use by SHR. We propose to reuse these triggers for LTM as well.

# 3	MRO enhancements for CHO with candidate SCGs

## 3.1	Use case definitions

- During RAN3#123-bis, some use-cases of possible CHO with candidate SCGs were presented and discussed in R3-241910. A summary is presented in Figure 1 below.

Figure 1: CHO with candidate SCGs failures use-cases, from R3-241910

This was further discussed during RAN3#124, and the following agreements were taken:

Do not distinguish between fast MCG recovery/no-fast MCG recovery for now (to simplify use cases).

RAN3 will start with the failure scenarios with UEs configured with CHO with candidate SCGs.

The concurrent error cases were also discussed and the result of this discussion was captured as below:

Concurrent error cases (MCG+SCG) is FFS. The definition of these error cases needs to be further clarified.

It corresponds to cases 1, 2, 3, 7, 8 and 9. However, this is a corner case, for which there is often no possible enhancement from a network point of view (e.g. UE enters a tunnel). Therefore, and considering the high number of cases to be discussed, it is proposed to down-prioritized concurrent MCG and SCG failure.

Concurrent MCG and SCG failure is down-prioritized.

Cases 4, 5 and 6 can also be down-prioritized because these are CHO failures, which should be covered by previous releases of MRO features.

Cases 4, 5 and 6 are down-prioritized.

Case 9 needs to be clarified. The failure case described is:

- CHO execution condition is met (event is fulfilled)

- CHO execution is unmet (event is not fulfilled anymore)

- CPAC execution condition is met

- Failure

However, the figure shows twice the event 3 above, whereas event 2 is not shown.

Clarify case 9 and correct Figure 1.

## Reducing the time between associated conditions

One of the main differences when we compare CHO with candidate SCGs to plain-old CHO+CPAC is that one CHO configuration and one CPAC configuration can now been linked together. In fact, for the UE to apply the PCell and PSCell configuration, both conditions (the one related to PCell and the one related to PSCell) need to be fulfilled. The goal for an optimized feature is then to have both conditions fulfilled at the same time, or close. If one of the conditions is not fulfilled, the CHO with candidate SCGs will not be executed at all. Which is sub-optimal, to say the least, or can even lead to failure:

- RLF due to UE waiting for the CPAC condition to be fulfilled.

- SCG Failure due to UE waiting for the CHO condition to be fulfilled.

Therefore, one important point to be discussed and optimized is how to reduce the time between fulfilment of CHO execution condition and CPAC execution condition.

Study solutions to reduce the time between fulfilment of CHO execution condition and CPAC execution condition.

# 4	References

R2-125bis_Maint_eRedCap_SON-MDT_(Mattias)_20240419_Final.docx

3GPP TS 38.300, "NR and NG-RAN Overall Description; Stage 2"

R3-243846, Summary of Offline Discussion on MRO for LTM

# 5	Conclusion

- For MRO enhancements for LTM:

- RAN3 to agree the MRO use cases for LTM:

- For too late LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

    - Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and performs reestablishment procedure.

    - Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell, detects HOF with the selected LTM cell. [This use case is FFS.]

    - Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM execution with the selected LTM candidate cell.

- For too early LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

    - Case 2a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

    - Case 2b: the UE detects HOF/RLF in the LTM target cell, selects the source cell which is also an LTM candidate cell, detects HOF with the source cell, and performs reestablishment procedure.

    - Case 2c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected source cell which is also an LTM candidate cell.

- LTM to wrong cell, the following sub-cases are considered but we may down prioritize later (not limiting):

    - Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with a third cell, not being the source or target cell.

    - Case 3b: the UE detects HOF/RLF in the LTM target cell, selects an LTM candidate cell which is different from the source or target one, detects HOF with the selected LTM candidate cell, and performs reestablishment procedure.

    - Case 3c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected LTM candidate cell which is different from the source or target one.

Agree LTM problem definitions in TP in Annex 1.

For LTM, in case of RLF/HOF, and for root cause analysis, the network needs to know the reason if and why LTM Recovery was not attempted (i.e. the selected cell was not a candidate cell, or LTM Recovery was not allowed).

RAN3 to copy the CHO solution for the case of LTM failure with a subsequent successful LTM recovery, quickly followed by a new failure and a reestablishment.

RAN3 to discuss the differentiation of RACH-less access from RACH-based access when performing root cause analysis of failure and near failure cases.

- When attempting a LTM cell switch, and a RACH-less access is configured in the UE, the UE will first attempt a RACH-less access. If that fails, the UE may fall back to a RACH-based access.

When attempting a LTM cell switch, and if no RACH-less access is configured in the UE (i.e. no TA is available for that candidate), the UE will attempt a RACH-based access.

UE to generate a RLF Report after a failed LTM cell switch.

For current RLF Reports, there is no possibility to observe whether a LTM cell switch failure attempted both RACH-less and RACH-based access, or only RACH-based access.

Enhance RLF Report with information indicating that RACH-less access was attempted but failed.

If the UE falls back from RACH-less LTM falls back to RACH-based LTM, this will increase the LTM interruption time which is a sub-optimal scenario.

Introduce fallback from RACH-less LTM to RACH-based LTM as a new trigger to log SHR.

Enhance SHR with information indicating that RACH-less access was attempted but failed.

Include an indication of fallback from RACH-less LTM to RACH-based LTM (e.g. TA validity timer has expired or that UE failed to autonomously calculate TA) inside RA-InformationCommon to enable logging it in the RA-report, the RLF-Report and the SHR.

DU is in charge of optimisations related to LTM trigger (i.e. sending the MAC CE command).

For the purpose of LTM, the CU shall transfer the RLF Report to the DU(s), using existing mechanism (F1), and the ACCESS AND MOBILITY INDICATION message.

There are triggers defined for T304, T310 and T312 for use by SHR. We propose to reuse these triggers for LTM as well.

- For MRO enhancements for CHO with candidate SCGs:

Concurrent MCG and SCG failure is down-prioritized.

Cases 4, 5 and 6 are down-prioritized.

Clarify case 9 and correct Figure 1.

Study solutions to reduce the time between fulfilment of CHO execution condition and CPAC execution condition.

# Annex 1:	TP to 38.300 on MRO Use Cases

First change

##### 15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

For LTM, connection failures that occur due to Too Early or Too Late LTM cell switch, or LTM cell switch to Wrong Cell are defined as follows:

-	Too Late LTM cell switch: an RLF occurs after the UE was configured with at least one LTM configuration; the UE attempts LTM recovery or to re-establish the radio link connection in a candidate cell different from the last serving cell.

Editor's note: In the above definition it is FFS if the failed LTM recovery should be considered as a too late LTM switch.

-	Too Early LTM cell switch: an RLF occurs shortly after a successful LTM cell switch from a source cell to a target cell or a LTM cell switch failure occurs during the LTM cell switch procedure; the UE attempts LTM recovery or to re-establish the radio link connection in the source cell.

-	LTM cell switch to Wrong Cell: an RLF occurs shortly after a successful LTM cell switch from a source cell to a target cell or a LTM cell switch failure occurs during the LTM cell switch procedure; the UE attempts LTM recovery or to re-establish the radio link connection in a cell other than the source cell or the target cell.

In the definition above, the "successful LTM switch" refers to the UE state, namely the successful completion of the PDCCH order-based early UL sync or the RA procedure.

End of changes