3GPP TSG-RAN WG3 #125	R3-244434

# Maastricht, Netherlands, 19th – 23rd August 2024

Agenda Item:	10.3.1

Source:	Ericsson

Title: 	SON/MDT enhancements for NTN

Document for:	Discussion

# 1	Introduction

RAN plenary decided during RAN#103 to support further data collection for Self-organizing Networks (SON). One of the topics that was endorsed was the SON enhancements for Non-terrestrial networks.

During previous meeting, the following agreements were taken:

- Logged MDT enhancement based on scenarios to be worked first, work on immediate MDT if time allows.

- MRO mechanisms for intra-NTN should be addressed based on failure scenarios first, work on near failure scenarios later.

- Take the connection failure scenarios in §15.5.2.2.2 of TS 38.300 as starting point with necessary update, if needed, in MRO for intra-NTN mobility.

- MRO for intra-NTN mobility shall consider the following trigger conditions:

- Measurement-based trigger condition;

- A time-based trigger condition;

- A location-based trigger condition;

- A time-based and Measurement-based combined trigger condition;

- A location-based and Measurement-based combined trigger condition;

MRO for intra-NTN mobility shall consider both conditional and non-conditional intra-NTN handover.

Additionally, the following ways forward were also discussed:

FFS on whether there are any enhancements to RLF report due to support for multiple trigger conditions in intra-NTN mobility.

FFS on whether to consider any enhancement for MDT configuration e.g., area scope and trigger condition for NTN.

Discuss MRO for RACH-less HO of NTN, only for those different issues identified compared with MRO for LTM.

Time information when RLF occurs needs to be included in the RLF report, other information to be included can be checked as well in next meeting.

This paper builds on the previous submissions by discussing SON for NTN on a range of different topics such as RLF enhancements, SHR enhancements and MDT area scope related enhancements.

# 2	Discussion

### 2.1 MDT related enhancements for NTN

Using the existing MDT framework in an NTN network requires further considerations and enhancements to cope with the specificities of NTN. One of the key differences between the NTN and the TN networks is the fact that, compared to the TN, in NTN the mapping between the coverage of an NTN cell and the geographical area covered by that cell changes over the time. Indeed, as a satellite follows its orbit the coverage of the satellite cells swipes the surface of the earth meaning that, at two different points in time the same geographical area may be covered by two different NTN cells.  In this context it may be difficult to define an Area Scope for MDT that enables measuring the NTN service coverage in a given geographical area. It is therefore proposed to extend the definition of the MDT Area Scope to enable collection of immediate and logged MDT measurements for the NTN coverage in a specific geographical area.

Proposal 1: RAN3 to extend the definition of the MDT Area Scope to enable collection immediate and logged MDT measurements for the NTN coverage in a specific geographical area.

The Area Scope of MDT needs specific enhancements related to indicate to the UE, that the MDT has to be collected in NTN networks. To do so, we need to add an NTN wide indicator in the Area Scope of MDT. This will allow the network to indicate to the UE to collect MDT only in NTN. There is currently no method in the specifications to indicate that MDT must be collected in a specific type of network, NTN in this case. This will allow an operator to build a coverage map of NTN network and detect problems specific to that type of RAT.

Other options such as indicating the CGI of the cell can be used, however they have an upper limit on the number of identifiers and as such is unsuitable for this scenario.

Proposal 2: Include NTN wide indication in the MDT Area Scope to enable MDT collection

Another aspect of NTN that requires further consideration with respect to MDT is the reporting of UE MDT measurements. Indeed, compared to TN, reporting of UE measurements over the NTN uplink comes with a much higher energy cost on the UE side. This cost is the natural consequence of the fact that for the NTN uplink the distance between the transmitter (the UE on the surface of the earth) and the receiver (the satellite high in orbit) is an order of magnitude higher compared to the distance between the transmitter and the receiver in case of TN uplink (e,g.,  hundreds/thousands of km for NTN vs hundreds/ thousands of meters for TN). In this context and whenever possible, it is preferable to avoid wasting UE’s battery on sending the MDT reports while the UE is under the service coverage of a NTN and to differ these reports for the times when the UE moves back under the coverage of a TN.

Proposal 3: RAN3 to extend the MDT framework to enable instructing the UE and/or the network to avoid triggering UE reporting of MDT measurements while the UE is connected to NTN.

### 2.2 RLF related enhancements for NTN

Current RLF framework has been standardized with terrestrial networks in mind. When a UE declares RLF when in an NTN cell, there are currently no mechanisms to log NTN specific information in the RLF report. Thus, the network, upon receiving this RLF report, cannot fix any specific issues that a NTN network might have.

Certain addition of NTN related information in the RLF report, for e.g. ephemeris information, NTN related causes such as GNSS failures, can help the network determine the RLF cause. This is critical to ensure a better non-terrestrial network functioning.

Some might argue that RLF falls more under RAN2 work, however it is good to keep in mind that RAN2 has very little time to discuss SON related topics during rel-19. Additionally, RAN3 is the leading group for the WI so we think it is prudent to start discussion early and report our outcomes to RAN2.

Observation 1: RAN2 has limited time to discuss SON related topics in rel-19 and RAN3 is the leading group for the WI.

Proposal 4: RAN3 to agree on what additions are needed in the RLF Report and to send an LS to RAN2 for final decision.

One information that needs to be added for HOF/RLF is the indication that the cell switch was performed using a satellite switch with re-sync procedure that was standardized for NTN in rel-18. This is a new procedure, and this information needs to be included in order for the network to accurately analyze the RLF Report and the failure cause.

Proposal 5: RAN3 to require RAN2 to add in the RLF Report information that the failure occurred during a satellite switch with re-sync procedure.

2.3 SHR related enhancements for NTN

During RAN3#124, it was agreed to discuss MRO and specifically the different trigger conditions.

- MRO for intra-NTN mobility shall consider the following trigger conditions:

- Measurement-based trigger condition;

- A time-based trigger condition;

- A location-based trigger condition;

- A time-based and Measurement-based combined trigger condition;

- A location-based and Measurement-based combined trigger condition;

It is important to indicate that the time-based and measurement-based combined trigger, and the location-based and measurement-based combination trigger are needed. RAN3 needs to indicate this to RAN2, and RAN2 needs to discuss these two conditions and include these in the trigger conditions.

Proposal 6: RAN3 to indicate to RAN2 that SHR generated when a UE is configured with a combination of trigger conditions needs to be studied. The two trigger conditions are namely, time-based and measurement-based, and location-based and measurement-based combination triggers.

For a scenario where a CHO configuration includes a time-based trigger condition and a channel quality measurement-based trigger condition, it would be beneficial for the network to understand if the time-window almost expired before CHO execution (i.e. before the quality measurement-based trigger condition is fulfilled). This would be categorized as a near-failure, and therefore SHR could be reused. To solve that issue, a new SHR trigger (threshold) could be introduced, for the UE to generate an SHR when more than X% of the time window of the time-based trigger condition had elapsed. This would be similar to T304/T310/T312 threshold.

Proposal 7: Introduce a new SHR trigger (threshold) for the UE to generate an SHR when more than X% of the time window of the time-based trigger condition had elapsed.

These combination of trigger conditions which were introduced for NTN has not been studied fully for SON and thus there are more aspects that need to be studied. These combination triggers give rise to a situation where the fulfilment and monitoring of these conditions might be ambiguously executed, leading to confusion at the network on what exactly happened. One example is the case when a time-based trigger condition is combined with a channel quality measurement-based trigger condition whether the UE measured the channel-quality only once the time-based trigger condition is satisfied or the UE was already measuring the channel quality.

Proposal 8: RAN3 should discuss when the trigger conditions for MRO are executed and their impact.

## 2.4 RACH-less HO related enhancements for NTN

During the previous meeting, there was an FFS agreed related to the enhancements for RACH-less HO. This should be agreed as explained below.

“Discuss MRO for RACH-less HO of NTN, only for those different issues identified compared with MRO for LTM.”

RACH-less handover was standardized in rel-18 for Non-terrestrial networks that was further applicable to other Wis such as mobile IAB among others. RACH-less HO for non-terrestrial networks needs to be optimized as SON enhancements for RACH-less HO were not considered previously. RACH-less HO optimizations are also being discussed in other Work Items, therefore we propose the following.

Proposal 9: RAN3 to discuss SON related enhancements for the RACH-less HO procedure for NTN that are different than the ideas identified for LTM.

Proposal 10: Any stage 2 and stage 3 implementations should be taken in the LTM work item.

# 3	Summary

In this contribution, some observations and proposals related to SON/MDT for NTN were made:

Proposal 1: RAN3 to extend the definition of the MDT Area Scope to enable collection immediate and logged MDT measurements for the NTN coverage in a specific geographical area.

Proposal 2: Include NTN wide indication in the MDT Area Scope to enable MDT collection

Proposal 3: RAN3 to extend the MDT framework to enable instructing the UE and/or the network to avoid triggering UE reporting of MDT measurements while the UE is connected to NTN.

Observation 1: RAN2 has limited time to discuss SON related topics in rel-19 and RAN3 is the leading group for the WI.

Proposal 4: RAN3 to agree on what additions are needed in the RLF Report and to send an LS to RAN2 for final decision.

Proposal 5: RAN3 to require RAN2 to add in the RLF Report information that the failure occurred during a satellite switch with re-sync procedure.

Proposal 6: RAN3 to indicate to RAN2 that SHR generated when a UE is configured with a combination of trigger conditions needs to be studied. The two trigger conditions are namely, time-based and measurement-based, and location-based and measurement-based combination triggers.

Proposal 7: Introduce a new SHR trigger (threshold) for the UE to generate an SHR when more than X% of the time window of the time-based trigger condition had elapsed.

Proposal 8: RAN3 should discuss when the trigger conditions for MRO are executed and their impact.

Proposal 9: RAN3 to discuss SON related enhancements for the RACH-less HO procedure for NTN that are different than the ideas identified for LTM.

Proposal 10: Any stage 2 and stage 3 implementations should be taken in the LTM work item.

# 4	References

- RP-234038, “New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4”, RAN#102, Edinburgh, Scotland, December 2023.