3GPP TSG-RAN WG3 #125	R3-244437

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda Item:	10.4

Source:	Ericsson

Title: 	Further discussion on RACH optimization for SDT

Document for:	Discussion and Approval

# 1	Introduction

At previous meeting the following agreements were taken with respect to RACH optimization for SDT

Work on scenario of RACH optimization for SDT first.

There is no need for UE to report SDT configurations, e.g., RSRP threshold, data volume threshold, T319a, and other configurations.

In this paper we further discuss some scenario and related proposals related to RACH optimization for SDT.

# 2	Discussion

RACH optimization for SDT should consider whether the RA-SDT resources are used according to network dimensioning. For instance, an operator may dimension resources for CG-SDT and RA-SDT assuming a balanced used of CG-SDT and RA-SDT for the UEs and have reduced common RACH resources.

In Rel-18 SDT solutions, depending on the network configuration, when a UE fails to perform CG-SDT can fall back to RA-SDT or common RACH resources. However, if the UE fails to initiate CG-SDT and instead performs RA-SDT for majority of the time, it may overload the dimensioned RA-SDT resources. Moreover, if UE fails to initiate both CG-SDT and RA-SDT, thereby select common RACH resources, it may again have adverse impact on the RA performance of the network.

UE failing to initiate CG-SDT and eventual fallback to RA-SDT or using common RACH resources has adverse impact on the RACH resource allocation by the network.

However, if the network can be made aware of the fact that UE intended to initiate CG-SDT and failed, and thus resorted to using RA-SDT resources or common RACH resources, network can optimize the RA-SDT resources or the common RACH resources.

Network can optimize the allocated RA-SDT resources or common RACH resources if it is made aware that the UE failed to perform CG-SDT

It should be noted that in RAN2#126 meeting, the group has agreed that UE would include uplink buffered data volume and downlink RSRP information in RA report upon UE evaluating SDT procedure.

- For the purpose of SON enhancements for SDT, include RSRP/data volume related information

- Downlink RSRP value and buffered uplink data volume at the time when the UE evaluates if it should perform SDT.

However, when the UE evaluates whether it should perform SDT or not, the downlink RSRP value is evaluated against the same threshold (sdt-RSRP-Threshold) used for both RA-SDT and CG-SDT. And we have the same situation for the buffered uplink data volume where sdt-DataVolumeThreshold is used. So, when the network receives an RA report with the information agreed in RAN2 (i.e., downlink RSRP and buffered uplink data volume), it cannot deduce whether the UE intended to perform RA-SDT or CG-SDT.

From the agreed information in RAN2 (downlink RSRP and uplink buffered data volume), it is not possible for the network to deduce if the UE intended to initiate RA-SDT or CG-SDT.

Hence, UE should explicitly indicate in the RA report whether it intended to initialize CG-SDT or RA-SDT.

UE indicates in the RA report that it failed to initiate RA-SDT or CG-SDT, details of the indication is left for RAN2 discussion.

# 3 Conclusions

In this contribution the following observations and proposal were derived:

Observation 1	UE failing to initiate CG-SDT and eventual fallback to RA-SDT or using common RACH resources has adverse impact on the RACH resource allocation by the network.

Observation 2	Network can optimize the allocated RA-SDT resources or common RACH resources if it is made aware that the UE failed to perform CG-SDT

Observation 3	From the agreed information in RAN2 (dowlink RSRP and uplink buffered data volume), it is not possible for the network to deduce if the UE intended to initiate RA-SDT or CG-SDT.

Proposal 1	UE indicates in the RA report that it failed to initiate RA-SDT or CG-SDT, details of the indication is left for RAN2 discussion.