3GPP TSG-RAN WG3 Meeting #125	R3-244439

Maastricht, NL, 19th Aug – 23rd Aug, 2024

Title: 	TP on Mobility Optimization for NR-DC

Source: 	Ericsson, InterDigital

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

Mobility Optimization for NR-DC is one of the Rel-18 leftovers to be studied.

At RAN3#123-bis meeting, it was agreed to take as baseline that AI/ML inference is assumed only at the MN, and conditional procedures are not in scope:

Agree to take as baseline for the Mobility Optimization for NR-DC that the use case is studied assuming inference at the MN. The main use case is limited to dual connectivity only (e.g. no conditional procedures are in scope).

At RAN3#124 meeting some progress was achieved and it was agreed to enhance existing dual connectivity procedure to trigger the collection of measured UE performance:

Enhance the Dual Connectivity procedure (e.g., MN-initiated SN addition, MN-initiated SN change) to trigger the collection of measured UE performance.

In this contribution, we further discuss the details to be captured in the TR 38.743 to support for AI/ML assisted Mobility optimization for NR-DC.

# 2. Discussion

The current text agreed in the TR 38.743 is very basic, as reported below:

5.1.1 Use case description

Mobility Optimization for NR-DC is studied by assuming inference at the MN only. The main use case is limited to Dual Connectivity only (e.g., no conditional Dual Connectivity procedures are in scope).

First, we think it is appropriate to clarify in the TR that this use case reuses the same type of solutions considered for the Mobility Optimization for single connectivity in rel-18.

Proposal 1: The solutions considered to support AI/ML-based Mobility Optimization for NR-DC are:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

Based on the agreements reached so far, the MN can trigger collection of measured UE performance using existing dual connectivity procedures. Therefore, a clear standard impact is that, to enable the MN to request UE performance to the SN, the existing dual connectivity procedures will need to be used as triggering conditions for the reporting of UE performance feedback.

Proposal 2: Enable existing dual connectivity procedures (e.g., MN-initiated SN addition, MN-initiated SN change) to trigger the collection of measured UE performance at the SN.

At some point in time, the SN will provide the UE performance to the MN, therefore, some criteria are needed, to regulate the data collection and the reporting, for example: when the SN should start collecting UE performance, when it should stop (exit conditions), when and how it should report the UE performance to the MN. Apart from NR-DC specific details that need be discussed during the normative phase, we think that, to support AI/ML based Mobility Optimization in NR-DC, a straightforward solution is to follow the same logic as in rel-18 for single connectivity, that is:

- reuse the Data Collection Reporting Initiation Xn procedure to define what the SN should report (and how)

- reuse the Data Collection Reporting Xn procedure for the reporting.

Let’s consider, for example, a case of SN Addition: the UE is in single connectivity and served by gNB1 (future MN). gNB1 initiates a Data Collection Reporting Initiation towards the gNB2:

- reusing the rel-18 solution, gNB1 requests gNB2 to provide load metrics, e.g., predicted radio resource utilisation, or measured Energy Cost and this information is used to infer a decision to initiate the SN Addition towards gNB2.

- to support AI/ML based Mobility Optimization in NR-DC, gNB1 also requests gNB2 to provide UE performance measurements to be used as feedback. gNB2 starts to collect the data when it becomes the SN for the UE, and later reports it to the MN.

Also in case of MN-initiated SN Change, the reuse of the Data Collection Reporting Initiation and Data Collection Reporting procedures can support the NR-DC use case, as shown in the example below, where the UE is in NR-DC with gNB1 (MN) and gNB2 (source SN):

- reusing the rel-18 solution, the MN initiates a Data Collection Reporting Initiation towards gNB3 (target SN) and it requests gNB3 to provide load metrics, e.g., predicted radio resource utilization, or measured Energy Cost.

- to support AI/ML based Mobility Optimization in NR-DC, gNB1 requests gNB2 to provide UE performance measurements when the UE is served by gNB2. gNB1 uses this information to infer a decision to initiate the SN Change towards gNB3.

From the provided examples, it is clear that the existing Data Collection Reporting Initiation and Data Collection Reporting procedures are good tools that can support AI/ML based Mobility Optimization for NR-DC, therefore we propose to reuse both procedures for this use case.

Proposal 3: For AI/ML based Mobility Optimization in NR-DC, reuse the Data Collection Reporting Initiation and Data Collection Reporting XnAP procedures.

The Data Collection Reporting Initiation and Data Collection Reporting procedures could also be extended, e.g., to collect and report additional non-UE Associated metrics that can be used as inputs to infer the next SN addition or SN change.

An example of non-UE associated metric that can be advantageous to collect is the propagation delay over the F1 interface that affects an SN/PSCell. Such latency is often the largest delay component in the traffic propagation delay over an NR-DC leg, and it is mainly depending on the transport network performance. It is known that NR-DC configurations do not deliver a good performance if the traffic propagation delay between the two legs of the NR-DC configuration is unbalanced (i.e. very different delays between the MCG and SCG branches and/or very high delays on both branches). It is therefore very useful for an MN to understand what is the F1 propagation delay of the SN/PSCell to select in the future those target SN/PSCell that are affected by more favourable propagation delays.

Proposal 4: For AI/ML based Mobility Optimization in NR-DC, add support for requesting and reporting of non-UE associated metrics, e.g., the F1 propagation delay for a SN/PSCell.

Since the AI/ML inference is at the MN, when the MN triggers a dual connectivity procedure (e.g., an SN Addition) and requests the SN to report UE performance, the reported information is meaningful if it helps the MN to understand whether triggering that procedure was a good choice or not. Regarding the collection of UE performance, as an example, we can assume that the SN has chosen Cell A as PSCell, and it collects UE performance in that cell. However, the information that Cell A was the PSCell for which UE feedback was collected is not – as of today – reported to the MN. We note that the lack of this information has some implications for the NR-DC case, compared to the case of mobility in single connectivity (Rel-18), where the cell to which UE performance refers to is implicitly known by the source gNB, since it can only be the target cell of the handover. In case of mobility in single connectivity, if the source gNB assumes that a certain target cell can provide good performance to the UE, it can trigger the handover towards that cell, and it can also verify whether its assumption was appropriate after receiving the measured UE performance from the target gNB. For example: a source gNB1 receives from a potential target gNB2 some load metrics (measured or predicted), and Cell A of gNB2 has good available capacity. Then, source gNB1 triggers handover towards Cell A of gNB2, the gNB2 measures the UE performance, reports it to the gNB1, and the gNB1 uses it to understand whether the handover decision was good or not. The gNB1 does not receive cell information in the UE Performance explicitly, but it knows which cell is associated with the UE performance, since it can only be the target cell.

Observation 1: UE performance is collected and reported for a certain cell, and the identity of the cell to which the UE performance refers to is currently not reported because it is known by the source cell. For mobility optimization in single connectivity, the cell is unequivocally identified as the target cell of the handover.

For the NR-DC case, the current structure of the UE performance does not offer the same possibility to the MN (as to the source gNB in single connectivity), since the MN is not aware of which cell the SN selects as PSCell. Therefore, when the SN reports the UE performance, the MN will not understand to which cell the reported UE performance refers to. It could be commented that the SN signals to the MN the CG-Config IE, where information about the PSCell selected by the SN may be configured. However, the MN shall not be mandated to peek into the CG-Config for the purpose of understanding to which PSCell the UE performance feedback is associated to.

We propose an example to analyse the reason why this lack of information is limiting.

In light of the above, the following is proposed:

Proposal 5: Extend UE performance to indicate the cell the UE performance refers to.

In addition to using the existing MN-initiated dual connectivity procedures to request measured UE performance, we think that some other enhancements of these procedures are essential, and should be discussed during normative phase. In particular, the MN needs to specify to the SN the conditions under which the selection of the SN as the best predicted SN has been made.

Namely, an AI/ML algorithm will infer that a given SN is the best possible SN to add on the basis of the UE performance feedback received in the past for UEs served by previous SNs on specific PSCells. Therefore, when an MN predicts which one is the best SN, it does so also by assuming that a specific PSCell will be selected for the UE. This is obvious because the UE performance feedback received in the past for each UE is specific to a PSCell. Hence, if UEs were served well by an SN, on one or more PSCells, the MN is more likely to predict that the SN and one or more specific PSCells, are the best options for a DC configuration at the UE.

Therefore, we propose that, at SN Addition, the MN could tell the SN that MN assumes that a specific cell, e.g. Cell A, is selected as PSCell. The SN is free to decide independently, but it can try to select Cell A as PSCell if possible.

Another interaction needed between MN and SN concerns the SN terminated PDU Session Resource setup from MN to SN-

Namely, when the MN carries out an AI/ML generated SN additions, it may request to the SN the establishment of PDU Session Resources for SN Terminated bearers. However, the inferred decision is based on the assumption that specific QoS flows will be served by the SN (as SCG). This is not in line with what could happen today, where an SN may decide that all the QoS flows for which the MN requests SN Terminated PDU Session Resources, shall be served by MCGs.

Therefore, when the MN derives an AI/ML based request to the SN for some resources for SN terminated bearers, it should tell the SN that this request was made assuming that SN could serve some or all of the QoS flows as SCGs. After receiving this notification, the SN is left free to decide whether to serve the QoS flows as MCG or SCG, but it can use the information and try to accommodate the MN assumptions if possible.

Proposal 6: Enhance dual connectivity procedures (e.g., MN-initiated SN addition, MN-initiated SN change) to indicate MN assumptions at the basis of the AI/ML based inference on the SN to add (e.g., which cell the MN assumed as PSCell, which QoS flows the MN assumed to be served as SCGs).

Based on the discussion provided, we propose to agree on the TP to TR 38.743 provided in the Annex.

Proposal 7: Agree on the TP to TR 38.743 provided in the Annex.

# 3. Conclusion

Based on the discussion in this paper, we make the following observation and proposals.

Proposal 1: The solutions considered to support AI/ML-based Mobility Optimization for NR-DC are:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

Proposal 2: Enable the existing dual connectivity procedures (e.g., MN-initiated SN addition, MN-initiated SN change) to trigger the collection of measured UE performance at the SN.

Proposal 3: For AI/ML based Mobility Optimization in NR-DC, reuse the Data Collection Reporting Initiation and Data Collection Reporting XnAP procedures.

Proposal 4: For AI/ML based Mobility Optimization in NR-DC, add support for requesting and reporting of non-UE associated metrics, e.g., the F1 propagation delay for a SN/PSCell.

Observation 1: UE performance is collected and reported for a certain cell, and the identity of the cell to which the UE performance refers to is currently not reported because it is known by the source cell. For mobility optimization in single connectivity, the cell is unequivocally identified as the target cell of the handover.

Proposal 5: Extend UE performance to indicate the cell the UE performance refers to.

Proposal 6: Enhance dual connectivity procedures (e.g., MN-initiated SN addition, MN-initiated SN change) to indicate MN assumptions at the basis of the AI/ML based inference on the SN to add (e.g., which cell the MN assumed as PSCell, which QoS flows the MN assumed to be served as SCGs).

Proposal 7: Agree on the TP to TR 38.743 provided in the Annex.

# Annex: TP to TR 38.743

-------------------------------------------------------- Start of changes --------------------------------------------------------

# Rel-18 Leftovers and solutions

## 5.1	Mobility optimization for NR-DC

### 5.1.1 Use case description

When NR Dual Connectivity (NR-DC) is used, a UE is configured to utilise resources provided by two different nodes connected via non-ideal backhaul, both providing NR access. One node acts as the MN and the other as the SN. 
While, on one hand, use of NR-DC operation is meant to provide a boost in UE performance, it comes at the expense of quite complex operations, with many possible procedures (i.e. interactions) between the nodes involved, and various options to handle UE mobility (e.g., either the MN or the SN can initiate modification of the current setup).

Mobility optimization for NR-DC can take advantage of AI/ML support to optimal NR-DC actions and configuration.

In Rel-19, AI/ML-based Mobility optimization for NR-DC assumes inference only at the MN and Conditional Dual Connectivity procedures are out of scope.

### 5.1.2	Solutions and standard impacts

#### 5.1.2.1 Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based Mobility optimization for NR-DC:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the MN.

- AI/ML Model Training and AI/ML Model Inference are both located in the MN.

#### 5.1.2.2	AI/ML Model Training in the OAM and AI/ML Model Inference in the MN

In this solution, the MN takes mobility optimization decisions for NR-DC using AI/ML model trained from OAM.

Figure 5.1.2.2-1. AI/ML Model Training in OAM and AI/ML Model Inference in the MN

In the flowchart, the MN is represented by gNB1, and the SN is represented by gNB2.

Step 1: The gNB1 configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report message(s) to the gNB1.

Step 4a: The UE measurements are sent to the OAM as input data for AI/ML model training.

Step 4b-4c: The gNB1 and gNB2 provide input data for training the AI/ML model. For example, measured/predicted radio resource status.

Step 5: Model Training at the OAM.

Step 6: The OAM deploys/updates the AI/ML Model to gNB1.

Note: This step is out of RAN3 scope.

Step 7a: The UE sends the UE measurement report(s) to the gNB1 as input for AI/ML model inference.

Step 7b: The gNB2 sends to the gNB1 input data for AI/ML model inference.

Step 8: The gNB1 predicts an action for Mobility optimization for NR-DC.

Step 9: According to the prediction, recommended actions or configuration, the MN, the SN, and the UE perform the Mobility optimization for NR-DC (e.g., SN Addition).

Step 11: The gNB2 provides feedback to the gNB1.

#### 5.1.2.3	AI/ML Model Training and AI/ML Model Inference in the MN

In this solution, the MN takes mobility optimization decisions for NR-DC using AI/ML model trained in the MN.

Figure 5.1.2.3-1. AI/ML Model Training and AI/ML Model Inference in the MN

In the flowchart, the MN is represented by gNB1, and the SN is represented by gNB2.

Step 1: The gNB1 configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report message(s) to the gNB1.

Step 4: The gNB2 provides input data for training the AI/ML model. For example, measured/predicted radio resource status.

Step 5: Model Training at the gNB1 trains AI/ML model for AI/ML based Mobility Optimization for NR-DC.

Step 6: The UE sends the UE measurement report(s) to the gNB1 as input for AI/ML model inference.

Step 7: The gNB2 sends to the gNB1 input data for AI/ML model inference.

Step 8: The gNB1 predicts an action for Mobility Optimization for NR-DC.

Step 9: According to the prediction, recommended actions or configuration, the MN, the SN, and the UE perform the Mobility Optimization for NR-DC (e.g., SN Addition).

Step 10: The gNB2 provides feedback to the gNB1.

#### 5.1.2.4		Input of AI/ML-based Mobility Optimization for NR-DC

The following data is required as input data for mobility optimization for NR-DC.

From local node:

-	Measured/Predicted radio resource status

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

-	Mobility History Information.

From neighbouring gNBs:

-	Measured/Predicted radio resource status

- Measured/Predicted F1-U propagation delays

#### 5.1.2.5  Output of AI/ML-based Mobility Optimization for NR-DC

AI/ML-based mobility optimization for NR-DC can generate following output:

-	MN triggered dual connectivity procedures (e.g. SN Addition, MN-Initiated SN Change)

-	Predicted SN to add and assumptions for Dual Connectivity configuration (e.g., assumed PSCell, assumed QoS flows served via SN terminated SCG bearers)

#### 5.1.2.6	Feedback of AI/ML-based Mobility Optimization for NR-DC

The following data is required as feedback data for mobility optimization for NR-DC.

-	UE performance per PSCell while the UE is served at the SN, in terms of QoS parameters such as throughput, packet delay of the handed-over UE, etc.

-	Resource status information updates from SN.

#### 5.1.2.7	Standard impacts

Following standard impacts are listed for subsequent Rel-19 normative work.

Xn interface:

- Enhancement for Data Collection Reporting Initiation and Data Collection Reporting procedures

- Enhancement in the reporting of UE associated measurements (e.g., UE performance per cell)

- Reporting of non-UE associated performance (e.g., measured F1 propagation delay)

- Enhancements of MN-initiated dual connectivity procedures (e.g., SN Addition, MN-Initiated SN Change)

-------------------------------------------------------- Next change --------------------------------------------------------

# 6	Conclusion

The AI/ML functionality and the following use cases are recommended to be specified in Rel-19 normative phase:

-	AI/ML-based Mobility Optimization for NR-DC

Recommendations for each use case take the section of “Solutions and standard impacts” for each use case as basis.

-------------------------------------------End of changes-------------------------------------------