3GPP TSG-RAN WG3 #125	R3-244440

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda Item:	11.4

Source:	Ericsson

Title: 	Improved granularity for Energy Cost

Document for:	Discussion and Approval

# Background

During the RAN3#124 meeting, the following agreement was captured in the meeting minutes:

Measured Energy Cost can be transferred from DU to CU.

Furthermore, there were discussion about enhancing the per-node Energy Cost metric with a finer granularity level. In this matter, the following was captured in the meeting minutes:

Only discuss on finer granularity of energy cost in Aug meeting and make the conclusion.

In this contribution, we discuss a potential enhancement of the granularity of the Energy Cost metric.

# Discussion

During the RAN3#119bis-e meeting, the following agreement was captured in the meeting minutes:

It is agreed that the Energy Cost is a node level parameter. Further EC granularities are out of scope of Rel18.

Although many possible Network Energy Saving (NES) actions (e.g., cell deactivation, cell DTX/DRX) are cell-level actions, RAN3 agreed to only consider Energy Cost (EC) as a node-level parameter during Rel-18 as a way to move the discussion forward. The main problem with considering EC with finer granularity is that it is common that several hardware components are shared among cells in a gNB. At best, the gNB could try to heuristically split the energy consumption from shared hardware among the involved cells; however, this could lead to problems in interpretability of a per-cell EC.

Observation 1:  Shared hardware components among different cells in a gNB make it difficult to obtain a per-cell EC value since energy consumption can only be measured on the physical hardware.

It is important to stress that agreeing on a cell-level EC metric could potentially lead to multiple intractable problems. We recall that the EC metric may be used as input or feedback for AI/ML algorithms optimizing NES actions. In the case that a particular cell has dedicated hardware that it is not shared with other cells, the reported cell-level EC values would reflect the true outcome of any applied NES actions on that specific cell. An AI/ML algorithm that is trained with this sort of values will show a good behaviour.

However, in the case of cells which share hardware components, reporting an EC per cell would be based on an estimation of the energy consumption, not a measurement. An AI/ML algorithm that is trained with this sort of “noisy” values will behave very differently depending on the goodness of the estimation. In the best-case scenario, this could degrade the performance of the AI/ML algorithm; in the worst, it could make the AI/ML algorithm propose bad NES actions. Therefore, a per-cell EC metric should not be provided if it is not possible to measure it at hardware level.

Proposal 1:  RAN3 should not consider a cell-level EC granularity for cases where the cell is served by hardware shared by other cells as well and where the measurable EC is for such shared hardware.

The exchange of Energy Cost between gNBs enables a local gNB to assess the energy impact of an action, e.g., offloading UEs and shutting off a cell (locally or in a neighbour gNB). Thus the measured EC received from neighbouring node and produced locally helps deriving the overall energy consumption of a RAN cluster before and after taking NES actions. In this way, AI/ML algorithms can be trained and employed such that the overall Energy Cost (and energy consumption) of the RAN is decreased.

However, it can be difficult to assess the energy impact of an action, e.g., shutting off a cell or handing over some UEs, due to the coarse-grained nature of the node-level EC metric as specified in Rel-18. If a neighbour gNB has multiple cells, the energy impact of an individual action may be hardly visible in the reported EC.

A gNB’s energy consumption can be impacted by fluctuations of user data traffic and load, UE mobility (actions), e.g., handover, and concurrent NW optimization actions of various forms, among others. The energy impact of an individual action may “drown” in the noise created by all such kind of energy impacting factors to all the cells of the neighbour gNB.

Observation 2:  Node-level EC might be too coarse to provide a good understanding of the energy impact of AI/ML NES actions in gNBs with multiple cells.

A finer, yet measurable, granularity for EC may thus allow AI/ML algorithms to better understand the impact of NES actions. For example, a gNB may group together cells that share hardware components and signal such grouping to its neighbour NG-RAN nodes, namely the cells that are included within a cell group for the purpose of EC reporting will be known to neighbour NG-RAN nodes. The gNB may therefore provide a measurable EC metric for the hardware components serving a specific group of cells. This approach overcomes the issue of “estimating” the EC, since the different groups of cells do not share hardware among them; it is only the cells inside a group that share hardware components. At the same time, the group-level EC provides a finer granularity than node-level EC.

Proposal 2:  RAN3 to study the exchange of cell groups and the reporting of EC for groups of cells between gNBs.

The mapping between the cells and groups in a gNB would depend on the specific hardware that is installed and thus, it would be up to the gNB implementation to group cells in a certain way. For example, if a certain cell has dedicated hardware that is not shared with other cells, that cell could form its own group; in this way, a single-cell-level EC is possible, when the deployment and implementation permits it. The benefit of a group-level EC is that it allows flexibility to provide the finest EC granularity which is still measurable.

Proposal 3:  RAN3 to agree that reporting of EC for groups of cells provides a flexible framework that allows to signal a measurable cell-level EC when it is possible.

In a split-architecture scenario, the mapping between the cells and groups of cells would be up to the gNB-DU implementation. The measured EC would be signalled from gNB-DU to gNB-CU per group of cells, and the gNB-CU would share this information over Xn with neighbour gNBs that have subscribed to the specific cells or group of cells.

Proposal 4:  RAN3 to discuss the reporting of EC for groups of cells in split-architecture.

# Conclusions

In this contribution, we discussed a potential enhancement of the Energy Cost metric, namely an approach for improving the granularity of the Energy Cost metric, and made the following observations and proposals:

Observation 1:  Shared hardware components among different cells in a gNB make it difficult to obtain a per-cell EC value since energy consumption can only be measured on the physical hardware.

Proposal 1:  RAN3 should not consider a cell-level EC granularity for cases where the cell is served by hardware shared by other cells as well and where the measurable EC is for such shared hardware.

Observation 2:  Node-level EC might be too coarse to provide a good understanding of the energy impact of AI/ML NES actions in gNBs with multiple cells.

Proposal 2:  RAN3 to study the exchange of cell groups and the reporting of EC for groups of cells between gNBs.Proposal 3:  RAN3 to agree that reporting of EC for groups of cells provides a flexible framework that allows to signal a measurable cell-level EC when it is possible.

Proposal 4:  RAN3 to discuss the reporting of EC for groups of cells in split-architecture.

A TP reflecting the proposals and observations above is available in R3-244441.