3GPP TSG-RAN WG3 Meeting #125	R3-244441

Maastricht, Netherlands, 19th – 23rd August 2024

Title: 	TP on Improved granularity for Energy Cost

Source: 	Ericsson

Agenda item:	11.4

Document Type:	Discussion and decision

# TP to TR 38.743

This TP mirrors the discussions and proposals in R3-244440.

## 5.3	Energy saving enhancements

Editor Note: Capture the description and its potential standard impacts.

------------------------------------------- Start of changes -------------------------------------------

### 5.3.1	Use case description

A description of the AI/ML-based Network Energy Saving use case as such is presented in TR 37.817. Based on the conclusions in TR 37.817, a metric called Energy Cost was introduced during Rel-18 normative work; this metric is defined as an index representation of the energy consumption at the NG-RAN.

In Rel-18, the metric of Energy Cost can be exchanged between NG-RAN nodes over the Xn interface with a per NG-RAN node granularity. However, a node-level Energy Cost metric might be too coarse to provide a good understanding of the energy impact of AI/ML-based network energy saving actions in NG-RAN nodes with multiple cells. An improvement of the granularity of Energy Cost enables the NG-RAN to take better network energy saving decisions.

### 5.3.2	Solutions and standard impacts

Some solutions and standard impacts of the AI/ML-based Network Energy Saving use case are presented in TR 37.817.

During Rel-18 normative work, the Energy Cost (EC) metric was defined on an NG-RAN node-level granularity even though many possible Network Energy Saving (NES) actions (e.g., cell deactivation, cell DTX/DRX) are cell-level actions. The main problem with considering EC with finer granularity is that it is common that several hardware components are shared among cells in an NG-RAN node. Since energy consumption is measured at the hardware level, a cell-level EC metric may only be an estimation after heuristically splitting the energy consumption from shared hardware among the involved cells.

A finer, yet measurable, granularity for the EC metric is achieved by grouping together cells that share hardware components and exchange between gNBs information concerning the cells forming each group as well as the EC metric at cell group level. This approach overcomes the issue of estimating the EC, since the different groups of cells do not share hardware among them; it is only the cells inside a group that share hardware components and for that the per cell group EC can be measured. At the same time, the group-level EC provides a finer granularity than node-level EC.

The following additional input and feedback is needed for AI/ML-based Network Energy Saving, and the resulting standard impacts with respect to what was specified in Rel-18 are listed for subsequent Rel-19 normative work.

#### 5.3.2.1	Additional input for AI/ML-based Network Energy Saving

To predict the optimized network energy saving decisions, NG-RAN may need the following additional information as input for AI/ML-based network energy saving:

Input information from local NG-RAN node:

- Measured Energy Cost for a group of cells

Input information from neighbouring NG-RAN nodes:

- Measured Energy Cost for a group of cells

#### 5.3.2.2	Additional feedback for AI/ML-based Network Energy Saving

To optimize the performance of an AI/ML-based network energy saving model, the following additional feedback may be collected from NG-RAN nodes:

- Measured Energy Cost for a group of cells

#### 5.3.2.3	Standard impacts

Potential Xn interface impact:

- Measured Energy Cost per groups of cells

- A description of the groups of cells sharing an Energy Cost

------------------------------------------- End of changes ------------------------------------------