3GPP TSG-RAN WG3 Meeting #125	R3-244442

# Maastricht, Netherlands, 19 – 23 August, 2024

Agenda item:	11.2

Source:	Nokia, Jio

Title:	AI/ML Network Slicing Discussion

Document for:	Discussion

# 1	Introduction

Several agreements have been made in previous meetings related to AI/ML slicing use case. A good starting point is captured in TR 38.743. During RAN3#123bis, initial agreements were reached on AI/ML slicing for both non-split and split architecture:

-	AI/ML Model Training may be located in the OAM and AI/ML Model Inference may be located in the NG-RAN node (gNB-CU).

-	AI/ML Model Training and AI/ML Model Inference may be both located in the NG-RAN node (gNB-CU).

Also in RAN3 #124 we made the following agreement:

Agree to improve the granularity of UE performance feedback in Rel-19 for slicing.

This agreement was captured in the TR with an FFS related to feedback of AI/ML based network slicing:

Finer granularity UE performance feedback for those UEs handed over from the source gNB. FFS on the exact level of finer granularity.

In RAN3 #124 we also captured the following open points:

FFS whether to include measured/predicted UE throughput per slice as input from local node.

FFS whether to introduce delivery of slice level UE throughput prediction/measurement during handover procedure.

FFS on SLA requirements fulfilment information exchange.

In this paper, we provide our views on remaining open points of the AI/ML Slicing solution. We provide a TP to TR 38.743, reflecting the proposals in this paper, in [2].

# 2	Addressing remaining open points

In this section, we address the open points captured in RAN3 #124. The first open point pertains to “FFS whether to include measured/predicted UE throughput per slice as input from local node.”. The proponents of this aspect claimed that using measured or predicted UE throughput per slice information at the local node as input could be beneficial for the AI/ML model in order to optimize RAN slice resource allocation, namely increase or decrease resources for a slice effectively in real-time. From TS 28.552, measured UE throughput per slice in the UL or DL direction are available at the local node.

Observation 1: UE throughput per slice can be measured at the local node.

However, behaviour of an individual UE is unpredictable, so providing a prediction of  the UE throughput at the target node has in our view very limited benefit.

Observation 2: Predicted UE throughput per slice may not be always feasible to calculate.

Another open point from the previous meeting is: “FFS whether to introduce delivery of slice level UE throughput prediction/measurement during handover procedure.”. The rationale behind this proposal was that slice level UE throughput information could assist the target node in deciding whether to accept the handed-over UE or not. However, the measured UE traffic at the source node may be completely different from the traffic at the target node. Similarly, the source node may not be able to accurately predict the traffic per UE in the target node since the source node does not have information about the UE behaviour in the target.

Observation 3: It is not feasible for the source node to predict slice level UE throughput at the target node.

On the other hand, in our view, traffic information at the local node could be useful for resource allocation, but does not need to be per UE granularity. As brought up by several companies in previous meetings, from RAN3 standardization perspective the main reason for an NG-RAN node not to meet target requirements derived from the SLA in terms of performance (e.g. UE throughput, latency) would be an overload situation. Such overload may be prevented by access control and pre-emption mechanisms. So, an NG-RAN node should also be enabled to avoid slice overload by optimized resource planning based on predicted information about slice traffic. Predicted slice traffic information, or load per slice, may be locally available today in the NG-RAN node but there is currently no mechanism for an NG-RAN node to collect information about predicted incoming traffic from neighbouring nodes. Having information on the actual traffic that is predicted to be handled by a node enables this node to allocate its slice resources proactively in order to accommodate all the predicted incoming traffic, e.g. for high-mobility UEs, and anticipate resource management between slices to ensure that the required resources are available when needed. Absence of such information can make it challenging to e.g. ensure the continuity of sliced services for high-mobility UEs.

Observation 4: Overall traffic information at the local node could be useful for resource allocation.

Proposal 1: We propose to exchange the overall predicted traffic per slice between NG-RAN nodes in order to enhance a node’s ability to anticipate incoming traffic from different network nodes.

Another open point relates to the following: FFS on SLA requirements fulfilment information exchange.

During previous RAN3 meetings, it was discussed that the exchange of measured/predicted slice SLA requirement fulfilment, in addition to the measured/predicted slice available capacity, between RAN nodes is required to enable the source node to offload a portion of its traffic to a neighbouring node, thereby improving the service of the RAN slice affected by poor performance. However, an open question remains: What are the scenarios where a RAN slice is not overloaded, yet SLA requirements are not met?

One such scenario discussed was when a UE’s SLA is not met because even though the RAN node is not overloaded the UE is located in difficult radio conditions, e.g. at the cell edge. This scenario, however, does not seem to relate to a slicing issue, but rather to a CCO issue or RRM issue. In such cases, the UE may need to be handed over to a neighboring node with better coverage or the CCO algorithm at the node serving the UE needs to be adjusted to improve UE performance. Also, even though such situations may exist in a cell for one or more UEs, it doesn’t mean that the cell is not suitable as target in the general case. A source node may also receive UE performance feedback from the target cell, and in this way learn in which situations a neighbour cell is not a suitable target cell. Exchange of measured/predicted slice SLA requirement fulfilment will not be beneficial for a source cell in this case.

Another scenario discussed was that the F1-U delay may result in SLA requirements not being met even though the RAN slice is not overloaded. We believe that high F1-U delay could either come as a result of high load in the transport network used for the fronthaul, or the F1-U delay would not be load related but rather related to the chosen deployment options. In the first case, the definition of Composite Available Capacity, which represents the overall available resource level in the cell in either Downlink or Uplink, will also take into account load levels in the transport network and hence load-related F1-U delays will be covered. However, if the F1-U delay is not related to load, but relates to other factors such as deployment options chosen by the operator, mitigation of this issue is not in the realm of the RAN and should be handled via operator intervention instead. Therefore, slice available capacity, derived based on the composite available capacity, is sufficient for target cell selection, hence eliminating the need for the exchange of SLA requirement fulfilment information over Xn.

Related to this topic some companies discussed the potential benefits of signaling to the OAM per gNB measured/predicted slice requirement fulfilment levels. However, OAM has already defined many performance measurements (TS 28.552) and performance KPIs (TS 28.554) related to RAN slicing that are reported by the gNB. These are sufficient for OAM to observe if RAN slice SLA requirements are fulfilled or not.

Observation 5: The current load metric to evaluate slice performance is sufficient.

Finally, an FFS that we would like to address is related to the finer granularity of UE Performance feedback for UEs that are being handed over from a source gNB to a target.

In Rel.18, UE Performance feedback is introduced as a mechanism to provide a measured UE Performance from a target node to a source node, in terms of an Average UE Throughput in UL or DL direction, Average Packet Delay and Average Packet Loss DL of a handed over UE. However, the reported UE performance is measured to be the total average UE Performance. In practice, a UE may support multiple slices. Each slice is associated with one or more PDU session that provides the path for user data transfer. Each PDU session may support a number of QoS flows, which may receive different treatment within the slice. However, currently only legacy UE performance feedback is agreed as feedback information towards the AI/ML model. It seems clear that reporting the overall UE Performance (e.g. average over all slices) does not provide sufficient information on the impact of the handover on the performance of different PDU sessions (slices) or QoS flows. For the purpose of AI/ML for slicing for the handover scenario, we therefore support a finer granularity of UE Performance, e.g. per PDU session or per QoS flow. However, as we discuss in [1], UE performance reporting with DRB granularity would be needed to support NR-DC scenarios. As a consequence, taking into account possible remapping of QoS flows to DRBs in the target node, UE performance reporting per QoS flow might be a good way forward.

Proposal 2: Enhance measured UE Performance reporting to also provide finer granularity of reporting per QoS flow.

Proposal 3: Agree the TP to TR 38.743 in [2].

# 3	Conclusion

In this contribution, we make the following observations and proposals:

Observation 1: UE throughput per slice can be measured at the local node.

Observation 2: Predicted UE throughput per slice may not be always feasible to calculate.

Observation 3: It is not feasible for the source node to predict slice level UE throughput at the target node.

Observation 4: Overall traffic information at the local node could be useful for resource allocation.

Proposal 1: We propose to exchange the overall predicted traffic per slice between NG-RAN nodes in order to enhance a node’s ability to anticipate incoming traffic from different network nodes.

Observation 5: The current load metric to evaluate slice performance is sufficient.

Proposal 2: Enhance measured UE Performance reporting to also provide finer granularity of reporting per QoS flow.

Proposal 3: Agree the TP to TR 38.743 in [2].

# References

[1]		R3-244443, Discussion on AI/ML Mobility Optimization for NR-DC, Nokia

[2]	R3-244472, (TP to TR 38.743) AI/ML Network Slicing, Nokia, Jio