3GPP TSG-RAN WG3 Meeting #125	R3-244443

Maastricht, Nedherlands, 18 – 23 August, 2024

Agenda item:	11.4

Source:	Nokia

Title:	Discussion on AI/ML Mobility Optimization for NR-DC

Document for:	Discussion

# 1	Introduction

In RAN3#123bis, we made the following agreement for handling of Mobility optimization for NR-DC as Rel-18 leftover in Rel-19:

Agree to take as baseline for the Mobility Optimization for NR-DC that the use case is studied assuming inference at the MN. The main use case is limited to dual connectivity only (e.g. no conditional procedures are in scope).

In addition, in RAN3#124 we made the following agreement related to Mobility optimization for NR-DC as Rel-18 leftover in Rel-19:

Enhance the Dual Connectivity procedure (e.g., MN-initiated SN addition, MN-initiated SN change) to trigger the collection of measured UE performance.

In RAN3#124 we also discussed under AI 9.2, based on [1], several issues that could motivate Rel-18 corrections for UE performance measurements in handover scenarios with multiple DRBs configured in the UE. The discussion also has relevance for UE performance monitoring in NR-DC scenarios. On top of this discussion, we further analyse UE performance feedback in case of NR-DC scenarios, and propose a way forward for closure of the present study item and follow-up in normative phase. We provide a TP to TR 38.743, reflecting the proposals of this paper, in [2].

# 2	Discussion

## 2.1	Scenarios

The agreement from RAN3#123bis indicates that, as a baseline, the present use case assumes Model Inference at the MN, and, hence, Model Inference at the SN is not considered. However, in the current agreements it is not clear whether it is possible for a target gNB to enable DC, while UE Performance monitoring is requested from it by the source gNB. Even though this scenario does not require inference neither in the (target) MN, nor in the SN, and might be considered to enhance support for handover optimization, and not NR-DC optimization, we believe the strong link to NR-DC makes it reasonable to handle this scenario as part of the NR-DC optimization use case.

Proposal 1: Include as part of the Mobility optimization for NR-DC use case the scenario of SN addition in the target gNB monitoring UE performance requested by the source gNB.

## 2.2	Bearer Concept for NR-DC

In 5G, we use a PDU Session to provide end-to-end user plane connectivity between the UE and a specific Data Network  through the User Plane Function (UPF). A PDU Session supports one or more QoS Flows. There is a one-to-one mapping between a QoS Flow and a QoS profile, i.e. all packets belonging to a specific QoS Flow have the same “5G Quality of Service Identifier” (5Ql). There are three bearer types: MCG bearer, SCG bearer and split bearer. These three bearer types are depicted in Figure 1 for MR-DC with 5GC (NG-RAN E-UTRA-NR Dual Connectivity – NGEN-DC, NR-E-UTRA Dual Connectivity - NE-DC and NR-DC), as sourced from TS 37.340.

Figure 1: Radio Protocol Architecture for MCG, SCG and split bearers from a UE perspective in MR-DC with 5GC (NGEN-DC, NE-DC and NR-DC) – source: TS 37.340.

From a network perspective, each bearer (MCG, SCG and split bearer) can be terminated either in MN or in SN. Network side protocol termination options are shown in Figure 2 for MR-DC with 5GC (NGEN-DC, NE-DC and NR-DC) as sourced from TS 37.340.

Figure 2: Network side protocol termination options for MCG, SCG and split bearers in MR-DC with 5GC (NGEN-DC, NE-DC and NR-DC) – source: TS 37.340.

## 2.3	UE Performance Feedback metrics from Rel-18

In Rel. 18, as part of the AI/ML in NG-RAN use case of Mobility Optimization, it was agreed to report the UE performance feedback from target NG-RAN node to source NG-RAN node after successful Handover. The actual triggering of the UE Performance measurement is performed through a Handover event when a UE is being handed over from a source NG-RAN node to a target NG-RAN node. The intention was to use this information related to QoS provided by the UE shortly after a successful handover to validate the handover decision as an output of the AI/ML model in the source NG-RAN node. The UE performance Feedback contains the following metrics (ref. clause 9.2.3.179 of TS 38.423):

- Average UE throughput in UL and DL

- Average Packet delay in UL and DL

- Average Packet loss in DL

For Rel-19, RAN3#124 agreed to enable use of UE performance feedback also to validate decisions relative to NR-DC:

Enhance the Dual Connectivity procedure (e.g., MN-initiated SN addition, MN-initiated SN change) to trigger the collection of measured UE performance.

As discussed in [1], Rel-18 specification doesn’t define how the UE performance measurements are obtained and there is no reference to specification of existing metric (TS 38.314, TS 28.552). It is also observed in [1] that e.g. the standardized UE throughput measurement is measured based on RLC SDUs and hence done at DRB level. Still, there was no consensus at last meeting to proceed with corrections of the UE performance measurements on this aspect in Rel-18, but the following was minuted as a common understanding:

UE level performance is derived as the average value of per DRB level measurements of the UE.

However, several companies believed such average, even if weighted according to some criterion, is not a good way forward to obtain an exploitable UE performance metric, e.g. based on the following example from [1]:

Assume that there are two flows for a UE, one requires strict packet delay / packet loss whereas the other requires loose one.

- Before handover the former flow is in high bit rate whereas the latter flow is in low bit rate—as a result, the “correct” UE-level packet delay / data loss is probably quite good.

- After handover, the bit rate of the former flow turns low whereas the one of the latter flow turns high—as a result the “correct” UE-level packet delay / data loss probably “deteriorates” significantly, even though the service level for each flow is entirely kept the same.

Furthermore, UE performance feedback may be shared between two NG-RAN nodes of different vendors, and implementation-based solutions could lead to improper actions. As can be seen from the discussion in section 2.4, support of UE performance feedback in NR-DC scenarios provides additional reasons to clarify ambiguity in the standardized UE Performance measurements.

## 2.4	UE Performance Feedback for different NR-DC scenarios

One specific aspect of NR-DC is split bearers with data transmission on two legs (MCG and SCG) from RLC to physical layer and their common handling on PDCP layer in MN or SN node for MN and SN terminated bearers, respectively, as is depicted in Fig. 3a and 3b.

Figure 3a: Possible options (green colour) for UE performance feedback collection in the case of MN-terminated split bearer (blue highlighted path). Left hand side: UE performance measurements are collected at the RLC layer. Right hand side: UE performance measurements are collected at the PDCP layer in the MN.

Figure 3b: Possible options (green colour) for UE performance feedback collection in the case of SN-terminated split bearer (blue highlighted path). Left hand side: UE performance measurements are collected at the RLC layer. Right hand side: UE performance measurements are collected at the PDCP layer in the SN.

A similar situation exists for the cases of MN-terminated SCG bearer and SN-terminated MCG bearers, where, for the first case, RLC, MAC and physical layer is in SN, whereas PDCP layer in MN, while, for the second case, RLC, MAC and physical layer is in MN, whereas PDCP layer in SN, as depicted in Fig. 4a and 4b, respectively.

Figure 4a: Possible options (green colour) for UE performance feedback collection in the case of MN-terminated SCG bearer (blue highlighted path). Left hand side: UE performance measurements are collected at the RLC layer in the SN. Right hand side: UE performance measurements are collected at the PDCP layer in the MN.

Figure 4b: Possible option (green colour) for UE performance feedback collection in the case of SN-terminated MCG bearer (blue highlighted path), collected at the PDCP layer in the SN.

Observation 1: The processing layers for split bearer, MN terminated SCG bearer, and SN terminated MCG bearer cases are not within the same node.

In Fig. 5, SN terminated SCG bearer case is shown. As it may be seen all processing layers are within the SN node.

Figure 5: Possible options (green colour) for UE performance feedback collection in the case of SN-terminated SCG bearer (blue highlighted path). Left hand side: UE performance measurements are collected at the RLC layer in the SN. Right hand side: UE performance measurements are collected at the PDCP layer in the SN.

Observation 2: The processing layers for SN terminated SCG bearer are within the same node.

In the following part, we look at options of how to collect UE performance measurements for the NR-DC bearer types described above.

Packet delay:

RAN packet delay is composed of several components as described in TS 38.314.

For the DL direction:

-	D1 (DL delay in over-the-air interface), referring to Average delay DL air-interface in TS 28.552 [2] 5.1.1.1.1.

-	D2 (DL delay on gNB-DU), referring to Average delay in RLC sublayer of gNB-DU in TS 28.552 [2] 5.1.3.3.3.

-	D3 (DL delay on F1-U), referring to Average delay on F1-U in TS 28.552 [2] 5.1.3.3.2.

-	D4 (DL delay in CU-UP), referring to Average delay DL in CU-UP in TS 28.552 [2] 5.1.3.3.1.

For the UL direction:

-	D1 (UL PDCP packet average delay, as defined in clause 4.3.1.1).

-	D2.1 (average over-the-air interface packet delay, as defined in 4.2.1.2.2).

-	D2.2 (average RLC packet delay, as defined in 4.2.1.2.3).

-	D2.3 (average delay UL on F1-U, it is measured using the same metric as the average delay DL on F1-U defined in TS 28.552 [2] clause 5.1.3.3.2).

-	D2.4 (average PDCP re-ordering delay, as defined in 4.2.1.2.4).

It can be seen that some of these components (DL: D1, D2; UL: D2.1, D2.2) can be measured at the corresponding node, i.e. the node hosting RLC, and the other components, with the notable exception of UL D1, can be measured at the node hosting the PDCP entity. UL D1 is measured by the UE and signalled via RRC to the MN (over SRB1) and/or to the SN (over SRB3). However, a framework was introduced in Rel-16 for QoS monitoring enabling delay components to be transferred from the corresponding node to the node hosting PDCP via the user plane (TS 38.425). Based on this framework, the full packet delay information will be available in the MN for all MN-terminated bearers. For transfer of packet delay information from the SN to the MN for SN-terminated bearers a natural solution seems to be to enhance the Rel-18 Xn signalling to provide packet delay measurement per DRB (and not e.g. at UE level).

Proposal 2: For transfer of packet delay information from the SN to the MN for SN-terminated bearers enhance the Rel-18 Xn signalling to provide packet delay measurement per DRB.

Average UE throughput:

The UL/DL average UE throughput measurements defined in TS 28.552 are intended to reflect the throughput of data bursts that are large enough to require transmissions to be split across multiple slots and therefore need to be measured at the RLC layer (RLC SDU level). If the principle of this definition is followed for UE performance measurements in NR-DC scenarios, the MN will have UL/DL average UE throughput measurements available in own node for both MN-terminated and SN-terminated MCG bearers, but will need to request the SN to report UE throughput for SCG bearers.

It should still be noted that TS 28.552 “define[s] L2 measurements that is aggregated and often reported per a Managed Object class (e.g. NRCellDU)”. The natural place to define such measurement would in our view be TS 38.314, under RAN2’s responsibility. Also, TS 28.552 clause 5.1.0 “Relation to RAN L2 measurement specification” indicates:

-	The measurement definitions in TS 38.314 [29] are often defined to be reported 'per UE or per DRB', to support MDT and Trace use cases.

The current scope of TS 38.314 is “description and definition of the measurements performed by network or the UE that are transferred over the standardised interfaces in order to support NR radio link operations, radio resource management (RRM), network operations and maintenance (OAM), minimization of drive tests (MDT) and self-organising networks (SON)” (see TS 38.314 clause 1).  It therefore seems needed to involve RAN2 in Rel-19 normative phase for definition of the UE throughput measurement for UE performance and potential enhancement of TS 38.314 for this purpose.

Proposal 3: RAN3 to recommend RAN2 to be involved in Rel-19 normative phase for the definition of UE throughput measurement for UE performance per RLC SDU level in TS 38.314.

Average Packet Loss:

A measurement for DL air interface packet loss rate is defined in TS 38.314 (Packet Uu Loss Rate in the DL per DRB per UE), measured at the RLC-SDU level (gNB-DU). TS 28.552 additionally defines specific measurements for F1-U packet loss rate for DL.

In order to determine signalling impact of DL packet loss rate measurements in NR-DC scenarios, RAN3 needs to first determine the interfaces and protocol layers to be included by these measurements.

Proposal 4: RAN3 to discuss and agree in Rel-19 normative phase the interfaces and protocol layers to be included by DL average packet loss measurements in NR-DC scenarios.

Depending on the outcome of this discussion, RAN2 involvement for additional documentation of packet loss rate measurements for UE performance could be needed.

Observation 3: RAN2 involvement for additional documentation of packet loss rate measurements for UE performance could be needed in Rel-19 normative phase.

Proposal 5: Agree the TP to TR 38.743 in [2].

# 3	Conclusion

In this paper we make the following observations and proposals:

Proposal 1: Include as part of the Mobility optimization for NR-DC use case the scenario of SN addition in the target gNB monitoring UE performance requested by the source gNB.

Observation 1: The processing layers for split bearer, MN terminated SCG bearer, and SN terminated MCG bearer cases are not within the same node.

Observation 2: The processing layers for SN terminated SCG bearer are within the same node.

Proposal 2: For transfer of packet delay information from the SN to the MN for SN-terminated bearers enhance the Rel-18 Xn signalling to provide packet delay measurement per DRB.

Proposal 3: RAN3 to recommend RAN2 to be involved in Rel-19 normative phase for the definition of UE throughput measurement for UE performance per RLC SDU level in TS 38.314.

Proposal 4: RAN3 to discuss and agree in Rel-19 normative phase the interfaces and protocol layers to be included by DL average packet loss measurements in NR-DC scenarios.

Observation 3: RAN2 involvement for additional documentation of packet loss rate measurements for UE performance could be needed in Rel-19 normative phase.

Proposal 5: Agree the TP to TR 38.743 in [2].

# References

[1]	R3-243578, Discussion on definition of UE performance metrics, CATT, Deutsche Telekom, Nokia, China Telecom, China Unicom.

[2]	R3-244444, (TP to TR 38.743) Mobility Optimization for NR-DC, Nokia.