3GPP TSG-RAN WG3 Meeting #125	R3-244444

Maastricht, Nedherlands, 18 – 23 August, 2024

Agenda item:	11.4

Source:	Nokia

Title:	(TP to TR 38.743) Mobility Optimization for NR-DC

Document for:	Text Proposal

# 1	Introduction

In this paper we provide a TP to TR 38.743 based on the discussions in R3-244443.

# Annex TP for TR 38.743

<<< start of changes >>>

# 5	Rel-18 Leftovers and solutions

Editor Note: Such topics are listed here for further selection/down selection for normative work.

<<< skip unchanged text >>>

## 5.1	Mobility optimization for NR-DC

Editor Note: Capture the description and its potential standard impacts.

### 5.1.1 Use case description

Mobility Optimization for NR-DC is studied by assuming inference at the MN only. The main use case is limited to Dual Connectivity only (e.g., no conditional Dual Connectivity procedures are in scope).

#### 5.1.1.1	UE Performance Feedback for different NR-DC scenarios

One specific aspect of NR-DC is split bearers with data transmission on two legs (MCG and SCG) from RLC to physical layer and their common handling on PDCP layer in MN or SN node for MN and SN terminated bearers, respectively, as is depicted in Fig. 5.1.1.1-a and 5.1.1.1-b.

Figure 5.1.1.1-a: Possible options (green colour) for UE performance feedback collection in the case of MN-terminated split bearer (blue highlighted path). Left hand side: UE performance measurements are collected at the RLC layer. Right hand side: UE performance measurements are collected at the PDCP layer in the MN.

Figure 5.1.1.1-b: Possible options (green colour) for UE performance feedback collection in the case of SN-terminated split bearer (blue highlighted path). Left hand side: UE performance measurements are collected at the RLC layer. Right hand side: UE performance measurements are collected at the PDCP layer in the SN.

A similar situation exists for the cases of MN-terminated SCG bearer and SN-terminated MCG bearers, where, for the first case, RLC, MAC and physical layer is in SN, whereas PDCP layer in MN, while, for the second case, RLC, MAC and physical layer is in MN, whereas PDCP layer in SN, as depicted in Fig. 5.1.1.1-c and 5.1.1.1-d, respectively.

Figure 5.1.1.1-c: Possible options (green colour) for UE performance feedback collection in the case of MN-terminated SCG bearer (blue highlighted path). Left hand side: UE performance measurements are collected at the RLC layer in the SN. Right hand side: UE performance measurements are collected at the PDCP layer in the MN.

Figure 5.1.1.1-d: Possible option (green colour) for UE performance feedback collection in the case of SN-terminated MCG bearer (blue highlighted path), collected at the PDCP layer in the SN.

In Fig. 5.1.1.1-e, SN terminated SCG bearer case is shown. As it may be seen all processing layers are within the SN node.

Figure 5.1.1.1-e: Possible options (green colour) for UE performance feedback collection in the case of SN-terminated SCG bearer (blue highlighted path). Left hand side: UE performance measurements are collected at the RLC layer in the SN. Right hand side: UE performance measurements are collected at the PDCP layer in the SN.

### 5.1.2	Potential standard impacts

The following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

Potential stage 2 impact:

- Define the interfaces and protocol layers to be included by DL packet loss rate measurements in NR-DC scenarios.

Note: RAN3 recommends RAN2 involvement in normative phase for documentation of measurements for UE performance.

Potential Xn interface impact:

-	Enhance the Dual Connectivity procedures (e.g., MN-initiated SN addition, MN-initiated SN change) to trigger the collection of measured UE performance.

-	Enhance UE performance feedback reporting for SN addition/removal and SCG change procedures related to split, MN-terminated SCG bearer, and SN-terminated MCG bearer cases

-	Enhance UE Performance feedback reporting for HO scenario with SN addition at the target.

<<< end of changes >>>