3GPP TSG-RAN WG3 Meeting #125	R3-244445

# Maastricht, Netherlands, 19 – 23 August, 2024

Agenda item:	11.4

Source:	Nokia, Telecom Italia, Huawei

Title:	On the Support of Split Architecture for the Rel-18 AI/ML Use Cases

Document for:	Discussion

# 1	Introduction

In the last meeting we made the following agreements with respect to split architecture:

Transferring Measured UE performance from CU-UP to CU-CP.

Measured Energy Cost can be transferred from DU to CU.

In this contribution we provide further views related to the UE Performance measurements. We also provide our views on the expected standardization impacts over F1 and E1 interfaces. A TP to TR 38.743, reflecting the proposals in this paper, is provided in [1].

# 2	Discussion

## 2.1 Reporting of Energy Cost over F1

According to the agreement from the previous meeting a measured Energy Cost can be transferred from gNB-DU to gNB-CU. This means that F1AP signalling procedures are needed in order to enable a gNB-CU to configure an Energy Cost measurement from a gNB-DU and to subsequently enable also the reporting of the Energy Cost measurement from gNB-DU to gNB-CU. During Rel-18, we had identified two possible options:

- Reuse existing signalling procedures e.g., Resource Status Reporting Initiation and Resource Status Reporting procedures over F1 interface

- Introduce new procedures to align F1AP signalling procedures with the Data Collection Reporting Initiation and Data Collection Reporting procedures already defined in XnAP.

In our view, a clean way is to introduce new procedures and align F1AP with the Data Collection Reporting Initiation and Data Collection Reporting procedures defined in Xn interface. This would also be a more futureproof way forward that could enable to transfer other information related to AI/ML, possibly even related to new use cases, in the future. However we suggest detailed agreements on signalling procedures to take place in normative phase.

Proposal 1: Choice of signalling procedures for reporting of Energy Cost over F1 to take place in normative phase.

## 2.2 Procedures for configuration and reporting of UE Performance Feedback over E1 and F1

In the last meeting we also agreed to transfer UE Performance measurements from gNB-CU-UP to gNB-CU-CP. The UE Performance measurements defined in Rel-18 (UL/DL average UE throughput, average packet delay and average packet loss in the DL) can be measured in the gNB-DU and/or in the gNB-CU-UP. In some cases, both gNB-DU and gNB-CU-UP can measure (different instances of) UE Performance measurements e.g., in case of average packet delay measurements. In particular:

- Average UE Throughput UL: Is described in TS 28.552, clause 5.1.1.3.3 and provides the average UE throughput in uplink for data bursts that are large enough to require transmissions to be split across multiple slots. This measurement is measured by the gNB-DU.

- Average UE Throughput DL: Is described in TS 28.552, clause 5.1.1.3.1 and provides the average UE throughput in downlink for data bursts that are large enough to require transmissions to be split across multiple slots. This measurement is measured by the gNB-DU.

- Average Packet Delay: It includes the RAN part of the average packet delay in the uplink and downlink directions. In case of Average packet delay in DL it is the sum of D1+ D2+ D3+ D4 where those are defined as follows:

  - D1 (DL delay in over-the-air interface): This is the Average delay in the DL, providing the average time it takes for packet transmission over the air interface. It is measured at the gNB-DU. It is described in TS 28.552, clause 5.1.1.1.1.

  - D2 (DL delay on gNB-DU): This is the average RLC SDU delay in the DL within the gNB-DU, for initial transmission of all RLC packets. It is measured at the gNB-DU. It is described in TS 28.552, clause 5.1.3.3.3.

  - D3 (DL delay on F1-U): This is the average GTP packet delay DL on the F1-U interface. It is measured at the gNB-CU-UP. It is described in TS 28.552, clause 5.1.3.3.2.

  - D4 (DL delay in CU-UP): This is the average PDCP SDU delay on the downlink within the gNB-CU-UP, for all PDCP packets. It is measured at the gNB-CU-UP. It is described in TS 28.552, clause 5.1.3.3.1.

In case of Average Packet Delay in UL it is the sum of D1+ D2.1+ D2.3+ D2.4, where those are defined as follows:

  - D1 (UL PDCP packet average delay): This is the average PDCP delay in the UL. It is defined in TS 38.314, clause 4.3.1.1 and is measured by the gNB-CU-UP.

  - D2.1 (average over-the-air interface packet delay): This is the air interface UL delay. It is defined in TS 38.314, clause 4.2.1.2.2. It is measured at the gNB-DU.

  - D2.2 (average RLC packet delay): It measures RLC packet delay in the UL. It is defined in TS 38.314, clause 4.2.1.2.3. It is measured at the gNB-DU.

  - D2.3 (average delay UL on F1-U): This is the average GTP packet delay UL on the F1-U interface. It is measured by the gNB-CU-UP. It is described in TS 28.552, clause 5.1.3.3.2.

  - D2.4 (average PDCP re-ordering delay in the UL): It measures PDCP re-ordering delay in the UL. It is measured at the gNB-CU-UP. It is defined in TS 38.314, clause 4.2.1.2.4.

- Average Packet Loss in the DL: According to TS 38.314, clause 4.2.1.5.1, Packet Uu Loss Rate in the DL per DRB per UE is measured by the gNB-DU. Additionally, TS 28.552 clause 5.1.3.1.3 also describes DL packet loss over F1-U interface, measured at the gNB-DU and defined to be the fraction of PDCP SDU packets which are not successfully received at the gNB-DU.

Observation 1: Some UE Performance measurements are measured at the gNB-DU and some at the gNB-CU-UP.

In order to enable gNB-CU-CP to have visibility to UE Performance measurements to report this information to neighbouring nodes, in the previous meeting we agreed that UE Performance measurements are sent from gNB-CU-UP to gNB-CU-CP. However, as can be seen above, the majority of the measurements are measured at the gNB-DU only and gNB-CU-UP does not have visibility to those measurements.

Observation 2: The majority of UE Performance measurements (i.e., Average UE Throughput UL/DL, Average Packet Loss DL) are measured at the gNB-DU only and gNB-CU-UP does not have visibility to those measurements.

According to TS 28.552, Average UE Throughput UL/DL and Average Packet Loss in the DL are both measured at the gNB-DU. Therefore, the simplest way to make this information available to gNB-CU-CP is to send it directly over F1 interface.

Observation 3: The simplest way to send Average UE Throughput UL/DL and Average Packet Loss in the DL is over F1 interface since they are both measured at the gNB-DU.

Proposal 2: Send Average UE Throughput UL/DL and Average Packet Loss in the DL from gNB-DU to gNB-CU-CP.

The situation is a bit more complex when it comes to the Average Packet Delay UL/DL since the total measurement comprises different components, some of which are measured in the gNB-DU and some of which are measured in the gNB-CU-UP. Currently, there is already signalling defined for aggregation of the packet delay measurement in the CU-UP for sending it to the UPF (TS 38.425 UL Delay DU Result, DL Delay DU Result). Therefore it is possible that gNB-CU-CP retrieves the aggregated Average Packet Delay measurement from gNB-CU-UP. As another alternative, the different entities of the gNB (gNB-DU and gNB-CU-UP) can independently send their corresponding components of the Average Packet Delay to gNB-CU-CP over F1 and E1 respectively and it would be the responsibility of gNB-CU-CP to perform the measurement aggregation. We suggest these options for packet delay measurement to be further evaluated in normative phase, taking into account stage 3 impacts for both options.

Proposal 3: For Average Packet Delay measurement evaluate in normative phase 2 options:

- gNB-CU-CP retrieves the aggregated Average Packet Delay measurement from gNB-CU-UP.

- gNB-CU-CP retrieves separately the components of the delay that are measured in the gNB-DU and in the gNB-CU-UP and performs the aggregation.

Proposal 4 : Introduce in TR 38.743 the following objective:

- Enable the gNB-CU(-CP) to request and retrieve UE Performance measurements from gNB-DU.

The previously agreed text for the use case description of split architecture support for Rel-18 use cases has been lacking a clear use case description. Therefore, we propose a text proposal for completeness in [1], which provides also a text proposal for the potential standards impacts we envision.

Proposal 5: Agree the TP to TR 38.743 in [1].

# 3	Conclusion

In this contribution, we make the following observations and proposals:

Proposal 1: Choice of signalling procedures for reporting of Energy Cost over F1 to take place in normative phase.

Observation 1: Some UE Performance measurements are measured at the gNB-DU and some at the gNB-CU-UP.

Observation 2: The majority of UE Performance measurements (i.e., Average UE Throughput UL/DL, Average Packet Loss DL) are measured at the gNB-DU only and gNB-CU-UP does not have visibility to those measurements.

Observation 3: The simplest way to send Average UE Throughput UL/DL and Average Packet Loss in the DL is over F1 interface since they are both measured at the gNB-DU.

Proposal 2: Send Average UE Throughput UL/DL and Average Packet Loss in the DL from gNB-DU to gNB-CU-CP.

Proposal 3: For Average Packet Delay measurement evaluate in normative phase 2 options:

- gNB-CU-CP retrieves the aggregated Average Packet Delay measurement from gNB-CU-UP.

- gNB-CU-CP retrieves separately the components of the delay that are measured in the gNB-DU and in the gNB-CU-UP and performs the aggregation.

Proposal 4 : Introduce in TR 38.743 the following objective:

- Enable the gNB-CU(-CP) to request and retrieve UE Performance measurements from gNB-DU.

Proposal 5: Agree the TP to TR 38.743 in [1].

# References

[1]		R3-244446, (TP to TR 38.743) Split Architecture Support for the Rel-18 AI/ML Use Cases, Nokia, Telecom Italia, Huawei