3GPP TSG-RAN WG3 Meeting #125	R3-244447

# Maastricht, Netherlands, 19 – 23 August, 2024

Agenda item:	11.4

Source:	Nokia, Telecom Italia, T-Mobile USA, BT

Title:	Finer Granularity Energy Cost Discussion

Document for:	Discussion

# 1	Introduction

In RAN3#119bis, it was agreed that the Energy Cost is a node level parameter. In addition, in Rel-18 we took an agreement that further Energy Cost granularities are out of scope of the release. In subsequent meetings we extensively discussed the use case of AI/ML Energy Saving with respect to predicting an Energy Cost related to an offloading action either by the source or by the target node. Despite the extensive discussions we couldn’t reach any agreements on this use case.

In Rel-19 time frame (RAN3 #124), we have made a first agreement touching the Energy Cost topic related to transferring of the measured Energy Cost from gNB-DU, which can do the Energy Cost measurement, to the gNB-CU. Specifically, we agreed the following:

Measured Energy Cost can be transferred from DU to CU.

Since no progress was achieved with respect to predicting an Energy Cost measurement we considered a different angle of AI/ML Energy Efficiency enhancements, pertaining to refining of the granularity of the Energy Cost measurement. Specifically, we captured the following open point:

Only discuss on finer granularity of energy cost in Aug meeting and make the conclusion.

In this contribution we provide some possible options towards the definition of a finer granularity energy cost. We provide the corresponding TP to TR 38.743, based on these options, in [1].

# 2	Discussion

In the Rel-18 solution, the Energy Cost is a node level measurement which, by nature, provides very coarse Energy Cost related information. As one example, the reported value by the target may not only reflect the Energy Cost of actions related to the source node, but also actions corresponding to concurrent offloadings and handovers by other neighbouring nodes, as well as UEs establishing and releasing their RRC connections. Towards this end, the source node receiving the reported Energy Cost information by a neighbouring node will not be able to evaluate which portion of the reported node level Energy Cost corresponds to its own actions which may subsequently result in suboptimal Energy Saving decisions by the source node (e.g., if the information is used to train an AI/ML Model). In addition, even for the same load distribution, consecutive Energy Cost reporting measurements at node level may have large differences due to the various operations of the network components. This is an important shortcoming of the Energy Cost measurement metric since the exact cost of an offloading action with respect to energy cannot be evaluated.

Observation 1: A significant shortcoming of node level Energy Cost measurement is that it is too coarse to reflect the impact of an offloading action to a target node with respect to an Energy Cost.

Clearly, to evaluate the impact of a cell-level Energy Saving action e.g., cell switch-off, the cost used to measure this impact with respect to energy should also be at least at cell level. This would allow for a one-to-one mapping between an action and a cost associated to the action to the impacted node (e.g., node that receives the traffic). Therefore, for cell level actions it is ideal that the Energy Cost per target cell is exchanged between RAN nodes. However, there are practical challenges in measuring the Energy Cost per cell. Those include the fact that a) the physical hardware resources (e.g., Radio Unit) can be shared across multiple cells (resource pooling), b) Energy Consumption measurements are only feasible at physical hardware level and c) it is overly complex to obtain granular measurements, especially in virtualized platforms for split architecture nodes. Therefore, cell-level Energy Cost is not measurable in general, including, but not only in case, where cells share physical resources.

Observation 2: Cell-level Energy Cost is not measurable.

However, it is possible to “estimate” the cell-level Energy Cost, using the physical unit (e.g Distributed Unit) level Energy Consumption and the Cell Level PRB Utilization. For example, if the gNB-DU level Energy Consumption is X units and if there are 3 cells mapped to the Distributed Unit, per-cell energy consumption can be calculated as a percentage of X, e.g., based on the percentage of per-cell PRB Utilization compared to the total PRB utilization. Thus, a node could estimate the energy consumption at cell level, even though it is not possible to obtain an accurate cell-level Energy Cost measurement.

Observation 3: Cell-level energy cost can be estimated, but not accurately measured.

Therefore one option to enable finer level granularity compared to node-level Energy Cost is to enable a node to estimate its cell-level Energy Cost.

Proposal 1: Enable a node to “estimate” its cell-level Energy Cost.

A node can subsequently exchange this Energy Cost estimate with its neighbours, if such estimate can be calculated.

Proposal 2: A gNB reports cell-level Energy Cost estimate to its neighbours upon request, if such estimate can be calculated.

As another alternative of finer-level granularity in split architecture also gNB-DU level Energy Cost granularity can be considered. In case of split architecture we agreed in the previous meeting that gNB-DU level Energy Cost can be transferred from gNB-DU to gNB-CU. Therefore, a gNB-CU will receive Energy Cost information related to the different gNB-DUs it manages. Thus, another alternative of finer granularity Energy Cost in split architecture is gNB-DU level. With gNB-DU level Energy Cost reporting, a finer granularity is achieved by measuring and reporting Energy Cost from specific gNB-DU(s) hosting target cell(s) of the handed over UE(s). This improves the accuracy compared to node level Energy Cost since it only accounts for the Energy Cost of a specific gNB-DU(s) where the offloading takes place (instead of all gNB-DU(s) of the target node). Even though gNB-DU level Energy Cost reporting doesn’t solve some of the issues measured above (e.g., it may still measure Energy Cost pertaining to actions of other neighbouring nodes, as well as UEs establishing and releasing their RRC connections), the inaccuracy is smaller compared to a node level measurement. A drawback of this option is still that it exposes the internal gNB architecture on the Xn interface, but we believe we can still capture it in the TR and leave the final decision and details to normative phase.

Proposal 3: A node reports gNB-DU-level Energy Cost measurements, related to one or more gNB-DUs, upon request.

In all options above, the Energy Cost estimate/measurement still does not map to a single action, which is namely the cost that the handover of a UE entailed at the target node. Similarly to the cell-level Energy Cost, in our view the Energy Cost of an executed handover event or of a number of executed handover events can also be estimated by a node. Reporting of this information (though an estimate) to a source node could provide valuable information in order to determine how offloading of different UEs by the source effectively impacts the Energy Cost of the target node.

Proposal 4: A gNB reports Energy Cost estimate per executed handover event or per number of executed handover events to its neighbours upon request, if such estimate can be calculated.

Proposal 5: Agree the TP to TR 38.743 in [1].

# 3	Conclusion

In this contribution we make the following observations and proposals:

Observation 1: A significant shortcoming of node level Energy Cost measurement is that it is too coarse to reflect the impact of an offloading action to a target node with respect to an Energy Cost.

Observation 2: Cell-level Energy Cost is not measurable.

Observation 3: Cell-level energy cost can be estimated, but not accurately measured.

Proposal 1: Enable a node to “estimate” its cell-level Energy Cost.

Proposal 2: A gNB reports cell-level Energy Cost estimate to its neighbours upon request, if such estimate can be calculated.

Proposal 3: A node reports gNB-DU-level Energy Cost measurements, related to one or more gNB-DUs, upon request.

Proposal 4: A gNB reports Energy Cost estimate per executed handover event or per number of executed handover events to its neighbours upon request, if such estimate can be calculated.

Proposal 5: Agree the TP to TR 38.743 in [1].

# References

[1]		R3-244448, (TP to TR 38.743) Finer Granularity Energy Cost, Nokia, Telecom Italia, T-Mobile USA, BT